3GPP TSG-RAN WG4 Meeting #114	R4-2503078

Athens, Greece Feb 17 – Feb 21, 2025

Source:	Tejas Networks

Title:	Discussion on NR-NTN n252 UE RF requirements

Agenda item:	6.18.2

Document for:	Discussion

Introduction

The WI [1] on Introduction of NR-NTN S-band (MSS band 2000-2020 MHz UL and 2180-2200 MHz DL) has been approved in RAN#104. The objective includes the UE-to-UE coexistence analysis for adjacent bands and TN-NTN isolation distance.

In this paper, we provide our views on issues related to factors that potentially impact RAN4 requirements.

Discussion

During the RAN4 #113 meeting, RAN4 agreed to a WF [2], following are the agreements captured in the WF.

NTN-TN Isolation distance for FR1 NTN

Agreement: TN-NTN Isolation Distance - Offline Agreement: - The TN-NTN coexistence assumption of 1.5 km separation between NTN UE and edge of the TN coverage from TR 38.863 has been used to guarantee the minimal performance on both TN and NTN networks. Nevertheless, this might not represent a typical deployment, and an NTN UE might still be able to operate within this isolation distance - Revision of the general TN-NTN coexistence assumptions is not in scope of this WI

During RAN4 #112, it was agreed to study the coexistence of S-band 252 UL with the DL of n25 and n2, given that the band separation with n25 DL is 5 MHz and with n2 DL is 10 MHz.

At a separation distance of 1500 meters, the free-space path loss would be approximately 102 dB. With significantly higher path loss (between NTN S-band UEs and TN n2/n25 UEs) values, relaxation of UE coexistence requirements for the new S-band UE with TN bands n2/n25 could be considered. Alternatively, reducing the separation distance between TN and NTN could also be explored.

We simulated a scenario to assess the potential adjacent channel interference on the n2 and n25 bands caused by NTN UL transmission. In this setup, NTN UL acted as the aggressor, and TN DL was the victim. The simulation was run for different isolation distances between NTN and TN, ranging from 1500m to 0m, and for various ACIR values from 14 to 32 dB. The results are captured in Figure 2, which presents a graph of the 5th percentile throughput loss versus ACIR. Additionally, the table displays the 5%-tile throughput loss for various isolation distances, with the ACIR fixed at 33 dB.

Figure 1: 5%-tile Throughput loss vs Isolation distance

| Isolation Distance | Throughput Loss Type | Value |
| --- | --- | --- |
| 1500m | Average | 0.48 |
|  | 5%-tile | 1.73 |
| 1000m | Average | 0.63 |
|  | 5%-tile | 2.18 |
| 500m | Average | 0.7 |
|  | 5%-tile | 3.25 |
| 100m | Average | 0.75 |
|  | 5%-tile | 3.84 |
| 100m | Average | 0.78 |
|  | 5%-tile | 4.12 |

Table 1: Throughput loss values for ACIR = 33 dB

The simulation results demonstrate that the isolation distance has a minimal impact on throughput loss, as the increase in throughput loss is relatively small across varying distances. Even as the isolation distance decreases from 1500 meters to 100 meters, the throughput loss shows only slight variations.

Observation 1: The impact of NTN-TN isolation distance on throughput loss is minimal.

- Proposal 1: RAN4 should evaluate the possibility of reducing the NTN-TN isolation distance.

UE to UE Coexistence

#### UE to UE Coexistence between n252 UL and n2/b2/n25/b25 DL - Tentative Agreement: - Agree to two different additional spurious emission requirements, each associated to an NS value and corresponding A-MPR, for protection of band b25/n25/b2/n2 - FFS: -30 dBm/MHz for relaxed value. - FFS: This emission level applies and the associated A-MPR is allowed whether or not the NS value is signalled. Also, the protection level should a note equivalent to Note 15 in 38.101-1 which says “These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth.” - FFS: [-40] dBm/MHz for the stricter value based on current b23 specification - The requirement is applicable in every country where b25/n25/b2/n2 and n252 are applicable - Specify asymmetric channel bandwidths with 5, 10 and [15] MHz UL CHBW for band n252.

A partial geographic overlap of NTN and TN was agreed during the RAN4 #112bis meeting, and the results from the previous section support the feasibility of an NTN-TN overlapping scenario. During the same meeting, one of the contributions [4] demonstrated that the probability of active NTN UL transmission causing interference to TN DL is very low, as the satellite beam covers a wide area.

- Observation 2: The overall network throughput experiences minimal degradation due to the low likelihood of NTN and TN UEs being within 10 meters of each other and scheduled simultaneously, which is a positive indicator for NTN and TN network coexistence.

Although the likelihood of simultaneous NTN and TN UEs being scheduled to come close to each other is low, situations where two UEs with NTN and TN capabilities are within a 10-meter separation (such as inside a vehicle or a group gathering) are not rare.

To explore the possibility of such scenarios, we evaluated the coexistence of UEs in close proximity through simulations in a cell edge scenario. The UE emission level considered for the simulation is -50 dBm/MHz. The simulation setup is shown in Figure 2 below. Additionally, the Table 2 displays the average throughput loss for different UE separations ranging from 1 m to 60 m.

Single Cell Site
Rural Macro

isd=7500m

UE Drop Position Range – (0.5*isd-100) to (0.5*isd) m

TN UE to NTN UE Separation distance-{1,2,5,10,20}

TN gNB

TN UE

NTN UE

Figure 2: Simulation set up of TN-NTN overlap scenario

| Separation Distance(m) | Avg Throughput Loss (%) | Avg CL (dB) |
| --- | --- | --- |
| 1 | 98.35 | -38.85 |
| 2 | 95.69 | -44.58 |
| 5 | 84.76 | -52.54 |
| 10 | 73.25 | -58.56 |
| 20 | 62.56 | -64.58 |
| 30 | 45.84 | -68.75 |
| 40 | 21.06 | -69.9 |
| 50 | 7.26 | -72.54 |
| 60 | 2.25 | -74.9 |

- Table 2: Throughput Loss vs. UE Separation for NTN-TN Overlap Scenario

The simulation results indicate that interference can cause significant performance degradation for cell-edge users in TN when the separation distance between NTN and TN UEs is up to 50 meters, with an emission level of -50 dBm/MHz and simultaneous scheduling. Specifically, as the separation distance decreases, the average throughput loss increases, with substantial degradation observed at distances of 10 meters or less. At a 10-meter separation, the throughput loss for cell-edge users averages 73.25%.

These findings highlight that when NTN and TN UEs operate simultaneously within a 50-meter radius, interference can have a considerable impact, leading to a significant decline in performance. Although the likelihood of simultaneous scheduling is relatively low, we recommend reassessing the current operational requirements for such scenarios and considering potential revisions to mitigate this interference. This would help ensure the effective coexistence of NTN and TN systems in close proximity while maintaining acceptable performance levels.

- Observation 3: There is significant performance degradation when simultaneously scheduled NTN and TN UEs within 50 meters of separation in a cell edge scenario.

- Proposal 2: RAN4 should assess the UE-to-UE coexistence scenario of simultaneously scheduled NTN and TN UEs in close proximity.

If UE to UE coexistence is feasible with a small separation between NTN UE and TN UE, it implies that NTN UEs can operate within the TN coverage. In such a scenario, RAN4 may need to consider the coexistence or potential overlap of TN and NTN networks. Additionally, as agreed in the RAN4#113 meeting, a deployment scenario where NTN UEs operate within the isolation distance can also be considered. A key aspect for discussion in RAN4 would be defining the priority criteria between TN and NTN networks to ensure efficient resource allocation and interference management.

The decision on priority criteria should consider factors such as network reliability, service requirements, and user experience. From the UE perspective, power consumption becomes a critical factor, especially in power-limited scenarios or when the UE aims to conserve energy. The UE may need to evaluate which network (TN or NTN) is more reliable or energy-efficient for its specific use case. On the network side, considerations such as resource allocation, network planning, and interference mitigation strategies must be carefully addressed to ensure seamless coexistence. Furthermore, the potential need for coordination between TN and NTN networks to resolve interference or resource contention issues should also be explored. Additionally, aspects such as signaling overhead, latency, and quality of service (QoS) requirements should be evaluated at both the UE and network levels to ensure a balanced and efficient coexistence framework.

- Observation 4: The potential for a UE to connect to the NTN network while in TN coverage needs to be considered to ensure seamless coexistence.

- Proposal 3: RAN4 should discuss the prioritization criteria for a UE operating in both TN and NTN coverage.

Conclusion

This section summarizes our discussion by listing all the observations and proposals discussed in this contribution.

Observation 1: The impact of NTN-TN isolation distance on throughput loss is minimal.

- Observation 2: The overall network throughput experiences minimal degradation due to the low likelihood of NTN and TN UEs being within 10 meters of each other and scheduled simultaneously, which is a positive indicator for NTN and TN network coexistence.

- Observation 3: There is significant performance degradation when simultaneously scheduled NTN and TN UEs within 50 meters of separation.

- Observation 4: The potential for a UE to connect to the NTN network while in TN coverage needs to be considered to ensure seamless coexistence.

- Proposal 1: RAN4 should evaluate the possibility of reducing the NTN-TN isolation distance.

- Proposal 2: RAN4 should assess the UE-to-UE coexistence scenario of simultaneously scheduled NTN and TN UEs in close proximity.

- Proposal 3: RAN4 should discuss the prioritization criteria for a UE operating in both TN and NTN coverage.

- References

- R4-2420465, WF on requirements for NR IoT NTN bands, Inmarsat, Orlando, USA, November 18th – 22nd, 2024.

- RP-241885, Revised WID on Introduction of another NR NTN S-band (MSS band 2000-2020 MHz UL and 2180-2200 MHz DL), Qualcomm Incorporated, Melbourne, Australia, September 9-12, 2024

- RP-242615, Status Report to TSG, Madrid, Spain, December 9-12, 2024.

- R4-2416459, Co-existence considerations for the new S-band, Qualcomm Inc, Hefei, China, 14 – 18 October 2024