3GPP TSG-RAN WG4 Meeting #114	R4-2502850

# Athens, Greece, February 17th – 21st, 2025

Title: 	WF on NR and IoT NTN bands

Agenda Item:	6.18, 6.19, 6.20

Source: 	Moderator (Aalyria)

Document for:	Approval

Topic #1: NR-NTN S-band

Sub-topic 1-1: General Requirements and Coexistence Aspects

Issue 1-1-1: UE to UE Coexistence with band n2/n25

Agreement:

- Agree to two different additional spurious emission requirements, each associated to an NS value, and corresponding A-MPR, for protection of band b25/n25/b2/n2

  - -30 dBm/MHz (NS_XX)

  - -40 dBm/MHz (NS_YY)

- For the above requirements, the following statement will be captured in a normative way:

  - “The requirement applies based on coordination between operators and subject to regional/national regulation”

    - Apply the requirements to UE based on NS signal

- Agree to specify asymmetric channel bandwidths with 5, 10 and 15 MHz UL CHBW for band n252, with BCS0 including all permutation based on UL channel bandwidth

Table 5.3.6-1: FDD asymmetric UL and DL channel bandwidth combinations

| NR Band | Channel bandwidths for UL (MHz) | Channel bandwidths for DL (MHz) | Asymmetric channel bandwidth combination set |
| --- | --- | --- | --- |
| n252 | 5 | 10,15, 20 | 0 |
|  | 10 | 15, 20 |  |
|  | 15 | 20 |  |

T-Mobile: to “requirement applies based on coordination…”  coordination between operators.

Issue 1-1-2: UE NS For Additional Spurious emissions

- Way Forward:

  - Adopt the following as starting point for the definition of the two NS values for protection of b25/n25/b2/n2

Boost Mobile: Implementing NOTE is OK. The easier way is to implement it in MPR table.

Apple: Capture it in TR.

Qualcomm: Note is compromise way. We can make it clear that UE applies requirement based on NS signaling. It is now that we do not define the 20MHz for UL?

AT&T: It is clearer to add the note in the NS mapping table.

Boost Mobile: Aysmmetric bandwidth table?

Apple: 20MHz is [ ]. We need which bandwidth we should simulate.

T-Mobile: Symmetric includes 20MHz in the table.

Qualcomm: Agree with T-Mobile. WID has 5~20MHz. 20MHz should be part of table.

6.5.3.3.X	Requirement for network signalling value "NS_XX"

When "NS_XX" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.3.2-X. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth.

Table 6.5.3.3.2-X: Additional requirements for "NS_XX"

| Frequency range (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
|  | 5 MHz, 10 MHz, 15 MHz, 20MHz |  |  |
| 1930 ≤ f <1995 | -30 | 1MHz | XX |
| NOTE XX: The requirement applies based on coordination, subject to regional/national regulation. . |  |  |  |

Agreement:

- the note will be moved to NS mapping table, including the contents below

  - “The requirement applies based on coordination between operators and subject to regional/national regulation”

  - Apply the requirements to UE based on NS signal

6.5.3.3.Y	Requirement for network signalling value "NS_YY"

When "NS_YY" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.3.2-Y. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth.

Table 6.5.3.3.2-Y: Additional requirements for "NS_YY"

| Frequency range (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
|  | 5 MHz, 10 MHz, 15 MHz, 20MHz |  |  |
| 1930 ≤ f < 1995 | -40 | 1MHz | YY |
| NOTE YY: The requirement applies based on coordination subject to regional/national regulation |  |  |  |

Agreement:

- the note will be moved to NS mapping table, including the contents below

  - “The requirement applies based on coordination between operators and subject to regional/national regulation”

  - Apply the requirements to UE based on NS signal

Issue 1-1-3: UE A-MPR for corresponding NS values

- Way Forward:

  - Specify different A-MPR values for each NS depending on the UL CHBW, RB start and LcRB.

    - FFS on exact values

  - Adopt the following table skeleton

Table 6.2.3.1-1: Additional maximum power reduction (A-MPR)

| Network signalling label | Requirements (clause) | NR satellite Band | Channel bandwidth (MHz) | Resources blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| NS_01 |  | Table 5.2.2-1 (NOTE 3) | 5, 10, 15, 20 | Table 5.3.2-1 | N/A |
| NS_24 | 6.5.3.3.13 in 3GPP TS 38.101-1 [5] | n256 | 5, 10, 15, 20 | Table 6.2.3.15-1 in 3GPP TS 38.101-1 [5] | Clause 6.2.3.15 in 3GPP TS 38.101-1 [5]2 |
| NS_02N | 6.5.3.3.2 | n255 | 5, 10, 15, 20 |  | N/A |
| NS_100 | 6.5.2.4.2 in 3GPP TS 38.101-1 [5] | n2561 |  |  | Table 6.2.3.1-2 in 3GPP TS 38.101-1 [5] |
| NS_03N | 6.5.3.3.3 | n254 | 5, 10, 15 |  | Clause 6.2.3.2 |
| NS_04N | 6.5.2.3.1 6.5.3.3.4 | n254 | 5 |  | Clause 6.2.3.3 |
| NS_05N | 6.5.2.3.2 6.5.3.3.4 | n254 | 5, 10, 15 |  | Clause 6.2.3.4 |
| NS_XX | 6.5.3.3.X | n252 | 5, 10, 15, 20 |  | Clause 6.2.3.X |
| NS_YY | 6.5.3.3.Y | n252 | 5, 10, 15, 20 |  | Clause 6.2.3.Y |
| NOTE 1: This NS can be signalled for NR satellite bands that have UTRA services deployed. NOTE 2: A-MPR for the upper 5 MHz of the band is not specified, and therefore shall be used as a guard band. NOTE 3: The NS_01 label with the field additionalPmax [8] absent is default for all NR satellite bands. NOTE 4: The NS is signalled based on coordination between operators and subject to regional/national regulation |  |  |  |  |  |

6.2.3.2	A-MPR for NS_XX

Table 6.2.3.X-X: A-MPR regions for NS_XX

| UL Channel BW | Carrier Center Frequency | RB_start*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 5MHz |  |  |  |  |
| 10MHz |  |  |  |  |
| 15MHz |  |  |  |  |
| 20MHz |  |  |  |  |

Table 6.2.3.X-X: A-MPR for NS_XX

|  | Modulation | A1 | A2 | A3 | A4 | A5 | A6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DFT-s-OFDM | Pi/2 BPSK |  |  |  |  |  |  |
|  | QPSK |  |  |  |  |  |  |
|  | 16QAM |  |  |  |  |  |  |
|  | 64QAM |  |  |  |  |  |  |
|  | 256QAM |  |  |  |  |  |  |
| CP-OFDM | QPSK |  |  |  |  |  |  |
|  | 16QAM |  |  |  |  |  |  |
|  | 64QAM |  |  |  |  |  |  |
|  | 256QAM |  |  |  |  |  |  |

Issue 1-1-4: Protection of aeronautical radionavigation-satellite service (i.e. GNSS)

- Way Forward:

  - Specify the following additional spurious requirements for protection of the aeronautical radionavigation satellite service:

Additional spurious emission requirements for band n252

| Frequency range (MHz) | Channel bandwidth / Spectrum emission limit1 (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
|  | 5 MHz, 10 MHz, 15 MHz, 20 MHz |  |  |
| 1559≤ f < 1610 | -50 | 700 Hz | Averaged over any 2 millisecond active transmission interval |
| 1559 ≤ f < 1610 | -40 | 1MHz | Averaged over any 2 millisecond active transmission interval |
| NOTE: The EIRP requirement in regulation is converted to conducted requirement using a 0 dBi antenna. |  |  |  |

Sub-topic 1-2: UE RF

Sub-topic description: General UE RF Requirements

Issue 1-2-1: SS Raster

One proposal to keep only Case A, one proposal with Case A and Case B.

- Way Forward:

  - For the NTN FDD band with UE transmitting at 2000 - 2020 MHz and SAN transmitting at 2180 - 2200 MHz, specify the synchronization raster entries with SSB pattern:

    - Option 1: Case A only

    - Option 2: Case B

    - Option 3: Case C

    - Option 4: Case B and Case C.

Qualcomm: the way forward only lists all the options and no agreement. Option 1 is sufficient.

Moderator: this is pending from last meeting. We should decide what we are going to do for Case B,C.

Echostar: We agree to Case A and respect to Case B and C come back next meeting.

CATT: For current stage, there is no Sat operator confirmation of other cases. Case A is sufficient.

ZTE: There is no need to introduce both Case B and C.

MODERATOR NOTE:

- Case A has already been agreed several meetings ago.

- Previous agreement in RAN4#113 was as follows:

  - For the NTN FDD band with UE transmitting at 2000 - 2020 MHz and SAN transmitting at 2180 - 2200 MHz, the synchronization raster entries with SSB pattern Case B and Case C could be introduced.

Issue 1-2-2: UE Asymmetric Channel Bandwidths

- Way Forward:

  - Specify Asymmetric channel bandwidths as follows (see Draft CR)

Table 5.3.6-1: FDD asymmetric UL and DL channel bandwidth combinations

| NR Band | Channel bandwidths for UL (MHz) | Channel bandwidths for DL (MHz) | Asymmetric channel bandwidth combination set |
| --- | --- | --- | --- |
| n252 | 5 | 10,15, 20 | 0 |
|  | 10 | 15, 20 |  |
|  | 15 | 20 |  |
| NOTE 1: The assignment of the paired UL and DL channels are subject to a TX-RX separation as specified in clause 5.4.4. NOTE 2: As indicated in TS38.306 [11], it is mandatory for UEs to support asymmetric channel BCS0 if there is an asymmetric BCS0 defined for the band. |  |  |  |

Issue 1-2-3: UE REFSENS

Summary: 2 Proposals with n252 co-banding with n256/n65, 1 proposal no co-banding, but values may need revisiting

- Way Forward:

  - Specify n252 UE REFSENS based on

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |
|  | 15 | [-99.5] | [-96.3] | [-94.5] | [-93.3] |
| n252 | 30 |  | [-96.6] | [-94.6] | [-93.5] |
|  | 60 |  | [-97.0] | [-94.9] | [-93.7] |

Skyworks: we may do not have different requirements. Look at the numbers in the end.

Qualcomm: Intention is not to define two sets of requirements.

Issue 1-2-4: UE Out-of-band Blocking

- Way Forward:

  - Further discuss the Out-of-Band Blocking requirements for band n252 based on the options following

    - Option 1, with no co-banding assumption with n256 and n65

Table 2.2-1: Out of-band blocking for NR satellite bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

| Operating Band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -44 | -30 | -15 |
| n252 | Finterferer (CW) | MHz | -70 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -95 < f – FDL_low ≤ -70 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 95 or FDL_high + 85 ≤ f ≤ 12750 |
| n2542 | Finterferer (CW) | MHz | -60 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -85 < f – FDL_low ≤ -60 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 85 or FDL_high + 85 ≤ f ≤ 12750 |
| n255 | Finterferer (CW) | MHz | -60 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -85 < f – FDL_low ≤ -60 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 85 or FDL_high + 85 ≤ f ≤ 12750 |
| n2561 | Finterferer (CW) | MHz | -100 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -145 < f – FDL_low ≤ -100 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 145 or FDL_high + 85 ≤ f ≤ 12750 |
| NOTE 1: Band n256 lower frequency ranges are modified to enable specific implementations NOTE 2: Band n254 power level of the interferer (Pinterferer) for Range 3 shall be modified to -20 dBm for Finterferer > 2585 MHz and FInterferer < 2775 MHz. NOTE 3: void NOTE 4: void |  |  |  |  |  |

  - Option 2 with assumption of n252 co-banding with n256 and n65

Table 1: Out of-band blocking for n252

| Operating Band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -44 | -30 | -15 |
| n2521 | Finterferer (CW) | MHz | -110 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -155 < f – FDL_low ≤ -110 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 155 or FDL_high + 85 ≤ f ≤ 12750 |
| n2561 | Finterferer (CW) | MHz | -100 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -145 < f – FDL_low ≤ -100 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 145 or FDL_high + 85 ≤ f ≤ 12750 |
| NOTE 1: Band n256 and n252 lower frequency ranges are modified to enable specific implementations |  |  |  |  |  |

  - Option3: reuse the OOB requirements of band B23

Qualcomm: option 2.

Mediatek: three approaches.

Echostar: it should not be based on n65 which is not implemented.

Mediatek: We do not implement n65.

Sub-topic 1-3: SAN RF

Sub-topic description: SAN RF requirements for band n252

Issue 1-3-1: SAN SS Raster

- Way Forward:

  - For the NTN FDD band with UE transmitting at 2000 - 2020 MHz and SAN transmitting at 2180 - 2200 MHz, specify the synchronization raster entries with SSB pattern:

    - Option 1: Case A only

    - Option 2: Case B

    - Option 3: Case C

    - Option 4: Case B and Case C.

Sub-topic 1-4: RRM Requirements

Way Forward:

- Endorse the following draft CR

| R4-2501179 | ZTECorporation,Sanechips | Draft CR to TS 38.133: Introduction of a new NTN S-band |
| --- | --- | --- |

Topic #2 IoT-NTN S-band

Sub-topic 2-1: UE RF

Sub-topic description: UE RF requirements

Issue 2-1-1: UE TX-RX Separation

- Way Forward:

    - Specify B252 variable TX-RX frequency separation as follows for Cat M1

Table 5.4A.3-1: Default UE TX-RX frequency separation

| E-UTRA Operating Band | TX – RX carrier centre frequency separation |
| --- | --- |
| 252 | 80 MHz1 161.4 to 198.6 MHz2 |
| NOTE 1: Default TX-RX separation. NOTE 2: The verification of flexible TX-RX frequency separation within this range is limited to reference sensitivity. Further details are specified in clause 7.3A. |  |

Table 7.3A-4: TX – RX carrier centre frequency separation for REFSENS verification

| E-UTRA Operating Band | Channel bandwidth | TX – RX carrier centre frequency separation for REFSENS verification |
| --- | --- | --- |
| 252 | 1.4 MHz | 161.4 MHz, 198.6 MHz |

    - Specify B252 variable TX-RX frequency separation as follows for Cat NB1 and NB2

Table 5.4B.3-1: Default UE TX-RX frequency separation

| E-UTRA Operating Band | TX – RX carrier centre frequency separation |
| --- | --- |
| 252 | 80 MHz1 160.2 to -199.8 MHz2 |
| NOTE 1: Default Tx-Rx separation. NOTE 2: The verification of flexible TX-RX frequency separation within this range is limited to reference sensitivity. Further details are specified in clause 7.3B. |  |

Table 7.3B-2: TX – RX carrier centre frequency separation for REFSENS verification

| E-UTRA Operating Band | Channel bandwidth | TX – RX carrier centre frequency separation for REFSENS verification |
| --- | --- | --- |
| 252 | 0.2 MHz | 160.2 MHz, 199.8 MHz |

Issue 2-1-2: UE Out of band Blocking for Cat M1

Both Options 1/2 for B252 co-banded filter with B256, of which Option 2 also proposes co-banding with B65 and Option 1 values don’t align with B256

- Way Forward

  - Aligned with the assumptions agreed for NR.

  - Specify out of band blocking for Cat M1 as follows:

    - Option 1: Assuming co-banding with B256

Table X: Out of-band blocking parameters for category M1 UE

| Operating Band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -44 | -30 | -15 |
| 2523 | Finterferer (CW) | MHz | -70 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -95 < f – FDL_low ≤ -70 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 95 or FDL_high + 85 ≤ f ≤ 12750 |
| NOTE 3: Band 252 lower frequency ranges are modified to enable specific implementations. |  |  |  |  |  |

  - Option 2: Assuming co-banding with B256 and B65

Table 1: Out of-band blocking for 252 and 256

| Operating Band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -44 | -30 | -15 |
| 2521 | Finterferer (CW) | MHz | -110 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -155 < f – FDL_low ≤ -110 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 155 or FDL_high + 85 ≤ f ≤ 12750 |
| 2561 | Finterferer (CW) | MHz | -100 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -145 < f – FDL_low ≤ -100 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 145 or FDL_high + 85 ≤ f ≤ 12750 |
| NOTE 1: Band 256 and 252 lower frequency ranges are modified to enable specific implementations |  |  |  |  |  |

Issue 2-1-3: UE Out of band Blocking for Cat NB1/NB2

Both Options 1/2 for B252 co-banded filter with B256, of which Option 2 also proposes co-banding with B65 and Option 1 values don’t align with B256

- Way Forward

  - Aligned with the assumptions agreed for NR.

  - Specify out of band blocking for Cat NB1 and NB2 as follows:

    - Option 1: Assuming co-banding with B256

- Table Y: Out-of-band blocking parameters for category NB1 and NB2 UE

| Operating Band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pw | dBm | REFSENS + 6 dB |  |  |
|  | Pinterferer | dBm | -44 | -30 | -153 |
| 2526 | Finterferer (CW) | MHz | -70 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -95 < f – FDL_low ≤ -70 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 95 or FDL_high + 85 ≤ f ≤ 12750 |
| NOTE 6: Band 252 lower frequency ranges are modified to enable specific implementations. |  |  |  |  |  |

  - Option 2 : Assuming co-banding with B256 and B65:

Table 1: Out of-band blocking for 252 and 256

| Operating Band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -44 | -30 | -15 |
| 2521 | Finterferer (CW) | MHz | -110 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -155 < f – FDL_low ≤ -110 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 155 or FDL_high + 85 ≤ f ≤ 12750 |
| 2561 | Finterferer (CW) | MHz | -100 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -145 < f – FDL_low ≤ -100 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 145 or FDL_high + 85 ≤ f ≤ 12750 |
| NOTE 1: Band 256 and 252 lower frequency ranges are modified to enable specific implementations |  |  |  |  |  |

Sub-topic 2-2: RRM Requirements

Way Forward:

- Endorse the following draft CR.

| R4-2501181 | ZTECorporation,Sanechips | Draft CR to TS 36.133: Introduction of a new IoT NTN S-band |
| --- | --- | --- |

Sub-topic 2-3: TR 36.764 Update

Issue 2-3-1: TR Section Skeleton

Way Forward:

- Updated TR 36.764 adding the following section

8 FDD band B252 (IoT-NTN FDD S_band) 8.1 Regulation 8.1.1 Regional Applicability 8.2 UE requirements 8.2.1 Channel arrangement 8.2.2 UE transmitter characteristics 8.2.2.1 Maximum output power 8.2.2.2 Emission requirements and NS values 8.2.2.2.1 Spurious emission 8.2.3 UE receiver characteristics 8.2.3.1 Reference sensitivity 8.2.3.2 Blocking requirements 8.2.4 A-MPR Evaluation 8.3 BS requirements

Topic #3: NR-NTN L-bands

Sub-topic 3-1: General Aspects and Regulatory Input

Sub-topic description: Regulatory aspects

Issue 3-1-1: Capturing of ETSI EN Requirements

- Way Forward

  - Capture existing in-band and out-of-band emission requirements for L-bands n251, n250 and n253 as defined by ETSI EN 301 681 in a new table under a new sub-clause under Clause 6.5.2 (Conducted transmitter characteristics, Out-of-Band emissions) as Additional Regional Emission Requirements.

    - Introduce a NOTE specifying that the requirements are converted from radiated requirements (i.e. EIRP) in ETSI EN 301 681 to conducted requirements, assuming a 0dBi antenna with flat frequency response integral to the UE.

      - Detailed reference with specific version of EN 301 681 spec being referenced shall be included.

    - FFS for UE with high gain antenna or UE with non-integral antenna.

    - Come back in next meeting with specific NS values to be specified

Qualcomm: ETSI works on the new harmonized standard. Which out-of-band emission is under discussions. If that happes, the requirements EN 301 681 is not applicable, and we should remove the requirements from the spec.

Moderator: this issue was discussed for a long time. The WF is captured due to some confusion. Agree with Qualcomm comment. Welcome interested operators comment on this. We propose to capture it.

Inmarsat: Agree with Moderator and Qualcomm.

Qualcomm: Action in 3GPP should be based on the other document other than EN 301 681.

Issue 3-1-2: Mechanisms for Protection of Radioastronomy

- Way Forward:

  - For n251, n250, n253 protection of Radio Astronomy, consider the following two mechanisms as a baseline:

    - Network-controlled RB scheduling is considered as a baseline based on operator’s control

    - FFS whether a new NS value should be specified to signal the additional emission limit at the edge of the band, to signal cells in proximity or overlap to RA stations

      - Clarify the specific emission limits required and the frequency range applicability, and whether a specific value of geographical distance for the limit is considered, when defining proximity to a RA station

Qualcomm: the lowest of bullet should be sub-bullet. Distance is frequency-distance or geographical.

Sub-topic 3-2: UE RF

Sub-topic description: System parameters and UE RF aspects

Open issues and candidate options before meeting:

Issue 3-2-1: UE Out-of-band blocking

  - Way Forward:

    - Consider the following out-of-band blocking requirements for band n253, n251 and n250 as a starting point for further verification (come back in next meeting):

| Operating Band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -44 | -30 | -15 |
| n251, n250 | Finterferer (CW) | MHz | [-60 < f – FDL_low < -15 or 15 < f – FDL_high < 60] | [-85 < f – FDL_low ≤ -60 or 60 ≤ f – FDL_high < 85] | [1 ≤ f ≤ FDL_low – 85 or FDL_high + 85 ≤ f ≤ 12750] |
| n253 | Finterferer (CW) | MHz | [-60 < f – FDL_low < -15 or 15 < f – FDL_high < 94] | [-85 < f – FDL_low ≤ -60 or 94 ≤ f – FDL_high < 119] | [1 ≤ f ≤ FDL_low – 85 or FDL_high + 119 ≤ f ≤ 12750] |

Sub-topic 3-4: RRM Requirements

Sub-topic description: RRM aspects

- Way Forward:

  - Endorse the following draft CR.

| R4-2501180 | ZTECorporation,Sanechips | Draft CR to TS 38.133: Introduction of a new NTN L-band |
| --- | --- | --- |