3GPP TSG-RAN WG4 Meeting # 114											                 R4-2500515

# Athens, GR, 17th – 21th Feb., 2025

Agenda item:			4.5.1

Source:	Moderator (Xiaomi)

Title:	Topic summary for [114][201] Maintenance_up_to_R16

Document for:	Information

Introduction

This document provides summary for RRM related Tdocs submitted to the following AI

4.5.1		Rel-15/16 RRM requirements 	[WI code]

Please kindly take following notes for Tdoc handling in this topic thread.

- Open issues are based on Discussion papers.

- Based on Chair’s guidance, all CRs in this email thread will be first handled in NWM flagging procedure which will be triggered separately.

- Tdocs that are withdrawn or revised in the Tdocs list will not be handled in the summary document or the NWM flagging procedure.

- Cat-A CRs will not be handled in the summary document or the NWM flagging procedure.

Topic #1: R15 NR

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2500444 | Apple | Observation 1: RRM measurement delay requirements specified in TS36.133 apply for deactivated SCell and neighbor cells on deactivated SCC. Observation 2: RRM measurement delay requirements specified in TS38.133 only apply for deactivated SCell, but not for neighbor cells on deactivated SCC. Observation 3: requirements of interruption due to RRM measurement on deactivated SCC has been specified in RRM spec since LTE, including measurement on deactivated SCell and neighbor cells on deactivated SCC. Proposal 1: requirements of RRM measurement on deactivated SCell also apply for neighbor cells on deactivated SCC. |

Open issues summary

Sub-topic 1-1: RRM measurement on deactivated SCell

- Proposals

  - Option 1 (Apple):

    - Requirements of RRM measurement on deactivated SCell also apply for neighbor cells on deactivated SCC.

- Recommended WF

  - Discuss the option.

Topic #2: R16 NR_RRM_enh-Core

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2501298 | Ericsson | Proposal 1: UE vendors firstly to clarify whether the gap pattern mismatch will happen in real field. Proposal 2: RAN4 to send LS to RAN2 to clarify the UE behaviour on gap pattern reporting in both Rel-16 supportedGapPatterns-NRonly-r16 and Rel-15 supportedGapPatterns if UE has already implemented such mismatch reporting. Proposal 3: RAN4 to send LS to RAN2 to clarify that UE is not allowed to report ‘0’ for one pattern in supportedGapPatterns-NRonly-r16 if UE reports ‘1’ to support the same pattern in Rel-15 supportedGapPatterns if UE currently didn’t implement such mismatch reporting. |
| R4-2500252 | Apple | - While the UE is applying gradual timing adjustment, the TTD for the pair of TAGs or between PCell-PSCell could exceed the MTTD requirement - In certain cases, there might be an impact to UL transmission if the MTTD is exceeded. - For NR-DC, EN-DC and NE-DC if asynchronous mode is supported there might not be issue with UL transmission if TTD exceeds MTTD. - For NR-DC, EN-DC and NE-DC if asynchronous mode is not supported there would be an impact to UL transmission if TTD exceeds MTTD. - For NR CA if TTD exceed MTTD, there would be an impact to UL transmission. - In LTE when gradual timing adjustment results in TTD exceeding MTTD requirement, UE stops gradual timing adjustment on that TAG. - We have the following options to define behaviour when UE gradual timing adjustment exceeds MTTD: Option 1: UE stops gradual timing adjustment. Option 2: UE stops UL transmission on the secondary UL carrier - In LTE if the MTTD is exceeded because of a new TA command, but not gradual timing adjustment by UE, the UE is not expected to transmit on the secondary UL carrier. - With the approach of stopping UL transmission when MTTD is exceeded due to gradual timing adjustment, the reason would not be known to the network and could be mistaken as link failure - The timing issue would be resolved with new TA command on the TAG - If the gradual timing adjustment on PCell causes TTD to exceed MTTD, stopping UL transmission on PCell would be detrimental, while stopping UL transmission on secondary cell would not help the NW realize the issue. - The gradual timing adjustment is applied in the until timing error is within ±Te or the MRTD requirement depending on the configuration is not exceeded in the following cases: Inter-band CA Inter-band EN-DC with spectrum overlapping between LTE and NR, and UE doesn’t support asynchronous operation Intra-band EN-DC if UE doesn’t support asynchronous operation Inter-band NE-DC if UE doesn’t support asynchronous operation Inter-band NR-DC if UE only supports synchronous operation If MTTD is exceeded due to application of new TA command, the UE shall stop transmission on SCell or PSCell in the following cases: Inter-band CA Inter-band EN-DC with spectrum overlapping between LTE and NR, and UE doesn’t support asynchronous operation Intra-band EN-DC if UE doesn’t support asynchronous operation Inter-band NE-DC if UE doesn’t support asynchronous operation Inter-band NR-DC if UE only supports synchronous operation |

Open issues summary

Sub-topic 2-1: Clarification on mandatory gap patterns

- Proposals

  - Option 1 (Ericsson):

    - Proposal 1: UE vendors firstly to clarify whether the gap pattern mismatch will happen in real field.

    - Proposal 2: RAN4 to send LS to RAN2 to clarify the UE behaviour on gap pattern reporting in both Rel-16  supportedGapPatterns-NRonly-r16 and Rel-15 supportedGapPatterns if UE has already implemented such mismatch reporting.

    - Proposal 3: RAN4 to send LS to RAN2 to clarify that UE is not allowed to report ‘0’ for one pattern in supportedGapPatterns-NRonly-r16 if UE reports ‘1’ to support the same pattern in Rel-15 supportedGapPatterns if UE currently didn’t implement such mismatch reporting.

- Recommended WF

  - Discuss the option

Sub-topic 2-2: Gradual timing adjustment for CA and DC

- Proposals

  - Option 1 (Apple):

    - The gradual timing adjustment is applied in the until timing error is within ±Te or the MRTD requirement depending on the configuration is not exceeded in the following cases:
       Inter-band CA
       Inter-band EN-DC with spectrum overlapping between LTE and NR, and UE doesn’t support asynchronous operation
       Intra-band EN-DC if UE doesn’t support asynchronous operation
       Inter-band NE-DC if UE doesn’t support asynchronous operation
       Inter-band NR-DC if UE doesn’t support asynchronous operation

    - If MTTD is exceeded due to application of new TA command, the UE shall stop transmission on SCell or PSCell in the following cases:
       Inter-band CA
       Inter-band EN-DC with spectrum overlapping between LTE and NR, and UE doesn’t support       asynchronous operation 
       Intra-band EN-DC if UE doesn’t support asynchronous operation
       Inter-band NE-DC if UE doesn’t support asynchronous operation 
       Inter-band NR-DC if UE only supports synchronous operation

- Recommended WF

  - Discuss the option