3GPP TSG-RAN WG4 Meeting # 114					       			       			R4-2500522

Athens, Greece, 17– 21 February, 2025

Agenda item:			5.1.2

Source:	Moderator (Huawei)

Title:	Topic summary for [114][208] Netw_Energy_NR

Document for:	Information

# Introduction

This topic summary includes RRM core/perf requirements for Rel-18 network energy saving (5.27.1).

Recommended Topics to be treated online (in order of decreasing priority):

Issue 1-1-1: Neighbour cells on carrier of SSB-less SCell

Issue 1-1-2: EPRE conditions updating

# Topic #1: Adaptation of common signal/channel transmission

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2500234 | Apple | Proposal 1: to agree with RAN2 that “For inter-band SSB-less SCell(s), this field is not present” in the conditional presence of servingCellMO. Proposal 2: if neighbor cells on carrier of SSB-less SCell have SSB transmission, the measurement for those neighbor cells shall be treated as inter-frequency measurement without MG as long as the SSBs from those neighbor cells can be contained in the active BWP of SSB-less SCell. Proposal 3: open to clarify the NES case in the current condition for definition of inter-frequency measurement without MG, as following: A measurement is defined as an inter-frequency SSB based measurements without measurement gaps (either legacy measurement gap or NCSG) in active BWP, for UE capable of interFrequencyMeas-NoGap provided that - the UE supports interFrequencyMeas-Nogap-r16 [15], and - the SSB is completely contained in the active BWP of the UE, or - the SSB of neighbor cells is completely contained in the active BWP of SSB-less SCell of the UE - For inter-frequency SSB based measurements without measurement gaps, UE may cause scheduling restriction as specified in clause 9.3.9.3. - Note: Non-CA capable UE is not expected to indicate support of interFrequencyMeas-Nogap-r16 [15]. |
| R4-2500471 | Nokia, Nokia Shanghai Bell | Observation #1: For serving cells, the basic rule is that intra-frequency measurement requirements in section 9.2 applies. Observation #2: The current intra-frequency measurement definition does not account when the serving cell is an SSB-less SCell. Hence, it is not clear if it does fulfill the conditions for intra-frequency measurement as defined in TS 38.133. Observation #3: It would be beneficial to consider serving cell frequency measurements as intra-frequency to allow fluent mobility for SCell. Observation #4: The SSB-less SCell is a serving cell and the carrier of SSB-less SCell is a serving CC. - If the SSB-less SCell configuration include servingCellMO, the neighbor cell measurement in this MO shall be considered as intra-frequency measurement. In the MO indicated by ServingCellMO, network may configure ssbFrequency to indicate a carrier frequency for the SSB-less SCell so that UE can determine measurement as intra-frequency or inter-frequency measurement based on the ssbFrequency. |
| R4-2500472 | Nokia, Nokia Shanghai Bell | 38.133 CR on intra-frequency measurement with SSB-less SCell |
| R4-2500903 | ZTE Corporation, Sanechips | Proposal 1: Specify the threshold of Y as the value not smaller than 30dBm. Proposal 2: A-TRS based requirement is based on two TRS samples, not impacted by the threshold of X, Y. Observation 1: There is conflict between the current SSB-less SCell definition and the demand of SSB based neighbour cell measurement. Proposal 3: From RAN4 perspective, we should pay more attention on how to guarantee no conflict between the SSB-less SCell definition and the demand of SSB based neighbour cell measurement. Proposal 4: To meet the definition of SSB-less SCell, if SMTC is configured in the MO of SSB-less SCell, two solutions could be considered: - Solution 1: The MO should not be declared as ServingCellMo in ServingCellConfig. Then the SSB of this MO is only transmitted from the neighbour cell. - For the case of no SSB measurement demand at any neighbour cell of the same frequency, the MO of the SSB-less SCell can be declared as ServingCellMo in ServingCellConfig. - Solution 2: No restriction on configuration of ServingCellMo. But introduce explicit signaling to configure SSB-less SCell. Proposal 5: Try to conclude in this meeting, at least to send LS to RAN2 to inform them what does RAN4 concern and the subsequent issues to address if no progress achieved in RAN4. Proposal 6: To reduce the measurement delay, it is better to define such neighbour cell measurement as intra-frequency type since actually there is serving cell at the frequency. Proposal 7: Whether the neighbour cell measurement is performed with gap or without gap, follow the legacy rule specified Clause 9.2.1 for intra-frequency. |
| R4-2501215 | Huawei, HiSilicon | Observation 1: When the servingCellMO is configured, based on existing intra-frequency definition, the neighbor cell measurement is defined as intra-frequency when centre frequency of the SSB of the neighbour cell is same as that of SSB configured in servingCellMO; otherwise, it is defined as intra-frequency. Observation 2: When the servingCellMO is configured, whether MG is needed can be determined based on existing conditions as defined for intra-frequency/inter-frequency, and corresponding measurement requirements apply accordingly. Observation 3: When the servingCellMO is not configured, based on existing intra-frequency definition, the neighbor cell measurement is defined as inter-frequency as there is no “SSB configured in servingCellMO”. Observation 4: When the servingCellMO is not configured, whether MG is needed can be determined based on existing conditions as defined for inter-frequency measurement, and corresponding measurement requirements apply accordingly. Proposal 1: RAN4 to provide following input to RAN2 regarding the definition of intra-frequency/inter-frequency, whether MG is needed and which requirements apply: When the servingCellMO is configured: - The neighbor cell measurement is defined as intra-frequency when centre frequency of the SSB of the neighbour cell is same as that of SSB configured in servingCellMO; otherwise, it is defined as intra-frequency - Whether MG is needed can be determined based on existing conditions as defined for intra-frequency/inter-frequency, and corresponding measurement requirements apply accordingly. When the servingCellMO is not configured: - the neighbor cell measurement is defined as inter-frequency as there is no “SSB configured in servingCellMO - whether MG is needed can be determined based on existing conditions as defined for inter-frequency measurement, and corresponding measurement requirements apply accordingly. Proposal 2: Based Proposal 1, there is no further RAN4 spec impact. |
| R4-2501216 | Huawei, HiSilicon | (Netw_Energy_NR-Perf) CR on TC maintenance for Rel-18 NES SSB-less |
| R4-2501217 | Huawei, HiSilicon | (Netw_Energy_NR-Perf) CR on TC maintenance for Rel-18 NES CHO |
| R4-2502059 | Ericsson | - When EPRE difference is beyond 12 dB and below 34 dB, UE is allowed one additional P-TRS for the AGC settling. - RAN4 to send LS to RAN2 informing about disadvanges of not indicating the SSB less SCells frequnecy’s measurement object ID. - The measurement object on SSB less SCell frequency is treated as inter-frequency - Measurement requirements are different for inter-frequency measurement, which might impact the measurement performance. |
| R4-2502060 | Ericsson | CR to TS 38.133 on core requirement maintenance for NES |

## Open issues summary

Before Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 1-1 Core/Perf maintenance

Issue 1-1-1: Neighbour cells on carrier of SSB-less SCell

Background:

RAN2 agreements in RAN2#127bis Agreements 1 From RAN2 perspective, no benefit is identified for configuring servingCellMO for inter-band SSB-less SCell. 2 No LS is sent to R4. 3 Discuss in the next meeting on whether to capture “For inter-band SSB-less SCell(s), this field is not present” in the conditional presence of servingCellMO, depending on R4 progress. 4 Postpone the discussion on intra-band SSB-less SCell. RAN4 agreements in RAN4#113 Agreement: RAN4 provide analysis on the categorization of intra-frequency/inter-frequency, whether MG is needed and any other aspects (if identified) for following cases: - When servingCellMO is configured - When servingCellMO is not configured RAN4 provide the above analysis to RAN2 (if consensus can be reached) and wait for RAN2 decision on whether servingCellMO is present for carriers of SSB-less SCell.

Sub-issue 1: Definition on intra-frequency or inter-frequency

- Proposals

  - Option 1: (Huawei, Nokia)

    - When the servingCellMO is configured:

      - The neighbor cell measurement is defined as intra-frequency when centre frequency of the SSB of the neighbour cell is same as that of SSB configured in servingCellMO; otherwise, it is defined as inter-frequency

    - When the servingCellMO is not configured:

      - the neighbor cell measurement is defined as inter-frequency as there is no “SSB configured in servingCellMO

  - Option 1a: (Nokia)

    - In the MO indicated by ServingCellMO, network may configure ssbFrequency to indicate a carrier frequency for the SSB-less SCell so that UE can determine measurement as intra-frequency or inter-frequency measurement based on the ssbFrequency.

  - Option 2: Inter-frequency measurement provided that servingCellMO is not present. (Apple, Apple)

Sub-issue 2: Whether MG is needed.

- Proposals

  - Option 1: Whether to apply the requirement of inter-frequency measurement with MG or without MG follow the legacy principle and there is no spec impact. (Huawei, ZTE)

  - Option 2: Treated as inter-frequency measurement without MG as long as the SSBs from those neighbor cells can be contained in the active BWP of SSB-less SCell. (Apple)

Sub-issue 3: Which requirements to apply

- Proposals

  - Option 1: To reduce the measurement delay, it is better to define such neighbour cell measurement as intra-frequency type since actually there is serving cell at the frequency (ZTE)

  - Option 2: Whether MG is needed can be determined based on existing conditions as defined for intra-frequency/inter-frequency, and corresponding measurement requirements apply accordingly. (Huawei)

Sub-issue 4: Spec impact

- Proposals

  - Option 1: Apple

A measurement is defined as an inter-frequency SSB based measurements without measurement gaps (either legacy measurement gap or NCSG) in active BWP, for UE capable of interFrequencyMeas-NoGap provided that - the UE supports interFrequencyMeas-Nogap-r16 [15], and - the SSB is completely contained in the active BWP of the UE, or - the SSB of neighbor cells is completely contained in the active BWP of SSB-less SCell of the UE - For inter-frequency SSB based measurements without measurement gaps, UE may cause scheduling restriction as specified in clause 9.3.9.3. - Note: Non-CA capable UE is not expected to indicate support of interFrequencyMeas-Nogap-r16 [15].

  - Option 2: Nokia CR R4-2500472

### 9.2.1 Introduction A measurement is defined as a SSB based intra-frequency measurement provided the centre frequency of the SSB of the serving cell and the centre frequency of the SSB of the neighbour cell indicated for measurement are the same, and the subcarrier spacing of the two SSBs are also the same. If the UE supports ncd-SSB-BWP-Wor-r18, a measurement is defined as a SSB based intra-frequency measurement provided the centre frequency of the reference SSB of the serving cell and the centre frequency of the SSB of the neighbour cell are the same, and the subcarrier spacing of the two SSBs are also the same. The reference SSB is the SSB defined in BWP-specific servingCellMO under BWP-DownlinkDedicated of active DL BWP. If the field is absent, the reference SSB is the SSB defined in servingCellMO under ServingCellConfig [2]. If the UE supports scellWithoutSSB-InterBandCA-r18, a measurement is defined as a SSB based intra-frequency measurement provided the ssbfrequency of the SSB-less SCell and the centre frequency of the SSB of the neighbour cell are the same.

  - Option 3: No RAN4 spec impact (Huawei)

Sub-issue 5: Information to be provided to RAN2

- Proposals

  - Option 1: (ZTE)

    - From RAN4 perspective, we should pay more attention on how to guarantee no conflict between the SSB-less SCell definition and the demand of SSB based neighbour cell measurement.

    - To meet the definition of SSB-less SCell, if SMTC is configured in the MO of SSB-less SCell, two solutions could be considered:

      - Solution 1: The MO should not be declared as ServingCellMo in ServingCellConfig. Then the SSB of this MO is only transmitted from the neighbour cell.

        - -  For the case of no SSB measurement demand at any neighbour cell of the same frequency, the MO of the SSB-less SCell can be declared as ServingCellMo in ServingCellConfig.

      - Solution 2: No restriction on configuration of ServingCellMo. But introduce explicit signaling to configure SSB-less SCell.

    - Try to conclude in this meeting, at least to send LS to RAN2 to inform them what does RAN4 concern and the subsequent issues to address if no progress achieved in RAN4

  - Option 2: (Ericsson)

    - RAN4 to send LS to RAN2 informing about disadvanges of not indicating the SSB less SCells frequnecy’s measurement object ID.

      - The measurement object on SSB less SCell frequency is treated as inter-frequency

      - Measurement requirements are different for inter-frequency measurement, which might impact the measurement performance.

  - Option 3: (Huawei)

When the servingCellMO is configured:

- The neighbor cell measurement is defined as intra-frequency when centre frequency of the SSB of the neighbour cell is same as that of SSB configured in servingCellMO; otherwise, it is defined as intra-frequency

- Whether MG is needed can be determined based on existing conditions as defined for intra-frequency/inter-frequency, and corresponding measurement requirements apply accordingly.

When the servingCellMO is not configured:

- the neighbor cell measurement is defined as inter-frequency as there is no “SSB configured in servingCellMO

- whether MG is needed can be determined based on existing conditions as defined for inter-frequency measurement, and corresponding measurement requirements apply accordingly.

With above agreements, there is no further RAN4 spec impact, and it is up to RAN2 decision on whether servingCellMO can be present for SSB-less SCell.

- Moderator:

Based on guidance in last RAN4 meetings:

RAN4 provide analysis with/without servingCellMO, and whether servingCellMO is present is up to RAN2.

Based on above way forward, it is recommended to discuss based on following framework/TP.

- From RAN4 understanding, the definition of intra-frequency/inter-frequency, whether MG is needed and which requirements apply depends on whether servingCellMO is present or not.

- Based on the agreements in RAN4#113 meeting, RAN4 provide following information

  - When the servingCellMO is configured:

    - Intra/inter: The neighbor cell measurement is defined as intra-frequency when centre frequency of the SSB of the neighbour cell is same as that of SSB configured in servingCellMO; otherwise, it is defined as inter-frequency

    - MG: Whether MG is needed can be determined based on existing conditions as defined for intra-frequency/inter-frequency

    - Requirements:

      - Option 1: corresponding measurement requirements apply accordingly.

      - Option 2: Intra-frequency requirements

  - When the servingCellMO is not configured:

    - Intra/inter: the neighbor cell measurement is defined as inter-frequency

    - MG:

      - Option 1: whether MG is needed can be determined based on existing conditions as defined for inter-frequency measurement

      - Option 2: without MG as long as the SSBs from those neighbor cells can be contained in the active BWP of SSB-less SCell

    - Requirements: corresponding measurement requirements apply accordingly.

- Other information to RAN2:

  - Option 1: Based on above RAN4 conclusion, it is up to RAN2 to decide whether servingCellMO can be present or not.

  - Option 2: The potential conflict (if any) when servingCellMO is present is up to RAN2 to handle.

Issue 1-1-2: EPRE conditions updating

Background:

RAN4 WF in RAN4#112 R4-2414102 Issue 1-1-1: Power difference conditions Agreement: - For EPRE difference within X dB, keep the existing requirement. - X equals to or larger than 12 dB - For EPRE difference beyond X dB and no larger than Y dB, add one more P-TRS occurrence. - Further discuss whether the upper bound of Y is needed. If needed, Y is larger than 25dB. - From RAN4 perspective, AGC compensation for carrier frequency difference can be optionally implemented by UE. Remove “[after pre-compensation]” from the spec, i.e., not specify “AGC compensation” for carrier frequency difference in the spec. - Further discuss the X and Y values in this meeting. Try to conclude in the 2nd round on Thursday afternoon. Additional Agreement: X = 12; Y = [30] RAN4 WF in RAN4#113 Issue 1-1-2: EPRE conditions updating - Proposals - Option 1: When EPRE difference is beyond 12 dB and below 34 dB, UE is allowed one additional P-TRS for the AGC settling. (Ericsson) - Option 2: Specify the threshold of Y as the value not smaller than 30dBm. A-TRS based requirement is based on two TRS samples, not impacted by the threshold of X, Y. (ZTE) Agreement: Conclude the number without [] in the next meeting.

- Proposals

  - Option 1: When EPRE difference is beyond 12 dB and below 34 dB, UE is allowed one additional P-TRS for the AGC settling.  (Ericsson)

  - Option 2: Specify the threshold of Y as the value not smaller than 30dBm. A-TRS based requirement is based on two TRS samples, not impacted by the threshold of X, Y. (ZTE)

- Recommended WF:

  - Discuss whether further updating is needed.

### Sub-topic 1-2 CR handling

SSB-less Core

| Tdoc | Title | Source | Recommendation |
| --- | --- | --- | --- |
| R4-2502060 | CR to TS 38.133 on core requirement maintenance for NES | Ericsson | Pending on issue 1-1-2 |
| R4-2500472 | 38.133 CR on intra-frequency measurement with SSB-less SCell | Nokia, Nokia Shanghai Bell | Pending on issue 1-1-1 |

SSB-less Perf

| Tdoc | Title | Source | Recommendation |
| --- | --- | --- | --- |
| R4-2501216 | (Netw_Energy_NR-Perf) CR on TC maintenance for Rel-18 NES SSB-less | Huawei, HiSilicon | Discuss the CR |

CHO Perf

| Tdoc | Title | Source | Recommendation |
| --- | --- | --- | --- |
| R4-2501217 | (Netw_Energy_NR-Perf) CR on TC maintenance for Rel-18 NES CHO | Huawei, HiSilicon | Discuss the CR |