| 3GPP TR 38.771 V0.2.0 (2025-02) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; Study on NR frequency range 2 (FR2) OTA (Over the Air) testing Phase 3; (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	6

3.1	Terms	6

3.2	Symbols	7

3.3	Abbreviations	7

4	General	8

5	UE RF testing methodology for STxMP	9

5.1	General	9

5.2	Measurement setup	9

5.3	Test System Aspects	9

5.4	Test procedure	9

Annex A: Measurement uncertainty budget for STxMP UE RF testing methodology	10

Annex <X>: Change history		11

For definitive guidance on drafting 3GPP TSs and TRs, see 3GPP TS 21.801 supplemented by the 3GPP web page http://www.3gpp.org/specifications-groups/delegates-corner/writing-a-new-spec.

Ensure all blue guidance text is removed before submitting the TS/TR to the TSG for approval.

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In drafting the TS/TR, pay particular attention to the use of modal auxiliary verbs! TRs shall not contain any normative provisions.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The objectives for FR2-1 OTA testing for UEs with simultaneous transmission with multi-panel (STxMP) are as follows.

-	Study and define RF testing methodology for FR2 non-handheld UE that can transmit simultaneously with multi-panel

-	Define the measurement setup and test procedure for configured transmitted power requirements for simultaneous transmission to multiple directions

-	Selecting proper AoA pairs for verification perspective

-	Target PC1/PC5 devices as the 1st priority.

-	Develop the related preliminary uncertainty assessments for the test methodology

-	FR2 test methods for multi-Rx chain DL reception defined in TR 38.871 should be used as the baseline.

-	The tests shall take the test system reuse, test system complexity and test time into account to keep the whole test costs within a reasonable level.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.101-2: "User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone".

[3]	3GPP TR 38.810: "Study on test methods".

[4] 	3GPP TR 38.884: " Study on enhanced test methods for FR2 NR UEs".

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

# 3	Definitions of terms, symbols and abbreviations

<Editor’s note: this clause and its three subclauses are mandatory. The contents shall be shown as "void" if the TS/TR does not define any terms, symbols, or abbreviations.>

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Definition format (Normal)

<defined term>: <definition>.

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

Symbol format (EW)

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

Abbreviation format (EW)

<ABBREVIATION>	<Expansion>

# 4	General

<Editor’s note: general aspects related to the scope of the study or common study outcomes can be captured in this clause>

## 4.1	Objective

In Rel-18 WI on MIMO Evolution for Downlink and Uplink (Acronym: NR_MIMO_evo_DL_UL), one of the objectives is to study, and if needed, extend the specification to facilitate simultaneous transmission with multi-panel (STxMP), targeting CPE/FWA/vehicle/industrial devices. With the UE RF requirements for PC1/PC2/PC5/PC6 were specified for STxMP scenarios in RAN4, the test methodology for FR2 non-handheld UE supporting simultaneous transmission with multi-panel need to be enhanced. This document is to define the enhanced test methodology, measurement setup and test procedure for the verifications of configured transmitted power requirements for simultaneous transmission to multiple directions.

## 4.2	Devices Type

The development of test methods shall focus on PC1/PC5 devices as the 1st priority. Other non-handheld UE types such as vehicle and industrial devices are precluded due to lack of core requirements.

# 5	UE RF testing methodology for STxMP

## 5.1	General

<Editor’s note: general aspects related to the UE RF testing methodology for STxMP can be captured in this clause>

The test methodology defied in this document only applies for STxMP testing with two-layer transmission (i.e., one layer per TCI). The methodology for single layer transmission needs to be further studied with the corresponding core requirements.

## 5.2	Measurement setup

<Editor’s note: outcome of measurement setup for STxMP UE RF testing can be captured in this clause>

Through the analysis of system complexity, feasibility of guaranteeing the same relative angular orientations between specific UE test directions and probes, etc, it was confirmed that multi-Rx system is suitable for STxMP testing. Therefore, the measurement setup with full degrees of freedom for AoA1 with fixed angular Offset(s) between AoA1 and AoA2 and with all probes located on the xz plane specified in [TR 38.871] can be reused for STxMP testing.

The example multi-AoA measurement setups are similar to baseline setups for 2 AoA RRM testing [TS 38.508-1], see Clause B.2, as illustrated in Figure 5.2-1 or FR2 MIMO testing [TS 38.151], see Clause B.2, as illustrated in Figure 5.2-2.

Figure 5.2-1: Example RRM baseline system with two simultaneously active AoA using top: DFF setup, centre: Enhanced IFF setup, bottom: IFF+DFF setup.

Figure 5.2-2: Example FR2 MIMO OTA 3D-MPAC system.

## 5.3	Test System Aspects

<Editor’s note: outcome of  test system aspects for STxMP UE RF testing can be captured in this clause>

### 5.3.1	Absolute Probe Locations

Given the rotational symmetry, two systems shown in Figure 5.2-1 and Figure 5.2-2 with different absolute probe locations shall still be able to guarantee the same relative angular orientations between specific UE test directions and probes provided the baseline system with probes in the xz plane and angular sweeps of 0≤q<360° and 0≤f<180° are supported.

### 5.3.2	AoA1-AoA2 DL Orientation Vectors

It was agreed that the directionality of the AoA1-AoA2 DL orientation vectors matters. Given RAN4 has confirmed that the baseline multi-RX/STxMP system is suitable to guarantee the same relative angular separations between specific UE test orientations and probes, similar as multi-Rx test system specified in [TR 38.871], the impact of directionality of the AoA1-AoA2 orientation vectors shall also be accommodated in the measurement to ensure no testing bias is introduced.

### 5.3.3	Calibration measurement procedure

The calibration measurement per DFF probe and IFF probe specified in [TR 38.871] can be reused for STxMP testing.

## 5.4	Test procedure

<Editor’s note: outcome of test procedure for STxMP UE RF testing can be captured in this clause>

### Min peak EIRP

The min peak EIRP requirement for STxMP is defined at the beam peak direction for each of the indicated joint/UL TCI states. However, the beam peak search procedure is not suitable for STxMP due to the following reasons:

-	Since only several fixed AoA separation can be chosen in the test system, it is hard to guarantee the tested AoA is exactly aligned with beam peak direction from each panel.

-	The mapping between probe and UE panel may change during the beam peak search, and the test results of each probe contain the radiated power from both panels and cannot differentiate two beam peaks clearly.

Two possible tested method to verify the requirement without beam peak search are proposed:

-	Option 1: Consider similar as multi-Rx UE declaration approach for one AoA pair selection from the set of {30deg, 60deg, 90deg, 120deg, 150deg} and one proper UE orientation listed in TS 38.101-2 [2] for EIRP per TCI measurement.

-	Option 2: For the selection of AoA pair, one of AoA is at the beam peak direction of single CC operation without STxMP enable (from Beam Peak Search for MOP in 6.2 of TS 38.101-2 [2]), another AoA is decided by UE declared orientation (listed in TS 38.101-2 [2]) and AoA pair from set of {30deg, 60deg, 90deg, 120deg, 150deg}.

The main difference is whether let UE totally decide the AoA pair to be tested. The following simulation results show the EIRP at ideal beam peak of each panel and the maximum EIRP under declared AoA offset and UE orientation. The 1x4 panel is used as an example and two panels are located at opposite side.

Table 5.4.1-1 EIRP at ideal beam peak and maximum EIRP based on UE declaration

| Orientation#1 AoA offset | panel#1 peak angle [theta, phi] | panel#1 peak EIRP /dBm | panel#2 peak angle [theta, phi] | panel#2 peak EIRP /dBm | AoA pair when AoA1 EIRP is maximum [AoA1φ, AoA1θ] [AoA2φ, AoA2θ] | AoA pair EIRP [AoA1,AoA2]/dBm | AoA pair when AoA2 EIRP is maximum [AoA1φ, AoA1θ] [AoA2φ, AoA2θ] | AoA pair EIRP [AoA1,AoA2]/dBm |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 30° | [165,90] | 26.49 | [10,90] | 26.4 | [170,90] [170,120] | [26.44, 19.75] | [165,60] [165,90] | [19.29,26.49] |
| 60° |  |  |  |  | [165,90] [165,150] | [26.49,17.82] | [165,30] [165,90] | [15.34,26.49] |
| 90° |  |  |  |  | [165,90] [165,180] | [26.49,14.09] | [165,0] [165,90] | [16.48,26.49] |
| 120° |  |  |  |  | [165,90] [345,150] | [26.48,19.16] | [345,30] [165,90] | [17.84,26.49] |
| 150° |  |  |  |  | [165,90] [345,120] | [26.49,23.64] | [345,60] [165,90] | [23.00,23.49] |

| Orientation#2 AoA offset | panel#1 peak angle [theta, phi] | panel#1 peak EIRP /dBm | panel#2 peak angle [theta, phi] | panel#2 peak EIRP /dBm | AoA pair when AoA1 EIRP is maximum [AoA1φ, AoA1θ] [AoA2φ, AoA2θ] | AoA pair EIRP [AoA1,AoA2]/dBm | AoA pair when AoA2 EIRP is maximum [AoA1φ, AoA1θ] [AoA2φ, AoA2θ] | AoA pair EIRP [AoA1,AoA2]/dBm |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 30° | [90,105] | 26.49 | [270,100] | 26.4 | [90,100] [90,130] | [26.44, 18.87] | [270,130] [270,100] | [18.87,26.40] |
| 60° |  |  |  |  | [90,105] [90,165] | [26.49,19.39] | [90,45] [90,105] | [19.43,26.49] |
| 90° |  |  |  |  | [90,105] [270,165] | [26.49,20.49] | [90,15] [90,105] | [21.37,26.49] |
| 120° |  |  |  |  | [90,105] [270,135] | [26.49,24.26] | [270,15] [90,105] | [23.00,26.49] |
| 150° |  |  |  |  | [90,105] [270,105] | [26.49,26.31] | [270,45] [90,105] | [23.79,23.49] |

The results show that based on UE declaration, one of AoA is always align with the ideal beam peak direction of panels when maximum EIRP is achieved, and under proper UE orientation and AoA offset, a AoA pair that close to the peak for each antenna module can be found, i.e., 150°AoA offset for Orientation#2 in above table. Therefore, the beam peak direction of single CC operation can be one of the candidates for STxMP verification.

After RAN4 discussion, the conclusion on AoA pair (AoA1 and AoA2) selection is as following which is a combination of Option 1 and Option 2:

-	  If UE declares the AoA1, then

a) The declaration of AoA1 direction can be any direction.

b) AoA2 direction is to use AoA1 + or - UE declared offset from set of {30deg, 60deg, 90deg, 120deg, 150deg} in    xz plane of test system

c) AoA1+ UE declared offset or AoA1 - UE declared offset needs to be declared by UE

d) If AoA1 and AoA2 directions pass EIRP PUMAX,f,c,k requirements, the DUT pass the test.

-   If UE doesn’t declare AoA1, then

a) the default AoA1, i.e., the beam peak direction of single CC operation without STxMP is applied

b) AoA2 direction is to use AoA1 + and - UE declared offset from set of {30deg, 60deg, 90deg, 120deg, 150deg} in xz plane of test system

c) AoA2 direction is AoA1 + the declared offset

d) AoA2 direction is AoA1 - the declared offset

e) If AoA1 and any of the two AoA2 directions pass EIRP PUMAX,f,c,k requirements, the DUT pass the test.

Note that the above AoA pair (AoA1 and AoA2) selection is based on the UE declared UE orientation.

The EIRP shall be measured per TCI with the configuration of rank 2 PUSCH transmission under SDM scheme, i.e., UE transmit signals with different layers for AoA1 direction and AoA2 direction respectively.

Testing polarization selection also need to be investigated for STxMP. In the legacy single AoA testing there are four EIRP component testing (2 link pol x 2 measurement pol). The 2 measurement polarizations correspond to  and  EIRP component which can not be reduced. The 2 link polarizations are measured in order to get the max total EIRP between the two link pol. When EIRP comes to 2AoA case, the whole EIRP measurements would be 4 link pol combinations x 4 EIRP components = 16 EIRP testing. It is agreed to limit the polarization combinations to simplify the test with crossed link polarization (‘’ and ‘’) which also benefits lower interference between probes.

### 5.4.2 MPR/A-MPR

To avoid additional unwanted emission, 3dB relaxation is applied to the MPR/A-MPR of STxMP requirement and legacy MPR/A-MPR is reused for each of the indicated joint/UL TCI states. The UE can satisfy the requirement only if the legacy MPR/A-MPR requirement can be met when only one panel is activated, so it is concluded that the MPR/A-MPR is unnecessary to be verified.

### 5.4.3 EIRPmax

Unlike min peak EIRP, the radiated power from both panels needs to be aggregated for EIRPmax verification, and the aggregated power at any direction should not exceed the defined requirement. The following simulation results show the maximum aggregated power and related AoA pair, and the simulation setup is same as min peak EIRP.

Table 5.4.3-1 Maximum aggregated power and related AoA pair

|  | AoA offset | AoA pair of aggregated EIRPmax across all tested beam pair [AoA1φ, AoA1θ] [AoA2φ, AoA2θ] | EIRPmax | Beam peak direction when only one panel is activated [φ, θ] |
| --- | --- | --- | --- | --- |
| Panels in same side | 30° | [270,105] [270,75] | 29.22 | [270,105] |
|  | 60° | [270,105] [270,45] | 29.22 |  |
|  | 90° | [270,105] [270,15] | 29.22 |  |
|  | 120° | [270,105] [90,15] | 29.22 |  |
|  | 150° | [270,105] [90,45] | 29.22 |  |
| Panels in opposite side | 30° | [90,105] [90,135] | 26.97 | [90,105] |
|  | 60° | [90,95] [90,155] | 26.94 |  |
|  | 90° | [90,95] [270,175] | 26.94 |  |
|  | 120° | [90,95] [270,145] | 26.94 |  |
|  | 150° | [90,95] [270,115] | 26.94 |  |

For the AoA pair to achieve maximum aggregated power, one of AoA is align with the legacy beam peak direction, but the AoA separation is not always same as the UE declared in min peak EIRP which depends on UE implementation. Therefore, the following two sets of AoA separation and UE orientation needs to be tested for EIRPmax compliance

-	The AoA separation and UE orientation declared by UE for EIRPPUMAX,f,c,k testing.

-	One of AoA is at the beam peak direction of single CC operation without STxMP enabled, the other AoA is at the direction decided by UE declared orientation (listed in TS 38.101-2 [2]) and 30deg AoA separation.

In theory, as long as the measured the EIRPmax for single carrier operation is 3dB lower than the EIRPmax requirement, the measured EIRPmax under STxMP 2AoA transmission should also lower than the EIRPmax requirement. After RAN4 discussion, 0.5dB margin is applied to guarantee the regulation requirement can be satisfied for sure, so the following test skipping rule is agreed:

-   If the measured the EIRPmax for single carrier operation is 3.5dB lower than the EIRPmax requirement, the     EIRPmax test for STxMP can be skipped.

### 5.4.4 TRPmax

For TRPmax, same approach as min peak EIPR is used to decide the AoA pair and UE orientation.

In theory, as long as the measured the TRPmax for single carrier operation is 3dB lower than the TRPmax requirement, the measured TRPmax under STxMP 2AoA transmission should also lower than the TRPmax requirement. After RAN4 discussion, 0.5dB margin is applied to guarantee the regulation requirement can be satisfied for sure, so the following test skipping rule is agreed:

- 	If the measured the TRPmax for single carrier operation is 3.5dB lower than the TRPmax requirement, the EIRPmax test for STxMP can be skipped.

Annex A:
Measurement uncertainty budget for STxMP UE RF testing methodology

<Editor’s note: the outcome of MU budget for STxMP UE RF testing methodology can be captured in this clause>

Annex <X>:
Change history

This is the last annex for TS/TSs which details the change history using the following table.
This table is to be used for recording progress during the WG drafting process till TSG approval of this TS/TR.
For TRs under change control, use one line per approved Change Request
Date: use format YYYY-MM
CR: four digits, leading zeros as necessary
Rev: blank, or number (max two digits)
Cat: use one of the letters A, B, C, D, F
Subject/Comment: for TSs under change control, include full text of the subject field of the Change Request cover
New vers: use format [n]n.[n]n.[n]n

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2024-08 | R4#112 | R4-2413533 |  |  |  | Initial skeleton | 0.0.1 |
| 2024-11 | R4#113 | R4-2418164 |  |  |  | TR 38.771 for study on NR FR2 OTA testing phase 3 - R4-2419809, TP to TR 38.771 on the test procedure of sTxMP, vivo | 0.1.0 |
| 2025.02 | R4#114 | R4-2501728 |  |  |  | TR 38.771 for study on NR FR2 OTA testing phase 3 - R4-2500508, TP to TR 38.771 on STxMP test procedure, Samsung - R4-2501729, TP to TR 38.771 on general part, Qualcomm - R4-2501730, TP to TR 38.771 on test methodology for STxMP, Qualcomm | 0.2.0 |