3GPP RAN4 Meeting #114-bis		R4-2503138

Wuhan, China, 7th – 11th April, 2025

Agenda item:	7.24.4

Source:	Ericsson

Title:	Discussion on OTA testing for A-IoT device 1

WI/SI:	Ambient_IoT_Solutions-Core

Release:	Rel-19

Document for:	Approval

# Introduction

During RAN4 #114 meeting the following agreements were made in WF [1] concerning A-IoT device 1 OTA testing.

This contribution provides our views related to some of the remaining issues.

# Discussion

## Test System Reuse

TR 38.870 details OTA test methods for NR1 TRP and TRS using both Anechoic Chamber (AC) and Reverberation Chamber (RC). These specifications were developed mainly for smartphones, wearable redcap UEs, tablets and laptops. However, A-IoT device 1 is significantly different from these types of devices. Indeed, A-IoT device 1 does not generate the transmitted signal, it only backscatters an unmodulated incoming CW signal transmitted from a CW Transmitter (CWT). Hence, test methods given in TR 38.870 have to be modified to take into account this aspect. Furthermore, it is our view that A-IoT device 1 should be tested using realistic deployment scenarios, and this includes testing this device when backscattering a CW signal from an external CWT and using RF energy harvesting to recharge its Energy Storage Unit (ESU).

Observation 1: A-IoT device 1 does not generate the transmitted signal, it only backscatters an unmodulated incoming CW signal transmitted from a CWT, hence it should be tested using realistic deployment scenarios while backscattering a CW signal from an external CWT and using RF energy harvesting to recharge its Energy Storage Unit (ESU).

Proposal 1: A-IoT device 1 should be tested while backscattering a CW signal from an external CWT and using RF energy harvesting to recharge its Energy Storage Unit (ESU).

Due to A-IoT device 1 small ESU and likely omnidirectional antennas, to reduce the measurement time, a coarse measurement grid can be used for both TRP and TRS measurement. Given the table below from TR 38.870, we propose using 30 degrees for TRP and 45 degrees for TRS that lead to an acceptable deviation in the measured values.

Observation 2: Due to A-IoT device 1 very small Energy Storage Unit (ESU) and likely omnidirectional antennas, a coarse measurement grid can be used for both TRP and TRS measurement.

Proposal 2: Use a coarse measurement grid of 30 degrees for TRP and 45 degrees for TRS measurement.

Observation 3: Due to A-IoT device 1 small size, it is sufficient to use the legacy Quiet Zone (QZ) size of 30 cm for testing.

Proposal 3: Use a Quiet Zone (QZ) size of 30cm for A-IoT device 1 testing.

The minimum distances DCW2D and DMA2D can be calculated using the formulas for minimum range length considering a 30cm QZ in TR 38.870. This range is defined as the distance from the centre of the quiet zone to the aperture of the measurement probes as shown below in the figure below.

It is the maximum of the following three limits:

-	The phase uncertainty limit: RQZ+2Drad2/λ

-	The amplitude uncertainty limit: 3D

-	The reactive Near-Field limit: RQZ+2λ

where RQZis defined as the radius of the quiet zone, i.e., RQZ=D/2, and Dradis the diameter of the effective radiating aperture. The minimum range length calculations for D=30cm quiet zone size TRP-TRS OTA test systems assume Drad is 30cm below 1GHz and decrease linearly from 30cm to 5cm from 1GHz to 7.125GHz, respectively.

The table below show the minimum range length for a 30cm QZ as a function of the frequency.

Observation 4: Considering a 30cm QZ and n8 frequency band (900MHz), the minimum range length is 90cm, so we propose using 1.5m distance for both DCW2D and DMA2D for A-IoT device 1 testing.

Proposal 4: Use a distance DCW2D and DMA2D of 1.5m for A-IoT device 1 testing.

Observation 5: Considering 900MHz frequency band; distance DCW2D and DMA2D of 1.5m; backscattering efficiency of -6dB; and omnidirectional antennas at CWT and reader, the received signal at the measurement antenna will be attenuated by around 76dB. Hence, to insure a measurement SINR of [20]dB, IsoCW2M should be superior to 96dB. This level of isolation can be obtained by making DCW2M significantly larger than DCW2D and DMA2D and using interference cancellation techniques.

Proposal 5: If omnidirectional antennas are used at CWT and reader, use an isolation IsoCW2M, including interference cancellation gain, of 96dB for A-IoT device 1 testing.

Observation 6: Considering 900MHz frequency band; distance DCW2D and DMA2D of 1.5m; backscattering efficiency of -6dB; and high gain antennas (around 15dBi) at CWT and reader, the received signal at the measurement antenna will be attenuated by around 46dB. Hence, to insure a measurement SINR of [20]dB, IsoCW2M should be superior to 66dB.

Proposal 6: If high directive antennas (around 15dBi) are used at CWT and reader, use an isolation IsoCW2M, including potential interference cancellation gain, of 66dB for A-IoT device 1 testing.

## Measurement Uncertainty (MU) assessment

The MU assessment will depend on the used test system and test procedure. So, it is worth agreeing on a test system and test procedure before performing the MU assessment.

# Conclusions

This contribution provides our views related to OTA test methodology for A-IoT device 1. The contribution makes the following proposals.

Observation 1: A-IoT device 1 does not generate the transmitted signal, it only backscatters an unmodulated incoming CW signal transmitted from a CWT, hence it should be tested using realistic deployment scenarios while backscattering a CW signal from an external CWT and using RF energy harvesting to recharge its Energy Storage Unit (ESU).

Proposal 1: A-IoT device 1 should be tested while backscattering a CW signal from an external CWT and using RF energy harvesting to recharge its Energy Storage Unit (ESU).

Observation 2: Due to A-IoT device 1 very small Energy Storage Unit (ESU) and likely omnidirectional antennas, a coarse measurement grid can be used for both TRP and TRS measurement.

Proposal 2: Use a coarse measurement grid of 30 degrees for TRP and 45 degrees for TRS measurement.

Observation 3: Due to A-IoT device 1 small size, it is sufficient to use the legacy Quiet Zone (QZ) size of 30 cm for testing.

Proposal 3: Use a Quiet Zone (QZ) size of 30cm for A-IoT device 1 testing.

Observation 4: Considering a 30cm QZ and n8 frequency band (900MHz), the minimum range length is 90cm, so we propose using 1.5m distance for both DCW2D and DMA2D for A-IoT device 1 testing.

Proposal 4: Use a distance DCW2D and DMA2D of 1.5m for A-IoT device 1 testing.

Observation 5: Considering 900MHz frequency band; distance DCW2D and DMA2D of 1.5m; backscattering efficiency of -6dB; and omnidirectional antennas at CWT and reader, the received signal at the measurement antenna will be attenuated by around 76dB. Hence, to insure a measurement SINR of [20]dB, IsoCW2M should be superior to 96dB. This level of isolation can be obtained by making DCW2M significantly larger than DCW2D and DMA2D and using interference cancellation techniques.

Proposal 5: If omnidirectional antennas are used at CWT and reader, use an isolation IsoCW2M, including interference cancellation gain, of 96dB for A-IoT device 1 testing.

Observation 6: Considering 900MHz frequency band; distance DCW2D and DMA2D of 1.5m; backscattering efficiency of -6dB; and high gain antennas (around 15dBi) at CWT and reader, the received signal at the measurement antenna will be attenuated by around 46dB. Hence, to insure a measurement SINR of [20]dB, IsoCW2M should be superior to 66dB.

Proposal 6: If high directive antennas (around 15dBi) are used at CWT and reader, use an isolation IsoCW2M, including potential interference cancellation gain, of 66dB for A-IoT device 1 testing.

# References

R4-2502858, “WF on requirements for system parameters and A-IOT device” CMCC, 3GPP RAN4 #114.