3GPP TSG-RAN WG4 Meeting #114-Bis	R4-2503170

Wuhan, CN, Apr. 2025

Source	:	Qualcomm Incorporated

Title	:	On UE RF requirements for the LPWUR

Agenda item	:	7.26.2.2

Document for	:	Approval

# 1.	Introduction

In this contribution we propose some parameters for the FR2 LPWUR, and  address the impact of an agreement in the RRM session on ability to determine LPWUS MDR.

# 2. 	Discussion

## 2.1	Pertaining to Refsens

### 2.1.1	Side conditions for Refsens

The two agreed receiver types [4] have different impairments (NF + IM). To meet the coverage requirements, the LPWUS configurations that allow each receiver type must have threshold sensitivities that compensate for the impairment level associated with that receiver. The RAN1 agreement pertaining to SNR and front-end impairment [RAN1#117] is below:

- If UE NF + IM = 17.0 dB, required LPWUS threshold SNR = -3.0 dB

- If UE NF + IM = 14.0 dB, required LPWUS threshold SNR = -0.5 dB

For example, UEs supporting set 1 (NF+IM = 18.0 dB) may need an LPWUS with a threshold SNR of -4.0 dB (or better) for the REFSENS test. Similarly, UEs supporting set 2 (NF + IM = 13.5 dB) may need an LPWUS with a threshold SNR of 0.0 dB (or better). This could be achieved, for example, with different durations of LPWUS.

Proposal 1: As side condition, the Refsens requirement is specified with a LPWUS whose threshold SNR (for 1% MDR) is:

- -4.0 dB for OOK based receivers and

- 0.0 dB for OFDM based receivers

### 2.1.2	Consistency with RRM LPWUR/MR coex principles

In the parallel RRM session the following agreement was made regarding LPWUR operation in idle mode [6]:

Issue 1-1-9: LP-WUR status at legacy case (not at LP-WUS monitoring case/fully offloading(case 1) case/RRM relaxation (case 3) case) Agreement: At legacy state, when and how to turn on LR for serving cell measurement is up to UE implementation.

The key principle here is that it is up to the UE to choose when to deploy the LPWUR, and when to instead use the MR for paging occasion monitoring in idle mode. The desired network-level outcome is continued reliability of PDCCH reception. This is also true for connected mode: there is no way for the network (or TE) to know if a certain PDCCH decoding instance was LPWUS-triggered or whether the MR monitored the configured PDCCH windows directly without any power-optimizing guidance from the LPWUS.

Observation 1: The UE can autonomously choose between LR and MR while meeting the network level goal of reliable detection of PDCCH, and whether LR was used to trigger PDCCH monitoring  is not known outside the UE.

This can mean that a UE can have poor LPWUS MDR and yet meet the required PDCCH reliability, and this is RAN1 compliant behavior. Put another way, LPWUS reliability is not a necessary condition to ensure the network goal of PDCCH reliability, and the standard leaves it up to UE implementation when it can monitor LPWUS.

Furthermore, there is no way to maintain a UE in an ‘LPWUR-enabled’ state, at least per RAN1 design. The network has neither control nor ability to monitor whether the UE relies on its LPWUR. Consequently, without ability to ensure if the LPWUR itself is on, it is not possible to reliably test any RF parameter of the LPWUR.

Observation 2: MDR of LPWUS is not directly testable, because the network or TE cannot enforce or verify that a feature-compliant UE engages its LPWUR for the entire duration of the test.

A more meaningful way to test the LPWUS is to get confirmation of a successful PDCCH detection, perhaps as an ACK to a PDSCH grant for connected mode, or RACH MSG1/MSGA for idle mode UEs. The principle is that it is more relevant to move to a functional test, i.e. ‘testing the feature’ rather than ‘testing the hardware piece that enables the feature’.

Proposal 2: The UE RF requirements for the Rel-19 LPWUS feature are built around a functional metric, for example PDCCH MDR (triggered by LPWUS), rather than MDR of LPWUS.

### 2.1.3	Refsens impact of desensitization due to legacy RBs

RAN4 have agreed [4] to a general framework for ASCS:

Issue 2-3-2: in-band power level of ASCS Test case Agreements: - For ASCS, allowable de-sense due to introduction of NR RBs is [0-1dB]. - [0] guard RB is the baseline.

To satisfy the coverage criterion in field operation, typically in the presence of legacy DL RBs, it is self-evident that the sensitivity value derived from the agreed signal configuration (no legacy RBs) will be optimistic by the desensitization caused by the legacy RBs. It is therefore necessary to amend the sensitivity expression considering the newer agreement excerpted above.

Proposal 3:             Sensitivity = -174dBm + 10*log(RxBW) + NF + SNR + IM – desens_ASCS

## 2.2	ASCS

### 2.2.1	ASCS compromise

Since ASCS value impacts the applicable Refsens requirement, UEs must balance the implications of this choice against their implementation.

On the one hand, a high de-sensitization allowance (closer to 1 dB) would make filtering less challenging. On the other hand, any de-sensitization must be compensated in the Refsens requirement by an equal amount, to ensure coverage condition is met in the presence of legacy RBs. While this is the general case, the requirement for corner cases can be relaxed, for example: a 5MHz 30k SCS has no room for additional legacy RBs, and therefore no desensitization from the same.

Different receiver types may choose to address this compromise differently. For example, an OFDM-based receiver has natural orthogonality to legacy RBs and no de-sensitization is necessary. The 0 desensitization condition is indeed preferred so the sensitivity requirement is not further tightened. Similarly, an OOK-based receiver may choose to implement better noise figure to allow for practical filtering and interference from legacy RBs.

### 2.2.2	ASCS for OOK-based receivers

Ability to suppress interference from adjacent RBs in non-coherent receivers is an extremely strong function of the receiver topology pertaining to filtering. It is possible to come up with multiple feasible combinations of guard RBs and desense allowance, and it would be very challenging to organize implementations based on this detail. Multiple aspects go into this determination, like how to distribute filtering before and after digitization, power headroom for stages before full filtering, detector design for AGC, etc. For the simplest case where all the filtering is in the analog domain prior to digitization, 1 dB or lower desense is not feasible without at least 1 guard RB. To include implementations that are aggressive in their power consumption and simplicity goals, it may be better to lay out the inclusive observation above as a general functional guideline:

Proposal 4: For OOK-based receivers, desens_ASCS = 1 dB, and 1 guard RB is specified.

### 2.2.3	ASCS for OFDM-based receivers

Interference suppression in coherent receivers is much less challenging, due to natural orthogonality of legacy RBs and indeed, NR jammers that are close by. The ‘leakage’ from off-frequency interferers is limited to secondary effects like loss of orthogonality from time/frequency errors, or non-linear effects that are not sufficiently well managed by the AGC loop. For this reason, it is reasonable to stipulate that no guard RBs are specified, and simultaneously, no desens is required.

Proposal 5: For OFDM-based receivers, desens_ASCS = 0 dB, and no guard RBs are specified.

## 2.3	Jammer hardening

### 2.3.1	General philosophy of jammer requirements

While the actual ACS/IBB/OBB requirements can wait until the Refsens requirement is stable, it is important to establish some system level principles. The operating environment remains the same no matter the LPWUR type, and the system cannot provision a low interference environment exclusively for some select UEs. Consequently:

Proposal 6: The LPWUR ACS/IBB/OBB requirements are constructed with the understanding that the same interferers that motivated the legacy jammer requirements are also present for the LPWUR, regardless of type (OOK or OFDM based).

There are implications on filtering, ADC sample rates, RF headroom and phase noise of frequency translating LOs, but these are implementation details that need to optimized around network constraints like jammers in this case.

### 2.3.2	ACS requirements

If the SNR values of the test signals for the basic refsens test are also stabilized, for example as proposed in section 2.1.1, it is possible to derive the ACS requirement for LPWUR. The overall relationship between various parameters for ACS are taken from [9], figure reproduced to the right.

If:

- Rlegacy is the refsens PSD for the legacy requirement,

- SNRlegacy is the SNR that causes the receiver to run at 95% throughput, typically -1 dB

- CBW is the channel NRB BW in Hz

- ACSlegacy is the legacy ACS requirement for that CBW

- IMACS is an implementation margin specific to ACS

The absolute interferer level at the UE input can be calculated as

Intrfrlegacy=Rlegacy-SNRlegacy+10logCBW+10log101410-1+ACSlegacy+IMACS

This same equation can be written for the absolute level of the interferer for the LPWUS case, assuming the LPWUS (wanted) signal is also 14 dB stronger than the LPWUS refsens:

IntrfrLPWUS=RLPWUS-SNRLPWUS+10logBWLPWUS+10log101410-1+ACSLPWUS+IMACS

If the absolute interferer power levels are the same for both cases, the right-hand sides of the two equations can be directly compared:

RLPWUS-SNRLPWUS+10logBWLPWUS+ACSLPWUS= Rlegacy-SNRlegacy+10logCBW+ACSlegacy

Or,

ACSLPWUS= Rlegacy-RLPWUS-SNRlegacy-SNRLPWUS+10logCBW/BWLPWUS+ACSlegacy

Now, the basic refsens PSDs are given by:

Rlegacy= -174+NFlegacy+IMNF+SNRlegacy, and

RLPWUS= -174+NFLPWUS+IMNF+SNRLPWUS

So,

Rlegacy-RLPWUS-SNRlegacy-SNRLPWUS= NFlegacy-NFLPWUS

The expression for ACSLPWUS then simplifies to:

ACSLPWUS=  NFlegacy-NFLPWUS+10logCBW/BWLPWUS+ACSlegacy

This expression can be used as the high-level principle for specifying the ACS requirement for the LPWUR

Proposal 7: LPWUS ACS is specified as (assuming higher wanted signal is higher than refsens by 14 dB):

ACSLPWUS=  NFlegacy-NFLPWUS+10logNRB/RBLPWUS+ACSlegacy

The detailed ACS side conditions can be constructed once the ACS requirement is clear.

## 2.4	Max. input power

Legacy practice for this parameter is to define the requirement for all possible channel bandwidths. The challenge for the LPWUR is that input power can be differently distributed between LPWUS and legacy DL RBs, depending on channel bandwidth. Moreover, since competitive implementations are expected to use AGC, the additional challenge is to set the AGC correctly, i.e. not saturate components while leaving enough LPWUS power for detection. It is also expected that LPWUS will be transmitted with similar (or same) PSD as legacy DL like PDSCH, so in the general case, it is not expected that the LPWUS Finally, the core requirement should ensure all channel BWs have a specified max. power guarantee.

Proposal 8: The max. input power level is specified for all channel bandwidths, in the presence of legacy RBs with the same PSD as the LPWUS RBs.

It can be left up to RAN5 how to streamline test cases.

## 2.5	Idle vs connected

RAN1 has already agreed to creating distinct capabilities for idle mode and connected mode UEs. See partial excerpt below from current state of RAN1 UE feature list [7]

| Features | Index | Feature group | : |
| --- | --- | --- | --- |
| 62. NR_LPWUS | 62-1 | LP-WUS operation in IDLE/INACTIVE mode based on OOK signal | : |
| 62. NR_LPWUS | 62-1a | LP-WUS operation in IDLE/INACTIVE mode based on OFDM [overlaid] sequence | : |
| 62. NR_LPWUS | 62-2 | LP-WUS operation in CONNECTED mode based on OOK signal | : |
| 62. NR_LPWUS | 62-2a | LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence | : |

Since support of one capability does not automatically imply support of the other, RAN4 must treat the two capabilities separately and apply requirements to UEs supporting each of those capabilities. Requirements here include side conditions that may be different between the two modes.

Proposal 9: Two sets (each set further broken out into OOK-based and OFDM-based) of LPWUS requirements are specified, one set applies to UEs supporting LPWUS in idle/inactive mode and another for UEs supporting LPWUS in connected mode.

## 2.6	FR2-specific LPWUR parameters

### 2.6.1	NF+IM assumption

RAN4 have agreed that ‘no combination gain across two Rx chains is considered for FR2 LP-WUR’ [4].  i.e., the receiver chain with the better signal or SNR is chosen (up to UE implementation). In [5], we showed that this type of scheme results in an automatic 3 dB degradation compared to the MR, but only in the worst-case polarization basis misalignment situation. Typically, the degradation is less. The resulting NF+IM projection is close enough for alignment with the agreed value for FR1 OFDM-based Rx (13.5 dB).

FR2 unlike FR1 does not lend itself to making a standalone ‘low power’ radio for LPWUS monitoring. Practical implementations will generally require significant front end sharing with the MR, and one of the benefits of that is good noise figure even for the OOK-based receiver. Consequently, both receiver types can share the same NF as OFDM FR1 type:

Proposal 10: For an FR2 LPWUR, NF+IM is 13.5 dB for both, OOK based and OFDM based topologies. This value includes impact of ‘Rx chain selection in REFSENS evaluation’.

### 2.6.2	Spherical coverage of LPWUR sensitivity

It is already agreed that single element beams would be used for the LPWUR [2]. Practical UEs are likely to share majority if not all the RF front end between the MR and the LR. Consequently, it is useful to compare UE spherical coverage with legacy module assumptions, first in legacy 4x1 configuration and then with only a single (dual pol.) element active per module. The figure on the right [5] shows the normalized spherical coverage CCDFs of sensitivity of a UE using realistic modules and MRC based on size of beam forming antenna (i.e. number of elements).

The reader is referred to [5] for further details, but the summary is below

Observation 3: The sensitivity degradation CCDF with single element module configuration is strongly correlated with the same for connected-mode PDSCH.

On the one hand, RAN4 could require an FR2 UE to demonstrate similar spherical coverage sensitivity degradation (i.e. ‘gain drop’) as connected mode, as laid out in [5]. On the other hand, if this UE has already demonstrated compliance with connected mode PDSCH sensitivity, with the front-end sharing assumption, it is automatically capable of meeting the LPWUS spherical coverage requirements if set according to the principle above. In this sense, there is no new aspect of the UE RF that is being verified.

Proposal 11: RAN4 to consider not setting an LPWUS spherical coverage sensitivity requirement for FR2 UEs, due to RF front end sharing assumption with the MR.

# 3.	Conclusions

Proposal 1: As side condition, the Refsens requirement is specified with a LPWUS whose threshold SNR (for 1% MDR) is:

- -4.0 dB for OOK based receivers and

- 0.0 dB for OFDM based receivers

Observation 1: The UE can autonomously choose between LR and MR while meeting the network level goal of reliable detection of PDCCH, and whether LR was used to trigger PDCCH monitoring is not known outside the UE.

Observation 2: MDR of LPWUS is not directly testable, because the network or TE cannot enforce or verify that a feature-compliant UE engages its LPWUR for the entire duration of the test.

Proposal 2: The UE RF requirements for the Rel-19 LPWUS feature are built around a functional metric, for example PDCCH MDR (triggered by LPWUS), rather than MDR of LPWUS.

Proposal 3:             Sensitivity = -174dBm + 10*log(RxBW) + NF + SNR + IM – desens_ASCS

Proposal 4: For OOK-based receivers, desens_ASCS = 1 dB, and 1 guard RB is specified.

Proposal 5: For OFDM-based receivers, desens_ASCS = 0 dB, and no guard RBs are specified.

Proposal 6: The LPWUR ACS/IBB/OBB requirements are constructed with the understanding that the same interferers that motivated the legacy jammer requirements are also present for the LPWUR, regardless of type (OOK or OFDM based).

Proposal 7: LPWUS ACS is specified as (assuming higher wanted signal is higher than refsens by 14 dB):

ACSLPWUS=  NFlegacy-NFLPWUS+10logCBW/BWLPWUS+ACSlegacy

Proposal 8: The max. input power level is specified for all channel bandwidths, in the presence of legacy RBs with the same PSD as the LPWUS RBs.

Proposal 9: Two sets (each set further broken out into OOK-based and OFDM-based) of LPWUS requirements are specified, one set applies to UEs supporting LPWUS in idle/inactive mode and another for UEs supporting LPWUS in connected mode.

Proposal 10: For an FR2 LPWUR, NF+IM is 13.5 dB for both, OOK based and OFDM based topologies. This value includes impact of ‘Rx chain selection in REFSENS evaluation’.

Observation 3: The sensitivity degradation CCDF with single element module configuration is strongly correlated with the same for connected-mode PDSCH.

Proposal 11: RAN4 to consider not setting an LPWUS spherical coverage sensitivity requirement for FR2 UEs, due to RF front end sharing assumption with the MR.

# 4.	References

[1] 	R4-2408032, ‘On LPWUR system considerations’, Qualcomm, RAN4#111, May 2024

[2] 	R4-2420394, ‘WF on LP-WUS UE RF requirements’, vivo, RAN4#113

[3] 	R4-2417630, ‘On UE Rx requirements for the LPWUR’, Qualcomm, RAN4#113

[4] 	R4-2503028, ‘WF on LP-WUS UE RF requirements’, vivo, RAN4#114

[5] 	R4-2500109, ‘On UE Rx requirements for the LPWUR’, Qualcomm, RAN4#114

[6]	R4-2502600, ‘WF on RRM requirements for NR_LPWUS’, vivo RAN4#114

[7]	R1-2501397, ‘Initial RAN1 UE features list for Rel-19 NR after RAN1 #120’

[8]	R4-091982, ‘ACS Calculation’, Nokia, May 2009