3GPP TSG RAN WG4 #114bis															     R4-2503236

Wuhan, China, April 7th – 11th 2025

Agenda Item:		7.28.2

Source:				InterDigital Inc.

Title:		NR_XR_Ph3 remaining issues

Document for:		Discussion and decision

# Background

In the last RAN4 meeting #114, the discussions on this topic progressed. As a result, a WF has been agreed in the meeting in [1].

# Discussion

In this contribution, we share our analysis on gap skipping measurements occasions for NR XR case and the NW scheduling mechanism.

In RAN4 meeting #114, the agreed WF in [1] treats a variety of topics and a certain number of issues still need to be sorted out. We will address them in the following sections.

# NR deployment scenarios and MEASUREMENT types

In the last meeting, the following agreements and were made:

## Issue 1-1: Deployment scenarios < Agreement>: - Deprioritize FR2-2 for enabling transmission/reception in gaps/restrictions that are caused by RRM measurements <Way forward>: - Further discussion needed for - NR-DC

One of the FFS points was about the NR dual connectivity support for this feature. In our view, dual connectivity scenarios are more complex due to the involvement of two scheduling entities, that may be uncorrelated when considering the UE measurements gap skipping through DCI signaling.

However, it has been mentioned in the last meeting that if we assume that the DCI skipping order applies only to the gaps for the cells configured per band or per cell group, the issue is resolved. In addition, the RAN1 endorsed CR (Introduction of XR (eXtended Reality) for NR Phase 3) seems to imply this approach. For easy reference of the RAN1 draft CR, please see the Annex of this document.

Observation 1: If the DCI skipping order applies to the gaps affecting the cells configured in a particular band, then at least inter-band NR-DC with gaps per UE and per FR can be supported.

Proposal 1:  If Inter-band NR-DC gap skipping is to be supported, then the rule of per band applicability for DCI gap skipping order is required and sufficient.

# Type of MEASUREMENTs with gaps

The categories of gaps to be considered under this WID have been listed in the WF as follows:

- Type-1 MG: Gap(s) configured via GapConfig without suffix - Type-2 MG: Gap(s) configured via GapConfig-r17 without preConfigInd-r17 or ncsgInd-r17 - Rel-18 Concurrent Gap: gap with assigned priority which can be also NCSG or Pre-MG

Type-1 MG is supported by all UEs, and it is something that shall be supported in this WID. However, the WID coverage for the other types is rather related to the required study time or specification impact. Without having a strong opinion, we are OK with any second priority types if the work can be completed under Rel-19 timeframe.

Observation 2:  As Type-1 MG is a mandatory feature for Rel-19, the other types of MG shall be treated as a second priority.

Proposal 2: At least Type-1 MG shall be covered in Rel-19 timeframe. We don’t have a strong opinion on the WID coverage for second priority MG types, if times allows.

# XR skipping requirements impact

In the last meeting RAN4 agreed to follow the NR-U model for measurement delay calculation.

<Agreement>: - Follow NR-U-like approach for designing the formulas for measurement delay extension.

The follow up was to have some equations for the delay calculation and two options have been retained:

<Way foward>: The purpose of the options provided below is for guidance and not to preclude any other options. - Option 1: RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as - Max(200 ms × CSSFinter, Ceil( 8 * Kgap) × Max(MGRP , SMTC period) × CSSFinter + Nskip × Max(MGRP , SMTC period)), - where Nskip is the number of skipped MG occasions in the measurement period. - Use this formula as example and follow similar approach for the other cases. - Option 2: RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as - Max(200 ms, Ceil(( 8 + Nskip ) * Kgap) Max(MGRP , SMTC period)) CSSFinter - where Nskip is the number of skipped MG occasions in the measurement period. - Use this formula as example and follow similar approach for the other cases.

In our opinion, Option 1 shows a more intuitive and direct way of computing the delay as it adds up the skipped measurements.

Observation 3:  Option 1 offers a more intuitive and direct way of computing the delay as it adds up the skipped measurements.

Proposal 3: Support Option 1:

- RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as

  - Max(200 ms × CSSFinter, Ceil( 8  * Kgap) × Max(MGRP , SMTC period) × CSSFinter + Nskip × Max(MGRP , SMTC period)),

    - where Nskip is the number of skipped MG occasions in the measurement period.

    - Use this formula as example and follow similar approach for the other cases.

Another issue that needs to be resolved is about the conditions for measurement skipping. In the WF, the following options have been retained for further discussions:

- Option 1: conditions are needed - Option 1a: For measurement gap skipping, the max number of gaps skipped during the measurement period should be limited/specified. - Option 1b: The information related to the maximum number of measurement gap occasions allowed for skipping/reallocated to XR over a time period (mentioned in RAN1 LS on UAI) or the maximum time between any consecutive samples can be defined in TS 38.133 - Option 1c: Maximum acceptable time separation between two measurement occasions available for RRM measurements - Option 2: No conditions are needed for measurement skipping regarding maximum number of occasions or MG configuration.

As RAN4 agreed that there will be a measurement delay that will be precisely defined, and the supported gaps are network configured, we believe that there is no need for supplementary conditions/requirements for the time separation between measurements occasions or maximum number of skipped gaps.

Observation 4:  As the measurement delay will be precisely defined, and the supported gaps are network configured, the need for extra conditions is not clear.

Proposal 4: Support Option 2:

Option 2: No conditions are needed for measurement skipping regarding maximum number of occasions or MG configuration.

# Need/feasibility of UE assistance information

The WF mention the following agreement:

<Agreement>: - Agree to define UAI based on the following assumptions, and further discuss the content of UAI - RAN4 will not specify any explicit or implicit requirement or expectation on gNB behaviour in response to any XR UAI (from RAN4 perspective) - Not define UE behaviour and the corresponding requirements based on UAI - Supporting of UAI is optional for UE with optional UE capability.

The above agreement stands as assumption for any further discussion on UAI. Since there is no behavior to be defined for neither base station nor UE, we believe that is difficult to justify UAI introduction.

Observation 5: The UAI agreed assumptions do not justify UAI introduction.

Thus, from the following two groups of proposals listed in the WF:

<Way forward>: - Proposals related to measurement occasions - Option 1: Do not define UAI related to measurement occasions. - Option 2: The UE indicates a pattern of unused gaps - Option 3: The UE indicates a ratio/number of occasions that can be skipped without measurement delay extension - Option 4: The UE indicates NW a status whether it can tolerate measurement cancellation. - Option 5: One of options 2, 3, or 4 but the information is reported as UE capability. - Proposals related to channel conditions - Option 1: Provide information on channel conditions - Option 2: do not provide information on channel conditions

we have the following proposal:

Proposal 5: Do not introduce UAI, and thus:

    - Support Option 1 on the measurement occasions:

        - Option 1: Do not define UAI related to measurement occasions.

Support Option 2 on the channel conditions:

        - Option 2: do not provide information on channel conditions

# additional signaling aspects

In the last meeting, in addition to the dynamic schemes, semi-static schemes were brought back into discussion even though RAN1 already discussed and excluded them one by one.

Here are the options on semi-static schemes retained in the WF:

<Way forward>: - Further discussion based on recommended options - Option 1: Do not consider semi-static solutions - Option 2: RAN4 to further evaluate semi-static skipping indication in addition to DCI-based solution. - Option 2a: RRC pattern-based - Option 2b: MAC CE bitmap for skipping indication - Option 2c: RRC configuration patterns with MAC CE activation - Option 3: Network indicates by RRC which gaps can be skipped via DCI.

We are reiterating our support for the DCI based dynamic scheme and suggest to not consider semi-static schemes in this WI.

Proposal 6: Support Option 1

  - Option 1: Do not consider semi-static solutions

# Skipping indication on frequency domain

In the last meeting, the following proposals were retained for further discussion:

<Way forward>: - Proposals - Proposal 1: For the gap/restriction skipping impact in frequency domain, - For the gap skipping, depend on whether the UE capable of per-FR gap - If capable of per-FR gap, the skipping is only valid for the gap instance in the same FR with scheduled XR transmission. - Otherwise, no need to distinguish different FR. - For the scheduling restriction skipping, reuse the principle defined in existing scheduling restriction to decide whether the intra-band/inter-band/same FR MO(s) would be skipped. - Proposal 2: - RAN4 to investigate scheduling restrictions for CA if DCI skipping indication is sent on one of the CCs. - RAN4 to discuss and clarify whether UE can conduct reception/transmission also on other cells that fall within the applicable range of carriers during the skipped gap/restriction, in addition to the cell to which the skipping was indicated. - Proposal 3 (Huawei): Depending on whether UE is configured with per-FR gap, if UE is configured with per-FR gap, the MG skipping only applies to the same FR and has no impacts on UE behavior on the other FR. If UE is configured with per-UE gap, the MG skipping applies to both FR. - Proposal 4 (vivo): RAN4 agrees to introduce RRC signaling of which gap or both gaps when per-FR gap is configured are to be skipped via further DCI indication.

After seeing the draft CR regarding the DCI gap skipping indication introduction from RAN1(see the Annex of this document for reference), we believe that Proposal 3 is in line with RAN1 understanding.

Proposal 7: Support Proposal 3 from the above list:

Proposal 3: Depending on whether UE is configured with per-FR gap, if UE is configured with per-FR gap, the MG skipping only applies to the same FR and has no impacts on UE behavior on the other FR. If UE is configured with per-UE gap, the MG skipping applies to both FR.

# Conclusions

This contribution discussed our analysis related to measurements occasions skipping for NR XR case. Based on the discussion, the following observations and proposals are made:

Observation 1: If the DCI skipping order applies to the gaps affecting the cells configured in a particular band, then at least inter-band NR-DC with gaps per UE and per FR can be supported.

Proposal 1:  If Inter-band NR-DC gap skipping is to be supported, then the rule of per band applicability for DCI gap skipping order is required and sufficient.

Observation 2:  As Type-1 MG is a mandatory feature for Rel-19, the other types of MG shall be treated as a second priority.

Proposal 2: At least Type-1 MG shall be covered in Rel-19 timeframe. We don’t have a strong opinion on the WID coverage for second priority MG types, if times allows.

Observation 3:  Option 1 offers a more intuitive and direct way of computing the delay as it adds up the skipped measurements.

Proposal 3: Support Option 1:

- RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as

  - Max(200 ms × CSSFinter, Ceil( 8  * Kgap) × Max(MGRP , SMTC period) × CSSFinter + Nskip × Max(MGRP , SMTC period)),

    - where Nskip is the number of skipped MG occasions in the measurement period.

    - Use this formula as example and follow similar approach for the other cases.

Observation 4:  As the measurement delay will be precisely defined, and the supported gaps are network configured, the need for extra conditions is not clear.

Proposal 4: Support Option 2:

Option 2: No conditions are needed for measurement skipping regarding maximum number of occasions or MG configuration.

Observation 5: The UAI agreed assumptions do not justify UAI introduction.

Proposal 5: Do not introduce UAI, and thus:

    - Support Option 1 on the measurement occasions:

        - Option 1: Do not define UAI related to measurement occasions.

Support Option 2 on the channel conditions:

        - Option 2: do not provide information on channel conditions

Proposal 6: Support Option 1

  - Option 1: Do not consider semi-static solutions

Proposal 7: Support Proposal 3 from the above list:

Proposal 3: Depending on whether UE is configured with per-FR gap, if UE is configured with per-FR gap, the MG skipping only applies to the same FR and has no impacts on UE behavior on the other FR. If UE is configured with per-UE gap, the MG skipping applies to both FR.

# References

- R4-2502700 “WF on RRM requirements for XR_Ph3, RAN4 Meeting #114, Athens, GR, February 2025

# Annex

38.213 CR: Introduction of XR (eXtended Reality) for NR Phase 3

## *** Unchanged parts are omitted ***

## 10.6	Indication for cancelation of RRM measurement gaps

A UE can be indicated by a value ‘1’ of an XYZ field in a DCI format [5, TS 38.212] provided by a PDCCH reception to cancel an earliest RRM measurement gap occasion where the UE would perform RRM measurements on all cells in the applicable range of the measurement gap occasion that include the one or more scheduled cells associated with the DCI format [10, TS 38.133]. The earliest RRM measurement gap occasion is at least 3 msec or 5 msec, as determined by a reported UE capability, after the end of the PDCCH reception.

If a UE indicates “Value 1” for FG-ABC, the UE ignores a value ‘0’ of the XYZ field in a DCI format.

If a UE indicates “Value 2” for FG-ABC, a value ‘0’ of the XYZ field in a DCI format provided by a PDCCH reception indicates to the UE that the UE behavior for the earliest RRM measurement gap occasion after the PDCCH reception where the UE would perform RRM measurements on cells in a same frequency band as the one or more scheduled cells associated with the DCI format is as described in [10, TS 38.133]. When the UE detects more than one DCI formats that include the XYZ field and are associated with a same earliest RRM measurement gap occasion where the UE would perform RRM measurements on cells in a same frequency band as the one or more scheduled cells associated with the more than one DCI formats, the UE applies the indication provided by the XYZ field of a last DCI format, from the more than one DCI formats, that is in a PDCCH reception that ends at least 3 msec or 5 msec, as determined by a reported UE capability, before the start of that earliest RRM measurement gap occasion.

## *** Unchanged parts are omitted ***