3GPP TSG-RAN4 Meeting #114bis		R4-2503253

# Wuhan, China 7th – 11th April, 2025

Source: 	Qualcomm Incorporated

Title: 	Generalization Aspects of AI-ML

Agenda item:	7.19.1

Document for:	Discussion

# Introduction

This paper focuses on the generalization aspects of AI-ML.

RAN4 made the following agreements several meetings ago [1]

Issue 1-3-2: Delay requirements Agreement: - The discussion should consider use-cases specific aspects Agreement: - Whether transition from AI to non-AI would be UE initiated or network initiated - This will be a decision for other groups - RAN4 to assess what RRM requirements are needed for each case.

Following relevant proposals came up during the last meeting:

- Define functionality activation delay, at least for beam management - Consider transition delay for AI to non-AI switching, and for AI deactivation

# Discussion

RAN4 discussed defining three types of delays for Rel-19 AI-ML use cases.

- Activation delay

- Deactivation delay

- Switching delay between two AI-ML functionalities

As the agreement of [1] showed, the discussion of above delay requirements should be case specific. Hence, we discuss these delay requirements on a case-by-case basis.

Figure 1: Steps for configuration and activation/deactivation of AI-ML functionalities

## Delay Requirements for CSI

Legacy non-AI-ML CSI measurement requirements were discussed in RAN1 and captured in RAN1 spec. That trend should follow to AI-ML based CSI prediction, too. RAN1 is the right place to discuss AI-ML based CSI activation and switching requirements.

Observation 1: Legacy non-AI-ML CSI measurement requirements were discussed in RAN1 and entirely captured in RAN1 spec.

Proposal 1: RAN4 should not define AI-ML based CSI activation/switching delay requirements and let RAN1/RAN2 to define it.

## Delay Requirements for Positioning

3GPP considered three UE sided AI-ML positioning cases in Rel-19. They are: 1) Case 1, 2) Case 2a and 3) Case 2b. During the last RAN plenary, Case 2a and 2b got excluded from the updated WID of AI-ML [3]. No RAN4 requirement has yet been agreed for Case 1.

RAN4 should wait to see the progress of case 1 before discussing activating and switching requirements of UE sided AI-ML based positioning.

Observation 2: Two of three UE sided AI-ML positioning use cases (case 2a/2b) have been removed from the AI-ML air-interface WID during the last RAN plenary.

Proposal 2: RAN4 should wait to see the progress of case 1 before discussing activating and switching requirements of UE sided AI-ML based positioning.

## Delay Requirements for AI-ML Beam Management

Legacy non-AI-ML based L1-RSRP reporting delay requirements were defined in RAN4. Hence, RAN4 is the appropriate arena to define delay requirements for AI-ML based beam prediction.

RAN2 sent an LS to RAN1 several meetings ago that shows the required steps to activate/deactivate AI-ML functionalities for inference/monitoring. These steps are mentioned in Figure 1.

The starting time to activate periodic AI-ML based beam management resources are not still finalized in RAN1/RAN2. For example, network can configure with periodic CSI-RS configurations as setB of a particular functionality via step 3 of Figure 1. However, UE can later indicate via step 4 that it cannot support this functionality. Based on UE report in step 4, network will deactivate that particular CSI-RS configuration in step 5. From that perspective, it is not clear where activation timelines for periodic AI-ML BM resources start from. Similar issues may arise for semi-persistent resources. RAN4 should wait until the starting time of periodic and semi-persistent setB resources for AI-ML based beam management are finalized in RAN1.

Figure 2: Activation delay timeline of aperiodic setB resources for AI-ML based beam management

The activation timeline of aperiodic setB resources for AI-ML based beam management is relatively clear. These aperiodic setB resources for AI-ML based beam prediction can only be activated after step 5 of Figure 1 is concluded. These aperiodic resources for setB would be configured by network. Network will also specify the uplink resources where UE is expected to convey inferred properties of setA. Figure 2 shows the details of this timeline. RAN4 can consider defining activation/switching delay timelines in this scenario.

Observation 3: The starting time to activate periodic AI-ML based beam management resources are not still finalized in RAN1/RAN2.

Proposal 3: RAN4 should wait until the starting time of periodic and semi-persistent setB resources for AI-ML based beam management are finalized in RAN1.

Proposal 4: RAN4 considers defining the delay requirements to activate a functionality in AI-ML beam management where setB resources are aperiodic.

- Note: The timeline starts when UE receives the network command that triggers the of setB resources. The timeline ends when UE is expected to convey the inferred properties of setA resources via uplink.

During the last meeting, several companies propose to define deactivation delay requirements for AI-ML based beam management. The end point of deactivating AI-ML based beam management is not clear. The steps to fall back to non-AI-ML beam management after deactivation requires additional configurations and transmissions of network (e.g., configuration of setA resources). It would be difficult to determine delay requirements that encapsulates all these steps.

Observation 4: The end point of deactivating AI-ML based beam management is not clear. The steps to fall back to non-AI-ML beam management after deactivation requires additional configurations and transmissions of network (e.g., configuration of setA resources).

Proposal 5: RAN4 does not define deactivation delay requirements for AI-ML based beam management.

# Conclusions

Observation 1: Legacy non-AI-ML CSI measurement requirements were discussed in RAN1 and entirely captured in RAN1 spec.

Observation 2: Two of three UE sided AI-ML positioning use cases (case 2a/2b) have been removed from the AI-ML air-interface WID during the last RAN plenary.

Observation 3: The starting time to activate periodic AI-ML based beam management resources are not still finalized in RAN1/RAN2.

Observation 4: The end point of deactivating AI-ML based beam management is not clear. The steps to fall back to non-AI-ML beam management after deactivation requires additional configurations and transmissions of network (e.g., configuration of setA resources).

Proposal 1: RAN4 should not define AI-ML based CSI activation/switching delay requirements and let RAN1/RAN2 to define it.

Proposal 2: RAN4 should wait to see the progress of case 1 before discussing activating and switching requirements of UE sided AI-ML based positioning.

Proposal 3: RAN4 should wait until the starting time of periodic and semi-persistent setB resources for AI-ML based beam management are finalized in RAN1.

Proposal 4: RAN4 considers defining the delay requirements to activate a functionality in AI-ML beam management where setB resources are aperiodic.

- Note: The timeline starts when UE receives the network command that triggers the of setB resources. The timeline ends when UE is expected to convey the inferred properties of setA resources via uplink.

Proposal 5: RAN4 does not define deactivation delay requirements for AI-ML based beam management.

# References

- RAN4 Chair’s Notes, 3GPP TSG RAN WG4 #113.

- R1-2407604, LS on applicable functionality reporting for beam management UE-sided model

- RP-250792, “Revised WID for NR_AIML_air”, 3GPP TSG RAN meeting #107.