# 3GPP TSG-RAN WG4 Meeting #114-bis                                                          R4-2503279

Wuhan, China, 7-11 April, 2025

Title: 	MPR evaluations for contiguous intra-band CA with NC allocation for PC1.5 TxDiv

Source: 	Huawei, HiSilicon

Agenda item:	7.1.1.1.2

Document for:	Discussion

# Introduction

In RAN4 114 meeting the WF[1] was agreed. One of the open issues to be addressed in this meeting was the PC1.5 MPR evaluations for intra-band contiguous ULCA with non-contiguous RB allocation (with TxD). In the following section our evaluations are provided.

# Discussion

The PA was calibrated based on the following assumptions:

- PC1.5 (2Tx: 26dBm+26dBm, 1LO)

- 1dB MPR: DFT-s-OFDM QPSK 20MHz, 100RB

- Carrier Leakage: 28dBc, Image: 28dBc, CIM3: 60dBc, CIM5: 70dBc

## MPR Evaluations for PC1.5 2Tx with an intra-band contiguous CA configuration: Non-contiguous allocations

We did MPR evaluations for the cases of 100MHz+100MHz (class C), with equal PSD. The details of MPR evaluations can be found in Annex 1, which include the revise-IMD and implementation margin. we propose the following MPR values:

Table 1: Non-contiguous RB allocation for Power Class 1.5 with dual Tx for handheld UE

| Modulation |  | MPR for bandwidth class C(dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Inner | Outer 1 | Outer2 |
| DFT-s-OFDM | QPSK | 4.2 | 10.5 | 17.3 |
|  | 16QAM | 4.3 |  |  |
|  | 64QAM | 5.7 |  |  |
|  | 256QAM | 8.8 |  |  |
| CP-OFDM | QPSK | 4.5 | 11 | 17.6 |
|  | 16QAM | 4.5 |  |  |
|  | 64QAM | 6.6 |  |  |
|  | 256QAM | 10.7 |  |  |

For FWA UEs our evaluations show that 0.5dB MPR reduction is feasible, as shown in Table 2.

Table 2: Non-contiguous RB allocation for Power Class 1.5 with dual Tx for FWA UE

| Modulation |  | MPR for bandwidth class C(dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Inner | Outer 1 | Outer2 |
| DFT-s-OFDM | QPSK | 3.7 | 10 | 16.8 |
|  | 16QAM | 3.8 |  |  |
|  | 64QAM | 5.2 |  |  |
|  | 256QAM | 8.3 |  |  |
| CP-OFDM | QPSK | 4 | 10.5 | 17.1 |
|  | 16QAM | 4 |  |  |
|  | 64QAM | 6.1 |  |  |
|  | 256QAM | 10.2 |  |  |

Proposal 1: Consider MPR evaluations of Table 1 and Table 2 for intra-band contiguous NR CA with non-contiguous RB allocation for PC 1.5 with dual Tx for handheld and FWA UE, respectively.

# Conclusions

Proposal 1: Consider MPR evaluations of Table 1 for intra-band contiguous NR CA with non-contiguous RB allocation for PC 1.5 with dual Tx.

# Reference

[1]	“R4-2502863, WF on HPUE for CA in TN, Samsung.”

# Annex 1

The presented data for handheld UE includes the reverse-IMD and Implementation margin.