# 3GPP TSG-RAN WG4 Meeting #114bis 	                                R4-2503285

Wuhan, China, 07th – 11th April, 2025

Source:                Tejas Networks Pvt. Ltd.

Title:                     Discussion on SBFD General aspects

Agenda Item:       7.23.1

Document for:     Discussion

# Introduction

This contribution presents our analysis of general issues related to SBFD, including DL/UL subband configurations.

# Discussion

## DL/UL subband configurations support for SBFD

In previous RAN4 discussion, specifically, in RAN4#113 [1], and RAN4#114 [2] the following agreements are achieved:

Agreement: RAN4#113 - For Rel-19, option 2b is adopted, with the following restrictions on SBFD configuration: - For DUD case, the UL subband should be placed in the middle of channel bandwidth, i.e., symmetric DUD - DL subband 1 and DL subband 2 shall have the same size or different sizes by only 1RB - For a specific channel BW supported for SBFD, maximum [2 or 3] specific UL subband sizes are allowed to be declared and shall be selected from the channel bandwidth sizes from Table 5.3.2-1 in TS38.104. - FFS different sizes for specific UL subband for DUD and DU/UD. - No further restriction is needed on the range of the ratio of the UL subband size relative to the channel bandwidth. - This restriction shall be applied to FR1 WA BS only, as the starting point - FFS the restriction is also applied to FR1 MR BS - For a specific channel BW and a specific UL subband size supported, only one DL subband size is allowed to be declared and tested for DU/UD case, and only one DL subband size is allowed to be declared and tested for DUD case - This restriction shall be applied to FR1 WA BS only, as the starting point.

RAN4#114 No new agreement achieved. to FR1 MR BS

In RAN4, it was decided that only DUD and DU patterns are permitted for sub-band configurations. The DU configuration has a single guard band between the DL and UL subbands, whereas the DUD configuration includes two guard bands. The total bandwidth occupied by these guard bands reduces the available transmission bandwidth. This reduction percentage should be considered when determining the minimum channel bandwidth for SBFD operating bands. In the DUD ({D, U, D}) case, where the UL subband is placed in the middle of the channel bandwidth—for instance, with a {40 MHz, 20 MHz, 40 MHz} distribution—there is an additional 40 MHz protection against leakage from adjacent channel CLI. Since SBFD symbols represented by {D, U, D} are susceptible to adjacent channel gNB-gNB CLI, UL subbands may experience interference from DL slots of legacy TDD transmissions from neighbouring gNBs that overlap with SBFD symbols.

For example, assuming a 100 MHz channel bandwidth with a [D, U, D] configuration:

- The effective bandwidth will be 98.28 MHz, corresponding to 273 RBs.

- With a guard band of 3.6 MHz (1.8 MHz * 2), five RBs will be required.

- This results in an approximate 3.7% transmission bandwidth reduction due to the SBFD inter-subband guard band.

- An optimal reduction value of approximately 3–4% can be considered.

Furthermore, when defining RF requirements for UL/DL subband configurations, a configuration-agnostic approach can be adopted. In this approach, RF requirements are specified independently of the subband configuration, allowing supported configurations to be declared without restricting SBFD deployment to a predefined set of configurations. From Table 5.3.2-1 in TS 38.104, initial selections can be made for 50 MHz and 100 MHz bandwidth cases. Specific UL subband sizes can be declared, provided they align with the channel bandwidth sizes listed in Table 5.3.2-1. For declaration and testing, DUD and DU/UD configurations can be defined using the following table. In the DUD case, DL subband 1 and DL subband 2 must either have: The same size, or a difference of only 1 RB, where DL1 may be 1 RB higher or lower compared to DL2.

Table 1: Sizes for specific UL subband for DUD and DU/UD.

|  | NRB, SBFD, DL | NRB, SBFD, UL | NRB, SBFD, GB |
| --- | --- | --- | --- |
| DU/UD @ SCS 30 kHz) 50 MHz 100 MHz | 104 | 24 | 5 |
|  | 217 | 51 | 5 |
| DUD (SCS 30 kHz) 50 MHz 100 MHz | NRB, SBFD, DL1 =50 NRB, SBFD, DL2 =49 | 24 | 5 |
|  | 106 | 51 | 5 |

Proposal 1: For 50 MHz and 100 MHz bandwidth cases, UL subband sizes must conform to those specified in Table 5.3.2-1 of TS 38.104. In DUD configurations, the two downlink subbands （DL1 and DL2） must either be equal in size or differ by no more than 1 RB.

Proposal 2: For FR1 WA BS, the declared DL subband sizes supporting SBFD shall be tested, while no restrictions should apply to other FR1 BS classes or FR2 BS.

# Conclusion

In this contribution, we discuss about SBFD general aspects with the following proposals and observations.

Proposal 1: For 50 MHz and 100 MHz bandwidth cases, UL subband sizes must conform to those specified in Table 5.3.2-1 of TS 38.104. In DUD configurations, the two downlink subbands (DL1 and DL2) must either be equal in size or differ by no more than 1 RB.

Proposal 2: For FR1 WA BS, the declared DL subband sizes supporting SBFD shall be tested, while no restrictions should apply to other FR1 BS classes or FR2 BS.

# Reference

- R4-2419907: Way forward for [113][306] NR_duplex_evo_General, Samsung.

- R4-2502310: Way forward for [114][306] NR_duplex_evo_General, Samsung.

- R4-2416134: SBFD general aspects, Ericsson