3GPP TSG-RAN WG4 Meeting #114bis	R4-2503292

# Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda item:	7.24.3

Source: 	vivo

Title: 	Initial discussion on RRM impacts for R19 A-IoT device

Document for:	Discussion

- Introduction

For the Ambient IoT, the study item [1] was completed and the corresponding WI was approved in [2] and further updated in [3]. The RRM discussion is scheduled to start from RAN4#114bis meeting. In this contribution, we provide our initial considerations on potential RRM impacts for Ambient-IoT.

- Discussion

The RRM scope in [3] is copied below for reference:

- RAN4 scope: - Specify RF requirements for Ambient-IoT BS, device 1, and CW - RF requirements for Type 1-C Ambient-IoT BS - RF requirements for device 1 - RF requirements for CW - Specify RRM core requirements for device 1, if necessary - Study and develop OTA test methodology for A-IoT device 1 - Consider test methods specified in TR 38.870 as starting point. Take test system reuse, test system complexity and test time into account, when developing test methods suitable for Ambient IoT. - Develop the preliminary Measurement Uncertainty (MU) assessment for the test system - Use band n8 as an example band

The target device and deployment scenario are included in the general scope of the WID and copied below where the device type to be specified and deployment scenario to be considered is described:

General Scope The definitions provided in TR 38.848, TR 38.769, and decisions, etc. made during the Rel-19 SI in RAN WGs are taken into this WI, and the following is the exclusive general scope: - The overall objective shall be to standardize the following Ambient IoT device: Device 1: ~1 µW peak power consumption, has energy storage, RF envelope detector receiver, initial sampling frequency offset (SFO) up to 10X ppm, neither R2D nor D2R amplification in the device. The device’s D2R transmission is backscattered on a carrier wave provided externally. - Deployment scenario 1 with Topology 1, according to D1T1-B. - FR1 licensed spectrum in FDD, with R2D in DL spectrum and D2R and CW in UL spectrum. - Spectrum deployment in-band to NR and standalone, with A-IoT BS located indoor. - Traffic types DO-DTT, DT, for rUC1 (indoor inventory) and rUC4 (indoor command). - Carrier wave transmission for waveform 1 only, without hopping, per the following cases in TR 38.769: - Case 1-4 for D1T1-B - Proximity determination via Solution 1 in TR 38.769 only. - Device (un)availability via Direction 1 in TR 38.769 only.

Checking RAN1 scope [3], it can be found that the only supported physical channels are PRDCH and PDRCH.

Checking RAN2 scope, the only procedures are A-IoT Paging, A-IoT Random access and A-IoT data transmission and only MAC layer is included.

- RAN2 scope:

  - Specify the necessary functions and procedures for an Ambient IoT compact protocol stack and lightweight signalling procedure to enable DO-DTT and DT data transmission:

    - A-IoT Paging, including subsequent paging for the same service. Support the options that a paging message contains one identifier, and that a paging message contains no identifier.

Note: RAN2 aims to design a paging message format such that multiple identifiers can be contained in one paging message, for forward compatibility purposes.

    - A-IoT Random access, including re-access for failure handling. Contention-based and contention-free cases are supported. For the contention-based random access, only Solution 1 (3-step only) is included.

    - A-IoT data transmission, including data (re-)transmission for failure handling. Segmentation is supported at least in D2R.

    - Only MAC layer is included

Considering the above scope, due to the simplicity of the target device, its corresponding physical layer and signalling are significantly simplified and most RRM requirements defined for a normal NR device will not be applicable.

Observation 1: Most RRM requirements defined for a normal NR UE will not be applicable for A-IoT WI.

Considering the A-IOT random access to be defined, there may be RRM impact with the considering that the random access procedure could also be a simplified one. For the other procedure with potential RRM impact, RAN4 could wait for more RAN1/2 progress.

Proposal 1: RAN4 study potential RRM requirements for A-IoT random access procedure and wait for more RAN1/2 progress on other procedures with potential RRM impact.

- Conclusions

In this contribution we provide our analysis for A-IoT WI and have the following observations and proposals:

Observation 1: Most RRM requirements defined for a normal NR UE will not be applicable for A-IoT WI.

Proposal 1: RAN4 study potential RRM requirements for A-IoT random access procedure and wait for more RAN1/2 progress on other procedures with potential RRM impact.

References

[1] RP‑234058 New SID: Study on solutions for Ambient IoT (Internet of Things) in NR

[2] RP-243326 New Work Item: Solutions for Ambient IoT (Internet of Things) in NR

[3] RP-250796 New Work Item: Solutions for Ambient IoT (Internet of Things) in NR