# 3GPP TSG-RAN WG4 Meeting #114bis	R4-2503305

# Wuhan, Hubei, China, 07th – 11th April, 2025

Source: 	Xiaomi

Title: 	Discussion on other RRM core requirements in NTN for NR Phase 3

Agenda Item:	7.29.4.2

Document for:	Discussion

# 1Introduction

In the last RAN4#113 meeting, discussion on RRM requirements for Rel-19 NR NTN phase3 occurred and a WF was approved as detailed in [1]. In this contribution, we would like to provide our further discussion on other RRM core requirements in NTN for NR Phase 3 and provide our proposals.

- Discussion

###### Issue 1-1: Cell Reselection Further discussion: - In cell reselection requirements, discuss whether any update is needed for case where SSB periodicity is larger than 20 ms: - Option 1 (Samsung): FFS - Option 2 (vivo): No update is needed ###### Issue 1-2-A: RRC Re-establishment (default TSMTC,i) Agreement (online): - In RRC re-establishment requirements, update is needed to the TSMTC,i value for case where the SMTC for the inter-frequency carrier is not provided. Further discussion: - In RRC re-establishment requirements, update the value of TSMTC,i to 160ms for case where the SMTC for the inter-frequency carrier is not provided. - FFS: whether the above will apply only for UEs capable of coverage enhancement feature, if such a UE capability is defined. - FFS: whether the above is for only inter-frequency or for both intra-frequency and inter-frequency ###### Issue 1-2-B: RRC Re-establishment (SSB Ês/Iot < -8dB and SSB periodicity >20 ms) Further discussion: - In RRC re-establishment requirements, discuss whether any update is needed to the TSMTC,i value for case where the SMTC for the inter-frequency carrier is not provided: - Option 1: Yes, an update is needed - Note: With the understanding that the by-default SSB periodicity for initial access is 160ms for Rel-19, from RRM requirement perspective. - Companies are encouraged to provide details on how to update the specification. - Other option is not precluded. ###### Issue 1-5: Paging Further discussion: - Further discuss whether any update is needed to paging interruption requirements for NGSO scenarios when the SSB periods is equal to 160 ms: - Option 1 (Huawei, vivo): no update is needed - Option 2 (Nokia): reduce the multiplying factor K in 4.2C.2.5

- Cell re-selection:

- Current cell re-selection measurement requirements are defined based on DRX cycle, of which the smallest value is 320ms. Originally, the SSB periodicity can be configured to up to 160ms, and the current cell re-selection requirements work. So, we don’t think there is any impact on the cell re-selection.

- Proposal 1: In cell reselection requirements, no update is needed.

- Paging interruption in RRC Idle/Inactive:

- Similar as the opinion on cell re-selection, we don’t think there is any impact on the paging interruption requirements.

- Proposal 2: In paging interruption requirements, no update is needed.

- RRC Re-establishment:

The current requirements involve description of UE’s assumption on SSB periodicity. As the default SSB periodicity can be up to 160ms, the related requirements need to be revisited. The legacy Re-establishment delay requirement include the components of the target intra-frequency NR cell identification time (Tidentify_intra_NR) and the target inter-frequency NR cell identification time (Tidentify_inter_NR,i). To derive the NR cell identification

- When serving cell SSB Ês/Iot≥-8, the NR cell identification time for drive from the SMTC periodicity. For inter-frequency case, the legacy requirements clearly clarify that if the periodicity of the SMTC occasion configured for the inter-frequency carrier i is not configured, the UE may assume that the target SSB periodicity is no larger than 20 ms. In this case, it is straightforward to modify the UE’s assumption on SSB periodicity to no larger than 160 ms.

- When serving cell SSB Ês/Iot＜-8, there is a Note saying “The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC,i > 20 ms and serving cell SSB Ês/Iot < -8 dB”. To align with the SSB periodicity extension, we understand that the corresponding extended SMTC periodicity should be configured. Then, we think the existing note can still work for the case SSB periodicity up to 160ms. We prefer not to update RRC re-establishment requirements under serving cell SSB Ês/Iot＜-8 case.

Proposal 3: In RRC re-establishment requirements,

- When Serving cell SSB Ês/Iot≥-8, update the value of TSMTC,i to 160ms for case where the SMTC for the inter-frequency carrier is not provided in NR-NTN downlink coverage enhancement scenario；

- When Serving cell SSB Ês/Iot＜-8, no update needed.

- Conclusion

- Proposal 1: In cell reselection requirements, no update is needed.

- Proposal 2: In paging interruption requirements, no update is needed.

Proposal 3: In RRC re-establishment requirements,

- When Serving cell SSB Ês/Iot≥-8, update the value of TSMTC,i to 160ms for case where the SMTC for the inter-frequency carrier is not provided in NR-NTN downlink coverage enhancement scenario；

- When Serving cell SSB Ês/Iot＜-8, no update needed.

- Reference

- R4-2502605, WF on RRM requirements for NR_NTN_Ph3_Part2, Qualcomm