3GPP TSG-RAN WG4 Meeting #114bis																		R4-2503314

Wuhan, Hubei, China, 07th – 11th April, 2025

Source: 	Xiaomi

Title: 	Discussion on AIoT system parameters

Agenda item:	7.24.1

Document for:	Discussion

- Introduction

This contribution provides our further analysis for the remaining issues of AIoT system parameters according to the WF [1] in last meeting.

- Discussion

  - R2D bandwidth

For R2D bandwidth, last RAN4 meeting status is as following,

Issue 2-1-4: Values of R2D bandwidth

Agreement:

- Define R2D transmission bandwidth configuration:

  - 1PRB, 2PRB, 3PRB, 4PRB with 15KHz SCS

  - FFS on other RB numbers

- Further discuss the R2D channel bandwidth considering additional guard band.

- If RAN1 has the agreement different than above, RAN4 will revisit the agreements

For the RB number, RAN1#120 agreements are as following,

Agreement

RAN1 does not specify the exact bandwidth a reader shall use for Btx,R2D.

- For R2D, specify the minimum Btx,R2D # of PRBs associated to each M value.

- For R2D, whether to specify the maximum Btx,R2D # of PRBs is up to RAN4

- Note: Reader can transmit any Btx,R2D subject to this minimum Btx,R2D and the agreed 15 kHz subcarrier spacing, and subject to other limitations that are up to RAN4.

Agreement

For R2D, for the OOK-4 modulation for M-chip per OFDM symbol transmission:

- The maximum M value is no less than 16, and to be down-selected from 32, 24 or 16 at RAN1#120bis.

According to the latest agreement in RAN1, no more PRB choices will be introduced besides 1, 2, 3, 4 PRBs. Although RAN1 agreed that more PRBs is allowed for the associate M value, RAN4 can define maximum Btx,R2D, our understanding is that RAN4 only need to define the RF requirements for minimum Btx,R2D # of PRBs associated to the M values, maximum Btx,R2D can be left to implementation and are not defined in RAN4 spec.

Proposal 1: RAN4 only define the RF requirements for minimum Btx,R2D # of PRBs associated to the M values, no maximum Btx,R2D # of PRBs are defined in RAN4 spec.

The remaining issue for R2D bandwidth is the guard band. The DFT-S-OFDM symbol generation procedure is the same with NR, it can be estimated the spectrum is similar to NR waveform. The guard band width depends on the BS Tx performance and the OBUE/ACLR requirements. It can be estimated that time domain windowing is still needed for OFDM symbol to improve the spectral characteristics.

Observation 1: R2D signal spectral characteristics is similar to NR waveform. Time domain windowing is still needed for to meet the OBUE and ACLR requirement if RF requirements similar to NR are reused.

For the size of the guard band, it should be decided considering the implementation complexity and the reader Tx OBUE/ACLR requirements.

2.2 D2R bandwidth

For D2R bandwidth, last RAN4 meeting status is as following,

Issue 2-1-6: D2R bandwidth

Agreement:

- D2R channel bandwidth for A-IoT device

  - Define D2R channel bandwidth for A-IoT device considering the following aspects:

    - [1SB or] 2SB

    - D2R maximum data rate (depending on RAN1 conclusion)

    - SFO

    - FDMA, i.e., considering the different device 1 has different frequency shift

    - Reference Filter performance

    - Other aspects are not precluded

RAN1#120 agreements related to D2R bandwidth are as following,

Agreement

For D2R, only 2SB (Double sideband) modulation is supported.

Agreement

For D2R transmission with small frequency shift +/- R/Tb Hz, where time duration Tb corresponding to a bit (that is after FEC if applied) and R = Tb/(2 × D2R chip length):

- the transmitted 2R chips for bit 1 and bit 0 are [0 1 0 1 …] and [1 0 1 0 …], respectively, for OOK modulation

- the transmitted 2R chips for bit 1 and bit 0 are [-1 +1 -1 +1 …] and [+1 -1 +1 -1 …], respectively, for BPSK modulation.

- Note: The case of R=1 is equivalent to using a Manchester line code without repeating each Manchester codeword.

Agreement

The following is supported for indicating the D2R chip duration:

- The D2R chip duration is indicated in R2D control information from predefined a set of D2R chip duration values

From the above agreement, we can see that DSB not SSB is supported in R19 device 1. The small frequency shift method is decided in RAN1 then spectrum with small frequency shift can be analysed in RAN4 now. The chip rate is still under discussion in RAN1.

For SSB or DSB, there was a proposal in last RAN4 meeting that although device 1 backscattering signal is DSB, D2R bandwidth may need to consider SSB because of SFO. We don’t think this proposal is technically correct. The requirement should be defined according to the spectrum of device Tx signal. If the SFO is 10^5 ppm, the sampling period becomes T±0.1T. The spectrum of real signal (DSB) is symmetric as shown in Figure 1. The SFO impact to SSB signal is shown in Figure 2 that the CBW uncertainty according to SFO is the same with DSB signal, the only difference is that SSB signal spectrum only includes one side band.

Figure 1: Spectrum of DSB signal, left: without frequency shift, right: with frequency shift f1

Figure 2: Spectrum of SSB signal, left: without frequency shift, right: with frequency shift f1

The simulated device DSB output signals for with/without small frequency shift are shown in Figure 3 and Figure 4.

Figure 3: D2R signal spectrum with R=1

Figure 3: D2R signal spectrum with R=4

From the above analysis, we have the following proposal,

Proposal 2: D2R bandwidth is defined according to DSB signal not SSB signal.

For the maximum data rate, it’s not decided in RAN1 yet. RAN4 can begin the discussion with some assumption for the general methodology. For example, if 1 kbps data rate is assumed, chip rate can be assumed 7.5kcps. The final CBW decision can be updated according to RAN1 decision.

Proposal 3: RAN4 begin the CBW discussion with one specific chip rate assumption, for example 7.5kcps, for the general methodology. The final CBW decision can be updated according to RAN1 decision.

For the impact of SFO, our understanding is that it brings difference between the actual device CBW and the nominal CBW from system aspect. In the system parameter definition, nominal CBW is defined without consideration of SFO. But the RF requirement related to CBW should consider the impacts if any. Like the center frequency, the theorical frequency is defined without the consideration the frequency error, phase noise, etc. But the RF requirement related to the non-ideal frequency is reflected in spec.

Proposal 4: Nominal channel bandwidth is defined without consideration of SFO. The RF requirements can be defined considering SFO impact.

The RF requirements related to SFO may be guard band, spectrum emission, etc. It can be discussed further.

For the aspect of FDMA, our understanding is that small frequency shift needn’t to be considered for D2R CBW from device perspective because device CBW is not changed for small frequency shift mode looking at Figure 1. However, it may need to discuss if D2R CBW at reader side need to be defined, if it will be defined, FDMA should be taken into account.

Proposal 5: FDMA is not considered for D2R CBW from device side.

Proposal 6: RAN4 should decide if D2R CBW from reader side needs be defined to consider FDMA.

  - Deployment scenario

There was an open issue in last RAN4 meeting as following,

Issue 2-2-2: Deployment scenario

Agreement:

- To specify RAN4 RF requirements based on the assumption that NR UE is only outdoor in Rel-19.

  - FFS on whether and how to capture the side condition of outdoor UE assumption in the specifications

For this issue, we slightly prefer not to capture the outdoor NR UE assumption in RAN4 spec. The requirement for AIoT is defined for AIoT readers and devices, not NR UE. The co-existence study is the assumption for all of the Wis in standard discussion, it can be captured in TR or WF, no need to be captured in the requirement specs.

Proposal 7: No need to capture the NR UE outdoor assumption in RAN4 TS specifications.

- Conclusion

This contribution provides our views on the remaining issues of AIoT system parameters. We have the following proposals and observations.

For R2D bandwidth,

Proposal 1: RAN4 only define the RF requirements for minimum Btx,R2D # of PRBs associated to the M values, no maximum Btx,R2D # of PRBs are defined in RAN4 spec.

Observation 1: R2D signal spectral characteristics is similar to NR waveform. Time domain windowing is still needed for to meet the OBUE and ACLR requirement if RF requirements similar to NR are reused.

For D2R bandwidth,

Proposal 2: D2R bandwidth is defined according to DSB signal not SSB signal.

Proposal 3: RAN4 begin the CBW discussion with one chip rate assumption, for example 7.5kcps, for the general methodology. The final CBW decision can be updated according to RAN1 decision.

Proposal 4: Nominal channel bandwidth is defined without consideration of SFO. The RF requirements can be defined considering SFO impact.

Proposal 5: FDMA is not considered for D2R CBW from device side.

Proposal 6: RAN4 should decide if D2R CBW from reader side needs be defined to consider FDMA function.

For deployment scenario,

Proposal 7: No need to capture the NR UE outdoor assumption in RAN4 TS specifications.

- References

[1] R4-2502858, “WF on requirements for A-IoT device”, CMCC, RAN#114.