3GPP TSG-RAN WG4 Meeting #114bis									R4-2503330

Wuhan, China, 7th – 11th April, 2025

Agenda item:	7.10.3.2

Source: 	Xiaomi

Title: 	Discussion on UE RF requirements for NTN less than 5MHz

Document for:	Discussion

Introduction

In RAN4#114 meeting, RAN4 discussed UE RF requirements for NTN less than 5MHz, and the related WF was approved in [1]. This contribution discusses the remaining issues for NTN less than 5MHz and provides our proposals.

Discussion

Issue 2-2-1: Reference sensitivity - Agreement - For band n254, [2.2] dB improvement compared with 5 MHz CBW; - For band n256, [1.7]dB improvement compared with 5 MHz CBW; - FFS for band n255

In last RAN4 meeting, the reference sensitivity requirements for NTN 3MHz CBW for band n254, n255 and n256 were discussed. In theory, for 3MHz channel bandwidth with 15 PRBs, the total received power is reduced by 2.2dB compared with 5MHz channel bandwidth. It is feasible for n254 due to larger Tx/Rx separation. However, for n255 and n256, as the Tx/Tx separation is narrow, the maximum input power and in-band blocking interferer signal level are reduced to -40 dBm, the Rx IIP2 effects needs to be considered. In addition, the Tx/Rx separation of n255 is narrower than that of n256. Thus, we support to 1.7dB improvement for both n255 and n256.

Proposal 1: For reference sensitivity requirement of 3MHz CBW, RAN4 to define the following requirements for band n256, n255 and n254:

- For band n254, 2.2 dB improvement compared with 5 MHz CBW;

- For band n255 and n256, 1.7dB improvement compared with 5 MHz CBW;

Conclusion

In this contribution, we discussed the remaining issue for UE RF requirements for NTN less than 5MHz and provides our proposals:

Proposal 1: For reference sensitivity requirement of 3MHz CBW, RAN4 to define the following requirements for band n256, n255 and n254:

- For band n254, 2.2 dB improvement compared with 5 MHz CBW;

- For band n255 and n256, 1.7dB improvement compared with 5 MHz CBW;

- Reference

[1]	R4-2502282	WF for [113][311] NR_IoT_NTN_less_than_5MHz_UERF		Xiaomi