# 3GPP TSG-RAN WG4 Meeting #114bis	R4-2503344

# Wuhan, Hubei, China, 07th – 11th April, 2025

Title: 	On RRM requirements of SSB adaptation

Source: 	OPPO

Agenda item:	7.25.2.2

Document for:	Discussion

Introduction

In last meeting, a WF was approved for SSB adaptation [1]. RAN2 sent LS reply [2] to RAN4 on SMTC impact due to SSB adaptation.

RAN2 thanks RAN4 for the LS in R4-2416911 on SSB adaptation. Regarding the question of SMTC impacts due to SSB adaptation Agreement on SSB adaptation: 1. RAN2 preference is to keep SMTC based L3 RRM framework and to introduce additional SMTC configuration according to SSB adaptation for L3 RRM measurement on SCell with SSB adaptation. 2. For L3 measurement, RAN2 assumes the adapted SSB on neighbor cell is measured based on legacy SMTC. ACTION: RAN2 respectfully asks RAN4 to take above agreements into account for RAN4’s future design and provide feedback if any concern.

In this contribution, we will provide discussion on RRM requirements for SSB adaptation.

Discussion

RAN4 has agreed to discuss L1/L3 measurement requirement impact as starting point [1]. There are different kinds of measurement to be considered by Rel-18, including L1 serving cell measurement, L1 neighbor cell measurement and L3 neighbor cell measurement.

Issue 1-3-1: L3 requirements for SSB adaptation – general Agreement: Deactivated SCell measurement: - Option 1: For SSB adaption, reuse the same conclusion from OD-SSB case#2 (scenario#4) discussion to define RRM requirements. - At least for the case when the SSB periodicity is adapted to be shorter than before - Option 2: Same as legacy - Other options are not precluded. The requirement can be updated based on further progress in other WG or other aspects are identified that the conclusion can not be directly reused. Issue 1-4-1: SCell activation requirement for SSB adaptation Agreement: For SCell activation: - Define the SCell activation requirements based on the SSB periodicity after SSB adaptation. - For the case SSB adaptation command comes together with SCell activation command. - The requirement can be updated based on further progress in other WG Issue 1-2-2: Transition requirements applicability & Issue 1-2-3: Transition requirements – details Agreement: - For L1-RSRP/L1-SINR/CBD - The measurement/evaluation period based on SSB periodicity after adaptation apply from Tfirst_SSB. - Tfirst_SSB is the uncertainty time from the timepoint when SSB adaptation occurs till the first SSB after SSB adaptation.

In last meeting, for L1 impact of SSB adaptation, RAN4 agreed SSB periodicity can be reused for L1-RSRP/L1-SINR/CBD. RAN4 also discussed the L3 impact of SSB adaptation on deactivated SCell measurement and SCell activation. For SSB adaptation on deactivated SCell measurement, we prefer the same requirements as legacy based on measCycleSCell can be used.

Proposal 1: For SSB adaptation on deactivated SCell measurement, the same L3 requirements as legacy based on measCyclescell can be used.

In our view, we don’t pursue SMTC-based L3 measurement on deactivated SCell activation, and provide 3 concerns from our side:

- Even though RAN4 has the common understanding that L3 measurement requirement in RAN4 is defined based on SMTC configuration rather than SSB configuration, it is not always true because enhanced requirements based on SSB periodicity were already introduced in R18 by replacing the SMTC periodicity.

- SSB adaptation and OD-SSB can be considered together. In the discussion of OD-SSB, most companies also support to use OD-SSB periodicity to speed up the measurement of deactivated SCell before and during SCell activation. But UE does not expect SMTC adaptation due to SSB adaptation or OD-SSB activation/deactivation as they may happen more frequently and dynamically, e.g., DCI or MAC-CE triggering.

- SMTC based on RRC configuration is used for measurements on both serving and neighbor cell on the same frequency layer, which should be better as persistent or semi-persistent. Thus, RAN4 should restrict the SMTC impact on all related cells besides some of them with SSB adaptation.

- As OD-SSB before SCell activation is mainly for fast SCell activation, it is hard to restrict the OD-SSB based L3 measurement to SMTC window because UE cannot wait for a coming SMTC window for L3 measurement to further extend the delay of SCell activation and the SMTC window is more designed for static SSB transmission. It is agreed that OD-SSB can be dynamically (de)activated with various periodicity.

- On the other hand, after the SCell has been activated, it seems OK to fallback and limit serving cell measurement as intra-frequency layers to the SMTC window as legacy. In addition, based on RAN2 LS reply [2], SMTC periodicity can be used as baseline for SSB adaptation on L3 measurement on neighbor cells.

Therefore, we have the following observations:

Observation 1: For SSB adaptation on deactivated SCell measurement, additional SMTC configuration is not needed.

Observation 2: For SSB adaptation during SCell activation, the requirements are based on the SSB periodicity after SSB adaptation, and additional SMTC configuration is also not needed.

Observation 3: After the SCell has been activated, L3 measurement on serving cell or intra-frequency can use legacy SMTC.

In total, RAN4 needs to discuss whether to introduce requirements related to additional SMTC since it may be transparent to RAN4. If not, then RAN4 can sent LS to inform RAN2.

Proposal 2: RAN4 to discuss the need of additional SMTC since it may be transparent to RAN4.

Proposal 3: Any new SMTC impact should be considered for OD-SSB and SSB adaptation together.

Conclusions

In this paper we provided our views on the SMTC impact on RRM requirements due to SSB adaptation and/or OD-SSB.

Proposal 1: For SSB adaptation on deactivated SCell measurement, the same L3 requirements as legacy based on measCyclescell can be used.

Observation 1: For SSB adaptation on deactivated SCell measurement, additional SMTC configuration is not needed.

Observation 2: For SSB adaptation during SCell activation, the requirements are based on the SSB periodicity after SSB adaptation, and additional SMTC configuration is also not needed.

Observation 3: After the SCell has been activated, L3 measurement on serving cell or intra-frequency can use legacy SMTC.

Proposal 2: RAN4 to discuss the need of additional SMTC since it may be transparent to RAN4.

Proposal 3: Any new SMTC impact should be considered for OD-SSB and SSB adaptation together.

Reference

- R4-2502689, WF on RRM requirements for Netw_Energy_NR_enh_Part2

- R4-2503117, Reply LS on SSB adaptation, RAN2