3GPP TSG-RAN WG4 Meeting # 114bis	R4-2503366

# Wuhan, China, 7th - 11th, Apr 2025

Source: 	Samsung

Title: 	On fallback behavior and signaling aspect for fragmented carriers

Agenda Item:	7.5.2

Document for:	Discussion

- Introduction

This paper discuss the leftover issues captured in WF [1].

- 2. Discussion

Though the current study is primarily focusing on 2CC case per the SID, we think we must also consider its high order configurations to make the design future proof enough to avoid chaos in future, as we believe the free-up RF resources is in pursuit of either more CC(s) or more MIMO layer.

Observation 1: The free-up RF resources by single RF Rx Chain architecture is in pursuit of either more CC(s) or MIMO layer.

Proposal 1: When it comes to feature design, higher order configurations (CC number larger than 2) must also be taken into account to make it future proof.

One key issue being extensively discussed in last meeting was the “fallback” behavior when the in-gap interference is too large to prevent the 2CC of a band being processed well by single Rx RF chain, which will have significant impact on the signalling framework design. We illustrate the possible “fallback” path for different kinds of combos with different RF architectures in Figure 1~3. Note the “fallback” refers to either the real fallback(i.e., cell release) or “fallback”(MIMO layer decrease).

Figure 1. “Fallback” path for CA_nX(2A)

Figure 2. “Fallback” path for CA_nX(2A)-nZA

Figure 3. “Fallback” path for CA_nX(2A)-nY(2A)

First of all, as can be noticed in above Figures, some higher order combos can only be achieved with the architecture of fragmented carriers mode (i.e., one Rx RF Chain architecture), for example CA_nX(2A)-nY(2A) (X,Y as low band) for a UE equipped with only 2 low band antennas. Some combos can be achieved by both architectures. For some legacy or new UE, fragmented carriers architecture may not be supported at all.

As elaborated in previous meetings, generally we think the Rx requirements for fragmented mode should be relaxed, and separated from the current requirements. Therefore, a new UE capability must be introduced to inform the NW the UE performance clearly. Further, it is also preferred to leverage the existing band combination reporting scheme. For the detailed UE capability design, we propose to consider “absent/present with 0/present with 1” approach. More specific, for instance,

- When UE capability is absent, UE supports legacy mode only →For backward compatible

- When UE capability is present with parameter=0, UE supports fragmented mode only

- When UE capability is present with parameter=1, UE supports both legacy mode and fragmented mode.

The granularity of the UE capability should at least be per FS, maybe better to be FSPC to accommodate the case with 3DLCC within a band.

“Fragmented mode” here intends for the “one Rx RF chain architecture” and its associated requirements, “legacy mode” here intends for the “fully separated Rx chain architecture” or “partially shared Rx chain architecture” with the current requirements.

Proposal 2: With the understanding of separated requirements are expected for fragmented carriers mode, new UE capability should be introduced (leveraging the existing band combination reporting scheme) to clearly indicate to NW the applicable UE requirements is requirements of {legacy architecture, fragmented mode architecture, both}, granularity as FSPC.

- When UE capability is absent, UE supports current requirements only→For backward compatible

- Details up to RAN2

For the possible “fallback” paths, with the understanding that cell release and MIMO layer re-config is already under NW control in the existing mechanism, this legacy approach should still stand. We keep proposing the views presented in previous meetings:

Proposal 3: When the significant in-gap interferer precludes the UE to work in fragmented carriers mode, UE shall fallback under the NW full control, fallback here refers to:

- The real fallback (i.e., cell release) w/ and w/o UE architecture change

- “Fallback”(MIMO layer decrease) with UE architecture change

Note: “UE architecture change” here denotes that UE architecture is changed from fragmented carriers architecture (i.e., One Rx RF Chain) to legacy CA architecture (i.e., fully separated Rx Chain or partially shared Rx Chain) at least for one band of a band combination.

Proposal 4: The absolute power level of in-gap interferer and/or the PSD difference between in-gap interferer and target CC could be used to determine whether one Rx RF chain is enabled, like a threshold.

- The detailed measurement solution is left for RRM in following WI (if any in Rel-20)

Proposal 5: The determination of the reasonable/acceptable absolute power level of in-gap interferer and/or the PSD difference between in-gap interferer and target CC could consider the intra-band DLCA performance in fragmented carrier mode not be worse than the performance of its single CC operation.

- 3. Conclusion

Observation 1: The free-up RF resources by single RF Rx Chain architecture is in pursuit of either more CC(s) or MIMO layer.

Proposal 1: When it comes to feature design, higher order configurations (CC number larger than 2) must also be taken into account to make it future proof.

Proposal 2: With the understanding of separated requirements are expected for fragmented carriers mode, new UE capability should be introduced (leveraging the existing band combination reporting scheme) to clearly indicate to NW the applicable UE requirements is requirements of {legacy architecture, fragmented mode architecture, both}, granularity as FSPC.

- When UE capability is absent, UE supports current requirements only→For backward compatible

- Details up to RAN2

Proposal 3: When the significant in-gap interferer precludes the UE to work in fragmented carriers mode, UE shall fallback under the NW full control, fallback here refers to:

- The real fallback (i.e., cell release) w/ and w/o UE architecture change

- “Fallback”(MIMO layer decrease) with UE architecture change

Note: “UE architecture change” here denotes that UE architecture is changed from fragmented carriers architecture (i.e., One Rx Rf Chain) to legacy CA architecture (i.e., fully separated Rx Chain or partially shared Rx Chain) at least for one band of a band combination.

Proposal 4: The absolute power level of in-gap interferer and/or the PSD difference between in-gap interferer and target CC could be used to determine whether one Rx RF chain is enabled, like a threshold.

- The detailed measurement solution is left for RRM in following WI (if any in Rel-20)

Proposal 5: The determination of the reasonable/acceptable absolute power level of in-gap interferer and/or the PSD difference between in-gap interferer and target CC could consider the intra-band DLCA performance in fragmented carrier mode not be worse than the performance of its single CC operation.

- 4. Reference

[1] R4-2502873, WF on NR FR1 DL fragmented carriers study, Mediatek, RAN4#114