3GPP TSG-RAN WG4 Meeting #114bis				R4-2503393

Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda item:	7.24.4

Source:	CMCC

Title:	Discussion on OTA test method for A-IoT device

Document for:	Approval

1	Introduction

In RAN#106 meeting, a new WI Solutions for Ambient IoT (Internet of Things) in NR was approved [1]. following list the objective of this OTA test part.

- RAN4 scope:

  - Specify RF requirements for Ambient-IoT BS, device 1, and CW

    - RF requirements for Type 1-C Ambient-IoT BS

    - RF requirements for device 1

    - RF requirements for CW

  - Specify RRM core requirements for device 1, if necessary

  - Study and develop OTA test methodology for A-IoT device 1

    - Consider test methods specified in TR 38.870 as starting point. Take test system reuse, test system complexity and test time into account, when developing test methods suitable for Ambient IoT.

    - Develop the preliminary Measurement Uncertainty (MU) assessment for the test system

  - Use band n8 as an example band

- In this paper, we will focus on the following issues for OTA testing.

2	Discussion

Last meeting agreement is listed as below:

Issue 4-1-1: Scope of OTA test methodology Agreement: - For OTA test, the starting point of test methodology for Ambient IoT includes - Chamber setup - Position of Device - Test procedures - Minimum range/ QZ (Quiet Zone) size/QZ calibration - Preliminary MU assessment - Note: The performance metrics to be discussed in RF (RRM/Demodulation if any) requirements. And the test methodology takes sensitivity and backscatter power as example for discussion to develop test methodology.

- Test setup

- Now there is no agreements of performance metrics. According to experience of RF requirements, at least the output power and emission requirements are defined with TRP metric. According to 38.870, 3D-scan is required which is time consuming and test complex. RAN4 needs to consider how to reduce testing time. Tag usually use the dipole antenna with the maximum antenna gain appears at the phi = 90° plane which is the vertical plane of the dipole antenna long dimension center when tag longest dimension at x-axis. The antenna gain is symmetrical along the x-axis. So in theory the measurement point grid could be more sparse than UE testing. In last meeting, some contributions suggest to only test hemisphere towards the reader. But considering there may be different antenna for tag, it’s better to stil scan the 3D sphere with more sparse measurement grid.

- Proposal 1: it’s suggested to still use the 3D-scan for TRP performance metric but with more sparse measurement grids.

- In TR 38.870, for the position guideline, the UE point at x axis, i.e. UE is located in y-z plane, if we want to reuse the test set up as much as possible, the longest dimension of tag should point at z axis, i.e. tag is located in x-y plane. Besides, the CW node should be avoided to be located at the direction with negative antenna gain, i.e. for the figure 7.2-2 in TR38.870, the CW node should be avoided at φ direction.

- Proposal 2: the longest dimension of tag is suggested to be in x-y plane considering tag mainly use dipole antenna.

- CW will always be transmitting during the test of Tx requirements. One issue is how to avoid the such extra power from CW to impact final test value. Following list three solutions:

- Option 1: test with device and without device, the power difference between above two sets values is the pure power from tag. Such solution will not require extra isolation between CW antenna and receiver reference antenna. But this will double the testing which is not the target of this scope.

- Option 2: set isolation between the CW Tx antenna and receiver reference antenna, e.g. 50dB as listed in RFID spec. The tag antenna gain is negative along the tag antenna direction, CW Tx antenna should be avoided in those directions but the receiver reference antenna should be 3D-scan. Careful design is required to avoid that CW Tx antenna pointing at the direction of negative antenna gain.

- Option 3: usually CW only occupies very narrow bandwith and its emission is much perfect. We can just abstract the CW power at test equipment. Such solution will ignore the emission power of CW node which may incorrectly increase the testing power from tag especially for emission requirements.

- Observation 1: above list three kinds of solutions to avoid taking CW power into final test value.

- Following list the analysis of MU factors and show our initial thoughts.

| - MU contribution description | - Applicable for device |
| --- | --- |
| - Mismach uncertainty | - Can be reused |
| - Insertion loss | - Can be reused |
| - Influence of the antenna cable | - Can be reused |
| - Measurement receiver: uncertainty of the absolute level | - Can be reused |
| - CW tester: uncertainty of the absolute level | - may be updated |
| - Sensitivity measurement: output level step resolution | - Can be reused |
| - Measurement distance | - may be updated |
| - Quality of quiet zone | - Depends on the size discussion of quiet zone |
| - DUT Tx-power dirft | - may be updated |
| - DUT sensitivity drift | - may be updated |
| - Coarse samling grid | - Depends on the agreements of sampling grid |
| - Random uncertainty | - Can be reused |
| - Frequency response | - Can be reused |
| - Uncertainty of network analyser | - Can be reused |
| - Uncertainty of the gain/efficiency of the calibration antenna | - Can be reused |

- Proposal 3: above show our initial thoughts for MU.

3	Conclusion

This contribution makes the following observations and proposals.

- Proposal 1: it’s suggested to still use the 3D-scan for TRP performance metric but with more sparse measurement grids.

Proposal 2: the longest dimension of tag is suggested to be in x-y plane considering tag mainly use dipole antenna.

- Observation 1: above list three kinds of solutions to avoid taking CW power into final test value.

| - MU contribution description | - Applicable for device |
| --- | --- |
| - Mismach uncertainty | - Can be reused |
| - Insertion loss | - Can be reused |
| - Influence of the antenna cable | - Can be reused |
| - Measurement receiver: uncertainty of the absolute level | - Can be reused |
| - CW tester: uncertainty of the absolute level | - may be updated |
| - Sensitivity measurement: output level step resolution | - Can be reused |
| - Measurement distance | - may be updated |
| - Quality of quiet zone | - Depends on the size discussion of quiet zone |
| - DUT Tx-power dirft | - may be updated |
| - DUT sensitivity drift | - may be updated |
| - Coarse samling grid | - Depends on the agreements of sampling grid |
| - Random uncertainty | - Can be reused |
| - Frequency response | - Can be reused |
| - Uncertainty of network analyser | - Can be reused |
| - Uncertainty of the gain/efficiency of the calibration antenna | - Can be reused |

- Proposal 3: above show our initial thoughts for MU.

References

- RP-243312, New Work Item: Solutions for Ambient IoT (Internet of Things) in NR