3GPP TSG-RAN WG4 Meeting # 114-bis                                                        R4-25xxxxx

Wuhan, China, 7th –11th April, 2025

Agenda item:	7.26.2.2

Source: 	CMCC

Title: 	Discussion on LP-WUS UE RF Rx requirements of REFSENS, ASCS and ACS

Document for:	Discussion

# 1. Introduction

In RAN plenary #105 meeting, revised WID of LP-WUS is approved with following RF part objectives:

- Specify the necessary RAN4 core requirement(s) to support the feature (RAN4).

  - Specify UE low-power wake-up receiver requirements, at least REFSENS, ACS and ASCS requirements with consideration of possible new methodology to assess the low-power wake-up receiver performance

    - Define guard RBs for ACS and ASCS cases

    - Study testability of above requirements

    - Consider impacts of different architecture and impairments, and set requirements that enable all types of reasonable implementation

  - Study and if necessary specify or support by declaration, the corresponding BS requirements, e.g., dynamic range for LP-WUS/LP-SS.

    - Current NR BS requirements is baseline

In this contribution, we focus on discuss of UE LP-WUS RF requirements including REFSENSE, ACS and ASCS.

- Discussion

During the last meeting, RAN4 reached the following agreement below.

Issue 2-1-1: SNR, NF and IM value for FR1 LP-WUR Agreement: - For the FR1 requirements targeting at bands <2.5GHz - The IM+NF values are - Set 1: 18dB - Set 2: 13.5dB - Send an LS to RAN1 with RAN4 decision of two sets of : NF (assume IM is 2.5dB) - Ask RAN1 to check whether coverage target can be met based on above NF information. And ask for detailed signal configuration (e.g., chip rate, length of sequence) of LP-WUS. - LS should also cc RAN2 Issue 2-1-3: Timing error impacts for OOK-based and OFMD-based SNR Agreements - Timing error contribution to RAN4 RF simulation can be contribution driven. Issue 2-3-2: in-band power level of ASCS Test case Agreements: - For ASCS, allowable de-sense due to introduction of NR RBs is [0-1dB]. - [0] guard RB is the baseline. Issue 2-3-3: channel bandwidth of ASCS Test case Agreements: - For ASCS core requirements, it should be applied to all NR CBW. - For conformance test, for NR CBW can be further discussed and down-selected. - For ASCS test configuration, the NR RBs should be fully allocated with the same PSD of LP-WUS within the channel except for 11 LP-WUS RBs and potential guard RBs (if any). - Sub-topic 2-4 ACS simulation and requirements Issue 2-4-1: Baseline ACS requirements (same interference) and required guard RB Agreements - Same interference signal level as MR for LP-WUR ACS requirements - FFS at least for one set of Rx requirements. - The LP-WUS level is LP-WUS REFSENS+[X]dB, X=14 as starting point - FFS whether two sets of LP-WUS wanted signal level can be defined. - Final ACS will be decided by LP-WUR REFSENS and wanted signal relaxation - FFS number of guard RBs. Issue 2-4-2: ACS requirements (same interference) and required guard RB Way forward: - FFS, also depends on discussion for Issue 2-4-1 Issue 2-4-3: ACS requirements (lower interference level) and required guard RB Way forward: - FFS, also depends on discussion for Issue 2-4-1 Issue 2-4-3: Case 1 ACS test parameter Way forward: - FFS LPWUS ACS test case. - ACS requirement applies to all NR CBW

2.1 REFSENSE requirements

Considering that SNR is relevant to RF and demodulation, and that RAN1 has two options of OFDM signal and OOK signal, the required SNR for demodulation of OOK and OFDM signals could be different, so two sets of SNR values could be specified. Based on the simulation results, the OFDM receiver performs better than the OOK receiver. Since the case OOK-4 (M=4) has the worst performance, the target SNR should be chosen based on this case.

Table 1 SNR@1%BLER with 4bit ADC under AWGN

| Modulation | Channel structure | Receiver type | SNR@1% BLER |
| --- | --- | --- | --- |
| OOK-1 | 8bit | Envelope | -6.5dB |
|  |  | OFDM | -10.6dB |
|  | 16bit | Envelope | -6.1dB |
|  |  | OFDM | -10.3dB |
| OOK-4 M=2 | 8bit | Envelope | -4.7dB |
|  |  | OFDM | -7.6dB |
|  | 16bit | Envelope | -4.2dB |
|  |  | OFDM | -7.2dB |
| OOK-4 M=4 | 8bit | Envelope | -2.5dB |
|  |  | OFDM | -4.7dB |
|  | 16bit | Envelope | -2.2dB |
|  |  | OFDM | -4.3dB |

Proposal 1: -2.5dB SNR for OOK-based signal, and -4.7dB for OFDM-based signal.

2.1 ASCS and ACS

When defining ACS and ASCS for LP-WUR, two factors should be considered: the ASCS value on a dB scale and the required guard RB. If we only define the guard RB requirement without specifying an ASCS value, there would be no restriction on the in-band selectivity of LP-WUR. In extreme cases, even LP-WUR with the worst performance might pass the test, as there is only a guard RB limitation. Conversely, if we only define the ASCS value without any guard RB limitation, LP-WUR with the worst performance could also pass the test in extreme cases, as long as the guard RB is large enough.

Proposal 2: For ASCS and ACS requirement, RAN4 should specify the both guard RB and ASCS/ACS requirements.

Since ASCS requirement cannot be specified directly, we should define the test case for the ASCS requirement in the specification. The test parameters should include the power level, bandwidth, MDR, and other relevant parameters. The MDR will be verified in the test with the guard RBs as defined in the specification. And we could use the following table as the starting point to define the ASCS test parameters.

Table 3: Test parameters for LP-WUS ASCS

| RX parameter | Units | Channel bandwidth (MHz) |
| --- | --- | --- |
|  |  | 5-100MHz |
| LP-WUS Power | dBm | REFSENS |
| In-band Pinterferer | dBm | NR signal, same as LP-WUS |
| Guard RB between LP-WUS and NR |  | [0 or 1] at each side |
| MDR |  | 1% |

Proposal 3: Use the table 3 as the starting point of the ASCS test parameters.

For ACS requirement, we can use the legacy requirement as the starting point, and further consider to relax the value with guard RB. Based on the current simulation assumptions, we conducted simulations of 3rd-order and 5th-order filters under the conditions of OOK-4 and M=4, and the results are shown in the figure below. The simulation results show that the number of guard RBs has a significant impact on the ACS value. Under the condition of 1 guard RB, the ACS value ranges from 22 to 24 dB. Considering the impact of 1 RB, and combining the above analysis, we recommend selecting 1 RB as the guard RB and setting the ACS value to [22-24 dB].

Proposal 4: Select 1 RB as the guard RB and setting the ACS value to [22-24 dB].

# 3. Conclusions

In this contribution, ASCS, ACS and REFSENSE are discussed with following observations and proposals.

Proposal 1: -2.5dB SNR for OOK-based signal, and 4.7dB for OFDM-based signal.

Proposal 2: For ASCS and ACS requirement, RAN4 should specify the both guard RB and ASCS/ACS requirements.

Proposal 3: Use the table 3 as the starting point of the ASCS test parameters.

Table 3: Test parameters for LP-WUS ASCS

| RX parameter | Units | Channel bandwidth (MHz) |
| --- | --- | --- |
|  |  | 5-100MHz |
| LP-WUS Power | dBm | REFSENS |
| In-band Pinterferer | dBm | NR signal, same as LP-WUS |
| Guard RB between LP-WUS and NR |  | [1 or 0] at each side |
| MDR |  | 1% |

Proposal 4: Select 1 RB as the guard RB and setting the ACS value to [22-24 dB].

# 4. Reference

- R4-2503028, WF on UE RF requirements for LP-WUS