3GPP RAN WG4 Meeting #114bis	R4-2503406

Wuhan, CN, April 7th – April 11th, 2025

Title: 	Discussion on CSI reporting requirement framework for CSI prediction

Source: 	CATT

Agenda item:	7.19.2

Document for:	Discussion

Introduction

In RAN4#114 meeting, RAN4 discussed Rel-19 AI/ML for NR air interface and agreements were captured in [1]. There are still some open issues that need discussion. In this contribution, we will present our views on those issues.

Discussion

Codebook for CSI prediction requirement/test

Some proposals were brought up to determine the test baseline for AI/ML CSI prediction and candidate codebooks are reproduced below:

Issue 1-1: Test baseline - Proposals - Option 1: use Rel-18 eType II Doppler codebook for AI/ML based CSI predictions - Option 2: use Rel-16 eType II codebook for AI/ML based CSI predictions - Option 3: wait for RAN1 to make enough progress - Option 4: use Rel-15 Type I codebook for AI/ML based CSI prediction

In RAN1#120 meeting, RAN1 made progress on CSI prediction reporting [2] and RAN4 can have an agreement based on RAN1 progress:

Agreement For CSI prediction using UE-side model, at least for inference, Rel-18 CSI framework is reused. - For CSI-RS resource type for CMR, periodic, semi-persistent, and aperiodic CSI-RS are supported. - For inference report, - for N4>=1, support Rel-18 codebook ('typeII-Doppler-r18')

According to RAN1 agreement, Rel-18 Doppler codebook will be used for AI/ML CSI prediction reporting. Therefore RAN4 can define requirements and tests.

Proposal 1: Use Rel-18 Doppler codebook to define requirements and tests for AI/ML CSI prediction.

Training and test data consistency

Another open issue is that whether extra work is required to keep consistency between training and test dataset. Two options are provided:

Agreement: - further discuss the following options - Option 1: Consistency between training data and test data should be ensured - RAN4 to study how to ensure consistency between training data(simulation) and test data (conformance test) and whether anything special should be done during the test - Option 2: Consistency will be implicit as test emulated channel will be very similar to the one used in RAN4 simulations

Since one-sided AI/ML models are used in CSI prediction and training dataset will be generated by UE vendors based on test configurations, therefore we think the consistency between training and test dataset is implicitly kept if test configurations provide enough information. RAN4 can further discuss what necessary information should be included in test configurations to ensure that UE vendors have enough knowledge to generate their own training datasets.

Observation 1: Consistency between training and test dataset is implicitly kept if test configurations include enough information to help UE vendors generate consistent training datasets.

Proposal 2: RAN4 to discuss necessary information that needs capturing in test configurations when defining test configurations.

Conclusions

This contribution discussed the issues related to AI/ML enabled CSI prediction and some proposals are provided:

Proposal 1: Use Rel-18 Doppler codebook to define requirements and tests for AI/ML CSI prediction.

Observation 1: Consistency between training and test dataset is implicitly kept if test configurations include enough information to help UE vendors generate consistent training datasets.

Proposal 2: RAN4 to discuss necessary information that needs capturing in test configurations when defining test configurations.

References

[1] R4-2502856, WF on the requirements for AI/ML air interface, Qualcomm, RAN4#114.

[2] RAN1 Chair’s Notes eom1, RAN1#120.