3GPP RAN WG4 Meeting #114bis	R4-2503407

Wuhan, CN, April 7th – April 11th, 2025

Title: 	Discussion on RRM core requirements testing framework for BM

Source: 	CATT

Agenda item:	7.19.3

Document for:	Discussion

Introduction

In RAN4#114 meeting, RAN4 discussed beam management in Rel-19 AI/ML for NR air interface and agreements are captured in [1]. In this contribution, we will continue to discuss the open issues summarized in [2].

Discussion

Beam prediction KPIs

Regarding beam prediction KPIs, the following KPIs were captured in TR [3] and no down-selection was made since the beginning of Rel-19.

Different metrics/KPIs have been discussed and were captured in the TR: For metrics for beam management requirements/tests, the following test metrics are identified and could be considered - Option 1: RSRP accuracy - Option 2: Beam prediction accuracy -Top-1 (%) : the percentage of "the Top-1 strongest beam is Top-1 predicted beam" -Top-K/1 (%) : the percentage of "the Top-1 strongest beam is one of the Top-K predicted beams" -Top-1/K (%) : the percentage of "the Top-1 predicted beam is one of the Top-K strongest beams" - Option 3: The successful rate for the correct prediction which is considered as maximum RSRP among top-K predicted beams is larger than the RSRP of the strongest beam – x dB, -Related measurement accuracy can be considered to determine x - Option 4: combinations of above options

And RAN4 made a small step forward in last meeting:

Issue 2-1: Metrics/KPIs for beam prediction If RSRP and beam ID are reported: - RSRP accuracy will be one of the KPIs. - How option 1 is applied is FFS - Beam ID prediction accuracy will be one of the KPIs - FFS whether beam ID prediction accuracy refers to a combination of Option 2 and/or Option 3 or modified Option 2/3 or other options If only beam ID is reported: - Beam ID prediction accuracy will be used as the KPI - FFS whether beam ID prediction accuracy refers to a combination of Option 2 and/or Option 3 or modified Option 2/3 or other options

Currently, RAN4 is also performing a simulation on AI/ML beam management studying the impact of measurement error on prediction accuracy. Multiple KPIs are used in the simulation assumption for result alignment. Therefore RAN4 can wait for more observations and understanding based on simulation results and then RAN4 can make down- selection with confidence.

Proposal 1: RAN4 postpone down-selection of KPIs after having more observations and understanding based on simulation results.

Measurement period for inference

In RAN4#113 meeting, RAN4 agreed that inference delay includes two parts:

Issue 3-5: Prediction delay requirements Agreement: - Prediction delay includes: - Measurement for prediction - FFS on whether the legacy measurement delay can be reused - Inference delay - Other components are FFS

And in last meeting, there are candidate options for how to defining core requirements for BM:

- Proposals - Option 1: Use “legacy” L1-RSRP measurement as baseline - FFS whether same values for M,N,P should be reused - Option 2: Differentiate case 1 and case 2 - Reuse L1 measurement period for Case 1 - Introduce configurable number of measurement occasions for case 2 - Option 3: Study the measurement period based on simulations - Option 4: Others

We will discuss prediction delay for BM-Case 1 and BM-Case 2 respectively.

BM-Case 1:

For the measurement part, UE behaviour is exactly the same as legacy UE. Thus, we think the legacy measurement requirements can be reuse for this part. For inference delay, since UE does not need to measure reference signals and the inference is completely a UE internal procedure which can be carried out with other UE procedures in parallel or some time gaps, e.g., SSB processing and interval between SSB occasions. Thus, the legacy L1-RSRP measurement requirement can be reused.

Observation 1: For BM-Case 1, UE measures SetB beams and predict results for SetA beams in spatial domain. The legacy L1-RSRP measurement requirement can be reused for BM-Case 1.

BM-Case 2:

In RAN1#120 meeting, RAN1 made the following agreements for BM-Case 2:

Agreement For inference, for BM-Case 2 of UE-side model, - The time gap between two consecutive future time instances is configured by RRC, and the number of future time instance(s) N is configured by RRC. - time gap is [10ms, 20ms, 40ms, 80ms, 160ms] - N = [1, 2, 4, 8] - Reference time of the earliest time instance for the predicted results is based on the most recent occasion of the CSI-RS/SSB resource in Set B for measurement - Where the most recent occasion of the CSI-RS/SSB resource of set B is the latest CSI-RS/SSB occasion no later than the corresponding CSI reference resource of the corresponding inference report.

According to RAN1 agreements, network will configure the time gap between two consecutive future time instances and the number of future time instance(s) N via RRC. Though these parameters are configurable, we do not think they have directly impacts on RRM core requirements. In BM-Case 2, the UE behaviour that takes time is measurement on SetB beams. After UE completes measurements on SetB beams, it can directly predict results for SetA beams, which is a short internally procedure and it can also be covered time interval between reference signals.

Observation 2: For BM-Case 2, UE measures SetB beams and predict results for SetA beams in time domain. The legacy L1-RSRP measurement requirement can also be reused for BM-Case 2.

Proposal 2: RAN4 to reuse legacy L1-RSRP measurement requirement for both BM-Case 1 and BM-Case 2.

UE Rx beam knowledge

In RAN4#113 meeting, RAN4 discussed the issue that whether UE knows the Rx beam for the predicted best DL Tx beam and the way forward is reproduced:

RAN4#113 way forward: - Whether the TCI state associated with a predicted Tx beam is known or unknown is FFS - RAN4 to further discuss whether to mandate that the UE has to predict the Rx beam paired with the predicted Tx beam or not. - Other way to know UE Rx beams is not precluded - UE capability is one of the solutions

As we discussed in our paper submitted in last meeting, the prediction is performed based on measurement results of SetB beams. How SetB beams are measured by UE should be known to UE itself, i.e., UE should know the Rx beam that it uses to receive those SetB beams. However, based on RAN4 discussions, we found that the slow progress is caused by the involvement of UE Rx beam sweeping. We will briefly analyse the case when UE Rx beam sweeping is considered in the following contents.

In general, there are two patterns for UE Rx beams: 1) UE receives SetB beams with a fixed Rx beam; 2) UE sweeps its Rx beams. For pattern 1, it is clear that the fixed Rx beam should be used for receiving the predicted DL Tx beam. For pattern 2, though there is not a straightforward way to decide which Rx beam should be used for the predicted DL Tx beam, UE can choose an Rx beam according to the spatial relationship between the predicted DL Tx beam and SetB beams, which depends on UE implementation. Otherwise, the ‘predicted best DL Tx beam’ is meaningless if UE does not which Rx beam should be used to receive it.

In addition, the reliability of choosing UE Rx beam also depends on the training datasets. For examples, if model is trained with a dataset containing RSRP results among all Tx beam and Rx beam pairs, then UE can directly know which Rx beam is the most appropriate beam to be used for the predicted beam. Therefor RAN4 should assume that the UE Rx beam is known for the predicted beam.

But, according to the latest WID [3], this use case is to predict the DL Tx beam prediction:

- Beam management - DL Tx beam prediction for both UE-sided model and NW-sided model, encompassing [RAN1/RAN2]: - Spatial-domain DL Tx beam prediction for Set A of beams based on measurement results of Set B of beams (“BM-Case1”) - Temporal DL Tx beam prediction for Set A of beams based on the historic measurement results of Set B of beams (“BM-Case2”) - Specify necessary signalling/mechanism(s) to facilitate LCM operations specific to the Beam Management use cases, if any - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE NOTE: Strive for common framework design to support both BM-Case1 and BM-Case2

Hence, in order to make test simple, it would be better for UE to fix its Rx beam when measuring SetB beams, in which case UE Rx beam will be known to UE itself for a predicted best DL Tx beam.

Proposal 3: For DL Tx prediction, UE Rx beam should be known to UE itself.

Requirement goals for BM

For AI/ML based beam management, the test target depends on the way of how the performance requirements will be defined. When defining performance requirements, different levels of overhead reduction need to be taken into account for a given SINR. For example, predicting 64 beams based on measuring 8 beams or 16 beams respectively, leading to 87.5% and 75% overhead reduction, and different performance requirements are defined for both cases. Now assuming that the reduction ratio is 0, which means SetB is the same as SetA, and then prediction is performed. In this case, the performance requirements of AI/ML based BM shall not be worse than the legacy requirements given the same SINR, i.e., no performance degradation is expected.

Next assuming that reduction ratio increases, whether lower RSRP results are allowed is the key point in this issue. If the legacy accuracy values are reused for all overhead reduction ratios, it means only one set of performance requirements will be defined and it is exactly the same as the legacy requirements. Option 2 means that higher RSRP results will be defined, but it is not the main purpose of this feature which is to reduce overhead rather than improving performance. Option 3 is that lower RSRP results are allowed. Considering different overhead reduction ratios, larger margin values can be defined for higher overhead reduction ratios. For instance, ±1 dB margin can be defined for overhead reduction ratio of 50% and ±2 dB margin for 75%.

Observation 3: Test target of AI/ML based BM depends on the way of how performance requirements will be defined.

Proposal 4: For AI/ML based beam management, two options can be considered for defining performance requirements:

- Option 1: The legacy performance requirements are reused, regardless of overhead reduction ratio.

- Option 2: The legacy performance requirements are relaxed with margins, and larger margin values are used for higher overhead reduction ratios.

Conclusions

This paper discussed of RRM core requirement issues related to AI/ML for beam management, and following proposals are provided:

Proposal 1: RAN4 postpone down-selection of KPIs after having more observations and understanding based on simulation results.

Observation 1: For BM-Case 1, UE measures SetB beams and predict results for SetA beams in spatial domain. The legacy L1-RSRP measurement requirement can be reused for BM-Case 1.

Observation 2: For BM-Case 2, UE measures SetB beams and predict results for SetA beams in time domain. The legacy L1-RSRP measurement requirement can also be reused for BM-Case 2.

Proposal 2: RAN4 to reuse legacy L1-RSRP measurement requirement for both BM-Case 1 and BM-Case 2.

Proposal 3: For DL Tx prediction, UE Rx beam should be known to UE itself.

Observation 3: Test target of AI/ML based BM depends on the way of how performance requirements will be defined.

Proposal 4: For AI/ML based beam management, two options can be considered for defining performance requirements:

- Option 1: The legacy performance requirements are reused, regardless of overhead reduction ratio.

- Option 2: The legacy performance requirements are relaxed with margins, and larger margin values are used for higher overhead reduction ratios.

References

[1] R4-2502856, WF on the requirements for AI/ML air interface, Qualcomm Incorporated, RAN4#114.

[2] R4-2500683, Topic summary for [114][131] NR_AIML_air, Qualcomm Incorporated, RAN4#114.

[3] TR 38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface, V18.0.0.