3GPP TSG-RAN WG4 Meeting #114-bis	R4-2503424

# Wuhan , CN, 7th – 11st Apr, 2025

Title: 	Discussion on  less than 5MHz for NTN RRM performance requirements

Source: 	CATT

Agenda item:	7.10.3.5

Document for:	Discussion

- Introduction

In last RAN4 meeting, RAN4 has reached good progress in defining the core RRM requirements and darft CRs has been endorsed [1]. In this contribution, we discuss and provide our initial view on the performance requirements based on the agreed core requirements.

- Discussion

  - 2.1 Test scope

For NTN with less than 5MHz, according to the draft big CR endorsed for core requirement in the last meeting [1], some new test cases will be added in the existing NTN test cases, which will also be defined in Clause A.14.

Proposal 1: For NTN with less than 5MHz, some new test cases will be added in the existing NTN test cases, which will also be defined in Clause A.14.

In our view, the legacy principle for TN with less than 5MHz can be reused as baseline, so we propose:

Proposal 2: RAN4 to additionally define the following test cases for NTN with less than 5MHz:

| Handover | TC 1: Intra-frequency handover from FR1 to FR1; unknown target cell operating with 12 PRB SSB bandwidth |
| --- | --- |
|  | TC 2: Intra-frequency SAN time-based conditional Handover from FR1 to FR1 |
|  | TC 3: Intra-frequency SAN distance-based conditional Handover from FR1 to FR1 |
|  | TC 4: RACH-based Hard Satellite switching with re-synchronization from FR1 to FR1 |
|  | TC 5: RACH-less Soft Satellite switching with re-synchronization from FR1 to FR1 |
| Signalling characteristics | TC 6: Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for UE operating on a cell with less than 5 MHz BW |
|  | TC 7: Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for UE operating on a cell with less than 5 MHz BW |
|  | TC 8: Radio Link Monitoring In-sync Test for FR1 PCell with 3 MHz Channel Bandwidth configured with SSB-based RLM RS in non-DRX mode |
|  | TC 9: Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode for a UE operating on a cell with less than 5 MHz BW |
| Measurement procedure | TC 10: SA event triggered reporting tests without gap under non-DRX with SSB index reading and 12 PRB SSB for intra-frequency Measurements |

  - 2.2 Test configurations

- General principle

When defining the specific test cases for NTN with less than 5MHz, existing NTN test cases can be used as a baseline. Since many test procedure and parameters defined for NTN can be reused for NTN with less than 5MHz, so we think that the same principle for defining less than 5MHz test cases in TN can be reused, that is, RAN4 only to clarify the procedure and parameters which can be reused from NTN, and then define new parameters applicable to less than 5MHz.

Proposal 3: The existing NTN test cases can be used as a baseline for NTN with less than 5MHz.

Proposal 4: The principle is that RAN4 only to clarify the procedure and parameters which can be reused from NTN, and then define new parameters applicable to less than 5MHz.

- General test configuration

For general test configuration, since NTN with less than 5MHz introduced the new channel bandwidth, so the test configurations need to discussed. The test configurations for NTN with less than 5MHz can be as follows:

- For GSO: NR FDD, 15 kHz SSB SCS, 3 MHz bandwidth with 15PRB CBW

- For NGSO: NR FDD, 15 kHz SSB SCS, 3 MHz bandwidth with 15PRB CBW

Proposal 5: RAN4 to consider the following two general test configurations as follows:

| Configuration | Description |
| --- | --- |
| 1 | GSO, NR FDD, 15 kHz SSB SCS, 3 MHz bandwidth with 15PRB CBW |
| 2 | NGSO, NR FDD, 15 kHz SSB SCS, 3 MHz bandwidth with 15PRB CBW |

- Test parameters

For the test parameters, at least the follwing should be revised compared to NTN, and the parameters used in TN with less than 5MHz can be reused considering 15PRB CBW and 12 PRB SSB bandwidth.

- BWchannel: 3: NPRB,c = 15 MHz

- BWP BW: NPRB,c = 15 MHz

- SSB Configuration: SSB.13 FR1

Proposal 6: Considering that 15PRB CBW and 12 PRB SSB bandwidth, at least the following test parameters can be defined as below:

- BWchannel: 3: NPRB,c = 15 MHz

- BWP BW: NPRB,c = 15 MHz

- SSB Configuration: SSB.13 FR1

- Measurement accuracy

Regarding the measurement accuracy for TN with less than 5MHz, RAN4 reused the L3 and L1 accuracy requirement of legacy TN. Therefore, for NTN with less than 5MHz, we think that RAN4 can also reuse existing L1/L3 measurement accuracy requirements of NTN.

Proposal 7: For NTN with less than 5MHz, RAN4 to reuse existing L1/L3 measurement accuracy requirements of NTN.

- Conclusion

This document discussed the issues for RRM performance requirements for R19 NTN with less than 5MHz and presented following proposals:

Proposal 1: For NTN with less than 5MHz, some new test cases will be added in the existing NTN test cases, which will also be defined in Clause A.14.

Proposal 2: RAN4 to additionally define the following test cases for NTN with less than 5MHz:

| Handover | TC 1: Intra-frequency handover from FR1 to FR1; unknown target cell operating with 12 PRB SSB bandwidth |
| --- | --- |
|  | TC 2: Intra-frequency SAN time-based conditional Handover from FR1 to FR1 |
|  | TC 3: Intra-frequency SAN distance-based conditional Handover from FR1 to FR1 |
|  | TC 4: RACH-based Hard Satellite switching with re-synchronization from FR1 to FR1 |
|  | TC 5: RACH-less Soft Satellite switching with re-synchronization from FR1 to FR1 |
| Signalling characteristics | TC 6: Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for UE operating on a cell with less than 5 MHz BW |
|  | TC 7: Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for UE operating on a cell with less than 5 MHz BW |
|  | TC 8: Radio Link Monitoring In-sync Test for FR1 PCell with 3 MHz Channel Bandwidth configured with SSB-based RLM RS in non-DRX mode |
|  | TC 9: Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode for a UE operating on a cell with less than 5 MHz BW |
| Measurement procedure | TC 10: SA event triggered reporting tests without gap under non-DRX with SSB index reading and 12 PRB SSB for intra-frequency Measurements |

Proposal 3: The existing NTN test cases can be used as a baseline for NTN with less than 5MHz.

Proposal 4: The principle is that RAN4 only to clarify the procedure and parameters which can be reused from NTN, and then define new parameters applicable to less than 5MHz.

Proposal 5: RAN4 to consider the following two general test configurations as follows:

| Configuration | Description |
| --- | --- |
| 1 | GSO, NR FDD, 15 kHz SSB SCS, 3 MHz bandwidth with 15PRB CBW |
| 2 | NGSO, NR FDD, 15 kHz SSB SCS, 3 MHz bandwidth with 15PRB CBW |

Proposal 6: Considering that 15PRB CBW and 12 PRB SSB bandwidth, at least the following test parameters can be defined as below:

- BWchannel: 3: NPRB,c = 15 MHz

- BWP BW: NPRB,c = 15 MHz

- SSB Configuration: SSB.13 FR1

Proposal 7: For NTN with less than 5MHz, RAN4 to reuse existing L1/L3 measurement accuracy requirements of NTN.

- Reference

[1]	R4-2502615, Draft big CR on RRM requirements for NTN less than 5MHz in Rel-19, Xiaomi