3GPP TSG-RAN WG4 Meeting #114-bis                                                                                       R4-2503439

Wuhan, CN, Apr. 7 –Apr. 11, 2025

Source: 	CATT

Title: 	Discussion on SAN RF requirements for the NTN new S-band

Agenda item:	6.13.2

Document for:	Approval

- Introduction

In RAN4 #114 meeting, a WF [1] on NR NTN support new S-band SAN RF requirement was agreed. However, there are still some remaining issues need to be discussed in the next meeting.

In this contribution, we would like to share some views on SAN RF requirements for introducing the NTN new S-band.

- Discussion

The system parameters for NR NTN support new S band have been discussed and concluded for SAN and NTN UE in the RAN4 #112-bis meeting. However, for Sync raster, there are still some remaining issues, as shown below:

#### Issue 1-3-5: SAN SS Raster - Agreement: - Agree Option 2 with both Case A and [Case B] - Need check if the 30KHz SCS SS raster is needed and whether case B is correct Table 5.4.3.3-1: Applicable SS raster entries per operating band (FR1-NTN) SAN operating band; SS Block SCS; SS Block pattern (NOTE); Range of GSCN (First – <Step size> – Last). n252; 15 kHz; Case A; 5456 – <1> – 5494. 30 kHz; Case B; 5460 – <1> – 5488. NOTE: SS Block pattern is defined in clause 4.1 in TS 38.213 [7]..

During RAN4 #113 meeting, a tentative agreement was achieved, as shown below:

Tentative agreement: - For the NTN FDD band with UE transmitting at 2000 - 2020 MHz and SAN transmitting at 2180 - 2200 MHz, the synchronization raster entries with SSB pattern Case B and Case C could be introduced.

Meanwhile, the SSB pattern was discussed during RAN4 #114 meeting, but no agreement was achieved.

As described in TS 38.213, there are a total of three SSB patterns defined for FR1 and FR1-NTN:

- Case A: 15 kHz SSB SCS, for situations where operator requires a minimum system CBW of 5 MHz. Meanwhile, only Case A could support the 15 KHz SCS scenario.

- Case B: 30 KHz SSB SCS, for situations where operator requires Dynamic Spectrum Sharing deployment, i.e. LTE and NR in the same carrier. In Case B pattern, NR SSB transmission will not be impacted by LTE PDCCH / CRS, thus ensuring coexistence with LTE when transmitted in the same carrier.

- Case C: 30 KHz SSB SCS, for situations where operator requires 60 kHz SCS sharing channel configuration.

In general, the SSB patterns for brand new operating bands are usually requested by operators. For SSB pattern Case A, it is supported by default in almost all FR1 operating bands, unless 15 kHz SCS is not defined.

For Case B, it is introduced if the operator needs to guarantee the coexistence of the new band with the LTE legacy band, as at least one SSB transmission will be protected from interference from LTE PDCCH / CRS transmissions. However, looking back at the discussions at previous meetings, we do not see a strong demand for case B from operators.

For Case C, since the maximum Tx CBW for new NTN S band is 20 MHz, the incentive to introduce 60 kHz SCS does not seem to be too strong, where 60 KHz SCS is usually used to support 100 MHz CBW scenarios.

In addition, it is not appropriate to assume that the SSB pattern of each operating bands were defined by duplex mode. Regarding to NTN SAN spec TS 38.108, the SAN SSB pattern of each FR1 operating band are shown as below:

Table 5.4.3.3-1: Applicable SS raster entries per operating band (FR1-NTN) for above 3 MHz channel bandwidth SAN operating band; SS Block SCS; SS Block pattern (NOTE); Range of GSCN (First – <Step size> – Last). n256; 15 kHz; Case A; 5429 – <1> – 5494. n255; 15 kHz; Case A; 3818 – <1> – 3892. 30 kHz; Case B; 3824 – <1> – 3886. n254; 15 kHz; Case A; 6215 – <1> – 6244. 30kHz; Case C; 6220 – <1> – 6238. NOTE: SS Block pattern is defined in clause 4.1 in TS 38.213 [7]..

It is obviously that for all FR1-NTN bands, the duplex mode is FDD. However, for bands n255 and n254, the SSB pattern is different, where Case B is used for n255 and Case C is used for n254.

Observation 1: For the NTN FDD band n255 and n254, the SSB pattern is different, where Case B is used for n255 and Case C is used for n254.

Apparently, the SSB pattern for bands n255 and n254 are not the focus of this paper. However, for band n252, where Case A is more basic and has been agreed upon, it may be more appropriate to wait for the operator's conclusion as to whether other SSB pattern are needed.

Therefore, we believed that for SAN synchronization raster of new NTN S band, the introduction of SSB pattern Case B and Case C should be triggered by the real demand of operator.

Proposal 1: For the NTN FDD band with UE transmitting at 2000 - 2020 MHz and SAN transmitting at 2180 - 2200 MHz, the introduction of sync raster entries Case B or Case C should be triggered by the real demand of operator.

- Conclusion

This contribution provides our initial views on SAN requirements for introduction of the NTN new S-band. The following proposals are concluded as follows:

Observation 1: For the NTN FDD band n255 and n254, the SSB pattern is different, where Case B is used for n255 and Case C is used for n254.

Proposal 1: For the NTN FDD band with UE transmitting at 2000 - 2020 MHz and SAN transmitting at 2180 - 2200 MHz, the introduction of sync raster entries Case B or Case C should be triggered by the real demand of operator.

- References

- R4-2502850, WF on NTN and IoT-NTN bands, Aalyria, RAN4 #114