3GPP TSG-RAN WG4 Meeting # 114-bis	R4-2503442

Wuhan, CN, 7h – 11th Apr, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.108 | CR | Draft | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DraftCR for TS 38.108, Introduction on system parameters for SAN supporting combined L band |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_combinedLband-Core |  |  |  |  |  | Date: |  |  | 2025-03-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To introduce system parameters for SAN supporting combined L band |  |  |  |  |  |  |  |  |
| Summary of change: |  | To introduce channel raster entries for SAN supporting combined L band To introduce SS raster entries for SAN supporting combined L band |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The system parameters of SAN supporting combined L band would be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.2, 5.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change>

5.4.2	Channel raster

5.4.2.1	NR-ARFCN and channel raster

The global frequency raster defines a set of RF reference frequencies FREF. The RF reference frequency is used in signalling to identify the position of RF channels, SS blocks and other elements. The global frequency raster is defined for all frequencies from 0 to 100 GHz. The granularity of the global frequency raster is ΔFGlobal.

RF reference frequencies are designated by an NR Absolute Radio Frequency Channel Number (NR-ARFCN) in the range [0…3279165] on the global frequency raster. The relation between the NR-ARFCN and the RF reference frequency FREF in MHz is given by the following equation, where FREF-Offs and NRef-Offs are given in table 5.4.2.1-1 and NREF is the NR-ARFCN.

FREF = FREF-Offs + ΔFGlobal (NREF – NREF-Offs)

Table 5.4.2.1-1: NR-ARFCN parameters for the global frequency raster

| Range of frequencies (MHz) | ΔFGlobal (kHz) | FREF-Offs (MHz) | NREF-Offs | Range of NREF |
| --- | --- | --- | --- | --- |
| 0 – 3000 | 5 | 0 | 0 | 0 – 599999 |
| 3000 – 24250 | 15 | 3000 | 600000 | 600000 – 2016666 |
| 24250 – 30000 | 60 | 24250.08 | 2016667 | 2016667 – 2112499 |

The channel raster defines a subset of RF reference frequencies that can be used to identify the RF channel position in the uplink and downlink. The RF reference frequency for an RF channel maps to a resource element on the carrier. For each operating band, a subset of frequencies from the global frequency raster are applicable for that band and forms a channel raster with a granularity ΔFRaster, which may be equal to or larger than ΔFGlobal.

For the uplink of FDD FR1-NTN bands defined in Table 5.2-1

FREF, shift = FREF + Δshift, Δshift = 0 kHz or 7.5 kHz.

where Δshift is signalled by the network in higher layer parameter frequencyShift7p5khz [7].

The mapping between the channel raster and corresponding resource element is given in clause 5.4.2.2. The applicable entries for each operating band are defined in clause 5.4.2.3.

5.4.2.1A	NB-IoT carrier frequency numbering

The NB-IoT carrier frequency numbering (EARFCN) is defined in clause 5.4B of TS 36.108 [17].

[NOTE:	Signalling of Δshift = 7.5 kHz for the uplink of the FDD FR1 NTN channel can ensure co-existence between the in-band NB-IoT and associated NR NTN channels.]

5.4.2.2	Channel raster to resource element mapping

The mapping between the RF reference frequency on the channel raster and the corresponding resource element is given in table 5.4.2.2-1 and can be used to identify the RF channel position. The mapping depends on the total number of RBs that are allocated in the channel and applies to both UL and DL. The mapping must apply to at least one numerology supported by the SAN.

Table 5.4.2.2-1: Channel Raster to Resource Element Mapping

|  | NRBmod2=0 | NRBmod2=1 |
| --- | --- | --- |
| Resource element index | 0 | 6 |
| Physical resource block index |  |  |

NRB is the transmission bandwidth configuration specified in sub-clause 5.3.2, is the PRB index within the NRB, and  is the resource element index within this PRB.

5.4.2.3	Channel raster entries for each operating band

The RF channel positions on the channel raster in each SAN operating band are given through the applicable NR-ARFCN in table 5.4.2.3-1 for FR1-NTN and table 5.4.2.3-3 for FR2-NTN, using the channel raster to resource element mapping in clause 5.4.2.2.

For SAN operating bands with 100 kHz channel raster, ΔFRaster = 20 × ΔFGlobal. In this case, every 20th NR-ARFCN within the operating band are applicable for the channel raster within the operating band and the step size for the channel raster in table 5.4.2.3-1 is given as <20>.

For SAN operating bands with 60 kHz channel raster above 3 GHz, ΔFRaster = I ×ΔFGlobal, where I ϵ {4, 8}. In this case, every Ith NR-ARFCN within the operating band are applicable for the channel raster within the operating band and the step size for the channel raster in table 5.4.2.3-3 is given as <I>.

Table 5.4.2.3-1: Applicable NR-ARFCN per operating band in FR1-NTN

| SAN operating band | ΔFRaster (kHz) | Uplink range of NREF (First – <Step size> – Last) | Downlink range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n256 | 100 | 396000 – <20> – 402000 | 434000 – <20> – 440000 |
| n255 | 100 | 325300 – <20> – 332100 | 305000 – <20> – 311800 |
| n254 | 100 | 322000 – <20> – 325300 | 496700 – <20> – 500000 |
| [n253] | 100 | 333600 – <20> – 335000 | 303600 – <20> – 305000 |
| [n251] | 100 | 325300 – <20> – 332100 | 303600 – <20> – 311800 |
| [n250] | 100 | 333600 – <20> – 335000 | 303600 – <20> – 311800 |

For SAN operating bands with 100 kHz channel raster, enhanced channel raster is defined with ΔFRaster = 2 × ΔFGlobal. In this case every 2nd NR-ARFCN within the operating band are applicable for the channel raster within the operating band and the step size for the channel raster in table 5.4.2.3-1 is <2>.

Table 5.4.2.3-2: Applicable NR-ARFCN per operating band for enhanced channel raster

| SAN operating band | ΔFRaster (kHz) | Uplink range of NREF (First – <Step size> – Last) | Downlink range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n256 | 10 | 396000 – <2> – 402000 | 434000 – <2> – 440000 |
| n255 | 10 | 325300 – <2> – 332100 | 305000 – <2> – 311800 |
| n254 | 10 | 322000 – <2> – 325300 | 496700 – <2> – 500000 |
| [n253] | 10 | 333600 – <2> – 335000 | 303600 – <2> – 305000 |
| [n251] | 10 | 325300 – <2> – 332100 | 303600 – <2> – 311800 |
| [n250] | 10 | 333600 – <2> – 335000 | 303600 – <2> – 311800 |
| NOTE 1: The channel numbers that designate carrier frequencies so close to the operating band edges that the carrier extends beyond the operating band edge shall not be used. These channel numbers shall also be such that the minimum guard band for each channel bandwidth and SCS specified in Table 5.3.3-1 are met for carriers located at the upper or lower edge of an operating band. |  |  |  |

Table 5.4.2.3-3: Applicable NR-ARFCN per operating band in FR2-NTN

| SAN operating band | ΔFRaster (kHz) | Uplink range of NREF (First – <Step size> – Last) | Downlink range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n512 | 60 | 2070833 – <1> – 2112499 | 1553336 – <4> – 1746664 |
|  | 120 | 2070833 – <2> – 2112499 | 1553336 – <8> – 1746664 |
| n511 | 60 | 2084999 – <1> –2112499 | 1553336 – <4> – 1746664 |
|  | 120 | 2084999 – <2> –2112499 | 1553336 – <8> – 1746664 |
| n510 | 60 | 2070833 – <1> – 2084999 | 1553336 – <4> – 1746664 |

<Next Change>

5.4.3	Synchronization raster

5.4.3.1	Synchronization raster and numbering

The synchronization raster indicates the frequency positions of the synchronization block that can be used by the UE for system acquisition when explicit signalling of the synchronization block position is not present.

A global synchronization raster is defined for all frequencies. The frequency position of the SS block is defined as SSREF with corresponding number GSCN. The parameters defining the SSREF and GSCN for all the frequency ranges are in table 5.4.3.1-1 for above 3 MHz channel bandwidth and in Table 5.4.3.1-2 for 3 MHz channel bandwidth.

The resource element corresponding to the SS block reference frequency SSREF is given in clause 5.4.3.2. The synchronization raster and the subcarrier spacing of the synchronization block are defined separately for each band.

The synchronization raster and the corresponding SS block do not cover all possible RF channel bandwidth and locations on enhanced channel raster.

Table 5.4.3.1-1: GSCN parameters for the global frequency raster for above 3 MHz channel bandwidth

| Range of frequencies (MHz) | SS block frequency position SSREF | GSCN | Range of GSCN |
| --- | --- | --- | --- |
| 0 – 3000 | N * 1200 kHz + M * 50 kHz, N = 1:2499, M ϵ {1,3,5} (Note) | 3N + (M-3)/2 | 2 – 7498 |
| 3000 – 24250 | 3000 MHz + N * 1.44 MHz, N = 0:14756 | 7499 + N | 7499 – 22255 |
| NOTE: The default value for operating bands which only support SCS spaced channel raster(s) is M=3. |  |  |  |

Table 5.4.3.1-2: GSCN parameters for the global frequency for 3 MHz channel bandwidth

| Range of frequencies (MHz) | SS block frequency position SSREF | GSCN | Range of GSCN |
| --- | --- | --- | --- |
| 0 – 3000 | N * 600 kHz + M * 50 kHz + 300 kHz, N = 1:1665, M ϵ {1,3,5} (Note 1) | 26638+3N + (M-3)/2 | 26640 – 41636 |
| NOTE: Only applicable for 15 PRB transmission bandwidth configuration within 3 MHz channel bandwidth with punctured PBCH defined in TS 38.211 [5] clause 7.4.3.1. |  |  |  |

5.4.3.2	Synchronization raster to synchronization block resource element mapping

The mapping between the synchronization raster and the corresponding resource element of the SS block is given in table 5.4.3.2-1.

Table 5.4.3.2-1: Synchronization Raster to SS block Resource Element Mapping

| Resource element index k | 120 |
| --- | --- |

k is the subcarrier number of SS/PBCH block defined in TS 38.211 clause 7.4.3.1 [5].

5.4.3.3	Synchronization raster entries for each operating band

For FR1-NTN and for above 3 MHz channel bandwidth, the synchronization raster for each band is given in table 5.4.3.3-1. The distance between applicable GSCN entries is given by the <Step size> indicated in table 5.4.3.3-1.

For FR1-NTN and for 3 MHz channel bandwidth, the synchronization raster for each band is given in table 5.4.3.3-3. The distance between applicable GSCN entries is given by the <Step size> indicated in table 5.4.3.3-3.

For FR2-NTN, the synchronization raster for each band is given in table 5.4.3.3-2. The distance between applicable GSCN entries is given by the <Step size> indicated in table 5.4.3.3-2.

Table 5.4.3.3-1: Applicable SS raster entries per operating band (FR1-NTN) for above 3 MHz channel bandwidth

| SAN operating band | SS Block SCS | SS Block pattern (NOTE) | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n256 | 15 kHz | Case A | 5429 – <1> – 5494 |
| n255 | 15 kHz | Case A | 3818 – <1> – 3892 |
|  | 30 kHz | Case B | 3824 – <1> – 3886 |
| n254 | 15 kHz | Case A | 6215 – <1> – 6244 |
|  | 30kHz | Case C | 6220 – <1> – 6238 |
| [n253] | 15 kHz | Case A | 3800 – <1> – 3807 |
| [n251] | 15 kHz | Case A | 3800 – <1> – 3892 |
|  | 30 kHz | Case B | 3806 – <1> – 3886 |
| [n250] | 15 kHz | Case A | 3800 – <1> – 3892 |
| NOTE: SS Block pattern is defined in clause 4.1 in TS 38.213 [7]. |  |  |  |

Table 5.4.3.3-2: Applicable SS raster entries per operating band (FR2-NTN)

| SAN operating band | SS Block SCS | SS Block pattern (NOTE) | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n512 | 120 kHz | Case D | 17448 – <12> – 19428 |
|  | 240 kHz | Case E | 17472 – <24> – 19416 |
| n511 | 120 kHz | Case D | 17448 – <12> – 19428 |
|  | 240 kHz | Case E | 17472 – <24> – 19416 |
| n510 | 120 kHz | Case D | 17448 – <12> – 19428 |
|  | 240 kHz | Case E | 17472 – <24> – 19416 |
| NOTE: SS Block pattern is defined in section 4.1 in TS 38.213 [7]. |  |  |  |

Table 5.4.3.3-3: Applicable SS raster entries per operating band (FR1-NTN) for 3 MHz channel bandwidth

| NR operating band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n256 | 15 kHz | Case A | 37492 – <1> – 37629 |
| n255 | 15 kHz | Case A | 34267 – <1> – 34424 |
| n254 | 15 kHz | Case A | 39060 – <1> – 39129 |
| NOTE: SS Block pattern is defined in clause 4.1 in TS 38.213 [7]. |  |  |  |

<End of Change>