3GPP TSG-RAN WG4 Meeting #114-bis                                                                                       R4-2503446

Wuhan, CN, Apr. 7 –Apr. 11, 2025

Source: 	CATT

Title: 	Discussion on system parameter for NTN support spectrum less than 5 MHz

Agenda item:	7.10.3.1

Document for:	Approval

- Introduction

In RAN4 #114 meeting, a WF [1] on NR NTN support spectrum less than 5 MHz system parameter was agreed. However, there are still some remaining issues need to be discussed in the next meeting.

In this contribution, we would like to share some views on system parameter for NTN support spectrum less than 5 MHz.

- Discussion

The open issues on system parameter for NR NTN support spectrum less than 5 MHz was discussed during the previous meetings, as shown below:

# System parameters Issue 1-1: Whether 3MHz channel bandwidth for NR-NTN in FR1-NTN bands is mandatory or optional? - Agreement: - Introduce 3MHz Channel bandwidth for NR-NTN in FR1-NTN bands as optional feature in Rel-19. - Further discuss whether 3MHz channel bandwidth is mandatory on certain bands in Rel-20 if there is a specific request from operators. Issue 1-2: UE capability Signalling - Agreement: - The exiting UE capabilities signalling of support3MHz-ChannelBW-Symmetric-r18 and support3MHz-ChannelBW-Asymmetric-r18 is reused for FR1 NTN bands. - The description of exiting UE capabilities signalling should be updated to support FR1 NTN bands.

According to the description of WID, the introduction of channel bandwidths less than 5 MHz in NTN is due to spectrum limitations/constraints encountered in existing NTN systems. Since the WID only considers FR1-NTN bands, unlike TN networks, the frequency bands are not multiples of 5 MHz. For TN networks, most of the operating bands were defined before the 3 MHz CBW feature was introduced, so the new 3MHz feature will only be defined as optional to ensure legacy UE could work properly. However, for the FR1-NTN UE, since the 3MHz Channel bandwidth for NR-NTN in FR1-NTN bands will be introduced as optional feature in Rel-19, the impact of introducing 3MHz CBW is negligible. Therefore, the mandatory support of the 3MHz CBW feature of NTN UE in feature release such as Rel-20 will maximize the use of the limited NTN spectrum, which is beneficial to the entire NTN system.

Proposal 1: 3MHz channel bandwidth for NR-NTN in the FR1-NTN bands could be mandatory in Rel-20 if the introduction of NTN UE supporting 3MHz CBW does not lead to a fragmented market.

The 3 MHz channel bandwidth UE capabilities signalling are defined in TS 38.306 [2] are shown as below:

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| support3MHz-ChannelBW-Asymmetric-r18 Indicates whether the UE supports 3 MHz channel bandwidth in uplink with larger than 3 MHz channel BW in DL, including short RACH preamble formats with 15kHz SCS, and long PRACH formats with 1.25kHz SCS. This feature is supported for 15kHz SCS only. It is applicable only to single-carrier operation and applies to bands where the UE indicates support for asymmetricBandwidthCombinationSet with 3 MHz UL according to clause 5.3.6 of TS 38.101-1 [2]. This feature is not applicable to UEs indicating supportOfRedCap-r17 or supportOfERedCap-r18. NOTE 1: The UE supporting this feature supports configuration of 15 PRB UL BWP operation. NOTE 2: If the UE indicates support in asymmetricBandwidthCombinationSet for a 3MHz UL in a band according to clause 5.3.6 of 38.101-1 [2], this feature shall be indicated for the band. | Band | No | FDD only | FR1 only |
| support3MHz-ChannelBW-Symmetric-r18 Indicates whether the UE supports 3 MHz symmetric channel bandwidth in DL and UL, including the following functional components: - Reception of 12 PRB PBCH based on RB-level puncturing; - Short RACH preamble formats with 15kHz SCS, and long PRACH formats with 1.25kHz SCS; - Reception of 15 PRB CORESET0. This feature is supported for 15kHz SCS only. It is applicable only to single-carrier operation and when an associated SS/PBCH block is located according to Table 5.4.3.3-2 in TS 38.101-1 [2]. This feature is not applicable to UEs indicating supportOfRedCap-r17 or supportOfERedCap-r18. NOTE: The UE supporting this capability supports configuration of 15 PRB BWP operation in DL and UL. | Band | No | FDD only | FR1 only |

For support3MHz-ChannelBW-Asymmetric-r18 and support3MHz-ChannelBW-Symmetric-r18 capabilities, the asymmetricBandwidthCombinationSet for asymmetric CBW and associated SS/PBCH block for symmetric CBW are both defined in TS 38.101-1 but not TS 38.101-5. Therefore, the descriptions of TS 38.101-5 need to be supplemented.

Since RAN4 has concluded that the exiting UE capabilities signalling of support3MHz-ChannelBW-Symmetric-r18 and support3MHz-ChannelBW-Asymmetric-r18 will be reused for FR1 NTN bands, RAN4 need to send an LS to RAN2 to update the description of the above UE capabilities signalling to include TS 38.101-5.

Proposal 2: RAN4 need to send LS to RAN2 to update the description of the UE capabilities signalling support3MHz-ChannelBW-Symmetric-r18 and support3MHz-ChannelBW-Asymmetric-r18 to include TS 38.101-5.

Meanwhile, since 3 MHz channel bandwidth will be introduced, we believe that the behaviour of UE to support 3 MHz channel bandwidth can be introduced accordingly. Considering that a normal SSB of 5 MHz contains 20 RBs and its occupied bandwidth is 3.6 MHz (15 KHz SCS), it means that its SSB bandwidth will be larger than the 3 MHz channel bandwidth, and thus a UE operating on 3 MHz channel bandwidth will not be able to decode the SSB correctly. For TN networks supporting 3 MHz channel bandwidth, RAN1 has designed an RB-level puncturing method that reduce the number of RBs in the PBCH from 20 RBs to 12 RBs, thereby reducing the bandwidth occupied by the SSB to 2.16 MHz. This approach allows the SSBs to be correctly placed in 3 MHz channel bandwidths, and therefore we believe this approach could be reused for NTN networks supporting 3 MHz channel bandwidth.

Proposal 3: For NR-NTN UE supporting 3MHz channel bandwidth in the FR1-NTN bands, the reception of 12 RB PBCH based on RB-level puncturing should be supported.

- Conclusion

This contribution provides our initial views on system parameters for NTN support spectrum less than 5 MHz. The following proposals are concluded as follows:

Proposal 1: 3MHz channel bandwidth for NR-NTN in the FR1-NTN bands could be mandatory in Rel-20 if the introduction of NTN UE supporting 3MHz CBW does not lead to a fragmented market.

Proposal 2: RAN4 need to send LS to RAN2 to update the description of the UE capabilities signalling support3MHz-ChannelBW-Symmetric-r18 and support3MHz-ChannelBW-Asymmetric-r18.

Proposal 3: For NR-NTN UE supporting 3MHz channel bandwidth in the FR1-NTN bands, the reception of 12 RB PBCH based on RB-level puncturing should be supported.

- References

- R4-2502282, WF for [114][311] NR_IoT_NTN_less_than_5MHz_UERF, Xiaomi, RAN4 #114