3GPP TSG-RAN WG4 Meeting #114-bis                                                                                       R4-2503448

Wuhan, CN, Apr. 7 –Apr. 11, 2025

Source: 	CATT

Title: 	Discussion on UE Tx requirement for NTN support HPUE

Agenda item:	7.10.2.2.1

Document for:	Approval

- Introduction

In RAN4 #114 meeting, a WF [1] on UE Tx requirement for NTN support HPUE was agreed. However, there are still some remaining issues need to be discussed in the next meeting.

In this contribution, we would like to share some initial views on UE Tx requirement for NTN support HPUE for NR NTN HPUE.

- Discussion

The open issues on UE Tx requirements for NTN support HPUE was concluded during the previous meetings, as shown below:

# NR-NTN HPUE Tx requirements Issue 2-1: NR-NTN PC2 Agreement: - Define the requirements for NR-NTN PC2 covering the following cases and try to define the common requirements as much as possible - 1Tx handheld - 2Tx handheld - 1Tx non-handheld - 2Tx non-handheld - If the difficulty is identified to define the requirements for the cases above, discuss how to down-select the cases in Rel-19 Issue 2-3: NR-NTN PC1.5 Agreement: - Define the requirements for PC1.5 for non-handheld NR-NTN UE with 2Tx in Rel-19. - FFS on other cases if time is allowed Issue 2-3: NR-NTN PC1 Agreement: - RAN4 to consider non-handheld UE with 1Tx and [4Tx] in Rel-19. Issue 2-4-1: PC2 ACLR Agreement: - PC2 ACLR is [30 or 31] dBc for NR-NTN UE Issue 2-4-2: PC1.5 ACLR Agreement: - PC1.5 ACLR is [30 or 31] dBc for NR-NTN UE Issue 2-4-3: PC1 ACLR Agreement: - PC1 ACLR is [31 or 37] dBc for NR-NTN UE - Further study averaging value, i.e. 34. Issue 2-5: SEM Agreement: - Apply SEM from NR-NTN PC3 to other PCs for NR-NTN as baseline. - Further discuss whether and how to capture the FCC 47 CFR part 25 specification in clause 202 Issue 2-6: MPR/A-MPR Agreement: - For PC 2 and PC 1.5 MPR: - Option 1: Existing MPR requirements for PC2 and PC1.5 in TS 38.101-1 apply, if PC2/PC1.5 ACLR reused TN PC2/PC1.5 ACLR as 31 dBc. - Option 2: Aim for MPR analysis for PC2 and PC1.5 if PC/PC1.5 ACLR agreed as 30 dBc. - For PC 1 MPR: - Option 1: If PC1 ACLR reused TN PC1 ACLR as 37 dBc: - Option 1-1: Existing MPR requirements for PC1 in TS 38.101-1 applies - Option 1-2: More analysis is needed to revisit the MPR requirements from TN spec. - Option 2: Aim for MPR analysis for PC1 for other ACLR values [31 or 34] dBc - Aim for MPR/A-MPR analysis in #114bis for different power classes - For PA model calibration: - DFT-s-OFDM QPSK 20 MHz - 100RB0 - 4dB post PA loss - 1dB MPR - Carrier leakage 28dBc - IQ image:28 dBc - CIM3: 60 dBc - ACLR: PC2 [31]dB, PC1.5 [31]dB, PC1 [31]dB - SEM: existing PC3 SEM - EVM: 17.5%

For NTN HPUE, the results of the coexistence study show that there is some difference in the required ACIR values for NTN HPUE at 1.5 km isolation distance and 0 km isolation distance. For 1.5 km isolation distances, the ACIR is relatively relaxed, meaning that defining a less stringent ACLR requirement may not cause problems for NTN HPUE. However, for 0 km isolation distances, some companies have observed that ACIR requirements are still stringent.

The TN HPUE ACLR requirements for PC 3/2/1.5/1 are shown as below in TS 38.101-1:

Table 6.5.2.4.1-2: NR ACLR requirement Power class 1; Power class 1.5; Power class 2; Power class 3. NR ACLR; 37 dB; 31 dB; 31 dB; 30 dB. NOTE 1: Void.

For PC 2 NTN HPUE ACLR requirements, it was discussed at the last meeting and decided to reuse the TN HPUE ACLR requirements. The agreements focused on how to reuse the ACLR requirements for TN HPUE, and resulted in two approaches. Regarding to simulation results, it seems acceptable to reuse PC 3 TN HPUE ACLR requirements for PC 2 NTN HPUE. However, considering the previous 3GPP approach, the requirements from same power class will be reused (when 3GPP try to reuse something), it is also reasonable to reuse the PC 2 TN ACLR requirements. In addition, with the increasing number of NTN HPUE users, the current simulation assumptions of the coexistence simulation may not be entirely realistic. Considering that ACLR requirement is the key point for coexistence, it might be wise to reserve some margin for ACLR requirements to ensure that network deployments are future-proof.

Therefore, it is acceptable to reuse PC 3 TN HPUE ACLR requirement for PC 2 NTN HPUE. Meanwhile, reuse of PC 2 TN HPUE ACLR requirement could also be considered.

Proposal 1: It is acceptable to reuse PC 3 TN HPUE ACLR requirement for PC 2 NTN HPUE. Meanwhile, reuse of PC 2 TN HPUE ACLR requirement could also be considered.

For PC 1.5 NTN HPUE ACLR requirements, the same approach could be considered. It is acceptable to reuse PC 3 TN HPUE ACLR requirement for PC 1.5 NTN HPUE. Meanwhile, reuse of PC 1.5 TN HPUE ACLR requirement could also be considered

Proposal 2: It is acceptable to reuse PC 3 TN HPUE ACLR requirement for PC 1.5 NTN HPUE. Meanwhile, reuse of PC 1.5 TN HPUE ACLR requirement could also be considered.

- Conclusion

This contribution provides our views on how to conduct the UE RF requirements for NR NTN HPUE. The following proposals are concluded as follows:

Proposal 1: It is acceptable to reuse PC 3 TN HPUE ACLR requirement for PC 2 NTN HPUE. Meanwhile, reuse of PC 2 TN HPUE ACLR requirement could also be considered.

Proposal 2: It is acceptable to reuse PC 3 TN HPUE ACLR requirement for PC 1.5 NTN HPUE. Meanwhile, reuse of PC 1.5 TN HPUE ACLR requirement could also be considered.

- References

- R4-2502954, WF on UE RF requirements for NTN HPUE, Samsung, RAN4 #114