3GPP TSG-RAN WG4 Meeting # 114bis                                                        R4-2503471

Wuhan, Hubei, China, 07th – 11th April, 2025

Title: 	                 Discussion on RF requirements for CW for D1T1

Source: 	CATT

Agenda item:	7.24.2.3

Document for:	Discussion

- Introduction

This contribution provides our further analysis for RF Requirements for CW for scenario D1T1-B according to the WF [1].

- RF Requirement for CW for D1T1

2.1 General

In WID [2], RF requirements for Type 1-C Ambient-IoT BS need to be specified. Scenario is D1T1-B, R2D is located in DL spectrum and D2R and CW are located in UL spectrum. Spectrum deployment in-band to NR and standalone, with A-IoT BS located indoor. R2D supports only OOK-4 modulation, one solution for CP handling. D2R backscattering supports only OOK and BPSK modulations. For scenario D1T1-B, the CW is transmitted from outside the topology.

2.2 CW requirements

2.3.1 Output power

Figure 1： Distribution diagram of BSs, devices and CW nodes in A-IoT system

For the D1T1-B scenario, the CW source is deployed separately from the D2R A-IoT BS. CW interference signal level at A-IoT BS connector can’t block the reception of the reader. In addition, the total cancellation capability of CW interference signals can also be achieved.

The CW signal level at A-IoT BS receiver connector is depended on the CW output power, the distance between the CW node and the A-IoT BS, and the Tx and Rx antenna gain.

Figure 1 is distribution diagram of BSs, devices and CW nodes in A-IoT system, and it is based on TR [3]. In the D1T1-B case, the distance between readers is 20m, the maximum distance between the reader and the device is 15m, and minimum distance between active readers is 60m, CW nodes are co-located with A-IoT BSs. For each actived device, the nearest CW node will be activated during the simulation, and CW node is not co-located with any activated reader. It can be seen that the minimum distance between actived CW node and actived reader is equal to the distance between two readers.

Use InF-DH NLOS pathloss models, the isolation is 61dB at 20 meters distance. CW nodes and A-IoT nodes are deployed at the same height, and for TX and RX antennas, the antenna gain is about -5 dB at 90° offset from the boresight. The total isolation is 61+5+5=71dB. When the CW output power is 23dBm and 33dBm, the corresponding input CW interference signal level at the A-IoT BS receiver connector is -48dBm and -38dBm, respectively. The narrowband blocking requirements of existing Medium Range NB-IoT BS is -44dBm, A-IoT BS can reuse this value, therefore, the receiver will be blocked when CW output power is 33dBm.

Observation 1:  The receiver of A-IoT BS will be blocked when CW output power is 33dBm.

Proposal 1: CW output power should be less than 27dBm.

2.3.2 ON/OFF power

In the D1T1-B scenario, CW is transmitted from outside the topology, CW nodes only transmit signals and don’t receive signals, and the CW/D2R spectrum and R2D spectrum are different, so the ON/OFF power requirement is not necessary.

Proposal 2:  The ON/OFF power requirement is not necessary.

2.3.3 Phase noise

The phase noise frequency points overlap with the frequency range of the unwanted emission, the unwanted spurious is defined based on the continuous spectrum, and the impact of the phase noise can be included in it, so the requirement for the unwanted emission can be defined, and the phase noise requirement for the CW signal is not required.

Proposal 3: The impact of the phase noise of the CW signal can be included in unwanted emission requirement, so phase noise requirement is not required.

2.3.4 Unwanted emission

The unwanted emission requirement is needed, and the value of it can be analysed based on the output power of CW signal and frequency offset of CW signal and D2R signal.

Proposal 4: The unwanted emission requirement is needed.

Proposal 5: The value of unwanted emission requirement can be analysed based on the output power of CW signal and frequency offset of CW signal and D2R signal.

2.3.5 Intermodulation

If A-IoT BS and CW node are co-located, and the CW node works with other devices and other A-IoT BS, if the A-IoT BS receiver and the co-located CW node operate simultaneously, assume the coupling loss between them is 30dB, then the CW interference signal level at the A-IoT BS input will be large and it will have a great impact on the Rx blocking requirement of A-IoT BS, even will block the A-IoT BS receiver, so this scenario is not recommended.

Proposal 6: Scenario in which A-IoT BS receiver and the co-located CW node operate simultaneously is not recommended.

Since continuous waves with multiple tones are allowed in RAN1, reverse IM products from the same location/nearby continuous wave transmitter should be suppressed. However, in WID [2], it is specified that carrier wave is only waveform 1 and without hopping, so it is a single tone, and the intermodulation requirement is not needed.

Proposal 7: The intermodulation requirement is not needed.

- Summary

This contribution provides our analysis for RF Requirements for CW for scenario D1T1-B according to the WF [1], and our observations and proposals can be summarized in the following:

Observation 1:  The receiver of A-IoT BS will be blocked when CW output power is 33dBm.

Proposal 1: CW output power should be less than 27dBm.

Proposal 2:  The ON/OFF power requirement is not necessary.

Proposal 3: The impact of the phase noise of the CW signal can be included in unwanted emission requirement, so phase noise requirement is not required.

Proposal 4: The unwanted emission requirement is needed.

Proposal 5: The value of unwanted emission requirement can be analysed based on the output power of CW signal and frequency offset of CW signal and D2R signal.

Proposal 6: Scenario in which A-IoT BS receiver and the co-located CW node operate simultaneously is not recommended.

Proposal 7: The intermodulation requirement is not needed.

Reference

[1] R4-2502859, “WF on requirements for A-IoT BS and CW”, Huawei, RAN4#114

[2] RP-243326, “Solutions for Ambient IoT (Internet of Things) in NR”, Huawei, RAN#106

[3] TR 38.769