3GPP TSG-RAN WG4 Meeting # 114bis                                                        R4-2503472

Wuhan, Hubei, China, 07th – 11th April, 2025

Title: 	                 Discussion on system parameters for A-IoT

Source: 	CATT

Agenda item:	7.24.1

Document for:	Discussion

- Introduction

This contribution provides our further analysis for R2D and D2R system parameters according to the WF [1].

- Discussion

2.1 System parameter for R2D

- 2.1.1 R2D transmission bandwidth

RAN1#120 has conclusions on R2D transmission bandwidth, they won’t specify exact R2D transmission bandwidth Btx,R2D, they only specify the minimum Btx,R2D # of PRBs associated to each M value,  whether to specify the maximum Btx,R2D # of PRBs can be decided by RAN4. Reader can transmit any Btx,R2D subject to this minimum Btx,R2D and the agreed 15 kHz subcarrier spacing, and subject to other limitations that are up to RAN4. For R2D, for the OOK-4 modulation for M-chip per OFDM symbol transmission, the maximum M value is no less than 16, and to be down-selected from 32, 24 or 16.

The following table shows the required RB number for different date rate

Table 1: Starting point for M values and the associated minimum Btx,R2D value [2]

| M | Minimum Btx,R2D # of PRBs |
| --- | --- |
| 1 | 1 |
| 2 | 1 |
| 4 | 1 |
| 6 | 1 |
| 8 | 2 |
| 12 | 2 |
| 16 | 2 |
| 24 | 3 |
| 32 | 4 |

Observation 1: RAN1 won’t specify exact R2D transmission bandwidth Btx,R2D, whether to specify the maximum Btx,R2D # of PRBs can be decided by RAN4.

In WF [1], R2D transmission bandwidths are configured as 1PRB, 2PRB, 3PRB and 4PRB, and 15kHz SCS are used, and these configurations are consistent with RAN1's conclusions.

Whether multiple RBs resources are continuous will affect the definition of system parameters and RF requirements. Therefore, RAN4 to specify A-IoT requirements based on continuous RB allocation in Rel-19.

Proposal 1: RAN4 to specify A-IoT requirements based on continuous RB allocation in Rel-19.

- 2.1.2 R2D channel bandwidth

R2D channel bandwidth includes transmission bandwidth and guard band. The R2D SCS is defined as 15kHz, so the guardband adjacent to the low channel edge should be symmetric to the guardband adjacent to the high channel edge.

Proposal 2: The guardband adjacent to the low channel edge should be symmetric to the guardband adjacent to the high channel edge.

R2D channel bandwidth can reuse the channel bandwidth of NB-IoT standalone mode, i.e. 200kHz/1RB, 400kHz/2RB, 600kHz/3RB, 800kHz/4RB. The channel bandwidth, transmission bandwidth configuration, and guardband for different number of RBs are shown in figure 1 and table2.

- 1RB

- 2RB

- 3RB

- 4RB

Figure 1 Definition of Channel Bandwidth and Transmission Bandwidth Configuration for R2D

Table 2: Channel bandwidth and Transmission bandwidth configuration for R2D

| Transmission bandwidth configuration NRB | 1RB | 2RB | 3RB | 4RB |
| --- | --- | --- | --- | --- |
| Channel bandwidth (kHz) | 200 | 400 | 600 | 800 |
| Transmission bandwidth configuration Ntone 15kHz | 12 | 24 | 36 | 48 |
| Guardband (kHz) | 10 | 20 | 30 | 40 |

Proposal 3: The channel bandwidth, transmission bandwidth configuration, and guardband for different number of RBs are shown in figure 1 and table2.

- 2.1.3 R2D channel raster

The channel raster design should guarantee flexible placement of the A-IoT RB, and the A-IoT RB should be aligned with the NR RB. To do A-IoT in NR bands with 100 kHz channel raster, the channel raster of R2D should be 10kHz.

Proposal 4: The channel raster of R2D should be 10kHz.

2.2 System parameter for D2R

AN1#120 derived conclusions on some parameters related to the D2R channel bandwidth. For D2R, only 2SB (Double sideband) modulation is supported. For a D2R transmission, it is up to device implementation to use OOK or BPSK modulation. It is assumed that a device operates consistently with the same D2R modulation in the same inventory/command round.

Define D2R channel bandwidth for A-IoT device need to consider 2SB, D2R maximum data rate, SFO, FDMA and filter performance. We can decide the channel bandwidth after RAN1 has conclusions on D2R maximum data rate and SFO.

Proposal 5: We can decide the channel bandwidth of D2R after RAN1 has conclusions on D2R maximum data rate and SFO.

For device, symmetrical DL and UL RB numbers are better for implementation, and some standardized requirements definitions can be troublesome in the case of asymmetry.

Proposal 6: It is recommended that the DL and UL RB numbers of the device are symmetrical.

2.3 Deployment scenario

Practically if there are some certain rules to ensure that there is no indoor NR UE, then the same rules may also ensure that there is no NR UE near doorways, hence RAN4 can focus on specifying requirements for the scenario where all NR UEs are out door without capturing side conditions of outdoor UE assumption.

Proposal 7: RAN4 can focus on specifying requirements for the scenario where all NR UEs are out door without capturing side conditions of outdoor UE assumption.

- Summary

This contribution provides our further analysis for A-IoT system parameters. We have the following observation and proposals.

Observation 1: RAN1 won’t specify exact R2D transmission bandwidth Btx,R2D, whether to specify the maximum Btx,R2D # of PRBs can be decided by RAN4.

Proposal 1: RAN4 to specify A-IoT requirements based on continuous RB allocation in Rel-19.

Proposal 2: The guardband adjacent to the low channel edge should be symmetric to the guardband adjacent to the high channel edge.

Proposal 3: The channel bandwidth, transmission bandwidth configuration, and guardband for different number of RBs are shown in figure 1 and table2.

Proposal 4: The channel raster of R2D should be 10kHz.

Proposal 5: We can decide the channel bandwidth of D2R after RAN1 has conclusions on D2R maximum data rate and SFO.

Proposal 6: It is recommended that the DL and UL RB numbers of the device are symmetrical.

Proposal 7: RAN4 can focus on specifying requirements for the scenario where all NR UEs are out door without capturing side conditions of outdoor UE assumption.

Reference

[1] R4-2502858, “WF on requirements for system parameters and A-IOT device”, CMCC, RAN4#114

[2] TR 38.769