3GPP TSG RAN WG4 Meeting #114bis		R4-2503478

Wuhan, Hubei, China, April 07th – 11th , 2025

Title: 	On UE RF requirements and UE capability for low band CA via switching

Agenda Item:	7.7.2

Source: 	CATT

Document for:	Discussion

# 1 Introduction

In RAN4#114 which is the first working group meeting after the approval of the WID on low band CA via switching, RAN4 commenced the discussion and made some progress with a WF agreed [1]. And there are several issues which are still open as seen below:

Issue 2: Switching period values - Define switching periods between FDD low band and SDL low band with the following values as a UE capability to accommodate different UE implementations - 35us, 140us - RAN4 to further discuss 13us - LS will capture the currently confirmed values - It is not precluded to send another LS with more values in future meetings Issue 3: Switching period capability - Discuss whether the switching delay capability can be per band combination Issue 4: Time mask for switching - Discuss the time mask for switching based on the following options: - Option 1: Switching period can be either within carrier 1 (FDD) or within carrier 2 (SDL) - Option 2: Switching period is always within the switch-from carrier - Option 3: Switching period is contained within the switching gap configured by the network - Other options are not precluded

In this contribution, we present our views on these identified issues.

# 2 Discussion

Switching period values

In last meeting, in addition to the agreed values of 35μs and 140μs for switching period, other values such as 13us are also proposed [2]. In our understanding, low band CA via switching may benefit from a smaller switching period from different aspects: 1) Fast react to radio conditions thus able to improve overall achievable system performance; 2) More flexible and less stringent on semi-static switching pattern design. Therefore, 13us switching period could be considered.

Proposal 1: RAN4 to consider an additional switching period value of 13μs.

Switching period capability

Usually in UE implementation, low bands are treated in one group. And this feature is about switching in a band combination only consisting of low bands, therefore, the granularity of the switching is unnecessary on a band combination basis, i.e., can be per-UE basis.

Proposal 2: Switching period capability for low band CA via switching is per-UE basis.

Time mask for switching

Switching period should always be located at Scell (SDL). If a switching period is within the Pcell (FDD), the both UL and DL of the FDD cell will be blanked during the switching, where UL of the FDD cell is the only uplink in this band combination. Hence, it is preferrable to ensure the performance of the Pcell.

Proposal 3: Switching period is always within Carrier 2 (SDL).

# 3 Conclusion

In this contribution, we have the following proposals for UE RF requirements and UE capability for low band CA via switching:

Proposal 1: RAN4 to consider an additional switching period value of 13μs.

Proposal 2: Switching period capability for low band CA via switching is per-UE basis.

Proposal 3: Switching period is always within Carrier 2 (SDL).

# Reference

[1] R4-2503016, “WF on low-low band switching”, Apple

[2] R4-2501572, “Time masks for LB-LB CA switching”, Ericsson