3GPP TSG-RAN WG4 Meeting #114bis		R4-2503482

Wuhan, Hubei, China, Apr. 7th – 11th, 2025

Title:	Reply LS on UE capability for FDD-FDD inter-band CA simultaneous Rx/Tx

Response to:	R2-2401572

Release:	Release 19

Work Item:	NR_CADC_SUL_R19

Source:	RAN4

To:	RAN2

Cc:	RAN1

Contact person:

Name:	Aijun CAO

E-mail Address:	ajcao@catt.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

# 1	Overall description

RAN4 thanks RAN2 for the reply LS on UE capability for FDD-FDD inter-band CA simultaneous Rx/Tx and sharing RAN2 agreements and questions on for the example band the 2UL/2DL inter-band FDD-FDD CA_n5-n8 with the scheduling restriction of non-concurrent n5 DL and n8 UL.

For carrier aggregation on one low-low band combination where UL of one band is partially overlapped with DL of the other band, such as CA_n5-n8, RAN4 has studied various potential RF architectures, and in general these RF architectures can be categorized into several options by taking CA_n5-n8 as the example band combination:

- Option 1: Only support 1UL/2DL CA. Single UL in n5 (already in 38.101)

- Option 2: Support 2UL/2DL CA using non-concurrent n5 DL and n8 UL

- Option 3: Support 2UL/2DL CA. Dedicated filter to allow simultaneous n5 DL and n8 UL

where Option 1 and 2 are associated with full band n5 and n8 RF filters implementation, and Option 3 with dedicated RF filters implementation with partial frequency range. It is noteworthy that RF architecture implementing Option 3 does not have any scheduling restriction on both bands.

Moreover, RAN4 has already specified Option 1 in Rel-18, and intends to standardize Option 2 in Rel-19, which requires a future-proof signaling design to support that the capabilities for the simultaneous operation of nA-UL with nB-DL and nA-DL with nB-UL can be reported independently, as requested in the original LS R4-2420410. If only Option 2 is assumed, scheduling restriction at network can work however it is not a future-proof solution.

Regarding the questions raised, in-line answers could be found below:

- Whether it’s possible for a UE to support simultaneous Rx/Tx in inter-band FDD-FDD CA_n5-n8 without any scheduling restriction, or whether there may be a case where a UE can support simultaneous Rx/Tx in some part(s) of the overlapping FDD bands? If possible, from which release version?

Answer 1:     As stated above, there are possible RF architectures for a UE to support simultaneous Rx/Tx in inter-band FDD-FDD CA_n5-n8 without any scheduling restriction. Such support could be considered in Rel-20 or later.

- If a UE may be able to support simultaneous Rx/Tx in inter-band FDD-FDD CA_n5-n8 (or some other inter-band FDD-FDD CA combinations) in some form, whether it requires additional UE capabilities for the support/requirement of other features, e.g., full-duplex, switching time, or some frequency gap/guard band?

Answer 2:     At present, RAN4 does not identify additional UE capabilities or other features for supporting simultaneous Rx/Tx in inter-band FDD-FDD CA_n5-n8 or other inter-band FDD-FDD CA combinations with partially overlapped frequency range.

# 2	Actions

To RAN2

ACTION: 	RAN4 respectfully asks RAN2 to take the above information into account in the corresponding signalling design.

# 3	Dates of next TSG-RAN WG2 meetings

3GPP TSG RAN WG4#115	May 19th – 23rd, 2025			Malta