3GPP TSG-RAN WG4 Meeting #114bis												R4-2503494

Wuhan, China, 7 – 11 April, 2025

Agenda item:			7.24.4

Source:	Huawei, HiSilicon

Title:	on ambient IoT OTA tests

Document for:	approval

# Introduction

The previous meeting RAN4 #114 reached agreement on some aspects of OTA tests [1] and left the following issues to be agreed. In addition, Measurement Uncertainty (MU) has yet to be discussed. This contribution further discusses those issues.

Issue 4-1-4: Energy source

Agreement：

- Further discuss the energy source issue for device 1.

Issue 4-2-2: Device positioning guidance

Agreement：

Take following as a starting point and further discuss the details.

- The traditional test system of AC for handheld UE radiated testing can be considered as the starting point for A-IoT device testing.

- The following test setup parameters shall be defined for A-IoT device OTA testing:

  - DCW2D: distance between CW node and DUT

  - DMA2D: distance between measurement antenna and DUT

  - IsoCW2M: Isolation between CW node and measurement antenna

# Discussion

## Test setup

As discussed in R4-2500209 [2], the sizes of commercial RFID are around 15cm or under. There was discussion in last meeting about whether to reduce the size of quiet zone in order to reduce the chamber size for simplification. However, based on the calculation using the principle defined in TR 38.870 clause 7.7 for Sub 1GHz, a reduced quiet zone won’t significantly reduce the minimum range length. Assuming ambient IoT devices would be similar in size, this means that the current quiet zone size of 30cm in anechoic chambers should be sufficient to perform OTA tests for ambient IoT device type 1 and for future device types.

Proposal 1: keep the current 30cm quiet zone for anechoic chamber as specified in 38.870.

Proposal 2: use the same quiet zone ripple test procedure as specified in 38.870.

Proposal 3: use the same minimum range length as specified in 38.870

The reader and CW could be at the same distance from DUT (Device Under Test) as this arrangement would enable test antennas sharing a boom in anechoic chambers. In addition, there is no specific reason or need for reader and CW to have different distances from DUT.

Proposal 4: set both DCW2D and DMA2D equal to the minimum range length or larger.

Although CW and reader have the same distance to DUT, they are however separated in space. For example, they can be on the same arch or spherical surface. To better verify the radiation performance and limit the number of the measurement grids number, it is sufficient to scan hemisphere of the device. The position of A-IoT device (DUT) is on the X-Y plane inside the quiet zone with its long side aligned with Y-axis, short side with X-axis and its reflective side facing positive Z-axis with the reference coordinate system as defend in 38.870 as shown in Fig. 1. And the CW and reader are located above the DUT.

Figure 1 A-IoT device position with coordinate system

Proposal 5: The position of A-IoT device (DUT) is on the X-Y plane inside the quiet zone with its long side aligned with X-axis, short side with Y-axis and its reflective side facing Z-axis. And the CW and reader are located above the DUT

The values are [0], [30], [60], [90], [120], [150], [180], [210], [240], [270], [300], [330] and can be achieved using a turn-table.

Considering the test setup which is widely used for RFID, where four antennas are used at 0, 0 degrees.  Different from RFID, separate CW node should be considered as shown in figure 2.  During the actual measurement, only two antennas, namely CW and reader, are active. One reader antenna moves through positions from ant-1 to ant-4 , i.e. spherical coordinate in (, , R), (0, 0, R), (30, 0, R), (60, 0, R), and (90, 0, R), for a given CW position. The CW positions are (45, 180, R), where R is the minimum range length or larger. The setup shown in figure 2 can be achieved with two antennas, i.e. one for reader, the other for CW, or multiple antennas, e.g. 4 fixed antennas for reader.

Proposal 6: The reader is placed under spherical coordinate in (, , R), (0, 0, R), (30, 0, R), (60, 0, R), and (90, 0, R), for a given CW position. The CW position is (45, 180, R), subject to conclusions from RF core requirements discussion.

If RF core requirements necessitate more than one CW positions, the measurement process is repeated for each CW position.

Figure 2 proposed measurement setup

Proposal 7: use 30 degree step in in X-Y plane

Some isolation between CW and reader would be needed albeit interference cancellation could be available.

Proposal 8: use an isolation of 50 dB between CW and reader

## Measurement Uncertainty

Based on analysis above, it could be observed most TRP/TRS test setup/envorionment can be reused.The Measurement Uncertainty (MU) for ambient IoT OTA measurement can be based on the TRP and TRS MU tables in 38.870. The components serving the same purpose can be reused. The proposed changes are highlighted in the table below.

Table 1 Uncertainty contributions measurement for anechoic chamber method

| UID | Description of uncertainty contribution | Details in clause |
| --- | --- | --- |
| Stage 2: DUT measurement (Figure 7.2-1, Figure 7.2-2) |  |  |
| 1 | Mismatch of transmitter chain | B.2.1 |
| 2 | Insertion loss of transmitter chain | B.2.2 |
| 3 | Influence of the measurement antenna cable | B.2.3 |
| 4 | CW for backscatter measurement or R2D for sensitivity measurement: uncertainty of the absolute output level |  |
| 5 | Sensitivity measurement: output level step resolution | B.2.6 |
| 6 | Measurement distance | B.2.7 |
| 7 | Quality of quiet zone | B.2.8 |
| 8 | DUT sensitivity drift | B.2.10 |
| 9 | Uncertainty related to the use of phantoms | B.2.11 |
| 10 | Coarse sampling grid | B.2.12 |
| 11 | Random uncertainty | B.2.13 |
| 12 | Frequency Response | B.2.14 |
| Stage 1: Calibration measurement, network analyzer method (Figure 7.3-1) |  |  |
| 13 | Uncertainty of network analyzer | B.2.15 |
| 14 | Mismatch of transmitter chain | B.2.1 |
| 15 | Insertion loss of transmitter chain | B.2.2 |
| 16 | Mismatch in the connection of calibration antenna | B.2.1 |
| 17 | Influence of the calibration antenna feed cable | B.2.3 |
| 18 | Influence of the measurement antenna cable | B.2.3 |
| 19 | Uncertainty of the absolute gain/radiation efficiency of the calibration antenna | B.2.16 |
| 20 | Measurement distance | B.2.7 |
| 21 | Quality of quiet zone | B.2.8 |
| Systematic Errors |  |  |
| 22 | Systematic Error related to TRS grids | B.2.12 |

Table 2: Preliminary example of uncertainty budget for sensitivity or backscatter (see entry 4) measurement for anechoic chamber method

| UID | Uncertainty Source | Comment | Uncertainty Value [dB] | Prob Distr | Div | ci | Standard Uncertainty [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |
| 1 | Mismatch of receiver chain | Гcomm tester < 0.29 Гmeasurement antenna < 0.5 Cable attenuation > 3dB | 0.22 | U-shaped | 1.4 | 1 | 0.16 |
| 2 | Insertion loss of receiver chain | Systematic with Stage 1 (=> cancels) | 0 | Rectangular | 1.7 | 1 | 0.00 |
| 3 | Influence of the measurement antenna cable | Systematic with Stage 1 (=> cancels) | 0 | Rectangular | 1.7 | 1 | 0.00 |
| 4 | CW for backscatter measurement or R2D for sensitivity measurement uncertainty of the absolute output level | Manufacturer’s data sheet | 1.3 | Normal | 2 | 1 | 0.65 |
| 5 | Sensitivity measurement: output level step resolution | Systematic error that can be corrected | 0 | Rectangular | 1.73 | 1 | 0.00 |
| 6 | Measurement distance | DUT is not offset for hand-only phantom testing | 0 | Rectangular | 1.7 | 1 | 0.00 |
| 7 | Quality of quiet zone | Surface standard deviation of power measurements in ripple test | 0.5 | Actual | 1 | 1 | 0.50 |
| 8 | DUT sensitivity drift | Drift | 0.2 | Rectangular | 1.7 | 1 | 0.12 |
| 9 | Uncertainty related to the use of phantoms | Material Dielectric Constant, Material Conductivity, Geometry/Shape (incl. spacer), Data Mode Fixture | 0.64 | Rectangular | 1.7 | 1 | 0.37 |
| 10 | Coarse sampling grid | Sampling grids per Table B.2.12-1 | 0.10 | Actual | 1 | 1 | 0.10 |
| 11 | Random Uncertainty | Fixed MU to account for all the unknown, unquantifiable, etc. uncertainties including digital error rate | 0.4 | Normal | 2 | 1 | 0.20 |
| 12 | Frequency Response | Included in the output level step resolution | 0 | Rectangular | 1.7 | 1 | 0.00 |
| Stage 1: Calibration measurement, network analyzer method |  |  |  |  |  |  |  |
| 13 | Uncertainty of network analyzer | From datasheet of VNA with assessed transmission coefficients | 0.2 | Normal | 2 | 1 | 0.10 |
| 14 | Mismatch of receiver chain | Taken into account in VNA uncertainty term | 0 | U-shaped | 1.4 | 1 | 0.00 |
| 15 | Insertion loss of receiver chain | Systematic with Stage 2 (=> cancels) | 0 | Rectangular | 1.7 | 1 | 0.00 |
| 16 | Mismatch in the connection of calibration antenna | Taken in to account in VNA setup uncertainty | 0 | U-shaped | 1.4 | 1 | 0.00 |
| 17 | Influence of the calibration antenna feed cable | Gain calibration with a dipole | 0.3 | Rectangular | 1.7 | 1 | 0.17 |
| 18 | Influence of the measurement antenna cable | Systematic with Stage 2 (=> cancels) | 0 | Rectangular | 1.7 | 1 | 0.00 |
| 19 | Uncertainty of the absolute gain/ radiation efficiency of the calibration antenna | Calibration report with traceability to a National Metrology Institute | 0.58 | Normal | 2 | 1 | 0.29 |
| 20 | Measurement distance | Dipole: aligned with phase center | 0 | Rectangular | 1.7 | 1 | 0.00 |
| 21 | Quality of the Quiet Zone | Peak-to-null ripple | 0.5 | Rectangular | 1.7 | 1 | 0.29 |
| Combined standard uncertainty [dB] |  |  |  |  |  |  | 0.98 |
| Expanded uncertainty [dB] (Confidence interval of 95 %) |  |  |  |  |  |  | 1.65 |
| 22 | Systematic Error related to TRS grids | mean error | 0 | Actual | 1 | 1 | 0.00 |
| Total Expanded uncertainty [dB] (Confidence interval of 95 %) |  |  |  |  |  |  | 1.65 |

Table 2 is in fact two tables, one for backscattering, the other for sensitivity, depending on the value in entry 4.

Proposal 9: use Table 1 and 2 in R4-2503494 as baseline for MU estimation.

## Test procedure

Regarding the test procedure, we take sensitivity and backscatter power requirements to elaborate the OTA test procedure which should also apply to other requirements. For sensitivity, to better evaluate the Rx performance, the situation where D2R link becomes the bottleneck should be avoided. Thus, the configuration of CW power should be high enough for sensitivity test. CW power at DUT around -10dBm would be sufficient to ensure D2R link is not the bottleneck. Thus, the test procedure for sensitivity is proposed as follows:

Step 1：Conduct the calibration procedure as defined in clause 7.3 of 38.870

Step 2:   Place the DUT inside the QZ following the positioning guideline defined for A-IoT device.

Step 3：Set the target test frequency and transmit power for signal generator and CW signal.

- The transmit power of signal generator shall be set such that the received power at DUT’s antenna is lower than the minimum reference sensitivity requirement of device.

- The CW signal level is set as -10 dBm at DUT’s antenna

Step 4: Signal generators shall transmit the defined signalling (e.g. paging) and CW signal is transmitted continuously within the time period, during which A-IoT device is expected to response.

Step 5:  Determine whether DUT can send the correct response with correct timing relationship. If correct response is received, the sensitivity of this test point shall be calculated by use of the transmit power of signal generators and calibrated pathloss. Otherwise, repeat from step 3 and increase the transmit power of signal generator by 0.5 dB.

Step 6: Repeat step 3~5 for different measurement grid point.

- Note 1: The selected command used for test could be refined based on progress in other working groups.

Using the same principle for sensitivity test, the D2R link is not expected to be the bottleneck of Tx requirements testing.  For each measurement gird point, we propose to set the transmit power of signal generator 5 dB higher than that recorded for sensitivity test.

Step 1：Conduct the calibration procedure as defined in clause 7.3 of 38.870.

Step 2:   Place the DUT inside the QZ following the positioning guideline defined for A-IoT device.

Step 3：Set the target test frequency and transmit power for signal generator and CW signal.

- The transmit power of signal generator shall be set such that the received power at DUT’s antenna is equal to minimum sensitivity of device for the measurement grid + 5dB

- The CW signal level is set as TBD

Step 4: Signal generators shall transmit the defined signalling (e.g. paging) and CW signal is transmitted continuously within the time period, during which A-IoT device is expected to response.

Step 5:  Determine whether DUT can send the correct response with correct timing relationship. If correct response is received, the backscatter power at DUT’s antenna shall be calculated by calibrating input power at DUT’s antenna. Alternatively, backscatter loss can be calculated by subtracting the transmitted power.

Step 6: Repeat step 3~5 for different measurement grid point.

- Note 1: The selected command used for test could be refined based on progress in other working groups.

- Note 2: The power level of CW signal to be updated based on RF requirement discussion.

Proposal 10:  agree on following test procedures for sensitivity and backscatter test.

For sensitivity:

Step 1：Conduct the calibration procedure as defined in clause 7.3 of 38.870

Step 2:   Place the DUT inside the QZ following the positioning guideline defined for A-IoT device.

Step 3：Set the target test frequency and transmit power for signal generator and CW signal.

- The transmit power of signal generator shall be set such that the received power at DUT’s antenna is lower than the minimum reference sensitivity requirement of device.

- The CW signal level is set as -10 dBm at DUT’s antenna

Step 4: Signal generators shall transmit the defined signalling (e.g. paging) and CW signal is transmitted continuously within the time period, during which A-IoT device is expected to response.

Step 5:  Determine whether DUT can send the correct response with correct timing relationship. If correct response is received, the sensitivity of this test point shall be calculated by use of the transmit power of signal generators and calibrated pathloss. Otherwise, repeat from step 3 and increase the transmit power of signal generator by 0.5 dB.

Step 6: Repeat step 3~5 for different measurement grid point.

- Note 1: The selected command used for test could be refined based on progress in other working groups.

For backscatter:

Step 1：Conduct the calibration procedure as defined in clause 7.3 of 38.870.

Step 2:   Place the DUT inside the QZ following the positioning guideline defined for A-IoT device.

Step 3：Set the target test frequency and transmit power for signal generator and CW signal.

- The transmit power of signal generator shall be set such that the received power at DUT’s antenna is equal to minimum sensitivity of device for the measurement grid + 5dB

- The CW signal level is set as TBD

Step 4: Signal generators shall transmit the defined signalling (e.g. paging) and CW signal is transmitted continuously within the time period, during which A-IoT device is expected to response.

Step 5:  Determine whether DUT can send the correct response with correct timing relationship. If correct response is received, the backscatter power at DUT’s antenna shall be calculated by calibrating input power at DUT’s antenna. Alternatively, backscatter loss can be calculated by subtracting the transmitted power.

Step 6: Repeat step 3~5 for different measurement grid point.

- Note 1: The selected command used for test could be refined based on progress in other working groups.

- Note 2: The power level of CW signal to be updated based on RF requirement discussion.

Regarding whether 3D scan is needed (e.g. whether only to define EIPR-like requirements for single position), considering that corresponding RF requirements are undergoing, it is suggested to taking 3D scan with measurement grid mentioned above as baseline, thus we can get the EIS/EIRP for each measurement grid. Whether all grid points are needed and which results are used to formulate the RF requirements are up to RF requirements discussion.

Proposal 11: Taking 3D scan with measurement grid as baseline to get sensitivity level and backscatter results for each grid point. Whether all grid points are needed and which results are used to formulate the RF requirements are up to RF requirements discussion.

## Others

To test the performance across a band, it would be time efficient to select low, middle and high frequencies within a band. Band n8 has a bandwidth of 25MHz, 3 frequency samples should be sufficient.

Proposal 12: only test low, middle and high frequencies within a band and average the performance metrics from the three frequencies.

Based on the contributions and discussion in last meeting, companies seem to have interest to study the impact of energy source in the OTA test.  However, as presented in our paper, ambient IoT devices are low cost products and may be customized for different industry/use cases, thus the flexibility shall be considered for different device implementations. For instance, devices may have different sizes of capacitors/efficiency, which result in different charging/discharging time and dissimilar behaviours in various use cases. As the energizing behaviour and performance is not in the scope of the 3GPP discussion, RAN4 discussion shall focus on the communication performance and the energy supply can be up to implementation of device/TE.

Observation 1: The performance/behaviour of energy harvesting cannot be specified. Different implementations (e.g. capacitor size, energy source) may lead to different charging/discharging time and behaviour.

Since 3GPP/RAN4 is not going to define specific requirements/behaviour for this, the energy source shall only work as conditions to guarantee that a DUT will not fail a test due to lack of power.  For instance, for a certain type of device, the manufacture can declare the charging power level/duration. And TE can provide the required energy source long enough before sending paging to guarantee that the DUT will receive the paging when it is ready for reception for each test point.

Thus, the issue of energy source is outside the scope of this work item and should be left to implementation of test labs. However, it would suffice to state that “the DUT is assumed to have sufficient power for each test point. It is up to device manufacturers to provide detailed conditions/methods to guarantee that the DUT would not fail a test due to lack of sufficient power supply. Such detailed conditions/methods would not be specified in the specifications”.

The reason for such a statement is to accommodate a range of ambient IoT devices from various manufacturers and, as a result, to build up a sustainable eco-system for ambient IoT devices and associated testing.

Proposal 13:  adopt the following statement in the A-IoT specification that “the DUT is assumed to have sufficient power for each test point. It is up to device manufacturers to provide detailed conditions/methods to guarantee that the DUT would not fail a test due to lack of sufficient power supply. Such detailed conditions/methods would not be specified in the specifications”.

# Conclusion

This contribution provides the following proposals and observation.

Proposal 1: keep the current 30cm quiet zone for anechoic chamber as specified in 38.870.

Proposal 2: use the same quiet zone ripple test procedure as specified in 38.870.

Proposal 3: use the same minimum range length as specified in 38.870.

Proposal 4: set both DCW2D and DMA2D equal to the minimum range length or larger.

Proposal 5: The position of A-IoT device (DUT) is on the X-Y plane inside the quiet zone with its long side aligned with X-axis, short side with Y-axis and its reflective side facing Z-axis. And the CW and reader are located above the DUT.

Proposal 6: The reader is placed under spherical coordinate in (, , R), (0, 0, R), (30, 0, R), (60, 0, R), and (90, 0, R), for a given CW position. The CW positions are (45, 180, R) subject to conclusions from RF core requirements discussion.

Proposal 7: use 30 degree step in in X-Y plane

Proposal 8: use an isolation of 50 dB between CW and reader.

Proposal 9: use Table 1 and 2 in R4-2503494 as baseline for MU estimation.

Proposal 10:  agree on following test procedures for sensitivity and backscatter test.

For sensitivity:

Step 1：Conduct the calibration procedure as defined in clause 7.3 of 38.870

Step 2:   Place the DUT inside the QZ following the positioning guideline defined for A-IoT device.

Step 3：Set the target test frequency and transmit power for signal generator and CW signal.

- The transmit power of signal generator shall be set such that the received power at DUT’s antenna is lower than the minimum reference sensitivity requirement of device.

- The CW signal level is set as -10 dBm at DUT’s antenna

Step 4: Signal generators shall transmit the defined signalling (e.g. paging) and CW signal is transmitted continuously within the time period, during which A-IoT device is expected to response.

Step 5:  Determine whether DUT can send the correct response with correct timing relationship. If correct response is received, the sensitivity of this test point shall be calculated by use of the transmit power of signal generators and calibrated pathloss. Otherwise, repeat from step 3 and increase the transmit power of signal generator by 0.5 dB.

Step 6: Repeat step 3~5 for different measurement grid point.

Note 1: The selected command used for test could be refined based on progress in other working groups

For backscatter:

Step 1：Conduct the calibration procedure as defined in clause 7.3 of 38.870.

Step 2:   Place the DUT inside the QZ following the positioning guideline defined for A-IoT device.

Step 3：Set the target test frequency and transmit power for signal generator and CW signal.

- The transmit power of signal generator shall be set as that the received power at DUT’s antenna is equal to minimum sensitivity of device for the measurement grid + 5dB

- The CW signal level is set as TBD

Step 4: Signal generators shall transmit the defined signalling (e.g. paging) and CW signal is transmitted continuously within the time period, during which A-IoT device is expected to response.

Step 5:  Determine whether DUT can send the correct response with correct timing relationship. If correct response is received, the backscatter power at DUT’s antenna shall be calculated by calibrating input power at DUT’s antenna. Alternatively, backscatter loss can be calculated by subtracting the transmitted power.

Step 6: Repeat step 3~5 for different measurement grid point.

- Note 1: The selected command used for test could be refined based on progress in other working groups.

- Note 2: The power level of CW signal to be updated based on RF requirement discussion.

Proposal 11: Taking 3D scan with measurement grid as baseline to get sensitivity level and backscatter results for each grid point. Whether all grid points are needed and which results are used to formulate the RF requirements are up to RF requirements discussion.

Proposal 12: only test low, middle and high frequencies within a band and average the performance metrics from the three frequencies.

Observation 1: The performance/behaviour of energy harvesting cannot be specified. Different implementations (e.g. capacitor size, energy source) may lead to different charging/discharging time and behaviour.

Proposal 13:  adopt the following statement in the A-IoT specification that “the DUT is assumed to have sufficient power for each test point. It is up to device manufacturers to provide detailed conditions/methods to guarantee that the DUT would not fail a test due to lack of sufficient power supply. Such detailed conditions/methods would not be specified in the specifications”.

# Reference

- R4-2502858 WF on requirements for system parameters and A-IOT device

- R4-2500209 on OTA tests for ambient IoT devices

- TR 38.870 v18.4 Enhanced Over-the-Air (OTA) test methods for NR FR1 Total Radiated Power (TRP) and Total Radiated Sensitivity (TRS)