3GPP TSG-RAN WG4 Meeting #114bis	R4-2503499

Wuhan, China, April 7 – 11, 2025

Source:	Nokia, Nokia Shanghai Bell

Title:	RRM requirements for on-demand SSB SCell operation

Agenda item:	7.25.2.1

Document for:	Discussion

- Introduction

At RAN4#113 meeting, the R19 WI on network energy saving was further discussed and the agreements were captured in the WF [1]. In this contribution, we continue the discussion on the RRM impact from on-demand SSB SCell operation.

- General OD-SSB requirements

Processing time

In last meeting, RAN4 sent reply LS to RAN1 confirming the timing from OD-SSB transmission perspective [2]. Meanwhile RAN4 will continue the discussion on the UE processing/preparation time for OD-SSB reception.

Issue 1-1-1: RAN1 LS Agreement: - RAN4 to confirm that T_min equals to m+3Nslotsubframe,μ+1 from network transmission perspective. - Note 1: This does not imply UE shall be able to process/prepare the OD-SSB from time instance A from RAN4 requirement perspective. - Meanwhile, RAN4 is discussing the additional UE processing/preparation time for OD-SSB reception for RAN4 requirement.

In our understanding, the UE processing time was raised because UE needs additional time to get prepared for receiving the first OD-SSB. It only applies to OD-SSB reception but not regular SSBs as OD-SSB may be activated at any unexpected time. But this also means that in some cases if the UE has been prepared when receiving OD-SSB activation command, the UE processing time may not be needed.

For Case 1, only OD-SSB may be transmitted in an SCell. The UE would likely turn off the receiver while OD-SSB is not activated. It may take time to prepare for the first OD-SSB reception after receiving activation indication. But if OD-SSB has been activated recently, the UE may be able to receive the OD-SSB without additional processing time.

For Case 2, always-on SSB are being transmitted in addition to OD-SSB in an SCell. The UE is required to keep monitoring always-on SSBs which may provide the reference for OD-SSB reception. For example, if OD-SSB is transmitted on the same frequency with always-on SSB, the UE receiver setting used for always-on SSB can be well applied to OD-SSB reception hence the additional processing time may be also saved.

Observation 1: If OD-SSB has been activated recently, the UE may be able to receive the OD-SSB without additional processing time when it is activated again.

Observation 2: If OD-SSB is transmitted on the same frequency with always-on SSB, the UE receiver setting used for always-on SSB can be well applied to OD-SSB reception hence the additional processing time may be also saved.

Hence, we would like RAN4 to discuss the applicability of the UE processing time. The processing/preparation time is needed only when necessary and it shall not apply at least in above cases.

- The processing/preparation time for OD-SSB reception is needed only when necessary and it shall not apply at least in the following cases:

- If OD-SSB has been activated recently, or

- If OD-SSB is transmitted on the same frequency with always-on SSB.

Interruption

Another aspect which may be impacted by OD-SSB is the interruption requirement. In existing specification, interruption is allowed on active serving cells due to RF tuning during some RRM procedures e.g.

- 8.2.2.2.1 Interruptions at SCell addition/release: This defines the maximum allowed interruption during the RRC reconfiguration procedure for SCell addition/release. As UE needs to monitor SSB

- 8.2.2.2.3 Interruptions during measurements on deactivated SCC:  This defines the maximum allowed interruption due to measurements when an SCell is deactivated

In both clauses above, UE needs to tune on its RF in order to monitor SSBs for identifying or measuring the SCell. However, if the SCell is in OD-SSB operation in Case 1 i.e. without always-on SSB, UE may not need to switch on the RF immediately as there is no SSB being transmitted. Hence no interruption is allowed before OD-SSB is indicated when UE is commanded to add/release an OD-SSB SCell.

If the SCell is in OD-SSB operation in Case 2 i.e. with always-on SSB, UE needs to tune the RF in order to receive at least always-on SSB. Existing interruption requirement can still apply. In any case, the interruption requirements during SCell addition/release need to be updated at least for OD-SSB SCell in Case 1.

Observation 3: If the SCell is in OD-SSB operation in Case 1 i.e. without always-on SSB, UE may not need to switch on the RF immediately as there is no SSB being transmitted.

- No interruption is allowed during the RRC reconfiguration procedure for SCell addition/release before OD-SSB is indicated in Case 1.

Similarly, interruption shall not be allowed during the measurements on deactivated SCC before OD-SSB is indicated. This applies if the SCell is in OD-SSB operation in Case 1.

- No interruption is allowed during measurements on deactivated SCC before OD-SSB is indicated in Case 1.

On the other hand, interruption may be caused when OD-SSB is indicated. UE may need to retune its RF in order to receive OD-SSB when it is activated via RRC or MAC command. Similarly, as the discussion on additional processing time, the interruption may happen only under certain conditions. RAN4 needs to discuss if interruption is allowed when OD-SSB is indicated and when it is allowed if possible.

- Interruption may be caused when OD-SSB is indicated on the SCell.RAN4 needs to discuss if interruption is allowed when OD-SSB is indicated via RRC or MAC command and when it is allowed if possible.

L1 measurement in OD-SSB SCell

In last RAN4 meeting, it was confirmed UE does not perform L1 measurement in deactivated SCell hence we will only consider L1 measurement for activated SCells. With the following agreements, it is FFS how to define the requirement for the OD-SSB based L1 measurement after SCell activation when DRX is configured.

Issue 4-1: L1 measurement before SCell activation Agreement: - RAN4 not to support OD-SSB based L1 measurements for deactivated Scell. Issue 4-2: L1 measurement after SCell activation Agreement: - RAN4 to support OD-SSB L1 measurement during and after SCell activation. - When non-DRX is configured, the current requirement can be reused with the clarification of TSSB in TL1-RSRP_Measurement_Period_SSB is the SSB periodicity of OD-SSB - RAN4 to support OD-SSB L1 measurement during SCell activation. - The requirement when DRX is configured. - Based on OD-SSB periodicity Way forward: - RAN4 to support OD-SSB L1 measurement after SCell activation. - FFS the requirement when DRX is configured. - Option 1: Follow legacy, based on DRX cycle - Option 2: Based on OD-SSB periodicity - Option 3: FFS - Note: the OD-SSB is triggered before or together with the Scell activation command, the OD-SSB transmission kept after the SCell activation until the OD-SSB deactivation command received.

After the SCell is activated, UE may be configured with L1 measurement and reporting based on CSI-RS and/or SSB. As L1 measurement is mainly used for channel/beam management, it is not as urgent as the measurement on deactivated SCell where the fast measurement could help with the following SCell activation procedure. In our view, UE can follow the legacy L1 measurement requirement based on DRX cycle when DRX is configured for OD-SSB based L1 measurement.

- UE will follow the legacy L1 measurement requirement considering DRX cycle when DRX is configured.

OD-SSB impact on neighbor cell measurement

About neighbor cell measurement, it was in general concluded to reuse the legacy measurement requirements if there is OD-SSB SCell or neighbor cell in the MO. But for the scenarios where OD-SSB is an SCell, the conclusion only applies to FR1 and further discussion is needed for FR2.

Issue 7-2: OD-SSB based Neighbor cell measurement - Scenario 1a: There is OD-SSB neighbor cell in the same MO with legacy serving cell Agreement: - For Scenario 1a: No RAN4 spec. change. - Scenario 1b: Neighbor cell with legacy SSB in the same MO with OD-SSB serving cell Agreement: - For FR1, follow legacy requirement for neighbor cell measurement with legacy SSB. - The agreement can be revisited if not aligned with the further progress in other WGs. - Further discuss for FR2. - Scenario 1c: There is OD-SSB neighbor cell in the same MO with OD-SSB serving cell Agreement: - For FR1, follow legacy requirement for neighbor cell measurement. - The agreement can be revisited if not aligned with the further progress in other WGs. - Further discuss for FR2.

In previous meetings, it was agreed a UE is not required to measure OD-SSB before OD-SSB is activated in the SCell. Hence the measurement requirement for the deactivated OD-SSB SCell (being discussed in section 3 and 4) is applicable only if OD-SSB is activated. But this shall not impact the measurement on the neighbor cells in the MO. The UE is required to perform the intra-frequency measurement on the serving MO and existing measurement requirement for neighbor cells on the MO still apply. The same principle shall be applied to both FR1 and FR2.

- If there is an OD-SSB SCell on the serving CC, the legacy requirement for neighbor cell measurement applies for both FR1 and FR2.

- OD-SSB based deactivated SCell measurement (Case 1)

In last meeting, the UE measurement behavior on the deactivated OD-SSB SCell was further clarified. In general, a UE is expected to perform a fast measurement based on OD-SSB periodicity within a fast mode window when the SCell is activated (either directly activated or by MAC CE). Afterwards, UE can fall back to legacy deactivated SCell measurement for UE power saving. The length of the fast mode window was further agreed as below to ensure UE is able to obtain a valid measurement result within the time period.

Sub-issue 1-1 – The length of FMW Agreement: - The time window is defined with Tuncertain plus actual measurement duration. - Tuncertain equals Tmin(RAN1) plus additional processing/preparation time, if it allows. - The actual measurement duration equals min(Treport , Tidentify) from the end of uncertain time. - Treport is the first OD-SSB based L3 measurement reporting - Tidentify equals the minimum RAN4 OD-SSB meas. period requirement as follow. - A detected cell: OD-SSB based measurement period - A newly detectable cell: OD-SSB based cell identification period - No OD-SSB measurement requirement applies in fast window if NW deactivates the OD-SSB earlier than the end of min(Treport, Tidentify). Sub-issue 1-2: What’s the expected minimum interval(T2, sage 4) between multiple FMWs? Agreement: RAN4 to define measurement requirement based on T2. - When NW re-activates the OD-SSB within the T2, - Option 1: UE is not required to perform “fast measurement” but follows legacy deactivated SCell measurement requirement. - Option 2: UE is required to perform “fast measurement” without cell search. - The UE is assumed to skip the PSS/SSS detection and/or SSB index acquisition. - Option 3: UE is required to perform FMW after T2. - When NW re-activates/re-configures the OD-SSB transmission after T2, UE performs the OD-SSB based fast L3 measurement within FMW - The UE is assumed to meet fast cell identification and measurement. - The start point of T2 is based on - Option 1: the time point Tend - Option 2: the latter point of L3 measurement reporting for FMW and Tend - Option 3: the latter point of latest L3 measurement reporting before NW re-indicating OD-SSB transmission and Tend - Option 4: the time point of OD-SSB deactivation - Other options are not precluded Where, Tend equals the time point of OD-SSB indication plus Tuncertain +Tidentify applies to option 1,2,3. - The duration is - Option 1: 5*max(measCycleSCell, DRX cycles) [– OD-SSB periodicity] - Option 2: 5* measCycle when measCycle =160 ms - Other options are not precluded - The activation and deactivation of OD-SSB is fully up to NW.

On top of the measurement framework, companies further raised the issue of reactivation of OD-SSB and proposed to optimize the measurement based on a time period T2. There was also different understanding on where T2 starts and ends which make the requirement discussion very complicated.

In our view, when network decides to initiate OD-SSB activation, network is expecting UE to perform fast measurement on the SCell. This can be used for obtaining the first measurement reporting or updating the measurement result upon network request. If the OD-SSB has been activated recently i.e. OD-SSB is reactivated within a time period T2, UE may have just measured the SCell hence UE can perform fast measurement for a shorter time period for power saving i.e. a shorter fast mode window may apply for OD-SSB reactivation.

Observation 4: Network is expecting UE to perform fast measurement on the SCell every time OD-SSB is activated.

- If the OD-SSB is reactivated within a time period T2, UE can perform fast measurement for a shorter time period for power saving i.e. a shorter fast mode window may apply for OD-SSB reactivation.

According to the agreement, the length of fast mode window is defined as Tuncertain + min (Treport , Tidentify) and Tidentify is the minimum RAN4 OD-SSB meas. period requirement which differentiates between detected cell from a newly detectable cell. This already allows a shorter fast mode window if the cell is detected at the moment of OD-SSB (re)activation. When OD-SSB is reactivated within a time period T2, the SCell is likely a detected cell hence the shorter fast mode window for detected cell i.e. OD-SSB based measurement period is well applicable for UE power saving.

- When OD-SSB is reactivated within a time period T2, the shorter fast mode window for detected cell i.e. OD-SSB based measurement period can be applied for UE power saving.

In our understanding, T2 was introduced to minimize the fast measurement for UE power saving. However, the length of fast mode window has been designed taking this into account the need for UE OD-SSB. UE is required to use every OD-SSB during the fast mode window to measure the SCell while the UE is not required to measure all OD-SSB during the full window but only for a period if the SCell has been measured recently. In any case, it is unclear how much benefit the optimization based on time period T2 could bring to the requirement, while this would also complicate the UE behaviour and network implementation. Since we are discussing the minimum requirement, UE is allowed to optimize their measurement behaviour up to implementation. To simplify the discussion, we prefer not considering further optimization to the measurement requirement based on time period T2.

Observation 5: It is unclear how much benefit the optimization based on time period T2 could bring to the requirement, while this would also complicate the UE behaviour and network implementation.

- Do not consider further optimization to the measurement requirement based on time period T2.

Deactivated SCell measurement within fast mode window

For the measurement within the fast mode window, some company proposed to define a lower bound or to apply CSSF for the measurement period due to UE implementation restriction.

Sub-issue 3-1: Lower bound for measurement in FMW Way forward: - Option 1: RAN4 not to define lower bound to measurement in FMW. - Option 2: Define a lower bound with OD-SSB periodicity as [10ms]. - The lower bound is only applied to measurement in FMW. Sub-issue 3-2: OD-SSB measurement in FMW(Case 1-1): single OD-SSB carrier with multiple legacy SSB carriers Agreement: When NW configures single OD-SSB carrier with multiple legacy SSB carriers, in FMW - Option 1: The CSSF only counts OD-SSB layers - Option 2: The CSSF of OD-SSB carrier counts OD-SSB layers and other layers Sub-issue 3-3: OD-SSB measurement in FMW(Case 1-2): Multiple active OD-SSB carriers Agreement: RAN4 to prioritize single OD-SSB carrier with multiple legacy SSB carriers in deactivated SCell measurement. - FFS multiple OD-SSB carriers

With the lower bound, the UE may relax the measurement on the deactivated Scell during the FMW. For instance, the OD-SSB is being transmitted with a periodicity of 5ms while the UE is measuring OD-SSB every 20ms assuming a low bound of 100ms. From network point of view, it is not energy efficient to transmit OD-SSB often if they are not actively considered when defining the requirements and not assumed being used by the UE. Besides, UE is assumed be able to process SSB within 2ms during SCell activation, we wonder why additional lower bound is needed for processing OD-SSB based measurement. The lower bound needs to be well justified as it may reduce the network energy saving gain from OD-SSB transmission.

- RAN4 does not introduce a lower bound OD-SSB periodicity. Any lower bound needs to be well justified as it reduces the network energy saving gain from OD-SSB transmission.

Due to the same concern, UE vendors would like to apply CSSF as an alternative if lower bound is not to be introduced. This can also relax the UE measurement when multiple carriers are to be measured. In our view, the measurement performed in fast mode are prioritized - the OD-SSB based measurement during FMW, neither measCycleSCell or DRX cycle are considered. Hence, this fast measurement shall also be prioritized over the measurements on legacy carriers with regular SSBs. The CSSF for the measurement within fast mode window shall not count the number of legacy carriers configured with measurement hence no CSSF applies.

But if there are multiple OD-SSB SCells to be measured within the fast mode window, the number of OD-SSB SCells needs to be counted when deriving delay. However, it is a matter how many OD-SSB SCells are being measured in fast window and the fast windows may not always align in time. To account the workload of the discussion and finalize the work item, we suggest focusing on single OD-SSB SCell in this release, and only consider multiple simultaneously OD-SSB carriers if time allows or otherwise in a later release.

Observation 6: For the measurement within fast mode window, CSSF does not apply and shall not count the number of legacy carriers configured with measurement.

- When NW configures single OD-SSB carrier with multiple legacy SSB carriers, the CSSF does not apply for the measurement within fast mode window.

- Focus on single OD-SSB SCell and do not consider multiple OD-SSB carriers in this release.

- OD-SSB based deactivated SCell measurement (Case 2)

Always-on SSB and OD-SSB time/frequency domain

In last meeting, it was agreed in RAN4 to start from the following scenarios based on RAN1 LS. In addition, RAN1 sent another LS R1-2501633 informing more scenarios to be supported.

Issue 2-2: Always-on SSB and OD-SSB time/frequency domain relation Agreement: RAN4 to start from the following scenarios: - Scenario 1: OD-SSB and always-on SSB are with same frequency, same offset but different periodicities. - Scenario 4: OD-SSB and always-on SSB are with same frequency, different offset but same periodicities. - At least the offset between OD-SSB and AO-SSB equals the half of AO-SSB periodicity - The scenarios can be updated based on other WGs’ progress. RAN1 reply LS (R1-2501633): RAN1 discussed the relation in terms of time location between the always-on SSB and on-demand SSB and agreed the following in RAN1#120. Agreement Regarding the relation in terms of time location between the always-on SSB and on-demand SSB, - For the case when the center frequency locations of always-on SSB and on-demand SSB are same, - Alt Time-C: RAN1 specification has no restriction with regards to overlapping - From RAN1 perspective, - Alt Time-C1: The case that, during OD-SSB transmission, the union of AO-SSB transmission and OD-SSB transmission has a periodic time domain pattern is supported (the interval between SSB bursts is even and supported in legacy specification). - Alt Time-C2: The case that, during OD-SSB transmission, the union of AO-SSB transmission and OD-SSB transmission has a non-periodic time domain pattern is supported. - It is up to RAN4 to define requirements, if any, corresponding to both or either of Alt Time-C1 or Alt Time-C2 - At least the following is supported: PBCH payload for the same SSB index (other than SFN index, half frame index) is the same for AO-SSB and OD-SSB - FFS: Whether half frame index is the same or different for AO-SSB and OD-SSB - For the case when the center frequency locations of always-on SSB and on-demand SSB are different, - Alt Time-C: RAN1 specification has no restriction with regards to overlapping - UE assumes that frequency resources of always-on SSB are not overlapped with those of on-demand SSB in frequency domain. - AO-SSB and OD-SSB are located in the same BWP - FFS: PBCH payload for the same SSB index (other than SFN index, half frame index) should be the same for AO-SSB and OD-SSB - NOTE: AO-SSB periodicity is not adapted

Referring to Fig.2, RAN1 further agreed to support Case 2-3 and Case 2-4 in addition to Case 2-1 and Case 2-2. RAN4 shall discuss how to define the measurement requirements for these newly agreed scenarios accordingly.

- RAN4 shall further discuss how to define the measurement requirements for these newly agreed scenarios:

- Case 2-3: AO-SSB and OD-SSB are on same carrier frequency with different periodicity, and the union of them has a non-periodic time domain pattern i.e. the interval between SSB bursts is uneven.

- Case 2-4:  AO-SSB and OD-SSB are on different carrier frequencies in the same BWP, and they are non-overlapping in frequency domain.

Fig. 2 OD-SSB SCell operation in Case 2 and Scenario #2/#2A

Deactivated SCell measurement

When OD-SSB is triggered on a deactivated SCell, it needs to be discussed how a UE would perform the measurements and how to define the measurement requirement while both OD-SSB and regular SSBs are present. The following options were proposed in last meeting.

Issue 2-1: (Case 2) OD-SSB based deactivated SCell L3 measurement when OD-SSB transmission is triggered Agreement: When NW indicates the OD-SSB transmission for deactivated SCell measurement(Case 2 Scenario 2), - Option 1: The same approach in Case 1 can be reused. - UE performs the OD-SSB based fast L3 measurement following OD-SSB periodicity for deactivated SCell within a time window. After the time window, UE fallbacks to a slow mode measurement(with DRX and measCycleSCell). - Option 2: - If the serving cell is unknown, the same approach in Case 1 can be reused. - If the serving cell is known, UE follows legacy requirements based on measCycleSCell. - The known condition reuses the principle of legacy known condition. - This scenario applies at least when UE sends the meas. reporting. - FFS the case if the L3 meas. report cannot be triggered. - The above requirement is applied at least when the OD-SSB and AO-SSB are on the same frequency and spatial relation.

The deactivated SCell measurement in Case 2 is different from Case 1 in that there is always-on SSB before OD-SSB is activated. Having the prioritized Case 2-1/Case 2-2 in mind, as OD-SSB and always-on SSB are on the same frequency, the UE may have acquired a measurement result based on always-on SSB when OD-SSB is activated. In this case, the UE may skip the measurement on OD-SSB and continue with the legacy measurement based on always-on SSB.

Similar principle was captured in Option 2 above. However, known/unknown condition is defined for determining SCell activation delay. Besides the measurement reporting sent before SCell activation command, the known condition also considers other side conditions e.g. if the SSB remains detectable hence and the actual UE conditions may not be aligned between UE and network. To avoid the ambiguity, this can be formulated as the conditions that the UE has been measuring always-on SSB for a certain time period, or the UE has sent a measurement reporting for the OD-SSB SCell.

Otherwise, it shall follow the measurement procedure as discussed for Case 1, i.e. start the fast measurement within a fast mode window and fall back to legacy measurement on deactivated SCell based on measCycleSCell.

Observation 7: If OD-SSB and always-on SSB are on the same frequency, the UE may have acquired a measurement result based on always-on SSB when OD-SSB is activated.

Observation 8: The current known/unknown condition considers other side conditions besides measurement reporting hence may not be aligned between UE and network.

- If OD-SSB and AO-SSB are on the same frequency, UE may skip the measurement on OD-SSB if the UE has been measuring always-on SSB for a certain time period, or the UE has sent a measurement report for the OD-SSB SCell. Otherwise, the UE follows the measurement procedure as discussed for Case 1, i.e. start the fast measurement within a fast mode window and fall back to legacy measurement on deactivated SCell.

Although there are different scenarios when OD-SSB and AO-SSB are on the same carrier frequency i.e. Case 2-1,2-2,2-3, the same principle will apply. The only difference is the periodicity used for defining the measurement requirement in fast mode window:

- For Case 2-1 and Case 2-3, the measurement requirement in fast mode window is defined based on the OD-SSB periodicity; and

- For case 2-2, the measurement requirement in fast mode window is defined based on the half of AO-SSB/OD-SSB periodicity.

- The same principle in Proposal #14 applies to all the scenarios where AO-SSB and OD-SSB are on the same carrier frequency. The only difference is the measurement requirement in fast mode window:

- For Case 2-1 and Case 2-3, the measurement requirement in fast mode window is defined based on the OD-SSB periodicity; and

- For case 2-2, the measurement requirement in fast mode window is defined based on the half of AO-SSB/OD-SSB periodicity.

If the OD-SSB is on a different frequency from AO-SSB e.g. Case 2-4 in Fig.2, from existing configuration point of view they would reside in different measurement object (MO)s as no MO can be associated with multiple frequencies of SSB. This seems to be easiest approach in both RAN2 and RAN4. So we can assume the measurements and reporting on OD-SSB and always-on SSB are up to network configuration and can follow legacy behavior.

In addition, for Case 2-4 RAN1 agreed UE is not required to measure both AO-SSB and OD-SSB when OD-SSB is activated. This means UE is required to measure either of them in this case. To simplify the discussion, we can assume UE is required to measure OD-SSB when it is activated to align with the measurement behaviour in other scenarios/cases.

Agreement Regarding the relation in terms of frequency location (i.e., center frequency) between the always-on SSB and on-demand SSB, - Alt 1: If always-on SSB is CD-SSB on a synchronization raster, the frequency location of on-demand SSB is different from the frequency location of always-on SSB. - On-demand SSB is not on sync raster - AO-SSB and OD-SSB are located in the same BWP - FFS: Additional conditions - Subject to separate UE capability - Note: UE is not required to measure both AO-SSB and OD-SSB

- If OD-SSB is on a different frequency from always-on SSB e.g. Case 2-4, RAN4 to assume they reside in different MO and follow legacy intra-frequency measurement and reporting behaviour.

- UE is required to measure OD-SSB when it is activated if OD-SSB is on a different frequency from always-on SSB e.g. Case 2-4.

- OD-SSB based SCell activation delay

Known/unknown condition

On SCell activation delay, the following was agreed on the known/unknown condition for OD-SSB SCell.

RAN4#113: Issue 1-4-1: The known cell condition – measurement reporting Agreement: - In case 1, known/unknown condition should be defined based on only OD-SSB. - In case 2, known/unknown condition should be defined based on either always-on SSB or OD-SSB. - Note 1: whether the always-on SSB and OD-SSB can be combined is discussed in another issue. - Note 2: the agreement is valid at least for the scenario with the same frequency for always-on SSB and OD-SSB. - Note 3: RAN4 to further clarify the relation for the SSB measured for known cell condition and the SSB used in SCell activation procedure. Issue 1-4-5: SCell activation requirement – OD-SSB periodicity Agreement: - In Case 1, the SCell activation delay requirement is defined based on OD-SSB periodicity. - Existing SCell activation delay requirement shall apply based on OD-SSB periodicity. - In Case 2, RAN4 to define the SCell activation delay requirement based on OD-SSB periodicity if the time instance receiving OD-SSB activation indication is no later than the time instance when receiving the SCell activation command. Otherwise, RAN4 to define the requirement based on always-on SSB for SCell activation. - This agreement is valid under the assumption of the same spatial relation for OD-SSB and always-on SSB [with the same SSB index], and this can be revisited based on RAN1 LS reply. - Note: T_firstSSB_max for intra-band co-located scenario is discussed in another issue.

One open issue about known/unknown condition is the time period during which the UE has sent a measurement reporting before receiving SCell activation command. As we agreed to follow legacy deactivated SCell measurement period after the fast mode window, the validity of the measurement reporting can still refer to the legacy time period e.g. max(5*measCycleSCell,  5*DRX cycles) for FR1 SCell, irrespective if the measurement reporting is based on OD-SSB or always-on SSB. This can minimize the impact on known condition and keep the SCell in known state.

- In the known condition, the time period during which the UE has sent a measurement reporting before receiving SCell activation command is the same as that in legacy SCell activation e.g. max(5*measCycleSCell,  5*DRX cycles) for FR1 SCell.

Another issue being raised is to further clarify the relation for the SSB measured for known cell condition and the SSB used in SCell activation procedure. We understood this problem is valid only if OD-SSB is on a different frequency from always-on SSB. As discussed before, if we prioritize the case with the same frequency, it does not matter if the known condition is determined based on always-on SSB and/or OD-SSB and the SCell shall be activated as a known SCell.

- If the OD-SSB is on the same frequency with always-on SSB, the SCell shall be activated as a known SCell no matter if the known condition is determined based on always-on SSB and/or OD-SSB.

- Conclusion

In this contribution, we discussed the RRM requirements for OD-SSB SCell operation based on RAN1/2 progress and presented our views on the potential RAN4 impact. The following observations and proposals are made:

Observation 1: If OD-SSB has been activated recently, the UE may be able to receive the OD-SSB without additional processing time when it is activated again.

Observation 2: If OD-SSB is transmitted on the same frequency with always-on SSB, the UE receiver setting used for always-on SSB can be well applied to OD-SSB reception hence the additional processing time may be also saved.

- The processing/preparation time for OD-SSB reception is needed only when necessary and it shall not apply at least in the following cases:

- If OD-SSB has been activated recently, or

- If OD-SSB is transmitted on the same frequency with always-on SSB.

Observation 3: If the SCell is in OD-SSB operation in Case 1 i.e. without always-on SSB, UE may not need to switch on the RF immediately as there is no SSB being transmitted.

- No interruption is allowed during the RRC reconfiguration procedure for SCell addition/release before OD-SSB is indicated in Case 1.

- No interruption is allowed during measurements on deactivated SCC before OD-SSB is indicated in Case 1.

- Interruption may be caused when OD-SSB is indicated on the SCell.RAN4 needs to discuss if interruption is allowed when OD-SSB is indicated via RRC or MAC command and when it is allowed if possible.

- UE will follow the legacy L1 measurement requirement considering DRX cycle when DRX is configured.

- If there is an OD-SSB SCell on the serving CC, the legacy requirement for neighbor cell measurement applies for both FR1 and FR2.

Observation 4: Network is expecting UE to perform fast measurement on the SCell every time OD-SSB is activated.

- If the OD-SSB is reactivated within a time period T2, UE can perform fast measurement for a shorter time period for power saving i.e. a shorter fast mode window may apply for OD-SSB reactivation.

- When OD-SSB is reactivated within a time period T2, the shorter fast mode window for detected cell i.e. OD-SSB based measurement period can be applied for UE power saving.

Observation 5: It is unclear how much benefit the optimization based on time period T2 could bring to the requirement, while this would also complicate the UE behaviour and network implementation.

- Do not consider further optimization to the measurement requirement based on time period T2.

- RAN4 does not introduce a lower bound OD-SSB periodicity. Any lower bound needs to be well justified as it reduces the network energy saving gain from OD-SSB transmission.

Observation 6: For the measurement within fast mode window, CSSF does not apply and shall not count the number of legacy carriers configured with measurement.

- When NW configures single OD-SSB carrier with multiple legacy SSB carriers, the CSSF does not apply for the measurement within fast mode window.

- Focus on single OD-SSB SCell and do not consider multiple OD-SSB carriers in this release.

- RAN4 shall further discuss how to define the measurement requirements for these newly agreed scenarios:

- Case 2-3: AO-SSB and OD-SSB are on same carrier frequency with different periodicity, and the union of them has a non-periodic time domain pattern i.e. the interval between SSB bursts is uneven.

- Case 2-4:  AO-SSB and OD-SSB are on different carrier frequencies in the same BWP, and they are non-overlapping in frequency domain.

Observation 7: If OD-SSB and always-on SSB are on the same frequency, the UE may have acquired a measurement result based on always-on SSB when OD-SSB is activated.

Observation 8: The current known/unknown condition considers other side conditions besides measurement reporting hence may not be aligned between UE and network.

- If OD-SSB and AO-SSB are on the same frequency, UE may skip the measurement on OD-SSB if the UE has been measuring always-on SSB for a certain time period, or the UE has sent a measurement report for the OD-SSB SCell. Otherwise, the UE follows the measurement procedure as discussed for Case 1, i.e. start the fast measurement within a fast mode window and fall back to legacy measurement on deactivated SCell.

- The same principle in Proposal #14 applies to all the scenarios where AO-SSB and OD-SSB are on the same carrier frequency. The only difference is the measurement requirement in fast mode window:

- For Case 2-1 and Case 2-3, the measurement requirement in fast mode window is defined based on the OD-SSB periodicity; and

- For case 2-2, the measurement requirement in fast mode window is defined based on the half of AO-SSB/OD-SSB periodicity.

- If OD-SSB is on a different frequency from always-on SSB e.g. Case 2-4, RAN4 to assume they reside in different MO and follow legacy intra-frequency measurement and reporting behaviour.

- UE is required to measure OD-SSB when it is activated if OD-SSB is on a different frequency from always-on SSB e.g. Case 2-4.

- In the known condition, the time period during which the UE has sent a measurement reporting before receiving SCell activation command is the same as that in legacy SCell activation e.g. max(5*measCycleSCell,  5*DRX cycles) for FR1 SCell.

- If the OD-SSB is on the same frequency with always-on SSB, the SCell shall be activated as a known SCell no matter if the known condition is determined based on always-on SSB and/or OD-SSB.

- References

[1]	R4-2502598, “WF on Netw_Energy_NR_enh”, Ericsson

[2]	R1-2500020, Reply LS on timeline for On-demand SSB operation on SCell, RAN4, RAN1#120, Athens, Greece, February 17th – 21st, 2025