3GPP TSG-RAN WG4 Meeting #114bis                                                          R4-2503518

# Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda Item:	7.19.4

Source:	Apple

Title:	Discussion on RRM core requirement and testing framework for Positioning accuracy enhancement

Release:            Rel-19

Document for:	Discussion

Introduction

Rel-19 Work Item WI was approved on the Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface (WID in [1]). The application of AI/ML techniques to NR air interface has been studied in FS_NR_AIML_Air.

This work item provides normative support for the general framework of AI/ML concerning air interfaces. It also enables the implementation of recommended use cases outlined in the previous study. Furthermore, several study objectives within this project aim to address outstanding issues identified during the study, with the goal of enhancing understanding in preparation for future normative effort.

The current agreements on how to perform the RAN4 study on general issues for AI/ML, and issues related to interoperability/testing have been captured in the latest TR [2]

In this paper, we provide some additional views on the topic of testability and interoperability issues for positioning accuracy enhancement.

Discussion

Positioning Use Cases

In TR 38.843 the following test metrics for positioning enhancement have been captured:

Both direct AI/ML positioning and AI/ML assisted positioning are considered.

For metrics for positioning requirements/tests, the candidate options include

-	Option 1: positioning accuracy: Ground truth vs. reported

-	only option available for direct positioning

-	Option 2: CIR/PDP, channel estimation accuracy

-	Option 3: ToA, RSTD and RSRP, and RSRPP

-	Option 4: others (e.g., intermediate KPIs, LoS/NLoS)/combinations of the above

The feasibility and testability of different options should be further justified in WI.

RAN1 evaluation was mainly driven to understand if AI/ML model based positioning can outperform legacy NR positioning method in terms of achievable positioning accuracy in the considered scenario and to also investigate the generalization capability of the AI/ML models for positioning.

In RAN1 #110bis meeting, the following 5 cases of AI/ML based positioning accuracy enhancement are agreed for further study,

- Case 1: 	UE-based positioning with UE-side model, direct AI/ML or AI/ML assisted positioning

- Case 2a: 	UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning

- Case 2b: 	UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning

- Case 3a: 	NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning

- Case 3b: 	NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning

For case 1 and case2a, the AI/ML model is deployed at UE, both model input and model output may need data/reference signal transmission between UE and NW. For case 2b, case 3a and case 3b, the AI/ML model is deployed at NW side(e.g., gNB or LMF), only model input may need data transmission from UE to NW.

According to the discussions during the AI/ML ad-hoc meeting minutes [ R4-2410571] from RAN4#111 the following issues were discussed for the AI/ML positioning case:

Sub-topic 3-1

There are some proposals on whether to define requirements for case 1 (UE-based positioning with UE-side model, direct AI/ML positioning)

Issue 3-1: Requirements for case 1

- Proposals

  - Option 1: RAN4 should not define requirements for case 1

  - Option 2: RAN4 should postpone the discussion until other WG conclude on defining a reporting scheme or not

  - Option 3: Others

- Recommended WF

To be discussed

Note: currently there are no requirements for UE based positioning

Sub-topic 3-2

Issue 3-2: Requirements for case 2a

Agreement:

RAN4 should not define any positioning accuracy requirements because positioning is LMF based

Issue 3-3: Reported Metrics for case 2a

During RAN4 #112 the following agreement has been captured:

Agreement:

- RAN4 will take existing requirements for existing metrics as baseline

  - FFS whether to revisit these requirements

- RAN4 will study how to define requirement for any new reported metrics

FFS how to define requirements for LOS/NLOS

Issue 3-5: Reported Metrics for case 2b

Agreement:

Postpone discussion on what and how to define requirements until RAN1 agrees to what metrics are reported

Issue 3-6: Reported Metrics for case 3a/3b

Agreement:

Postpone discussion until RAN1 agrees on any metric to be reported by gNBs.

From previous meetings the following issues were discussed:

Sub-topic 3-3

Issue 3-3: Requirements for case 2b

- Proposals

  - Option 1: RAN4 should not define any positioning accuracy requirements because positioning is LMF based

  - Option 2: RAN4 should continue to discuss how to define requirements for case 2b, companies should bring more concrete proposals in future meetings

  - Option 3: Postpone discussion until other groups make more progress

  - Option 4: others

- Recommended WF

  - To be discussed, options are not exclusive

Agreement:

No requirements on LMF for positioning accuracy

FFS on RAN4 requirements for any UE reported measurements defined by other groups

Sub-topic 3-4

Some companies are proposing to already start discussing how to define requirements for possible reported metrics such ToA, LoS/NLoS, etc. These might be introduced for cases 2a/2b

Issue 3-4: Requirements for reported metrics

- Proposals

  - Option 1: RAN4 to postpone discussion until reported metrics become clear in other groups

  - Option 2: RAN4 to already start the discussion on how to define requirements for LoS/NLoS indicator

  - Option 3: RAN4 to start discussing how to define requirements for other reported values, e.g. CIR/PDP, ToA,

  - Option 4: others

- Recommended WF

  - To be discussed, options are not necessarily exclusive

We present our views in the sections below:

### 2.1.1 Case 1: UE-based positioning with UE-side model

In this use case AI/ML model is at the UE. The UE collects measurements or data needed for model inferencing. In this use case model inferencing provides the prediction of the UE location. This scenario is similar to the UE-based positioning in legacy NR positioning and no requirements are defined for such scenario.

There are concerns regarding the feasibility on how to perform tests for positioning accuracy in RAN4. It has been considered that TE may provide "Positioning test data set(s)" in advance for consideration. These data sets may comprise of model input-related information, such as PRS measurement results matched with positions or various model inputs derived from the measurement outcomes, along with positioning label. TE sends the model input-related data to UE, which then returns the positioning results to TE. This allows TE to compare UE's output positioning results with the local positioning label data, enabling the determination of positioning accuracy.

According to discussions from previous meetings, proponents of option 1 have concerns about defining requirements due to the uncertainty around whether the UE will report positioning. However, reviewing RAN1 agreements, one of the options (option B-1) involves the target UE sending its inference results to the LMF. While this option pertains to model performance monitoring, it demonstrates that inference results can be reported to the LMF if option B-1 is adopted. With this in mind, it is not urgent to make a decision yet. Whether to define requirements for case 1 can be further discussed after conclusions are reached by other working groups.

Proposal 1: RAN4 to further discuss the feasibility and how to define requirements for positioning accuracy for case 1. Defining requirements is also dependent on RAN1/2 conclusion regarding whether UE will report positioning

### 2.1.2 Case 2a: UE-assisted/LMF-based positioning with UE-sided model

In use case 2a, the AI/ML model resides within the UE. Through inference, this model predicts measurements relevant for UE positioning. Subsequently, these positioning measurements derived from AI/ML model inference are transmitted to the Location Measurement Function (LMF) for the actual estimation of UE position.

This scenario is similar to the UE-assisted positioning supported by legacy NR positioning specifications. It's important to note that RAN4 has established requirements for all measurements associated with UE-assisted positioning. Given that the accuracy of position estimation conducted by the LMF heavily depends on the measurements reported by the UE, RAN4 should define both core and performance requirements for use case 2a.

LOS/ NLOS indicator can serve as an input to the AI/ML or non AI/ML based algorithm at LMF for deriving the positioning co-ordinates.

In Rel-17, NR positioning enables the UE to report Line-of-Sight (LOS) and Non-Line-of-Sight (NLOS) indicators to the Location Measurement Function (LMF). When an AI/ML model predicts the LOS/NLOS indicator, it should be treated as an intermediate KPI or feature, with defined minimum performance requirements. The inclusion of the LOS/NLOS indicator as an input to the positioning algorithm can have a notable influence on the overall positioning accuracy.

For LOS/NLOS indicator, RAN1 just agreed at least LOS/NLOS indicator is supported for reporting in #116 meeting. Thus, the LOS/NLOS could be the intermediate feature to be reported and the details can be seen:

Agreement For AI/ML assisted positioning Case 2a, at least LOS/NLOS indicator and/or timing information are supported for reporting. - If LOS/NLOS indicator is reported, the indicator can be reported as soft indicator or hard indicator as defined in 38.214. - If timing information is reported, the timing information at least can be reported via DL RSTD or UE Rx-Tx time difference as defined in 38.215. - Note: details of the report are pending further discussion.

However, RAN4 has not established specific requirements for this measurement in legacy systems. As a result, RAN4 should assess whether and how to define requirements for LOS/NLOS indicators. Regarding Time of Arrival (ToA), there is no legacy reporting scheme—only time information related to RSTD is currently reported.

In Assisted AI/ML Positioning, the inference of RSRP, RSRPP, Time of Arrival (ToA), and Reference Signal Time Difference (RSTD) from the AI/ML model/functionality acts as an input for the positioning algorithm at the UE or Location Measurement Function (LMF). Consequently, it could significantly influence the positioning accuracy.

The measurement accuracy requirements for RSRP/RSRPP/ToA/RSTD  defined for legacy positioning necessitate further analysis and adaptation for AI/ML-based models or functionalities. Hence RSRP/RSRPP/ToA/RSTD should be considered as intermediate KPIs/ features.

When the measurements used in AI/ML positioning align with those defined in legacy positioning, the various target positioning requirement types for NR positioning in Rel-18, as outlined in TR38.859, can be considered as potential candidates for AI/ML positioning. Specifically, the functionality input could be one of the existing measurements, such as PRS-RSRP or PRS-RSRPP.

Additionally, if new measurements like CIR, DP, and/or PDP are agreed upon in RAN1 as functionality inputs, this would have implications for RAN4 in terms of defining new measurement accuracy requirements for these measurements.

The input to the AI/ML functionality must meet specific measurement accuracy requirements to ensure that the AI/ML system performs as expected. Therefore, RAN4 should establish requirements for the measurement types already supported in legacy positioning (e.g., RSTD, RSRP, and RSRPP). Additionally, RAN4 should explore how to define the requirements for any new measurements that may be introduced, such as CIR, DP, and PDP.

Proposal 2: For Assisted AIML Positioning, the KPI test metric (e.g., LOS/NLOS) needs to be considered for validating the positioning accuracy. RAN4 should investigate how to establish requirements for LOS/NLOS indicators, considering their potential impact on performance and accuracy in positioning systems.

Proposal 3: For AIML positioning, RAN4 should take as baseline the requirements on the measurements supported in legacy positioning (RSTD, PRS-RSRP, and PRS-RSRPP) and define the requirements on the new measurements (e.g., CIR, DP, and PDP) based on RAN1 progress.

### 2.1.3 Case 2b: UE-assisted/LMF-based positionin with LMF-sided model

In use case 2b, the AI/ML model resides within the Location Measurement Function (LMF). Here, the UE assists the LMF in positioning by providing the necessary measurements or data required for inference by the AI/ML model within the LMF. The output of the model in this scenario is the prediction of the UE's location

Given that the performance of the AI/ML model at the LMF heavily depends on the accuracy of measurements conducted by the UE, it is important for RAN4 to establish corresponding accuracy requirements.

Proposal 4: RAN4 to study defining performance accuracy requirements for any reported UE measurements defined by other groups.

According to the RAN1 agreement, the measurements that the UE can report for use case 2b can be either sample-based or path-based. From a requirements perspective, new core and performance standards must be established for this use case.

Agreement from RAN1# 116bis [4]

In Rel-19 AI/ML based positioning, regarding the time domain channel measurements, RAN1 investigate the following alternatives:

      - Alternative (a).  Sample-based measurements, where the timing information is an integer multiple of sampling periods.

      - Alternative (b).  Path-based measurements, where the timing information is according to the detected path timing and may not be an integer multiple of sampling periods.

The issues to be studied include, but not limited to, the following:

  - Trade-off of positioning accuracy and signalling overhead

      - Impact and necessary details of gNB/UE implementation to obtain the channel measurement values.

      - Whether the same Alternative(s) applies to all cases or not

      - Applicability and necessity of specifying the Alternative(s) to different cases

      - Note: different sub-cases may have different issues.

Note: In addition to timing information, the components for the channel measurement for model input may also include power and potentially phase. To provide the type of the channel measurement in their investigation.

Proposal 5: RAN4 should define measurement requirements for both path-based and sample-based reporting in relation to use case 2b.

Before we define requirements we will need to study the relationship between positioning accuracy at the LMF and measurement accuracy reported by the UE

Proposal 6: Before RAN4 investigates testing Case 2b in AI/ML for positioning, it is important to first examine the relationship between the measurement accuracy reported by the UE and the resulting positioning accuracy at the LMF.

### 2.1.3 Case 3a: NG-RAN node-assisted positioning with gNB-sided model

Use case 3a closely resembles the previously discussed use case 2a. The primary distinction between use case 2a and use case 3a is the location of the AI/ML model. In use case 3a, the AI/ML model resides within the gNB, whereas in use case 2a, it resides within the UE. Similar to use case 2a, the AI/ML model in use case 3a generates positioning measurements as its output. These measurements are then relayed to the Location Measurement Function (LMF) for the estimation of UE position

Regarding Option 1 on subtopic 3.2, it has been discussed in the WF [R4-2403712], RAN4 should not define positioning accuracy requirements for case 3a/3b (accuracy on inference results of AI/ML positioning model)

Regarding Option 3: RAN4 continues to discuss requirements on measurements reported by UE/gNB to the entity running the AI/ML positioning model:

Proposal 7: RAN4 to define performance accuracy requirements for use case 3a for the measurements reported by gNB. The specifics and scope of the core and performance requirements for use case 3a are contingent on the outcomes of discussions within RAN1 and RAN2

### 2.1.4 Case 3b: NG-RAN node-assisted positioning with LMF-sided model

Use case 3b bears a close resemblance to the previously discussed use case 2b, with the primary difference lying in the involvement of the gNB. In this scenario, the gNB provides the measurement data required by the AI/ML model within the Location Measurement Function (LMF) for inference, contrasting with the reliance on the UE as seen in use case 2b. Nonetheless, in use case 3b, the AI/ML model's output continues to be the prediction of the UE's location, mirroring use case 2b.

Regarding Option 1 on subtopic 3.2, it has been discussed in the WF [R4-2403712], RAN4 should not define positioning accuracy requirements for case 3a/3b (accuracy on inference results of AI/ML positioning model)

Regarding Option 3: RAN4 continues to discuss requirements on measurements reported by UE/gNB to the entity running the AI/ML positioning model:

Proposal 8: RAN4 to define performance accuracy requirements for measurements performed at gNB for use case 3b. The specifics and scope of the core and performance requirements for use case 3a are contingent on the outcomes of discussions within RAN1 and RAN2

# Conclusion

In this section we summarize the list of observations and proposals we discussed in this contribution:

Proposal 1: RAN4 to further discuss the feasibility and how to define requirements for positioning accuracy for case 1. Defining requirements is also dependent on RAN1/2 conclusion regarding whether UE will report positioning

Proposal 2: For Assisted AIML Positioning, the KPI test metric (e.g., LOS/NLOS) needs to be considered for validating the positioning accuracy. RAN4 should inevstiagte how to establish requirements for LOS/NLOS indicators, considering their potential impact on performance and accuracy in positioning systems.

Proposal 3: For AIML positioning, RAN4 should take as baseline the requirements on the measurements supported in legacy positioning (RSTD, PRS-RSRP, and PRS-RSRPP) and define the requirements on the new measurements (e.g., CIR, DP, and PDP) based on RAN1 progress.

Proposal 4: RAN4 to study defining performance accuracy requirements for any reported UE measurements defined by other groups.

Proposal 5: RAN4 should define measurement requirements for both path-based and sample-based reporting in relation to use case 2b.

Proposal 6: Before RAN4 investigates testing Case 2b in AI/ML for positioning, it is important to first examine the relationship between the measurement accuracy reported by the UE and the resulting positioning accuracy at the LMF.

Proposal 7: RAN4 to define performance accuracy requirements for use case 3a for the measurements reported by gNB. The specifics and scope of the core and performance requirements for use case 3a are contingent on the outcomes of discussions within RAN1 and RAN2

Proposal 8: RAN4 to define performance accuracy requirements for measurements performed at gNB for use case 3b. The specifics and scope of the core and performance requirements for use case 3a are contingent on the outcomes of discussions within RAN1 and RAN2

- References

[1] RP-242399 – “Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”

[2] 3GPP TR 38.843, “Technical Specification Group Radio Access Network;Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface”