3GPP TSG-RAN WG4 Meeting #114bis                                               R4-2503520

# Wuhan, Hubei, China, 07th – 11th April, 2025

Source:  Apple

Title: On Testability and Interoperability Issues for AIML Mobility in NR

Agenda item:     7.21.3

Document for:   Discussion

Introduction

RAN4 discussion on the AI (Artificial Intelligence)/ML (Machine Learning) for mobility in NR (FS_NR_AIML_Mob study item) starts from RAN4#112bis. The recent RAN#105 plenary has agreed the updated version of the SID with the following RAN4 objectives [1]:

- Evaluate testability, interoperability, and impacts on RRM requirements and performance [RAN4] - Study the impacts on requirements based on RAN2 assumptions, and coordinate with RAN2 if needed - Study the testability and interoperability based on RAN2 framework (e.g., number of cells to measure, beams etc.) - NOTE 4: Leverage the work from “AI/ML for NR air interface” led by RAN1 and avoid the duplicate study for testability and interoperability. - NOTE 5: Avoid the overlaps with RAN2 work for evaluation

It is expected the following aspects to be discussed during this meeting.

- Work Plan for RAN4 Discussions on AI/ML for Mobility in NR

- AI/ML Mobility Scenarios Evaluation and RRM Requirements

- Testability and Interoperability Issues in AI/ML Mobility

In this contribution we provide our views and discuss different aspects of Testability and Interoperability issues in RAN4 for AI/ML Mobility based on the RAN2 framework.

Discussion

During RAN4#112bis the following agreements were reached:

Issue 3-1-1: New testability aspects

- Agreement

  - According to the SID, RAN4 to leverage the work from “AI/ML for NR air interface” led by RAN1 and avoid the duplicate study for testability and interoperability.

  - RAN4 will need to consider at least the following testability aspects for AI/ML-enabled mobility SI:

    - Testing in FR1

    - Testing inter-carrier scenario, i.e., inter-frequency inter-cell prediction (for scenario 3)

    - Testing cell-level L3-RSRP measurements

    - Consideration of serving and neighbor cells

      - Issue 3-2-1: Testing in FR1

- What way of testing to consider for FR1:

  - Candidate Options:

    - Option 1: OTA testing

    - Option 2: Conducted testing

    - Option 3: Both OTA and Conducted

- Agreement:

  - RAN4 to consider Option 2 as a baseline option for testing in FR1.

    - Option 2: Conducted testing

For AI/ML solutions to be effectively deployed within the 3GPP framework, interoperability and testability are important. This is especially important when AI/ML functions reside on the UE side, as the network must be able to reliably validate these functions. Interoperability enables mobility-focused AI/ML solutions—such as measurement and event prediction, as well as radio link failure (RLF) prediction—to function smoothly across varied network environments and with devices from different vendors. This compatibility supports enhanced network optimization, efficient radio resource management, and proactive maintenance, all contributing to a seamless and improved user experience [2]

- Measurement Prediction

- Handover and Radio Link Failure Prediction, and

- Measurement Event Prediction

However, for better understanding and designing of testability and interoperability aspects, RAN4 needs to wait for further RAN2 progress as well as RAN4 agreements on work plan and final agreed scenarios and corresponding RRM requirements.

During RAN4#113 the following agreement was reached:

Issue 3-1-1: Testing setup

- Agreement

  - RAN4 to further discuss:

    - Whether and how legacy testing setup for L3 measurements can be reused for FR1 and FR2

Issue 3-2-1: Testing goal

- Agreement

  - As baseline, the testing goal is to verify whether the minimum performance of AI/ML functionality/feature can be achieved.

Testability aspects for measurement prediction use case

RRM measurement prediction including intra and inter-frequency (UE sided and NW sided model)

At RAN2#125bis, the following sub-use cases were agreed for RRM measurement prediction:

- Sub-use case 1: L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 cell-level measurement result is generated

- Sub-use case 2: L3 Cell-level measurement result(s) is predicted based on actual L3 cell-level measurement result(s)

- Sub-use case 3: L3 Cell-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s)

In the draft of TR 38.744 [3], the two main identified study goals are:

- First Goal; Measurement reduction. Achieve the same mobility performance, but with reduced DL radio measurements that the UE needs to take and report, which improves UE energy efficiency, reduces the required signalling, and network overhead of transmitting reference signals (RSs).

- Second Goal; Mobility optimization. Predict future UE radio measurements and use them to optimize HO performance and mobility decisions.

Three domains (or a combination them) have been identified for mobility predictions:

- Spatial: when UE is not measuring every cell and/or beam

- Frequency domain: The UE is not measuring every frequency carrier/bands

- Temporal: when UE may skip and predict some of the measurement time steps

Correspondingly, the following scenarios for RRM measurement prediction use case were captured and prioritized in RAN2#126:

Table 1: Prioritization of evaluation scenarios in RAN2 (Table 5.2.1-1 in TR 38.744 draft) for measurement prediction use-case.

| Scenario number | Priority | Evaluation scenario | Target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |

Based on the RAN4 objectives, the Interoperability and testing aspects of AI/ML Mobility should be based on the RAN2 framework. RAN4 should also study the requirements and testing frameworks to validate Measurement prediction use case considering the existing mobility requirements and test framework as the starting point.

Considering high priority scenarios listed in the above table, we can observe that the output of the AI/ML functionality may be different for different scenarios. For instance, for scenario 2, the output of the AI/ML functionality may be L3 cell-level measurements for the same cell as input measurements while in scenario 3, the output of the AI/ML functionality may be L3 cell-level measurements of a different cell than the cell of the input measurements to the AI/ML functionality.

Since, the agreement on the selected performance metric/KPI will impact the testability and testing setup related discussions, in our view, it is better to first agree on performance metric/KPI for different scenarios listed in Table1 before discussing testing setup for these KPIs

Therefore, accuracy of L3 cell-level measurements of source and target cells may be candidate performance metrics/KPIs in RAN4.

The test objectives can be divided into two parts corresponding to the two goals, separately.

First Goal; Measurement reduction. Verify the trade-off between the measurement and prediction accuracy performance degradation, the legacy performance is as baseline:

Intermediate KPIs and Metrics:

- Average L3 cell RSRP difference for intra-cell (scenario 2)

- L1-RSRP difference (dB) (FFS, beam level)

- Average L3 cell RSRP difference for inter-cell (scenario 3)

- For scenario 3 and prediction across multiple cells and multiple frequency layers,  topK candidate cells/frequency layers can be another KPI

- Reduction rates: MRRT for temporal domain; MRRS for spatial domain: as side conditions

- Prediction window (PW), observation window (OW) as side conditions

Second Goal; Mobility optimization. Predict future UE radio measurements and use them to optimize HO performance and mobility decisions. Test property: HO performance and mobility enhancement

Intermediate KPIs and Metrics:

System KPI: Performance KPIs defined in TR 36.839 as baseline for mobility performance KPIs

- The evaluation of the AI/ML aided mobility benefits should consider HO performance KPIs (e.g., Ping-pong HO, HOF/RLF, Time of stay, Handover interruption, prediction accuracy, and measurement reduction) etc.) and complexity tradeoffs [RAN2] - NOTE: Simulation assumption and methodology can leverage TR 38.901, 38.843 and 36.839. And leave the detail discussion to RAN2

Proposal 1: RAN4 should define performance metrics/KPIs and clarify the objectives, testing properties, and methodologies for evaluating AI/ML functionalities across each mobility use case and sub-use case addressing both Goal 1 (measurement reduction) and Goal 2 (mobility optimization):

- 1st goal: To assess the trade-off between measurement reduction and measurement prediction accuracy and its effect to mobility performance with legacy as a baseline with MRRT, SNR as side conditions

- 2nd goal: To determine if the AI/ML model achieves measurable improvements in handover (HO) performance with PW, OW, SNR as side conditions

Temporal domain prediction, e.g. scenario number 2, 4, are to predict cell level results in prediction window based on actual measurement result(s) in observation window. Sufficient randomness and variation in time are needed, which is similar in concept to the spatial domain prediction. The dynamic nature of AI/ML-assisted mobility may necessitate added complexity in the test environment beyond traditional scenarios. This could involve incorporating time-varying reception power and SNR influenced by UE trajectory, with each test iteration reflecting unique conditions to capture a range of random variations.

Proposal 2: For temporal domain prediction, to investigate whether legacy test setups can serve as a baseline and how to introduce controlled randomness and time-domain variation/correlation to validate testing objectives comparing predicted versus actual outcomes.

Proposal 3: Define multiple test iterations, each designed to reflect unique, time-varying conditions such as reception power and SNR influenced by UE trajectory, to ensure coverage of potential conditions in AI/ML-assisted mobility scenarios

## 2.2 Generalization / adaptability of the models

AI/ML model generalization is a challenging aspect of deep learning, referring to a trained model's ability to perform effectively on new, unseen inputs. In our view, generalization and adaptability of AI models are particularly valuable to study in mobility scenarios:

- In AI/ML-based mobility, numerous deployment scenarios (e.g., UMa, UMi), varying channel environments, UE movement patterns, speeds, and frequency bands all influence model performance [5]

- In L3 AI/ML mobility multiple cells will be present. However, variations in network and local radio conditions across different cells may introduce challenges. There is a risk that the model could overfit to specific characteristics of individual cells, potentially reducing its adaptability to other cells

- SNR conditions for inter-cell measurements

  - Prediction accuracy should be maintained across a range of SNR values (including low SNR)

  - Heterogeneity of SNRs across frequency layers

- Codebooks across different cells (frequency layers)

  - Codebook can be different across collocated or non-collocated site (inter-cell)

    - Beam sweep order can be different

    - Number of Tx beams can be different

    - Beam widths could be different

- Tx Antenna array size and layout and antenna element spacing (across frequency bands)

In AI/ML-based mobility, training and inference require a dataset or environment that ensures consistency between primary and secondary carrier components. This constraint is not present in the AI/ML BM use case and therefore is not considered in the scope of ongoing WI.

The primary challenge in achieving generalization for AI/ML models in AI/ML mobility is ensuring that a model trained with a specific cell or inter-cell can perform effectively across different cells and inter-cell environments. A critical question is whether an AI/ML model trained in one deployment scenario can be directly applied to a different scenario without retraining. Addressing this challenge is essential, as it could reduce the need for multiple AI/ML models, thereby enhancing adaptability and robustness in diverse and realistic radio conditions (SNR).

Observation 1: The primary challenge in achieving generalization for AI/ML models in AI/ML mobility is ensuring that a model trained with specific cell or inter-cell characteristics can perform effectively across different cells and inter-cell environments. A critical question is whether an AI/ML model trained in one deployment scenario can be directly applied to a different scenario without retraining. Addressing this challenge is essential, as it could reduce the need for multiple AI/ML models, thereby enhancing adaptability and robustness in diverse and realistic radio conditions (SNR).

Proposal 4: For Rel-19 AI/ML mobility, it is essential for RAN4 to investigate generalization-related tests and requirements arising from varying propagation conditions, SNR levels, and cell-specific Tx/codebook implementations across different cells and frequency layers.

Current discussions in AI/ML-based beam management are introducing the concept of an associated ID, which captures certain network element-dependent conditions (such as Set A, Set B configurations, and codebook specifics). This associated ID could influence AI/ML model functionality by integrating these unique conditions, potentially enhancing model generalization across varying environments. Different cells will create different propagation and SNR conditions, and the AI/ML functionality will need to support different cell Tx layouts and codebooks. Assistance information will be needed to achieve generalization.

Proposal 5: RAN4 should investigate the side conditions for RSRP prediction pertinent to AI/ML mobility, along with network assistance information required to guarantee generalization and effective operation across multiple target cells and different frequency layers. Additionally, it should leverage the discussions in AI/ML beam management regarding the signaling of associated IDs to ensure consistency between primary and secondary carrier components, thereby enhancing generalization capabilities.

## 2.3 Life Cycle Management (LCM) Related Requirements

With the integration of AI/ML models into essential network functions—such as managing mobility, facilitating handovers, and optimizing radio resource allocation—it is important to ensure these models operate efficiently in real-time, adapting to the diverse and dynamic conditions of wireless channels. This consideration is crucial for both UE and network-side implementations. To ensure the sustained effectiveness of these AI/ML models, it is important to establish and monitor KPIs like prediction accuracy and latency, which are indicative of their real-time operational performance. [2]

When an AI/ML model's performance drops below specified thresholds, it's critical for either the network or the UE to implement corrective actions. This could involve updating the model, tweaking its configurations, or modifying the input data. If performance significantly deteriorates, it may be necessary to disable the AI/ML model entirely and revert to traditional methods, such as legacy measurement and reporting systems, to ensure consistent and reliable network operation. This proactive approach helps to maintain network integrity, ensuring that the use of AI/ML does not undermine performance during instances of model failure or reduced efficacy

- Proposal 6:  RAN4 should leverage the LCM progress and requirements discussed in R-19 WI related to AI/ML Air Interface and investigate enhancements to these LCM procedures to RRM for mobility.

Conclusion

In this paper, we present our perspectives on the potential impacts for RAN4 arising from testability and interoperability challenges associated with AI/ML-based mobility use cases. We made the following observations and proposals:

Observation 1: The primary challenge in achieving generalization for AI/ML models in AI/ML mobility is ensuring that a model trained with specific cell or inter-cell characteristics can perform effectively across different cells and inter-cell environments. A critical question is whether an AI/ML model trained in one deployment scenario can be directly applied to a different scenario without retraining. Addressing this challenge is essential, as it could reduce the need for multiple AI/ML models, thereby enhancing adaptability and robustness in diverse and realistic radio conditions (SNR).

Proposal 1: RAN4 should define performance metrics/KPIs and clarify the objectives, testing properties, and methodologies for evaluating AI/ML functionalities across each mobility use case and sub-use case addressing both Goal 1 (measurement reduction) and Goal 2 (mobility optimization):

- 1st goal: To assess the trade-off between measurement reduction and measurement prediction accuracy and its effect to mobility performance with legacy as a baseline with MRRT, SNR as side conditions

- 2nd goal: To determine if the AI/ML model achieves measurable improvements in handover (HO) performance with PW, OW, SNR as side conditions

Proposal 2: For temporal domain prediction, to investigate whether legacy test setups can serve as a baseline and how to introduce controlled randomness and time-domain variation/correlation to validate testing objectives comparing predicted versus actual outcomes.

Proposal 3: Define multiple test iterations, each designed to reflect unique, time-varying conditions such as reception power and SNR influenced by UE trajectory, to ensure coverage of potential conditions in AI/ML-assisted mobility scenarios

Proposal 4: For Rel-19 AI/ML mobility, it is essential for RAN4 to investigate generalization-related tests and requirements arising from varying propagation conditions, SNR levels, and cell-specific Tx/codebook implementations across different cells and frequency layers.

Proposal 5: RAN4 should investigate the side conditions for RSRP prediction pertinent to AI/ML mobility, along with network assistance information required to guarantee generalization and effective operation across multiple target cells and different frequency layers. Additionally, it should leverage the discussions in AI/ML beam management regarding the signaling of associated IDs to ensure consistency between primary and secondary carrier components, thereby enhancing generalization capabilities.

- Proposal 6:  RAN4 should leverage the LCM progress and requirements discussed in R-19 WI related to AI/ML Air Interface and investigate enhancements to these LCM procedures to RRM for mobility.

- References

- RP-242393, Revised SID on AIML for mobility in NR , OPPO, 3GPP TSG RAN Plenary #105, Melbourne, Australia, September 9-12, 2024.

- R4- 2416441, On Testability and Interoperability Issues for AIML Mobility, Nokia, RAN4#112-bis, Hefei, China, 14th –18th October 2024.

- TR 38.744, Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR (Release 19)

- TR 38.903 V18.4.0, Derivation of test tolerances and measurement uncertainty for User Equipment (UE) conformance test cases (Release 18)

- R4-2415576, General discussion on AI mobility regarding testability and interoperability, Samsung, RAN4#112-bis, Hefei, China, 14th –18th October 2024.

- R4-2415348, Testability and interoperability for AI based mobility, Qualcomm, RAN4#112-bis, Hefei, China, 14th –18th October 2024