# 3GPP TSG-RAN WG4 Meeting #114bis	R4-2503533

# Wuhan, China, 7th – 11th April, 2025

Agenda item:	7.26.4.2

Source:	Apple

Title:	On RRM core requirements for LP-WUR

Document for:	Discussion

# Introduction

In last RAN4#114 meeting, the WF for RRM requirements for LP-WUS/WUR was agreed in [1], and open issues have been summarized in [2]. There was one LS[3] from RAN1 after last meeting, and some RAN4 actions are needed as following:

In RAN1#118bis, it was agreed to support 3 candidate values for the wake-up delay capability a UE reports for LP-WUS operation in IDLE/INACTIVE mode. In addition, the following agreement was made in RAN1#120: Agreement For UE capability report on the wake-up delay: - Alt 1: For the 3 candidate values for the wake-up delay capability report, support {[70ms], [500ms] and [900ms]}. - The reported values assume SSB periodicity of 20ms, where [70ms] assumes [3] SSBs needed for synchronization, [500ms] and [900ms] assume [5] SSBs needed for synchronization. - FFS: translation of wake-up delay for different SSB periodicities - FFS: different sets of 3 candidate value for different SSB periodicities Note that the definition of wake-up delay is the minimum gap time between LP-WUS reception and MR to start PDCCH monitoring. RAN1 respectfully asks RAN4 to check and confirm the number of SSBs currently assumed by RAN1 for MR synchronization for SSB periodicity of 20ms. RAN1 also respectfully asks RAN4 to confirm if it is reasonable to assume the same number of SSBs for MR synchronization for different SSB periodicities.

In this contribution, we continue the discussion on the open issues in [1].

# General aspects

Issue 1-1-1: Cases/states to be considered for RRM relaxation and serving cell measurement offloading

We think the case #1 is the case when UE has very good quality of serving cell, and UE can fully offload from MR to LR. But if the serving cell quality is not that good as case #1, UE may consider to use MR relaxation for serving and neighbor cell measurement. Case #2 is a little bit weird case to consider, since when the LR is ON and neighbor cell measurement is OFF on MR, it seems not necessary for UE to keep MR awake for a relaxed serving cell measurement. In our view the threshold to trigger corresponding offloading and relaxation can be as following figure,

Figure 1. threshold assumption for different cases

If the serving cell measurement shows above the threshold of neighbor cell measurement (or a new threshold of MR offloading), UE can turn off the MR, and only rely on LR to perform serving cell measurement, and no neighbor cell measurement is needed (including higher or equal/lower priority layers measurement). If the neighbor cell measurement is triggered (e.g., below the threshold of intra-frequency or inter-frequency measurement triggering condition) but serving cell quality is still above threshold of LP-WUS monitoring threshold, the MR can be ON with relaxation for serving and neighbor cell measurement while the LR is also ON for serving cell measurement, i.e., case #3. Then if UE is below LP-WUS monitoring threshold but above the not-at-cell-edge threshold for RRM relaxation, then UE will be in a legacy power saving mode, which means, LR is OFF and MR is ON with neighbor cell measurement relaxation (no serving cell relaxation). Even further, if UE is below not-at-cell-edge threshold for RRM relaxation, it means UE is in legacy IDLE/Inactive mode status without any RRM relaxation.

To support the above logic, we have some assumptions for such threshold design, even though the final threshold decision shall be made by RAN2. We assume the threshold for MR fully offloading to LR can be same as the threshold for neighbor cell measurement triggering (highest one between SIntraSearchP/SIntraSearchQ and SnonIntraSearchP/SnonIntraSearchQ), or at least offloading threshold can be not lower than threshold for neighbor cell measurement triggering.

Based on the illustration in figure 1, we believe the case#2 and case#4 can be removed at least at this stage, but of course we can revisit the cases if RAN1/2 has some conclusions conflicting with RAN4 assumptions. And the following case table can be assumed in RAN4:

| RRM measurement case index | MR serving cell measurement | MR neighboring cell measurement | LR measurement |
| --- | --- | --- | --- |
| #1 Fully offloading case | Off | Off | ON |
| #3 Relaxed case b | On with relaxation measurement | On with relaxation measurement | ON |

Proposal 1: at this stage, remove case#2 and case#4 from the last meeting WF, and only following cases are considered. RAN4 can revisit the cases if in the future RAN1/2 has any conclusion conflicting with RAN4 assumptions.

| RRM measurement case index | MR serving cell measurement | MR neighboring cell measurement | LR measurement |
| --- | --- | --- | --- |
| #1 Fully offloading case | Off | Off | ON |
| #3 Relaxed case b | On with relaxation measurement | On with relaxation measurement | ON |

Proposal 2: assumption of thresholds for RAN4 discussion can be:

- the threshold for MR fully offloading to LR can be same as the threshold for neighbor cell measurement triggering (highest one between SIntraSearchP/SIntraSearchQ and SnonIntraSearchP/SnonIntraSearchQ), or at least offloading threshold can be not lower than threshold for neighbor cell measurement triggering.

Issue 1-1-2: Threshold for switch between different cases

One thing to clarify that, LR measurement will be used for UE to decide switch from case#1 to case#3, that is, serving cell measurement on LR will be used to decide (1)if the neighbor cell measurement is triggered and (2)if the neighbor measurement can be relaxed. Previously in R16/17/18, the neighbor cell triggering and relaxation criteria checking is based on the MR measurement, and therefore it means LR measurement result shall be comparable to MR measurement or shall be equivalent to MR measurement with certain offset/margin (e.g., LR threshold is MR threshold + offset/margin).

This issue has been discussed in RAN4 for couple of meetings, but we cannot foresee RAN2’s plan on discussing it. Thus, one possible way is to directly send LS to ask for RAN2’s guidance.

Proposal 3: LR measurement can be used to check the criteria for neighbor cell measurement triggering/relaxation (in case #1).

Proposal 4: to support proposal 3, LR measurement result shall be comparable to MR measurement result or shall be equivalent to MR measurement result with certain offset/margin (e.g., LR threshold is MR threshold + offset/margin).

Proposal 5: if RAN4 cannot make conclusion on issue 1-1-2, we recommend RAN4 sending LS to RAN2 for guidance.

Issue 1-1-3-1: On RRM requirements to be specified for OOK-based LP-WUR based on LP-SS with overlaid OFDM sequence

Issue 1-1-3-2: On RRM requirements to be specified for SSB-based LP-WUR based only on LP-SS with and without overlaid OFDM sequence

Issue 1-1-3-1/1-1-3-2 has been concluded in RAN4#114.

Issue 1-1-5: Criteria (entry/exit conditions) for LP-WUR serving cell measurement and MR RRM measurement relaxation

Issue 1-1-6: Criteria (entry/exit conditions) for LP-WUS monitoring

In WID, another important enhancement is: Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions. Even though conditions for offloading RRM serving cell measurement from MR to LR and conditions for relaxation of MR serving cell and neighbor cell measurements is mainly decided/designed by RAN2, RAN4 can investigate the mobility performance corresponding to such relaxation, and the mismatch/bias between LP-WUR based measurement result and MR based measurement result. For instance, if LP-SS is used for the measurement to decide when the MR shall be waken up or not, such LP-SS based measurement result at least shall be comparable to the MR based measurement results or some bias offset shall be defined; otherwise, LP-SS triggers MR to wake up for mobility but MR based measurement results may tell that the mobility is not needed after MR wakes up.

In previous RAN2 meeting, there was also some discussion that,

RAN2#128: - The entry condition for serving cell RRM relaxation is at least ‘if serving cell quality measured by MR is higher than relaxation threshold, e.g. RSRP and/or RSRQ’. FFS if LR measurement is needed. - FFS on exit condition for serving cell RRM relaxation, e.g., whether a separate exit condition other than ‘not fulfilling the entry condition’ is needed, or whether exit condition include MR and/or LR-based measurements - FFS if the entry condition for serving cell RRM measurement relaxation is the same as neighbour cell RRM measurement relaxation. RAN2#127: Serving cell measurements related For serving cell measurement offloading (i.e., serving cell measurement fully offloaded to LR and no serving cell measurement via MR is required), RAN2 should focus on specifying the offloading criterion for serving cell for UEs supporting LP-WUS, and assume that RAN4 will define the measurement offloading requirements for serving cell. Neighboring cell measurements related RAN2 understand that the RRM measurement of the neighboring cell can only be performed by MR. Can discuss again if RAN1 inform us otherwise. RAN2 will further discuss the neighbor cell measurement relaxation criteria (if the UE is using LR to measure the serving cell), e.g., considering reuse Rel-16 criteria for ‘not at cell edge’ and ‘low mobility’. RAN2#127bis: Working assumption - For neighbor cell measurement relaxation for UEs capable of LP-WUS, do not define additional MR-based criterion over the R16 criteria. RAN2 assume ‘UE not at cell edge’ is reused, FFS on ‘UE with low mobility’. - FFS (if needed) on enhancements based on R16 criteria (e.g., based on the LR measurements) for the case when MR serving cell measurement results are not available.

Like in power saving feature, the criteria itself has been defined be RAN2 in TS38.304, and therefore here we also think that RAN2 is the main group for criteria (entry/exit conditions) design. However, for RRM relaxation requirement (e.g., scaling factor), RAN4 can work on the RRM measurement relaxations (e.g., scaling factor) and offloading mechanisms based on the criteria defined by RAN2.

Proposal 6: Criteria (entry/exit conditions) for LP-WUR serving cell measurement and MR RRM measurement relaxation can be left to RAN2 to decide.

Proposal 7: For the entry/exit condition for LP-WUS monitoring, follow RAN1/2 conclusions or can be triggered by other groups if necessary.

Issue 1-1-7: On jointly consideration on Criteria (entry/exit conditions) for fully offloading/MR RRM measurement relaxation and LP-WUS monitoring

RAN2 agreements in last meeting ### RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE The entry condition for MR serving cell RRM relaxation can include both MR and LR measurements. If LR threshold is configured, the entry condition is when both MR and LR measurement are above the configured thresholds.

We also think such discussion can be left to RAN2 as in proposal 6 and 7.

Issue 1-1-8: LP-WUR operating carrier frequency

In last RAN #106 meeting, a WF RP-243266 for LP-WUS was agreed.

- For idle/inactive mode, consider a standardized solution for intra-frequency case (e.g. solution 1, 2, and 3 in RP-242656). For Rel-19, inter-frequency solutions (e.g. solution 4) will not be considered. - Detailed discussions and final solution selection is left up to WGs.

The solution 1, 2, 3 was defined as in RP-242656, as following:

- Solution 1: MR is prioritized (via SIBs) to camp on the same band as LR (System information, Paging, RACH on LR band)

- Solution 2: MR is prioritized to camp on the same band as LR (System information, Paging on LR band; and RACH on another MR band based on reselection criteria)

- Solution 3: MR is not prioritized to camp on the same band as LR but can be redirected via dedicated signalling depending on LR band load (up to gNB implementation)

During the RAN plenary discussion, solution 2 is the solution which needs more further analysis. If the RACH can be performed on the different carrier from the SI/paging carrier, it’s unclear so far how UE can determine the RACH occasion and timing on the RACH carrier. Before concluding on solution 2, some aspects need to be clarified from our perspective, e.g.,:

- Is there any limitation on band combination to apply such RACH on different carrier? E.g., intra-band or inter-band?

- if DL synchronization is performed on one carrier which is for SI reading and paging, how to acquire timing and RO information for RACH on another carrier?

If network configures such RACH transmission mechanism of option 2 on intra-band contiguous or ono-contiguous carrier relative to the paging carrier and these two cells are collocated, the timing from PO cell can be roughly reused to decide the RACH timing for different cell, and RAN4 may further define the RTD limitation and power imbalance limitation to apply such mechanism, e.g., RTD<=CP and power imbalance limitation<=6dB.

Second question is how to acquire the RO information for RACH carrier/cell, it might be carried on SI in PO cell or might be carried on SI in RACH cell. If RO information is carried on RACH cell only, UE reads SIB only after receive paging on PO cell to trigger its RACH on RACH cell, and then UE needs to acquire timing/power on RACH cell as well as reading SIB for RO information.

Obviously, many open issues need to sort out before we can define requirements for such inter-carrier MR/LR scenario. In last RAN2 meeting, one LS has been agreed as following,

RAN2 would like to thank RAN1 for the LS on LP-WUS operation in IDLE/INACTIVE mode in R2-2407921/R1-2407559. In addition to RAN2 reply LS in R2-2410957, RAN2 further discussed the details/pros/cons of solutions based on the way forward from RAN plenary in RP-242656. The corresponding discussion report is in R2-2501443. And RAN2 achieved the following conclusions: - RAN2 understands that UE can report which band(s) is supported by LR to NW. - RAN2 understands that any potential overload issues could be addressed by current mechanism in spec.

According to the RAN2 reply, there is no new requirement needs to be defined in RAN4 for the above issue. However, RAN4 can wait one more meeting for RAN1 to make the decision based on RAN2’s reply.

Proposal 8: regarding RAN#106 WF RP-243266, to wait RAN1 for solution selection based on RAN2 reply LS (e.g. solution 1, 2, and 3 in RP-242656), and then RAN4 can work on the requirement design if needed.

Issue 1-1-9: LP-WUR status at legacy case (not at LP-WUS monitoring case/fully offloading(case 1) case/RRM relaxation (case 3) case)

Concluded in last RAN4 meeting.

Issue 1-1-10: Considerations on higher priority frequency layer

- P1: Support relaxed higher priority frequency layer search at case 1 (xiaomi CT ZTE Huawei vivo Ericsson CMCC QC Nokia LGE CATT) - Follow the Rel-16 requirement of relaxed higher priority frequency layer search for case 1 - P2: Higher priority layer neighbour cell measurement shall also be OFF for case #1. (vivo, MTK, OPPO)

In case #1, when UE fully offload from MR to LR, we assume measurement results from LP-WUR will decide whether the MR shall be waken up or not, however, the MR will be turned off or in ultra-deep sleep mode. The FFS part is mainly about whether the higher priority measurement shall still be kept on MR or not. Current higher priority measurement in RAN4 has two main delays: 1min and 1 hour, as followings.

In order to save the power of UE, if MR still needs to wake up every 1min, we didn’t see much gain for this feature, but we are open to discuss if higher priority measurement on MR can be kept and performed as slow as 1 hour per layer. For simplicity of this feature at UE, we prefer to not consider higher priority layer measurement on MR when the MR is fully offloaded. Thus, the MR neighbor cell measurement shall not be considered for case #1.

Considering one motivation of higher priority layer measurement can be load balance for network, we can compromise to option 1 only if relaxed high priority layer measurement is assumed (i.e., perform higher priority measurement per 1 hour, like the following R16 power saving feature).

4.2.2.10.4 Measurements for UE fulfilling low mobility and not-at-cell edge criterion This clause contains requirements for measurements on inter-frequency NR cells provided that: - T331 timer is not running for EMR measurements on inter-frequency NR carrier, and - T331 timer is not running for EMR measurements on inter-frequency NR carrier, and - UE is configured with both lowMobilityEvaluation [2] criterion and cellEdgeEvaluation [2] criterion, and - UE is configured with both lowMobilityEvaluation [2] criterion and cellEdgeEvaluation [2] criterion, and - both lowMobilityEvaluation [2] and cellEdgeEvaluation [2] criteria are fulfilled. ------omit unrelated part------------ When Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, the UE shall search for inter-frequency layers of higher priority at least every K2*Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2.7 and K2=60.

Proposal 9: agree to adopt either one of the following options:

- Option 1: Support relaxed higher priority frequency layer search at case 1, i.e., perform higher priority frequency layer search at least every 1 hour.

- Option 2: Higher priority layer neighbour cell measurement shall also be OFF for case #1.

Issue 1-1-11: RRM requirements for FR2

In the past RAN#105 plenary meeting, the FR2 LP-WUS scope has been discussed and agreed in [RP‑242361 Way forward on LP-WUS for FR2] as following:

It is proposed to endorse the following in RAN#105 for Rel-19 LP-WUS/WUR WI regarding FR2 support - Confirm in RAN#105 that Rel-19 LP-WUS work item scope includes both FR1 and FR2. - Request RAN1 to make a progress on the FFS “Whether the above is applicable to FR2” (under RAN1#117 agreement on LP-WUS and LP-SS bandwidth) in RAN1#118bis. - Reuse the FR1 design when possible - No additional guidance in RAN#105 for RAN4 work and TU allocation

Since the FR2 is confirmed as a scope for LP-WUS WI, RAN4 needs to discuss (1) whether we need to design requirement for FR2 LP-WUR based RRM, and (2) when to start the discussion for such requirement design. Our understanding is FR2 LP-WUR based RRM requirement can be discussed after we concluded on the FR1 LP-WUR based RRM requirement, since FR2 case would be more complicated than FR1 by considering the Rx beam sweeping.

In RAN4 Idle/inactive mode requirement for SSB, the Rx beam sweeping has been applied for serving cell measurement and evaluation as following examples:

In LP-SS based serving cell measurement, UE will follow the LP-SS periodicity to do the measurement instead of DRX in legacy case, then we will probably use the different sweeping factor for different LP-SS periodicity; and furthermore if LP-SS has multiple repetitions inside each LP-SS occasion, UE may perform Rx beam sweeping on these repetitions and save the N1 for LP-SS occasion measurement. The most straightforward requirement could be that: the different Rx beam sweeping factor(N1) shall be applied for different LP-SS periodicities, and the N1 shall be decreased when LP-SS periodicity increases. However, since the LP-SS design is still on-going in RAN1, RAN4 can discuss the detailed requirement in future meetings.

Proposal 10: FR2 LP-WUR based RRM requirement can be discussed after we concluded on the FR1 LP-WUR based RRM requirement.

# Detail LP-WUR requirements at RRC_IDLE/INACTIVE state

Issue 1-2-1: Accuracy requirements

Regarding the accuracy requirement, if we follow the legacy accuracy requirement principle, the dedicated accuracy requirements in TS38.133 section 10 is mainly for the measurement results which is reported to network, and the only measurement accuracy requirement for IDLE/Inactive mode is the EMR feature related measurement. For LR based RRM measurement, since UE will not report anything back to network in IDLE/Inactive mode, our preference is still not to define a dedicated accuracy requirement in TS38.133 section 10, but instead we can reflect such accuracy performance in the core requirement sections as a margin, like in the following example:

Proposal 11: For RAN4 requirement of LR based RRM measurement in Idle/inactive states, no dedicated accuracy requirement is defined in the performance section, and reflect the accuracy performance as a margin in the core requirement.

Issue 1-2-2: Periodicity for SSB based LP-WUR measurement delay requirements

The agreements from last meeting were:

Agreement: If LP-SS is not configured, periodicity for SSB based LP-WUR measurement delay requirements - based on LO periodicity - FFS whether to define an upper bound - Note: DRX cycle and LO periodicity are the same based on RAN1 agreement. Agreement: When LP-SS is configured, apply the same agreement as for when LP-SS is not configured - The above agreement applies if LO periodicity is equal to or larger than LP-SS periodicity, FFS if LO periodicity is smaller than LP-SS periodicity.

In previous meeting, RAN1 also discussed the LP-WUS in eDRX, but no conclusion has been made. In our view, besides LP-WUS, we also need to investigate the UE RRM measurement for LP-SS in eDRX, e.g., whether LP-SS measurement shall follow the eDRX/DRX cycle or not. If the LP-SS based measurement is not following eDRX cycle or DRX cycle in IDLE/Inactive mode, i.e., LP-SS based measurement will follow the LP-SS periodicity, the RRM requirement shall only be defined based on the LP-SS periodicity. However, if the LR is measuring the SSB, it’s unclear so far which periodicity UE shall follow. RAN1 had some agreements in laprevious meeting that,

Agreement Regarding whether to support additional sync signal to LP-SS, down-selection from the following: - Option 1A: No additional sync signal, LP-SS periodicity =320ms - Additionally support at least one of [80ms,160ms] - FFS: whether different values can be applicable for different M values - Option 1B: No additional sync signal, LP-SS periodicity =320ms - No additional support of other periodicities - Option 2: With additional sync signal, LP-SS periodicity =320ms - FFS additionally support other LP-SS periodicities - FFS details on additional sync signal FFS additional sync signal is configurable and/or conditionally present Agreement For LP-SS periodicity, support at least 320ms - FFS: 80ms,160ms, 640ms,1280ms, 2560ms, 5120ms, 10240ms Agreement Confirm the following working assumption for iDRX: For each UE, the periodicity of LO is the same as its iDRX cycle. Agreement For the mapping between LO and PO, supports at least Option 1 (UEs monitoring the same PO monitor the same LO). Agreement From RAN1 perspective, when a UE is monitoring LP-WUS (based on the entry/exit condition for LP-WUS), a UE is not required to monitor a PO if - it does not detect a LP-WUS on the monitored LO - or the LP-WUS does not indicate a wake-up indication for the UE’s corresponding subgroup

If LP-SS periodicity is configured but UE is using LR to measure SSB, the measurement interval shall be comparable the LO periodicity, since UE needs the T/F tracking information from SSB measurement to monitor/receive LO; i.e., if the SSB measurement periodicity is too much bigger than LO periodicity, UE may be not able to correctly receive the LO. In last meeting, we received comments from companies that if LO periodicity is smaller than LP-SS periodicity, anyway UE still needs to monitor every LO; and therefore, following LO periodicity for T/F tracking and measurement still make sense.

To summarize, regardless of whether the LP-SS is configured or not, as long as UE is using LR to measure SSB, the LO periodicity could be the factor to consider this measurement interval. Thus, we propose to use DRX cycle or LO periodicity for SSB measurement periodicity.

Proposal 12: regardless of whether LP-SS is configured or not, periodicity for SSB based LP-WUR measurement delay requirements is

- based on LO periodicity

- Note: DRX cycle and LO periodicity are the same based on RAN1 agreement.

Issue 1-2-5-2: On MR evaluation requirements

- Proposals - P1: For MR evaluation requirement for LP-WUR monitoring, case 1 and case 3, consider the following options (vivo): - Option 1: reuse Nserv ; - Option 2: reduce Nserv value by half; - Option 3: Nserv always = 1; (LG xiaomi)

In last RAN4 meeting, the generic part for the evaluation has been agreed as followings:

Issue 1-2-3: On requirements for entry/exit criteria(threshold) evaluation for WUS paging monitoring/Fully Offloading (Case 1)/MR RRM relaxation (Case 3) Agreement: When both MR and LR threshold are configured, MR or LR evaluates whether its own entry/exit threshold is satisfied separately. MR and LR evaluation requirements are defined separately. Issue 1-2-4-2-2: On single or multiple LR evaluation requirements Agreement: Define a single LR evaluation requirement for both LR entry and exit threshold evaluation for LP-WUS monitoring, case 1 and case 3, i.e., LR evaluation requirement is not differentiated on entry or exit; or on different cases. Note: RAN2’s discussion on timer will not impact on RAN4’s evaluation requirement. Issue 1-2-4-2-3: On how to define LR evaluation requirements Agreement: LR evaluation duration is [x1 samples]*LP-SS (for OOK LR) or [y1 samples] *LO (for SSB LR), assuming x or y samples are used to satisfy accuracy requirement and x1 > x and y1>y. Number x and y are defined as measurement requirement as [x samples]*LP-SS (for OOK LR) or [y samples] *LO (for SSB LR)

For the fully offloading threshold checking, we had some discussion in last meeting that the MR will enter an ultra/deep sleep after the offloading threshold is met, and therefore in this case we need an evaluation period to make sure there is no ping-pong effect for UE to turn off or turn on the MR. The same justification will be applied to MR evaluation when entering case 1, and we also think no need to differentiate the evaluation requirement for case 1 and case 3. Thus, option 1 is preferred from our perspective.

Proposal 13: For MR evaluation requirement for LP-WUR monitoring, case 1 and case 3, reuse Nserv in existing spec, and Nserv for case 3 also includes the MR relaxation factor.

Issue 1-2-6: On MR wake up delay related requirements for LP-WUR

In RAN1 LS[3], the key agreements are:

Agreement

For UE capability report on the wake-up delay:

- Alt 1: For the 3 candidate values for the wake-up delay capability report, support {[70ms], [500ms] and [900ms]}.

  - The reported values assume SSB periodicity of 20ms, where [70ms] assumes [3] SSBs needed for synchronization, [500ms] and [900ms] assume [5] SSBs needed for synchronization.

  - FFS: translation of wake-up delay for different SSB periodicities

  - FFS: different sets of 3 candidate value for different SSB periodicities

Note that the definition of wake-up delay is the minimum gap time between LP-WUS reception and MR to start PDCCH monitoring.

RAN1 respectfully asks RAN4 to check and confirm the number of SSBs currently assumed by RAN1 for MR synchronization for SSB periodicity of 20ms. RAN1 also respectfully asks RAN4 to confirm if it is reasonable to assume the same number of SSBs for MR synchronization for different SSB periodicities.

The definition of wake-up delay is: Minimum gap time between LP-WUS reception and MR to start PDCCH monitoring, that is same as what RAN4 discussed in last meeting, i.e., interruption requirement in P1 above. The gap between an LO and a PO is considered to be no less than the wake-up delay a UE supports if the gap between the end of the last LP-WUS MO the UE monitors in the LO and the start of the PO is no less than the wake-up delay. Network needs to make sure the gap between LO for UE to monitor and PO shall be NO less than this wake-up delay, and that means it will not have any interruption to the PO reception due to the wake-up delay.

In short, we think such interruption requirement has already been reflected by UE capability indication as agreed in RAN1, thus no dedicated interruption requirement needs to be defined for this case. RAN4 only needs to define the wake-up delay requirement as defined in RAN1, if such requirement is necessary.

Proposal 14: RAN4 only needs to define the wake-up delay requirement, if such requirement is necessary from majority companies’ view, where the wake-up delay means minimum gap time between LP-WUS reception and MR to start PDCCH monitoring.

In order to check the number of SSBs currently assumed by RAN1 for MR synchronization for SSB periodicity of 20ms, there are some aspects for consideration:

- (1)Will the number of SSB for synchronization be impacted by the LP-SS periodicity before LP-WUS reception?

- (2)Will the number of SSB for synchronization be impacted by whether LP-SS or SSB is used for time tracking before LP-WUS reception?

- (3)Will the number of SSB for synchronization be impacted by the SINR condition?

If LP-SS has other periodicities besides 320ms, like 80ms/160ms, for different LP-SS periodicities, UE may need different SSB sample on MR for AGC settling and/or T/F tracking, and the principle could be larger LP-SS periodicity on LR needs more SSB samples occasions on MR for MR waking-up procedure. However, since MR is waken up after UE successfully received the LP-WUS (read it successfully), the coarse timing information shall already be good enough from LP-SS for LP-WUS decoding. Thus, the answer to question 1 can be No.

If the SSB is used for timing tracking before LP-WUS, UE can use SSB for AGC settling directly, and then during the MR wake-up procedure the sample of SSBs for ACG settling can be saved compared with the “LP-SS based tracking before the LP-WUS”, i.e., SSB based tracking before LP-WUS may need less SSB samples for MR wake-up than LP-SS based tracking. In last meeting RAN1 agreed, the EPRE ratio between LP-WUS/LP-SS and SSB can be configured by the gNB. That means, the coarse AGC estimation from LP-SS symbols may be also usable for AGC settling during MR wake-up procedure. Thus, the answer to question 2 can be No.

In our view, the MR wake-up delay capability is applied to the MR wake-up in case 1 once LP-WUS is received (fully offloading). In the above figure 1, we assume the SINR when entering case can be higher than -3dB, and the wake-up delay for MR can consist of MR ramping-up time, AGC fine tuning time, and T/F tracking time. With such high SINR condition, the 5 SSBs are sufficient for AGC fine tuning and T/F tracking, but RAN4 may need to check if such sample number needs to be revisited for lower SINR condition, e.g., the SINR is dropped when UE received LP-WUS in case 1, like in the following figure 2. The principle could be: lower SINR case needs more SSB sample number on MR synchronization than the higher SINR case. Thus, the wake-up delay capability and the number of SSBs currently assumed by RAN1 for MR synchronization for SSB periodicity of 20ms shall be associated with specific target SINR condition, e.g., ≥-3dB.

In addition, we didn’t see any impact of the SSB periodicity to the number of SSB for MR synchronization.

Figure 2. the SINR condition for SSB sample number for MR wake-up

We propose to reply to RAN1 that,

RAN4 confirms the number of SSBs currently assumed by RAN1 for MR synchronization for SSB periodicity of 20ms is sufficient, and it is reasonable to assume the same number of SSBs for MR synchronization for different SSB periodicities.

In addition, RAN4 assumes this MR wake-up delay capability and the number of SSBs for MR synchronization shall be associated with specific target SINR condition, e.g., ≥-3dB.

Proposal 15: RAN4 to reply to RAN1 LS that,

- RAN4 confirms the number of SSBs currently assumed by RAN1 for MR synchronization for SSB periodicity of 20ms is sufficient, and it is reasonable to assume the same number of SSBs for MR synchronization for different SSB periodicities.

- In addition, RAN4 assumes this MR wake-up delay capability and the number of SSBs for MR synchronization shall be associated with specific target SINR condition, e.g., ≥-3dB.

Issue 1-2-7: On LP-WUR cell selection evaluation requirements for case 1

As illustrated in our figure 1, we think the typical case is the S criterion threshold is lower than the MR offloading threshold and neighbor cell measurement triggering threshold, and therefore we don’t need to discuss the LR based S criterion evaluation in case#1.

Proposal 16: No need to discuss the LR based S criterion evaluation in case#1.

Issue 1-2-12: LR measurement metrics used for entry/exit threshold evaluation for LP-WUS monitoring/RRM relaxation

In previous RAN2 meeting, it was agreed that,

### Procedure and configuration of LP-WUS in RRC_IDLE/INACTIVE - Baseline: The network does not need to be aware of whether the UE is monitoring LP-WUS or not in RRC_IDLE/INACTIVE - Separate entry/exit thresholds can be configured for OFDM-based and OOK-based WUR if a cell supports both types of LRs. Signalling details are FFS. - Working assumption (can revisit if R1/R4 reached different conclusions): If the entry/exit conditions are configured, besides MR-based thresholds, LP-WUS monitoring entry condition can also include LR-based thresholds. - The metrics for serving cell quality measured by MR/LR for entry condition includes (LP-)RSRP and optional (LP-)RSRQ. - The metrics for serving cell quality measured by LR for exit condition includes (LP-)RSRP and optional (LP-)RSRQ.

Regarding issue 1-2-6, it’s clear that RAN2 had some conclusions already and still working on the threshold design. We recommend to just follow the RAN2 working assumption and agreements, and no need to discuss it in RAN4.

Proposal 17: For LP-WUR, LR measurement metrics have (LP-)RSRP and optional (LP-)RSRQ.

# MR RRM relaxation

Issue 1-3-1: MR RRM relaxation for serving cell/neighbour cell for case 3

Concluded in last RAN4 meeting.

# LP-WUR CONNECTED mode

Issue 1-4-1: LP-WUR at CONNECTED mode

In the WID, the scope for connected mode is to discuss the procedures to allow UE MR PDCCH monitoring triggered by LP-WUS including activation and deactivation procedure of LP-WUS monitoring, however, like in R16 WUS design, it would not have any RRM impact by using LP-WUS in connected mode. Furthermore, WID mentioned that,

In CONNECTED mode, UE MR ultra-deep sleep is not considered, and UE RRM/RLM/BFD/CSI measurements are performed by MR

All the RRM/RLM/BFD/CSI measurement would be as same as legacy case, i.e., use MR to perform these functionalities. Thus, no RRM scope shall be needed for connected mode in this WI.

Proposal 18: no RRM objectives is needed for connected mode in this LP-WUR/LP-WUS WI.

# Others

Issue 1-5-1: LR based RRM with EMR in IDLE/inactive mode

In RAN4 #111 and 112 meeting, the different cases for MR offloading and relaxation are captured in the WF R4-2410296 and R4-2413899, and so far, at least case#1 and case#3 are valid case (like in proposal 13) to consider in this WI,

| RRM measurement case index | MR serving cell measurement | MR neighboring cell measurement | LR measurement |
| --- | --- | --- | --- |
| #1 Fully offloading case | Off | Off | ON |
| #3 Relaxed case b | On with relaxation measurement | On with relaxation measurement | ON |

However, the EMR measurement has not been considered yet for both case #1 and case #3. In legacy IDLE/Inactive mode RRM requirement, the relaxation is not allowed when EMR is configured on that frequency layer and T331 is running, like followings:

Then, if EMR measurement is also configured and TS331 is running for case#1 and case#3, the MR status may need further discussion. For a UE which supports idleInactiveNR-MeasReport-r16 or idleInactiveEUTRA-MeasReport-r16, and serving cell configures carriers for idle mode CA/DC measurement reporting with T331 running, 	UE may keep the MR ON for EMR measurement regardless of the MR offloading condition is met or not.

Proposal 19: For a UE which supports idleInactiveNR-MeasReport-r16 or idleInactiveEUTRA-MeasReport-r16, and serving cell configures carriers for idle mode CA/DC measurement reporting with T331 running, UE shall keep the MR ON for EMR measurement regardless of the MR offloading condition is met or not.

# Conclusion

In this contribution, we continue the discussion on the open issues in [2].

Proposal 1: at this stage, remove case#2 and case#4 from the last meeting WF, and only following cases are considered. RAN4 can revisit the cases if in the future RAN1/2 has any conclusion conflicting with RAN4 assumptions.

| RRM measurement case index | MR serving cell measurement | MR neighboring cell measurement | LR measurement |
| --- | --- | --- | --- |
| #1 Fully offloading case | Off | Off | ON |
| #3 Relaxed case b | On with relaxation measurement | On with relaxation measurement | ON |

Proposal 2: assumption of thresholds for RAN4 discussion can be:

- the threshold for MR fully offloading to LR can be same as the threshold for neighbor cell measurement triggering (highest one between SIntraSearchP/SIntraSearchQ and SnonIntraSearchP/SnonIntraSearchQ), or at least offloading threshold can be not lower than threshold for neighbor cell measurement triggering.

Proposal 3: LR measurement can be used to check the criteria for neighbor cell measurement triggering/relaxation (in case #1).

Proposal 4: to support proposal 3, LR measurement result shall be comparable to MR measurement result or shall be equivalent to MR measurement result with certain offset/margin (e.g., LR threshold is MR threshold + offset/margin).

Proposal 5: if RAN4 cannot make conclusion on issue 1-1-2, we recommend RAN4 sending LS to RAN2 for guidance.

Proposal 6: Criteria (entry/exit conditions) for LP-WUR serving cell measurement and MR RRM measurement relaxation can be left to RAN2 to decide.

Proposal 7: For the entry/exit condition for LP-WUS monitoring, follow RAN1/2 conclusions or can be triggered by other groups if necessary.

Proposal 8: regarding RAN#106 WF RP-243266, to wait RAN1 for solution selection based on RAN2 reply LS (e.g. solution 1, 2, and 3 in RP-242656), and then RAN4 can work on the requirement design if needed.

Proposal 9: agree to adopt either one of the following options:

- Option 1: Support relaxed higher priority frequency layer search at case 1, i.e., perform higher priority frequency layer search at least every 1 hour.

- Option 2: Higher priority layer neighbour cell measurement shall also be OFF for case #1.

Proposal 10: FR2 LP-WUR based RRM requirement can be discussed after we concluded on the FR1 LP-WUR based RRM requirement.

Proposal 11: For RAN4 requirement of LR based RRM measurement in Idle/inactive states, no dedicated accuracy requirement is defined in the performance section, and reflect the accuracy performance as a margin in the core requirement.

Proposal 12: regardless of whether LP-SS is configured or not, periodicity for SSB based LP-WUR measurement delay requirements is

- based on LO periodicity

- Note: DRX cycle and LO periodicity are the same based on RAN1 agreement.

Proposal 13: For MR evaluation requirement for LP-WUR monitoring, case 1 and case 3, reuse Nserv in existing spec, and Nserv for case 3 also includes the MR relaxation factor.

Proposal 14: RAN4 only needs to define the wake-up delay requirement, if such requirement is necessary from majority companies’ view, where the wake-up delay means minimum gap time between LP-WUS reception and MR to start PDCCH monitoring.

Proposal 15: RAN4 to reply to RAN1 LS that,

- RAN4 confirms the number of SSBs currently assumed by RAN1 for MR synchronization for SSB periodicity of 20ms is sufficient, and it is reasonable to assume the same number of SSBs for MR synchronization for different SSB periodicities.

- In addition, RAN4 assumes this MR wake-up delay capability and the number of SSBs for MR synchronization shall be associated with specific target SINR condition, e.g., ≥-3dB.

Proposal 16: No need to discuss the LR based S criterion evaluation in case#1.

Proposal 17: For LP-WUR, LR measurement metrics have (LP-)RSRP and optional (LP-)RSRQ.

Proposal 18: no RRM objectives is needed for connected mode in this LP-WUR/LP-WUS WI.

Proposal 19: For a UE which supports idleInactiveNR-MeasReport-r16 or idleInactiveEUTRA-MeasReport-r16, and serving cell configures carriers for idle mode CA/DC measurement reporting with T331 running, UE shall keep the MR ON for EMR measurement regardless of the MR offloading condition is met or not.

# References

[1] R4-2502600, WF on RRM requirements for NR_LPWUS, Moderator (vivo), RAN4 #114

[2] R4-2500539, Topic summary for [114][225] NR_LPWUS, vivo, RAN4 #114

[3] R1-2501624, LS on the wake-up delay for LP-WUS operation in IDLE/INACTIVE mode, RAN1, Apple, RAN1 #120

[4] R2-2501394, Reply LS on LP-WUS operation in IDLE/INACTIVE mode, vivo, RAN2 #129