3GPP RAN WG4 Meeting #114bis		R4-2503549

Wuhan, China, April 7th – 11th, 2025

Agenda item:	7.9.3

Source:	Apple

Title:	On Rx requirements for new satellite broadcast band

WI/SI:	LTE_band_5G_bcast_GSO-Core

Release:	Rel-19

Document for:	Approval

# 1	Introduction

Considering that there has been very strong interest from satellite and broadcast operators to work with 3GPP partners to promote the commercial implementation of 3GPP compliant 5G broadcast service utilizing a geosynchronous satellite in Asia-Pacific region, a new spectrum work item on “New band for 5G Broadcast Utilizing Geosynchronous Satellite” [1] was approved in RAN #107 meeting. The objective is to specify a new SDO band for LTE-based 5G broadcast operating in 1467 to 1492 MHz in Region 3 for GSO with the related UE and SAN RF core requirements for 10MHz PMCH.

In this contribution, we share our preliminary views on the UE RF core requirements for the new 5G broadcast band utilizing geosynchronous satellite, in particular, the REFSENS, maximum input level, and out-of-band blocking requirements.

# Discussion

The LTE-based 5G broadcast operation for terrestrial network has already been developed in 3GPP [2]. To our understanding, the UE RF core requirements were specified based on the underlining assumption that the application would leverage the existing handheld UE design for LTE. Therefore, the requirements were mostly derived from the LTE specifications [3]. Similarly, for the new 5G broadcast band utilizing geosynchronous satellite, it is also our understanding that the existing handheld UE design for LTE would be the targeted device.

Observation 1: For the new 5G broadcast band utilizing geosynchronous satellite, it is expected the existing handheld UE design for LTE would be the targeted device.

In the following sub-sections, we share our preliminary views on the REFSENS, maximum input level, and out-of-band blocking requirements for the new satellite broadcast band.

## 2.1	REFSENS

In RAN4, UE REFSENS requirements have been defined based on the following generic formula:

REFSENS (dBm) = -174 dBm/Hz + NF (dB) + 10*log10(BW (Hz)) - diversity gain (dB) + IM (dB) + SNR (dB)

For LTE-based broadcast operation, despite the Reference Measurement Channel (RMC) is PMCH instead of PDSCH as for LTE, considering the common device for both applications, there would be no differentiation for all the parameters in the above REFSENS formula between PMCH and PDSCH. As a result, the existing LTE REFSENS requirement for TDD band (not subject to Tx self-interference) for the similar frequency range as the new satellite broadcast band, such as Band 50, can be applied, i.e., -97 dBm for 10MHz channel bandwidth.

Proposal 1: The REFSENS for 10MHz PMCH for the new satellite broadcast band can be specified as -97 dBm.

## 2.2	Maximum input level

In 3GPP specifications, the maximum input level is the only UE RF requirement specified to verify the higher order modulations other than QPSK. Practically, maximum input level needs to take into account the maximum DL transmission power from base station and the minimum coupling loss between the base station and UE.

Since the new satellite broadcast band is utilizing the geosynchronous satellite which is approximately 35000 km above earth, the minimum coupling loss can be estimated by the free space path loss which is about 187 dB for 1.5GHz carrier. If the satellite transponder could not transmit more than 90dBm output power, the UE maximum input power would be close to REFSENS level or less. In that case, it would not be meaningful to specify maximum input level requirement.

Observation 2: For the new 5G broadcast band, if the satellite transponder could not transmit more than 90dBm output power, it would not be meaningful to specify maximum input level requirement considering the minimum coupling loss is no less than 187 dB.

Proposal 2: Maximum input level requirement needs to take into account the maximum DL transmission power from SAN and the minimum coupling loss between SAN and UE.

## 2.3	Out-of-band blocking

In 3GPP specifications, the out-of-band blocking requirements have been specified with wanted signal power level at (REFSENS + 6dB) or higher. Therefore, subject to the analysis of maximum input level, if the maximum signal power reaching UE antenna would be close to REFSENS or less, it would not be meaningful to specify out-of-band blocking requirements, and same for other Rx requirements than REFSENS.

Observation 3: If the maximum signal power reaching UE antenna would be close to REFSENS or less, it would not be meaningful to specify out-of-band blocking requirements, and same for other Rx requirements than REFSENS.

On the other hand, if RAN4 finds it necessary to specify out-of-band blocking requirements, in our view, the requirements shall be specified based on RF filter sharing with LTE Band 50 instead of being specified based on a new RF filter design specific for the new band frequency range to avoid the impact to the existing handheld UEs supporting LTE.

Proposal 3: The out-of-band blocking requirements for the new satellite broadcast band shall be specified based on RF filter sharing with LTE Band 50.

# 3	Conclusion

In this contribution, we share our preliminary views on the UE RF core requirements for the new 5G broadcast band utilizing geosynchronous satellite, in particular, the REFSENS, maximum input level, and out-of-band blocking requirements.

Observation 1: For the new 5G broadcast band utilizing geosynchronous satellite, it is expected the existing handheld UE design for LTE would be the targeted device.

Observation 2: For the new 5G broadcast band, if the satellite transponder could not transmit more than 90dBm output power, it would not be meaningful to specify maximum input level requirement considering the minimum coupling loss is no less than 187 dB.

Observation 3: If the maximum signal power reaching UE antenna would be close to REFSENS or less, it would not be meaningful to specify out-of-band blocking requirements, and same for other Rx requirements than REFSENS.

Proposal 1: The REFSENS for 10MHz PMCH for the new satellite broadcast band can be specified as -97 dBm.

Proposal 2: Maximum input level requirement needs to take into account the maximum DL transmission power from SAN and the minimum coupling loss between SAN and UE.

Proposal 3: The out-of-band blocking requirements for the new satellite broadcast band shall be specified based on RF filter sharing with LTE Band 50.

# 4	References

RP-250788 “New band for 5G Broadcast Utilizing Geosynchronous Satellite”, Astrum Mobile, Qualcomm, 3GPP TSG-RAN Meeting #107, Incheon, Korea, March 10th – 14th, 2025

RP-211145 “New bands and bandwidth allocation for LTE based 5G terrestrial broadcast - part 2”, EBU, Qualcomm, 3GPP TSG-RAN Meeting #92-e, Online, June 14th – 18th, 2021

3GPP TS 36.101 V19.0.0 (2024-12)