3GPP TSG-RAN4 Meeting #114Bis	R4-2503557

Wuhan, Hubei, China, 07th – 11th April, 2025

Title:	UE performance in one RX mode

Source:	Qualcomm Incorporated

Agenda item:	7.5.3

Release:	Rel-19

WI: 	FS_NR_FR1_DL_Frag_Carrier

Lead WG: 	RAN4

Document for:	Approval

# 1.	Introduction

Ran4 approved a WF [3] on DL Fragmented carriers study SI [1]. In discussion in the study a possible feature to measure in-gap aggressors has been introduced [8] and deemed feasible. While it is true that UE can perform measurements but it is unclear how these results can be interpreted. In this paper we discuss some complex issues related to the in-gap measurement procedure.

We also discuss the UE single chain receiver linearity and provide our view of the analysis tables put in to the TR [4].

# 2. 	Discussion

## 2.1	In-gap aggressor measurement handling

In RAN4 a discussion in the study, the feasibility of the UE to perform measurements has been discussed and assumption is that UE measures in-gap RSSI for possible aggressors. What has not been discussed is the accuracy and interpretation of the results.

When the UE measures an aggressor, it also has to measure the wanted signal power since the relative difference matters. In the proposed method, network would receive in-gap aggressor information from the UE and then instruct the UE to use either single RX or 2Rx to receive the two NC CCs depending on aggressor situation.

What is unclear is what information network would use to do the decision since one UE may be better at tolerating an aggressor than an other UE.

Observation 1: Mapping of in-gap RSSI level or any other measurement result to its impact to channel quality for this UE depends on UE implementation

RAN4 could come up with a table of aggressor levels vs degraded throughput performance, for example to extend the ACS and in-band blocking requirement table to cover multiple aggressor level and wanted signal levels. Since we have two wanted signals, RAN4 needs to consider multiple different wanted power levels in the table. This could be a table with test conditions without a requirements and then RAN5 could come up with a test procedure where the throughput results are stored and then network can access these results for this type of a UE, like from a serve AI/ML work item is planning.

Observation 2: In order to map the aggressor impact to the UE, ran4 needs to define a database with aggressor test cases and impact to throughput and make this accessible to the network.

The test list should also contain tests with multiple in-gap aggressors and also different kinds, depending on how the spectrum is deployed. Operators are encouraged to provide input on the specific spectrum usage for this list e.g. if GSM type aggressors should be accommodated or not.

Since this same UE must support same NC DL CA also with 2Rx chain solution, RAN4 should discuss if this same UE should be tested for 2Rx mode for the same tests or is it sufficient to rely on exiting conformance tests only. This maybe needed since the test procedure for in gap aggressor levels and corresponding result may vary when UE is configured for 2Rx mode.

Observation 3: RAN4 needs further discussions how to handle the UE measurement report of the in-gap aggressor measured in 2Rx mode.

Different solution to make this method better suitable to different UE implementation more simply what is discussed above is for network to request channel quality information under aggressor conditions. Network could deduct the UE specific information about how the measured aggressor impacts this UE’s channel from its CQI information from the same UE but then the purpose of measuring the in-gap aggressor is questionable since the end result, impact to channel quality is already available without the aggressor measurement.

Observation 4: If UE reports its channel quality when aggressor is present, then the in-gap aggressor measurement is redundant information.

What would simplify this concept is to focus on decisions if UE can operate in NC DL CA either with 1Rx or 2rx mode to be made based on its reported channel quality. We discuss and propose this method in our companion paper [6].

## 2.2	UE single RX linearity

WF [3] and TP [5] discuss relaxations for NC DL CA selectivity tests with single RX is used. We discussed linearity requirements in [7] and concluded that UE equipped with a receiver that can support 256QAM DL is capable for handling 39 dBc PSD difference and receive wanted signal without any throughput loss. The single CC ACS requirement is 33 dB for narrow bandwidths and that has been already relaxed to 17 dB as written in Table 7.5A.1-1a of TS 38.101-1.

And as discussed in [5], the RAN5 test procedure is not testing even this but sets the PCell level so that it does not interfere the test result.

In agreed TP’s [5] a table with needed relaxations was put in to the TR. This table and the resulting ΔRIBNC seems to depend greatly on the NF increase due to AGC adjustment which is implementation specific and not explained in the provided TDocs. In the analysis, the degradation is counted in full toward the relaxation on top of existing NC DL CA  ΔRIBNC. However, the NF degradation is counted from the case without aggressors but in the existing ACS and in-band blocking case, REFSENS is already relaxed by 14 dB which allrady accommodates the degradation.

Observation 5: In the analysis for the single RX chain impact to ΔRIBNC, the NF degradation due to AGC is counted on top of exiting NF degradation allowance

## 2.3	Network side requirements

In TP [4] system simulation results were provided for the case with aggressors in gap. The Figure 6-4 shows that aggressor RSRP is larger than wanted signal only in very rare cases. The graph does not show the values but it implies they are very small. Focusing on 95 % of the cases, the aggressor is lower PSD than the wanted signal.

Observation 6: Based on agreed TR for this study item, the current NC DL CA selectivity test is greatly overengineered for the real network cases.

This makes the discussion on the UE capability redundant since the aggressor can be specified to be on the same level as the wanted signal and as the other agreed text indicates, single RX chain design can accommodate level of aggressor without any degradation on REFSENS. RAN4 should also relax requirements for all NC DL CA cases and therefore avoid the complicated test procedures.

We performed out own simulations and also looked at different cases, such as non-collocated deployment and results are proposed for TR in [9]. Results reproduced in Figure 1 below:

Figure 1: Received RSRP of own and wanted signals.

It can be observed that aggressor can be much higher than what is presented in Figure 6-4.Teh SID scope is co-located and for that purpose, there is no aggressor stronger than own signal and RAN4 could relax the aggressor specification.

Observation 7: For co-located scenario, there is no need for aggressor stronger than wanted signal.

It might be better to adopt this approach than to relax sensitivity.

If RAN4 wants to cover also non-collocated scenarios and possibly protection from other systems with single rx design, then any selectivity or blocking requirement should not be relaxed for single RX requirement.

Proposal : In order to avoid degrading UE performance in the system in general, possible relaxed requirements for ACS, REFSENS, In-band blocking will apply only to single RX and co-located deployment scenario

# Conclusion

We discussed, observed and proposed:

Observation 1: Mapping of in-gap RSSI level or any other measurement result to its impact to channel quality for this UE depends on UE implementation

Observation 2: In order to map the aggressor impact to the UE, ran4 needs to define a database with aggressor test cases and impact to throughput and make this accessible to the network.

Observation 3: RAN4 needs further discussions how to handle the UE measurement report of the in-gap aggressor measured in 2Rx mode.

Observation 4: If UE reports its channel quality when aggressor is present, then the in-gap aggressor measurement is redundant information.

Observation 5: In the analysis for the single RX chain impact to ΔRIBNC, the NF degradation due to AGC is counted on top of exiting NF degradation allowance

Observation 6: Based on agreed TR for this study item, the current NC DL CA selectivity test is greatly overengineered for the real network cases.

Observation 7: For co-located scenario, there is no need for aggressor stronger than wanted signal.

Proposal : In order to avoid degrading UE performance in the system in general, possible relaxed requirements for ACS, REFSENS, In-band blocking will apply only to single RX and co-located deployment scenario

# Reference

[1]	RP-243002, “Revised SID: Study on NR FR1 DL fragmented carriers”, MediaTek Inc., 3GPP TSG RAN Meeting #106, Madrid, Spain, Dec 09-12, 2024

[2]	R4-2420519, “WF on NR FR1 DL fragmented carriers study”, MediaTek, 3GPP TSG-RAN WG4 Meeting #113, Orlando, US, 18th – 22nd November, 2024

[3]	R4-2502873, “WF on fragmented DL”, MediaTek, 3GPP TSG-RAN4 Meeting #114, Athens, Greece,  16th – 21st February, 2025

[4]	R4-2502925, “TP for TR38.755”,  MediaTek Inc., Nokia, 3GPP TSG-RAN4 Meeting #114Bis, Athens, Greece,  16th – 21st February, 2025

[5]	R4-2502949, “Impacts on UE RF requirements and DL performance impacts”, MediaTek Inc. 3GPP TSG-RAN4 Meeting #114Bis, Athens, Greece,  16th – 21st February, 2025

[6]	R4-2503556, “Procedures for fragmented carriers”, Qualcomm Incorporated, 3GPP TSG-RAN4 Meeting #114Bis,Wuhan, Hubei, China, 07th – 11th April, 2025

[7]	R4-2418089,” NC DL CA vs single CC ACS”, Qualcomm, 3GPP TSG-RAN4 Meeting #113 Orlando, US, 18th – 22nd November, 2024

[8]	R4-2500244,” On UE RF requirements for fragmented carriers”, Apple, 3GPP TSG-RAN4 Meeting  #114	Athens, GR, 17th – 21st February, 2025

[9]	R4-2503558, “TP for TR38.755: Company C evaluation results”, Qualcomm Incorporated, 3GPP TSG-RAN4 Meeting #114Bis,Wuhan, Hubei, China, 07th – 11th April, 2025