3GPP TSG-RAN4 Meeting #114Bis	R4-2503561

# Wuhan, Hubei, China, 07th – 11th April, 2025

Title:	RX Switching time, frequency and location

Source:	Qualcomm Incorporated

Agenda item:	7.7.2

Release:	Rel-19

WI: 	NR_LBCA_Sw-Core

Lead WG: 	RAN4

Document for:	Approval

# 1.	Introduction

For WI LB DL CA via switching, WF and LS were approved in RAN4#114. LS informed RAN1 of the switching times 35usec and 140 usec with notions that more maybe added, especially mentioning the 13 usec. WF also agreed that more bands can be aggregated with this type of band pair. This paper discusses more advanced way to indicate the switching period and some details on the switching time mask and period location.

# 2. 	Discussion

## 2.1	Switching time discussion

As discussed in [5], faster or even zero switching time is possible for some UEs and offline comments from some OEMs confirmed that antenna performance maybe compromised by few dB’s if antenna tuning is covering full bandwidth of the two bands. The performance loss of the antenna was not discussed further even it is common understanding that dual pole tuning for the antenna could recover some of the losses. From rest of the UE implementation side, UE supporting CA has two RX chains available for reception and therefore is capable for simultaneous reception, as also stated in [7]. The proposal for 13 usec in [6] did not disclose assumption of what parts are re-used and are capable for the 13 usec switching time, in other words, what parts in UE consume the 13 usec.

Observation 1: Faster switching periods than 35 usec are possible for UE in conducted domain

But with the discussion above combined with the agreement in Issue 1 in [3], the UE that may be able to support fast switching time, may need to slow down the switching when other bands are configured to it. It is therefore proposed

Proposal 1: Define UE switching period capability for this type band pair dependent on the number of simultaneously configured aggregated bands

In this way, UE can declare the switching period either for optimised set of dedicated hardware or when UE needs to re-use some hardware blocks since they are allocated to the other bands and UE needs more time to switch.

It is understood by the author that the scope of this WI is only two bands but respecting the agreement in Issue 1 and preparing to enable release independence from this release for more advanced band configurations, it would help discussions in next release to adopt this approach now.

## 2.2	Transient period and its location

It should be firstly noted that there is no such requirement as receive time mask in RAN4. Also there is only one TX chain involved and therefore TX does not need to be switched anywhere, it can always be configured to the only configured UL CC. Since the unwritten assumption of this work was that antenna is not able to operate on both bands simultaneously, it means that the FDD TX and SDL can not operate simultaneously and how puncturing of the FDD TX is done needs some discussions.

The time masks in section 6.3A are applicable when the UE congested with UL grants, i.e. when UE is expected to be transmitting on two bands without sufficient gap in between the grants for it to switch. Then UE is assumed to puncture the grant for the switching period. If UE is provided with a gap in UL grants that is longer than the switching period, UE is expected to use that gap to switch. Same approach should be adopted here

Proposal 2: When there is a gap in UL and DL grants between FDD TX and SDL, UE is assumed to switch in the gap without puncturing transmission or reception on either of the bands.

When UE is scheduled to receiver or transmit in such way that there is a conflict between grants and switching period, then a rule for the location of the switching period should be defined so that the UE behaviour is predictable to the network. This can be because of network making a mistake between the persistent and semi-persistent scheduling, i.e. HARQ and PUSCH grants, or that the timing advance causes an overlap at the UE. RAN1 discussing [8] placement of the switching period and it is possible that they will define a location that will never conflict but if such case is not defined in RAN1, we propose the following:

Proposal 3: In case of conflicting UL and DL grants in before and after switching instance, UE is to prioritize FDD UL, pending for confirmation from RAN1.

RAN4 could include a question of handling of this error case in liaison statement to RAN1.

## 2.3	Switching frequency

RAN4 BB room and also RAN1 are discussing the possible patterns in more detail but they are not considering the lost efficiency because the outage during the switching period. Since 35 and 140 usec were both accommodated in the WF and LS, [3], [4], it would be good to agree the following proposal:

Proposal 4: Switching frequency is at most once per slot from the UE perspective

# Conclusion

Observation 1: Faster switching periods than 35 usec are possible for UE in conducted domain

Proposal 1: Define UE switching period capability for this type band pair dependent on the number of simultaneously configured aggregated bands

Proposal 2: When there is a gap in UL and DL grants between FDD TX and SDL, UE is assumed to switch in the gap without puncturing transmission or reception on either of the bands.

Proposal 3: In case of conflicting UL and LD grants in before and after switching instance, UE is to prioritize FDD UL.

Proposal 4: Switching frequency is at most once per slot from the UE perspective

# Reference

[1]	RP-243317, “New WID on low band carrier aggregation via switching”, Apple, 3GPP TSG RAN Meeting #106, Madrid, Spain, December 9-12, 2024

[2]	RP-242532, “Motivation for a new work item on low band carrier aggregation via switching in Rel-19”, AT&T, 3GPP TSG RAN Meeting #106, Madrid, Spain, December 9-12, 2024

[3]	R4-2503016, “WF on low-low band switching”, Apple, 3GPP TSG-RAN WG4 Meeting #114, Athens, Greece, February 17-21, 2025

[4]	R4-2502877, “LS on switching periods for low-low band switching”, AT&T, 3GPP TSG-RAN WG4 Meeting #114, Athens, Greece, February 17-21, 2025

[5]	R4-2500492, “Initial aspects of RX Switching”, Qualcomm, 3GPP TSG-RAN WG4 Meeting #114, Athens, Greece, February 17-21, 2025

[6]	R4-2501572, “Time masks for LB-LB CA switching”, Ericsson, 3GPP TSG-RAN WG4 Meeting #114, Athens, Greece, February 17-21, 2025

[7]	R4-2500405, “NR_LBCA_Sw Considerations”, Nokia, 3GPP TSG-RAN WG4 Meeting #114, Athens, Greece, February 17-21, 2025

[8]	R1-2502860, “Low band carrier aggregation via switching”, Qualcomm Incorporated, 3GPP TSG-RAN4 Meeting #114Bis, Wuhan, Hubei, China, 07th – 11th April, 2025