# 3GPP TSG-RAN WG4 Meeting #114bis	                                                 R4-2503569

# Wuhan, China, 7 – 11 April, 2025

Agenda Item:	7.25.2.1

Source: 	Apple

Title:	Discussion on on-demand SSB SCell operation

Document for:	Discussion

# Introduction

RAN4 requirements for OD-SSB was widely discussed in the previous RAN4 meeting. The latest agreements can be found in [1]. There are quite many open issues as captured in [1].

In this contribution, we will discuss all open issues listed in RAN4 WF [1]. After discussion, corresponding proposals are provided.

# Discussion

### 2.1 Topic #1: On-demand SSB(OD-SSB) requirements

#### 2.1.1 OD-SSB based deactivated SCell L3 measurement procedure (Case 1)

- Background

  - The deactivated SCell measurement requirement in FR1 is as follow.

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | Ceil(5 x Kp) x measCycleSCell x CSSFintra |
| DRX cycle≤ 320ms | Ceil(5 x Kp) x max(measCycleSCell, 1.5xDRX cycle) x CSSFintra |
| DRX cycle> 320ms | Ceil(5 x Kp) x max(measCycleSCell, DRX cycle) x CSSFintra |

  - The OD-SSB based deactivated SCell measurement procedure is shown as follow.

- Sub-issue 1-1 – The length of FMW

Agreement:

- The time window is defined with Tuncertain plus actual measurement duration.

  - Tuncertain equals Tmin(RAN1) plus additional processing/preparation time, if it allows.

  - The actual measurement duration equals min(Treport , Tidentify) from the end of uncertain time.

    - Treport is the first OD-SSB based L3 measurement reporting

    - Tidentify equals the minimum RAN4 OD-SSB meas. period requirement as follow.

      - A detected cell: OD-SSB based measurement period

      - A newly detectable cell: OD-SSB based cell identification period

No OD-SSB measurement requirement applies in fast window if NW deactivates the OD-SSB earlier than the end of min(Treport, Tidentify).

- Sub-issue 1-2: What’s the expected minimum interval(T2, sage 4) between multiple FMWs?

Agreement:

RAN4 to define measurement requirement based on T2.

- When NW re-activates the OD-SSB within the T2,

  - Option 1: UE is not required to perform “fast measurement” but follows legacy deactivated SCell measurement requirement.

  - Option 2: UE is required to perform “fast measurement” without cell search.

    - The UE is assumed to skip the PSS/SSS detection and/or SSB index acquisition.

Option 3: UE is required to perform FMW after T2.

- When NW re-activates/re-configures the OD-SSB transmission after T2, UE performs the OD-SSB based fast L3 measurement within FMW

  - The UE is assumed to meet fast cell identification and measurement.

- The start point of T2 is based on

  - Option 1: the time point Tend

  - Option 2: the latter point of L3 measurement reporting for FMW and Tend

  - Option 3: the latter point of latest L3 measurement reporting before NW re-indicating OD-SSB transmission and Tend

  - Option 4: the time point of OD-SSB deactivation

  - Other options are not precluded

Where, Tend equals the time point of OD-SSB indication plus Tuncertain +Tidentify applies to option 1,2,3.

- The duration is

  - Option 1: 5*max(measCycleSCell,  DRX cycles) [– OD-SSB periodicity]

  - Option 2: 5* measCycle when measCycle =160 ms

  - Other options are not precluded

- The activation and deactivation of OD-SSB is fully up to NW.

The open issue is regarding T2 design. The first issue is about UE measurement behaviour when NW re-activates the OD-SSB within the T2. The key motivation of T2 is to avoid unnecessary UE power consumption. Therefore, it is rational to assume that UE doesn’t need to perform “fast measurement” but follow legacy deactivated SCell measurement requirement.

Regarding starting point of T2, it is proposed to have a simple design at both NW and UE side, i.e. a fixed time point rather than some dynamic time point. Existing option 4 (the time point of OD-SSB deactivation) seems a very good solution.

As for T2 duration, it is preferred to reuse existing time period in known cell definition, i.e.

-	for FR1, T2 equals to max(5*measCycleSCell,  5*DRX cycles)

-	for FR2, T2 equals to 4 s for UE supporting power class 1/5 and 3 s for UE supporting power class 2/3/4

Proposal 1: When NW re-activates the OD-SSB within the T2, UE is not required to perform “fast measurement” but follows legacy deactivated SCell measurement requirement.

Proposal 2: The starting point of T2 is the time point of OD-SSB deactivation.

Proposal 3: duration of T2:

-	for FR1, T2 equals to max(5*measCycleSCell,  5*DRX cycles)

-	for FR2, T2 equals to 4 s for UE supporting power class 1/5 and 3 s for UE supporting power class 2/3/4

One aspect that has never been discussed before is how to handle collision between FWM and measurement gap (MG).

Considering the complexity of cancelling measurement gap, as being discussed in XR, we propose the following UE measurement behavior:

- If FMW covers any measurement gap occasion in time domain, UE is allowed to skip OD-SSB measurement and perform measurement on other carriers within measurement gap occasion(s).

With this, there is no impact on gap-based RRM measurement.

Proposal 4: If FMW covers any measurement gap occasion in time domain, UE is allowed to skip OD-SSB measurement and perform measurement on other carriers within measurement gap occasion(s).

#### 2.1.2 OD-SSB based deactivated SCell L3 measurement (Case 2)

Issue 2-1: (Case 2) OD-SSB based deactivated SCell L3 measurement when OD-SSB transmission is triggered

Agreement:

When NW indicates the OD-SSB transmission for deactivated SCell measurement(Case 2 Scenario 2),

- Option 1: The same approach in Case 1 can be reused.

  - UE performs the OD-SSB based fast L3 measurement following OD-SSB periodicity for deactivated SCell within a time window. After the time window, UE fallbacks to a slow mode measurement(with DRX and measCycleSCell).

- Option 2:

  - If the serving cell is unknown, the same approach in Case 1 can be reused.

  - If the serving cell is known, UE follows legacy requirements based on measCycleSCell.

    - The known condition reuses the principle of legacy known condition.

    - This scenario applies at least when UE sends the meas. reporting.

  - FFS the case if the L3 meas. report cannot be triggered.

- The above requirement is applied at least when the OD-SSB and AO-SSB are on the same frequency and spatial relation.

We continue supporting not to have fast measurement window for case 2 since it is extremely similar with legacy. Before activation of OD-SSB, UE measures the SCell based on always-on SSB following measCycleSCell. The measurement period depends on several aspects, e.g. DRX cycle, number of carriers to measure, length of measCycleSCell, Kp and so on. Upon OD-SSB activation, if the SCell is already known, it is unnecessary for UE to perform fast measurement since the SCell can already be activated in a fast mode (fulfill known definition). But if the cell is somehow unknown upon OD-SSB activation, we can accept to perform fast measurement similarly with case 1 as a compromised solution.

Proposal 5: OD-SSB based deactivated SCell L3 measurement when OD-SSB transmission is triggered in Case 2:

- Option 1: RAN4 requirement is defined based on legacy measCycleSCell (preferred)

- Option 2:

  - When OD-SSB is activated, UE determines whether to perform fast measurement or not

    - If the serving cell is known (based on RRM on always-on SSB), UE doesn’t need to perform fast measurement. Existing measurement requirements based on measCycleSCell can be applied.

    - If the serving cell is unknown, UE performs measurement in a fast mode within a time window. After the time window, UE falls back to slow mode.

      - Design of fast mode, slow mode and time window can be same as that discussed in case 1.

Besides, RAN1 approved an LS to RAN4 regarding case 2:

Agreement Regarding the relation in terms of time location between the always-on SSB and on-demand SSB, - For the case when the center frequency locations of always-on SSB and on-demand SSB are same, - Alt Time-C: RAN1 specification has no restriction with regards to overlapping - From RAN1 perspective, - Alt Time-C1: The case that, during OD-SSB transmission, the union of AO-SSB transmission and OD-SSB transmission has a periodic time domain pattern is supported (the interval between SSB bursts is even and supported in legacy specification). - Alt Time-C2: The case that, during OD-SSB transmission, the union of AO-SSB transmission and OD-SSB transmission has a non-periodic time domain pattern is supported. - It is up to RAN4 to define requirements, if any, corresponding to both or either of Alt Time-C1 or Alt Time-C2 - At least the following is supported: PBCH payload for the same SSB index (other than SFN index, half frame index) is the same for AO-SSB and OD-SSB - FFS: Whether half frame index is the same or different for AO-SSB and OD-SSB - For the case when the center frequency locations of always-on SSB and on-demand SSB are different, - Alt Time-C: RAN1 specification has no restriction with regards to overlapping - UE assumes that frequency resources of always-on SSB are not overlapped with those of on-demand SSB in frequency domain. - AO-SSB and OD-SSB are located in the same BWP - FFS: PBCH payload for the same SSB index (other than SFN index, half frame index) should be the same for AO-SSB and OD-SSB - NOTE: AO-SSB periodicity is not adapted

For the case when the center frequency locations of always-on SSB and on-demand SSB are different, there are several aspects that we need to think about. For instance, does UE need to measure both AO-SSB and OD-SSB? We know that RAN1 agreed UE is not required to measure both AO-SSB and OD-SSB if always-on SSB is CD-SSB on a synchronization raster. But what if AO-SSB is not on a synchronization raster? Besides, how to interpretate serving cell MO and how handle neighbor cell measurement? Specifically, is there any impact on definition of intra/inter-frequency measurement? For instance, if UE only measures OD-SSB after OD-SSB activation, can we still follow intra-frequency measurement requirements to measure neighbor cell whose SSB is on the same frequency as AO-SSB?

Note that some of the above aspects may depend on RAN2 design. Considering we are coming to the end of core part design, we propose to deprioritize this scenario from RAN4 requirements point of view.

Proposal 6: deprioritize the case when the center frequency locations of always-on SSB and on-demand SSB are different in R19 NES from RAN4 requirement point of view.

#### 2.1.3 OD-SSB based deactivated SCell L3 measurement requirement in FMW (Case 1)

Sub-issue 3-2: OD-SSB measurement in FMW(Case 1-1): single OD-SSB carrier with multiple legacy SSB carriers

Agreement:

When NW configures single OD-SSB carrier with multiple legacy SSB carriers, in FMW

- Option 1: The CSSF only counts OD-SSB layers

- Option 2: The CSSF of OD-SSB carrier counts OD-SSB layers and other layers

Sub-issue 3-3: OD-SSB measurement in FMW(Case 1-2): Multiple active OD-SSB carriers

Agreement:

RAN4 to prioritize single OD-SSB carrier with multiple legacy SSB carriers in deactivated SCell measurement.

- FFS multiple OD-SSB carriers

There are two different cases. Case one is that there are more than one carriers with active OD-SSB to measure.  Case two is there is no any other carrier configured with active OD-SSB but there are other legacy carriers, e.g. with always-on SSB.

For case one, if it has to be included in the scope, CSSF is necessary to scale the corresponding measurement requirements since UE has only up to two searchers. One of them is assigned to PCell and the other one needs to be shared among carriers configured with active OD-SSB.

For case two, CSSF is also necessary since we don’t expect any change in searcher assumption in this work item. We can accept that carrier with active OD-SSB shall be prioritized, provided that T2 (in the following issue 1-2-3) is introduced. In other words, UE can suspend the measurement on legacy carriers considering that network may activate the SCell with OD-SSB soon. Another way to prioritize OD-SSB carrier is to assign more measurement opportunities, e.g. by applying different scaling factors between carriers with OD-SSB and legacy carriers.

Proposal 7: RAN4 shall not change searcher assumption in this work item.

Proposal 8: for the case where there are more than one carriers configured with active OD-SSB:

- Option 1: not supported in this release

- Option 2: introduce CSSF in corresponding measurement requirement

Proposal 9: for the case where there is at least one regular carrier (e.g. with legacy SSB) besides the carrier configured with active OD-SSB:

- Option 1: introduce CSSF in corresponding measurement requirement

- Option 2: prioritize measurement on OD-SSB carrier and allow extension on measurement on other carriers, provided that T2 is agreed.

#### 2.1.4 OD-SSB general issues

Issue 4-2: L1 measurement after SCell activation

Agreement:

- RAN4 to support OD-SSB L1 measurement during and after SCell activation.

  - When non-DRX is configured, the current requirement can be reused with the clarification of TSSB in TL1-RSRP_Measurement_Period_SSB is the SSB periodicity of OD-SSB

- RAN4 to support OD-SSB L1 measurement during SCell activation.

  - The requirement when DRX is configured.

    - Based on OD-SSB periodicity

Wayforward:

- RAN4 to support OD-SSB L1 measurement after SCell activation.

  - FFS the requirement when DRX is configured.

    - Option 1: Follow legacy, based on DRX cycle (vivo, Apple, OPPO, Xiaomi)

    - Option 2: Based on OD-SSB periodicity (Nokia, ZTE, Samsung)

    - Option 3: FFS (HW, QC, Nokia, CMCC, Xiaomi, CATT)

    - Note: the OD-SSB is triggered before or together with the Scell activation command, the OD-SSB transmission kept after the SCell activation until the OD-SSB deactivation command received.

Option 1 is quite straightforward to us. Existing requirement for L1-RSRP is subject to TSSB, DRX, Treport and so on. = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. They can be reused except that TSSB shall refer OD-SSB periodicity, if configured.

Proposal 10: for OD-SSB L1 measurement after SCell activation, Follow legacy, based on DRX cycle and clarify that TSSB is OD-SSB periodicity, if configured.

Issue 6-6: Interruption

Open Issue:

- RAN4 to investigate whether there is impact due to OD-SSB on the allowed interruptions during measurements on deactivated Scell.

For case 1, before OD-SSB is activated there is nothing to measure. There is no point for UE to keep the RF chain on. Upon OD-SSB activation, UE needs to perform fast measurement on OD-SSB immediately. UE would switch on the RF chain upon OD-SSB activation. If network doesn’t activate the SCell during the fast window. UE may switch off the RF chain after the fast window for power saving. Therefore, UE shall be allowed to cause two interruptions before and after the fast measurement window. After the fast window, interruption is same as legacy.

Proposal 11: upon OD-SSB activation, UE is allowed to cause one interruption before the first OD-SSB occasion within the fast measurement window.

Proposal 12: after OD-SSB activation, if UE does not receive SCell activation command, UE is allowed to cause one interruption at the end of fast measurement window. After the fast measurement window, interruption is same as legacy.

One last issue is about the additional preparation time. Although we already sent LS to RAN1 on this issue, there is still an open issue, which is about the additional preparation time for OD-SSB measurement. In our understanding MAC-CE processing time shall be same as legacy, e.g. it can be finished before n+m+3Nslotsubframe+1. However, from UE measurement point of view, some additional preparation time is expected since UE may need to update measurement scheme upon OD-SSB activation. As we all know, two separate searchers are assumed in RRM measurement requirements. Measurement on OD-SSB occupies one searcher. Typically once OD-SSB is activated, UE may assume network is going to activate target SCell soon (e.g. scenario 2A). UE may choose to prioritize OD-SSB measurement.

Even if UE doesn’t always prioritize OD-SSB, effectively there is one more MO to measure upon OD-SSB activation:

Observation 1: OD-SSB activation is effectively similar with that network uses MAC-CE to add one more measurement object.

Proposal 13: additional preparation time T△ on top of n+m+3Nslotsubframe+1 for UE to start measurement on OD-SSB is 5ms.

# Conclusion

In this contribution, we provide discussion on RAN4 requirements for OD-SSB. After discussion, the following conclusions are provided:

Proposal 1: When NW re-activates the OD-SSB within the T2, UE is not required to perform “fast measurement” but follows legacy deactivated SCell measurement requirement.

Proposal 2: The starting point of T2 is the time point of OD-SSB deactivation.

Proposal 3: duration of T2:

-	for FR1, T2 equals to max(5*measCycleSCell,  5*DRX cycles)

-	for FR2, T2 equals to 4 s for UE supporting power class 1/5 and 3 s for UE supporting power class 2/3/4

Proposal 4: OD-SSB based deactivated SCell L3 measurement when OD-SSB transmission is triggered in Case 2:

- Option 1: RAN4 requirement is defined based on legacy measCycleSCell (preferred)

- Option 2:

  - When OD-SSB is activated, UE determines whether to perform fast measurement or not

    - If the serving cell is known (based on RRM on always-on SSB), UE doesn’t need to perform fast measurement. Existing measurement requirements based on measCycleSCell can be applied.

    - If the serving cell is unknown, UE performs measurement in a fast mode within a time window. After the time window, UE falls back to slow mode.

      - Design of fast mode, slow mode and time window can be same as that discussed in case 1.

Proposal 4: If FMW covers any measurement gap occasion in time domain, UE is allowed to skip OD-SSB measurement and perform measurement on other carriers within measurement gap occasion(s).

Proposal 5: deprioritize the case when the center frequency locations of always-on SSB and on-demand SSB are different in R19 NES from RAN4 requirement point of view.

Proposal 6: RAN4 shall not change searcher assumption in this work item.

Proposal 7: for the case where there are more than one carriers configured with active OD-SSB:

- Option 1: not supported in this release

- Option 2: introduce CSSF in corresponding measurement requirement

Proposal 8: for the case where there is at least one regular carrier (e.g. with legacy SSB) besides the carrier configured with active OD-SSB:

- Option 1: introduce CSSF in corresponding measurement requirement

- Option 2: prioritize measurement on OD-SSB carrier and allow extension on measurement on other carriers, provided that T2 is agreed.

Proposal 9: for OD-SSB L1 measurement after SCell activation, Follow legacy, based on DRX cycle and clarify that TSSB is OD-SSB periodicity, if configured.

Proposal 10: upon OD-SSB activation, UE is allowed to cause one interruption before the first OD-SSB occasion within the fast measurement window.

Proposal 11: after OD-SSB activation, if UE does not receive SCell activation command, UE is allowed to cause one interruption at the end of fast measurement window. After the fast measurement window, interruption is same as legacy.

Observation 1: OD-SSB activation is effectively similar with that network uses MAC-CE to add one more measurement object.

Proposal 13: additional preparation time T△ on top of n+m+3Nslotsubframe+1 for UE to start measurement on OD-SSB is 5ms.

# Reference

- R4-2502598 WF on RRM requirements for Netw_Energy_NR_enh_Part1, Ericsson

- R1-2501633, time location of OD-SSB for SCell, LGE