# 3GPP TSG-RAN WG4 Meeting #114bis	                                                 R4-2503573

Wuhan, China, 7 – 11 April, 2025

Agenda Item:	7.27.2.3

Source: 	Apple

Title:	Discussion on conditional Intra-CU LTM

Document for:	Discussion

# Introduction

In RANP#105, an updated WID for NR mobility enhancement phase 4 was approved in [1]. According to the update, conditional intra-CU LTM is to be supported in this release:

- Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Limit specifying the conditional LTM to the scenario where the UE is in non-DC - Checkpoint at RAN#107 to review the objective on whether Intra-CU conditional LTM can be specified to DC scenarios and if so, to which cases. RAN WG work to not start before this checkpoint

Correspondingly, RAN4 needs to specify RRM requirements to support the new procedure. Conditional LTM was preliminarily discussed in RAN4#112bis. The latest agreements can be found in the approved WF [2]. In this contribution, we further discuss the open issues.

# Discussion

## General aspects on Conditional LTM

Issue 1-1-1: The applicable scenarios for conditional LTM

RAN4#113 < Agreement > ad hoc - FFS: RAN4 requirements of CLTM which is triggered by L1 measurement (e.g. event LTM3 and LTM5) does not apply to candidate cells on deactivated SCC, including both serving cell and neighbor cells. - Note: This can be revisited if RAN4 agreed to introduce measurement requirements for L1 measurement on deactivated SCC.

< Agreement >

  - For the case which CLTM is triggered by L3 measurement, deactivated SCell and the cells with same frequency as deactivated Scell are included.

  - For the case which CLTM is triggered by L1 measurement, FFS deactivated SCell and the cells with same frequency as deactivated Scell are included.

we would like to point out that if the CLTM is triggered by L1 measurement, then the LTM candidate cell shall be an active serving SCell in MCG. If CLTM is triggered by L3 measurement, then the LTM candidate cell could be either an active serving SCell or a deactivated SCell. Because if the CLTM is triggered by L1 measurement, Tmeasure for L1 needs to be included in CLTM cell switch delay requirements. However, RAN4 doesn’t have requirements for L1 measurement on deactivated SCell.

Proposal 1: For the case which CLTM is triggered by L1 measurement, deactivated SCell and the cells with same frequency as deactivated Scell are NOT included, given that L1 measurement requirements on deactivated SCell and neighbor cells with same frequency as deactivated SCell are not included in existing spec.

## Early synchronization and TCI state activation

Issue 1-2-1: UE based TA measurement

RAN4#113 < Way forward > FFS the following proposals: - Define RRM requirements for UE based TA measurement. (CTC, CMCC, vivo, QC, CATT) - Do not define RRM requirements for UE based TA measurement. (Apple, MTK)

< Way forward > FFS the following options:

  - Option 1: Define requirements for L1 measurement on deactivated Scell.

  - Option 2: Define requirements for UE based TA measurement.

    - Proponent: The feature is essential for C-LTM. No cell switch command carrying TA for R19 C-LTM. For C-LTM, PRACH based on PDCCH order is very complicated.

    - Concern: Network cannot trust the TA value from UE. Difficult to define proper requirements. There is other command carrying TA.

  - Option 3: Not define requirement for any of them.

We would like to point out that RAN2 already agreed how to handle TA in conditional LTM in November meeting:

- The Early TA is signalled to the UE from the source cell (i.e., not from the candidate cell directly to the UE). This agreement will be included in the LS to RAN1/3/4.

- The network can inform the candidate cell’s TA information to UE via new MAC CE, which is the TA value when UE switches to that candidate cell during CLTM.

- Candidate cell TA is maintained by a new timer.

- For L1-based conditional LTM the condition evaluation is at MAC level and for L3-based conditional LTM the condition evaluation is at RRC level.

Observation 1: RAN2 already agreed new design on how to handle TA for C-LTM. Therefore, C-LTM can work well without UE based TA measurement.

Observation 2: UE based TA measurement has been widely discussed in R18. Eventually RAN4 realized that it is very challenging to define corresponding RAN4 requirements.

Observation 3: UE based TA measurement is a R18 optional feature. It is not clearly in scope of RAN4 R19 requirements.

Proposal 2: Do not define RRM requirements for UE based TA measurement in R19.

## Conditional LTM delay

Issue 1-3-3: The components of conditional LTM delay

< Agreement > FFS the following options ad hoc

- Option 1: DCLTM = [TRRC] + TEvent_DU + Tmeasure + TCLTM_interrupt + TLTM-RRC-processing + Tfirst-RS + TRS-proc

  - TCLTM_interrupt = TLTM-processing + TLTM-IU ms

- Option 2: DCLTM = [TRRC] + TEvent_DU + Tmeasure + TCLTM_interrupt + Tfirst-RS + TRS-proc

  - TCLTM_interrupt = TLTM-RRC-processing + TLTM-processing + TLTM-IU ms

- Option 3: DCLTM = [TRRC] + TEvent_DU + Tmeasure + TCLTM_interrupt

  - TCLTM_interrupt = TLTM-RRC-processing + TLTM-processing + TLTM-IU + Tfirst-RS + TRS-proc ms

We support option 3 as minimum requirements. It also aligns with CHO framework.

Proposal 3: minimum requirements for CLTM delay:

DCLTM = TRRC + TEvent_DU + Tmeasure + TCLTM_interrupt

TCLTM_interrupt = TLTM-RRC-processing + TLTM-processing + TLTM-IU + Tfirst-RS + TRS-proc ms

Issue 1-3-5: TEvent_DU

< Way forward > FFS the following proposals:

- Proposal 1: TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a conditional LTM command until a condition exists at the measurement reference point which will trigger the conditional intra-CU LTM.

    - Proposal 1a: It can cover both cases:

- UE first completes early TCI state activation and/or PDCCH order PRACH and then performs execution condition evaluation

- Neither early TCI state activation nor PDCCH order PRACH is successfully performed before condition is met.

- Proposal 2: If certain process can guarantee that UE first completes DL sync and UL sync and then performs execution condition evaluation, the delay for DL sync and UL sync can be contained in TEvent_DU.

- Proposal 3: Wait for more RAN2 agreements.

We continue supporting proposal 1.

Proposal 4:  TEvent_DU is the delay uncertainty which is the time from when UE transmits RRCReconfigurationcomplete message for the previous CLTM until a condition exists at the measurement reference point which will trigger the subsequent CLTM.

Issue 1-3-6: Tmeasure

< Way forward > FFS the following proposals:

- Proposal 1: Tmeasure is measurements time. It is defined from the end of TEvent_DU until UE executes LTM to a target cell and interruption time starts.

    - Proposal 1a:

  - If SSB based L1-RSRP measurement is used in the event, Tmeasure is SSB based L1-RSRP measurement period in R18 LTM.

  - If SSB based L3-RSRP measurement is used in the event, Tmeasure is SSB based L3-RSRP measurement period in legacy.

  - If CSI-RS based L1-RSRP measurement is used in the event, Tmeasure is CSI-RS based L1-RSRP measurement period which is to introduced in R19.

  - FFS on CSI-RS based L3-RSRP measurement.

    - Proposal 1b:

      - When the condition is L1-based: Use the Rel-18 L1 measurement period for serving cell and candidate cells as the starting point to define Tmeasure.

      - When the condition is L3-based: As the starting point, Tmeasure to follow the definition of Tmeasure for CHO.

As we agreed that both L1 and L3 measurement can trigger CLTM, Tmeasure needs to cover both cases.

Proposal 5: Tmeasure is measurements time. It is defined from the end of TEvent_DU until UE executes LTM to a target cell and interruption time starts.

- If SSB based L1-RSRP measurement is used in the event, Tmeasure is SSB based L1-RSRP measurement period in R18 LTM.

- If SSB based L3-RSRP measurement is used in the event, Tmeasure is SSB based L3-RSRP measurement period in legacy.

- If CSI-RS based L1-RSRP measurement is used in the event, Tmeasure is CSI-RS based L1-RSRP measurement period which is to introduced in R19.

# Conclusion

In this contribution, we provide discussion on measurement related enhancements for LTM. After discussion, the following conclusions are provided:

Proposal 1: For the case which CLTM is triggered by L1 measurement, deactivated SCell and the cells with same frequency as deactivated Scell are NOT included, given that L1 measurement requirements on deactivated SCell and neighbor cells with same frequency as deactivated SCell are not included in existing spec.

Observation 1: RAN2 already agreed new design on how to handle TA for C-LTM. Therefore, C-LTM can work well without UE based TA measurement.

Observation 2: UE based TA measurement has been widely discussed in R18. Eventually RAN4 realized that it is very challenging to define corresponding RAN4 requirements.

Observation 3: UE based TA measurement is a R18 optional feature. It is not clearly in scope of RAN4 R19 requirements.

Proposal 2: Do not define RRM requirements for UE based TA measurement in R19.

Proposal 3: minimum requirements for CLTM delay:

DCLTM = TRRC + TEvent_DU + Tmeasure + TCLTM_interrupt

TCLTM_interrupt = TLTM-RRC-processing + TLTM-processing + TLTM-IU + Tfirst-RS + TRS-proc ms

Proposal 4:  TEvent_DU is the delay uncertainty which is the time from when UE transmits RRCReconfigurationcomplete message for the previous CLTM until a condition exists at the measurement reference point which will trigger the subsequent CLTM.

Proposal 5: Tmeasure is measurements time. It is defined from the end of TEvent_DU until UE executes LTM to a target cell and interruption time starts.

# Reference

- R4-2502602, WF on RRM requirements for NR_Mob_Ph4_Part2, China Telecom