# 3GPP TSG-RAN WG4 Meeting #114bis	                                                 R4-2503576

Wuhan, China, 7 – 11 April, 2025

Agenda Item:	8.2

Source: 	Apple

Title:	Reply LS on granularity definition of pdcch-RACH-AffectedBandsList-r18

Document for:	Discussion

# Introduction

RAN2 sent an LS [1] to RAN4 on granularity definition of pdcch-RACH-AffectedBandsList-r18 with RAN4 action included. In this contribution, we provide our view on the question raised by RAN2.

# Discussion

For information, RAN2 LS is duplicated here:

The UE capability pdcch-RACH-AffectedBandsList-r18 (FG39-4) is defined in RAN4 with the granularity as “Per band pair per band combination (between the target band for RACH transmission and band under UE’s current band combo)” For each reported capability, there are two possible values: interruption or noInterruption. Regarding the band pair (one source band +one target band) of the BC, RAN2 understands that the target band refers to the target band towards which the UE performs PDCCH-ordered early RACH. However, the definition of the source band of the band pair remains unclear in RAN2. - Understanding 1: The source band of the band pair represents the source band (or PCell) on which the PDCCH order is sent to the UE to perform PDCCH-ordered early RACH to the target cell/band. Regarding the interruption band(s), there are two further understandings, denoted by 1a and 1b: - Understanding 1a: The value “interruption/noInterruption” means that interruption is needed or not for all serving bands of the current band combination. - Understanding 1b: The value “interruption/noInterruption” means that interruption is needed or not for the source band (the same band in which the PDCCH order is sent) of the current band combination. For other serving bands, noInterruption can be always assumed. - Understanding 2: The source band of the band pair represents the specific source band that have interruption or not (depends on the value “interruption” or “noInterruption”) in the BC when UE performs PDCCH-ordered early RACH toward a target cell on target band. Please note that in Understanding 1, the UE cannot indicate which specific source band of the BC has an interruption or not (either all bands in current BC in 1a or the same source band that receives the PDCCH order in 1b). In Understanding 2, the information about which source band receives the PDCCH order is not provided.

In R18 discussion, RAN4 didn’t discuss on which band/cell UE receives the PDCCH order to trigger RACH. In fact, whether there is interruption on the band pair or not depends on RF architecture, rather than on which band/cell the PDDCH order is received. Therefore, understanding 1b is not correct from RAN4 point of view.

Between understanding 1a and 2, 1a can save some signaling overhead. However, understanding 2 provides more accurate interruption information. We support to confirm understanding 2.

Proposal 1: confirm understanding 2 from RAN2.

We also prepare a draft LS as in the annex.

# Conclusion

In this contribution, we provide discussion on. After discussion, the following conclusions are provided:

# Reference

- R2-2501392, LS to RAN4 on granularity definition of pdcch-RACH-AffectedBandsList-r18, MTK

# Annex

3GPP TSG-RAN WG4 Meeting #114bis	   	                                                      R4-250xxxx

Wuhan, China, 7 – 11 April, 2025

Title:	Reply LS on granularity definition of pdcch-RACH-AffectedBandsList-r18

Response to:	R2-2501392

Release:	Rel-18

Work Item:	NR_Mob_enh2-Core

Source:	RAN4

To:	RAN2

Cc:

Contact person:	Qiming Li

Li_qiming@apple.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# 1	Overall description

RAN4 thanks RAN2 for the LS on granularity definition of pdcch-RACH-AffectedBandsList-r18. RAN4 discussed the issue and confirm that understanding 2 is aligned with RAN4 design.

- Understanding 2: The source band of the band pair represents the specific source band that have interruption or not (depends on the value “interruption” or “noInterruption”) in the BC when UE performs PDCCH-ordered early RACH toward a target cell on target band.

# 2	Actions

To RAN2

ACTION: 	RAN4 kindly asks RAN2 to take the above understanding 2 into consideration in RAN2 design.

# 3	Dates of next TSG RAN WG 2 meetings

TSG RAN WG4 Meeting #115			19th – 23rd May 2025                       		Malta

TSG RAN WG4 Meeting #116			25th – 29th August 2025                  		India