3GPP TSG-RAN WG4 Meeting # 114bis	R4-2503591

Wuhan, China, 7th – 11st April, 2025

Source:	LG Electronics

Title:	Discussion on RRM requirements of on-demand SSB Scell operation for Rel-19 NES

Agenda item:	7.25.2.1

Document for:	Discussion

Introduction

In agreed WID for Rel-19 NES operation [1], following objective was captured for RAN4 RRM core requirements:

- Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum. - Specify the corresponding core requirements, for the above features [RAN4].

In this contribution, we provide our views on RRM core requirements for RRM requirements of on-demand SSB Scell operation for Rel-19 NES.

Discussion

General On-demand SSB requirements

L1 measurement for SCell in Case 1

In previous RAN4 meeting, L1 measurement has been discussed as follows:

Issue 4-1: L1 measurement before SCell activation Agreement: - RAN4 not to support OD-SSB based L1 measurements for deactivated Scell. Issue 4-2: L1 measurement after SCell activation Agreement: - RAN4 to support OD-SSB L1 measurement during and after SCell activation. - When non-DRX is configured, the current requirement can be reused with the clarification of TSSB in TL1-RSRP_Measurement_Period_SSB is the SSB periodicity of OD-SSB - RAN4 to support OD-SSB L1 measurement during SCell activation. - The requirement when DRX is configured. - Based on OD-SSB periodicity Wayforward: - RAN4 to support OD-SSB L1 measurement after SCell activation. - FFS the requirement when DRX is configured. - Option 1: Follow legacy, based on DRX cycle (vivo, Apple, OPPO, Xiaomi) - Option 2: Based on OD-SSB periodicity (Nokia, ZTE, Samsung) - Option 3: FFS (HW, QC, Nokia, CMCC, Xiaomi, CATT) - Note: the OD-SSB is triggered before or together with the Scell activation command, the OD-SSB transmission kept after the SCell activation until the OD-SSB deactivation command received.

One open issue is whether to follow DRX cycle in case of OD-SSB L1 measurement after SCell activation. As commented in wayforward the OD-SSB transmission is kept after the SCell activation until the OD-SSB deactivation command. In deactivated SCell measurement or during SCell activation, the principle followed by RAN4 is to prioritize the OD-SSB measurement. The motivation is to make SCell known to NW or boost the SCell activation. In other words, once SCell is activated, we think the principle is no longer valid. If OD-SSB based periodicity is applied regardless of DRX, it means that the UE cannot transfer to OFF, which consumes more power. In that sense, we prefer to keep the OD-SSB L1 measurement after SCell activation follows legacy requirement.

Proposal 1: Follow legacy requirement for OD-SSB L1 measurement after SCell activation when DRX is configured

Additional reqiurement for MAC-CE processing

When a UE receives OD-SSB activation indication, NW transmits actual SSB burst staring from Time instance A. In some case, Time instance A can be located within the SSB burst. Per RAN1 agreement, NW is allowed to transmit the first actually transmitted SSB index within the first possible OD-SSB burst as illustrated in Figure 1. For instance, NW transmites SSB bursts except SSB index 0. From the UE perspective, the UE is required to measure every index, however, some SSB indices are missing. After the first OD-SSB burst, if the UE wants to combine the SSB to fulfil the required accuracy, the number of combination between SSB indexes can be different. In order to avoid such problem, we suggest to start measurement at the next OD-SSB burst, if Time instance A is located within the first possible OD-SSB burst.

Figure 1 Time instance A within OD-SSB burst

Proposal 2: If Time instance A is located within the first possible OD-SSB burst, start measurement at the next OD-SSB burst

On-demand based deactivated SCell L3 requirements (Case 1)

In RAN4#112bis, RAN4 introduced the time window in which a UE is required to measure OD-SSB. The motivation of the time window is to reduce the UE power consumption, since NW may not guarantee OD-SSB deactivation. According to the agreement, the UE performs ‘fast L3 measurement’ during the time window, otherwise ‘slow L3 measurement’ is expected. In RAN4#113, slow L3 measurement was defined as using legacy deactivated SCell measurement, which means that the UE shall meet the requirements using measCycleSCell for measurement periodicity. In RAN4#114, the length of fast measurement window was defined.

The measurement requirement based on interval between multiple FMW (T2), the configuration of the time window including the relationship with OD-SSB deactivation indication, and the lower bound for fast measurement requirement are still open issue. We would like to touch upon one by one.

The relation between time window and OD-SSB deactivation command in the time domain

In the previous meeting, RAN1 draw the agreement on supporting two options for deactivating OD-SSB transmission as follows:

Agreement For a cell supporting on-demand SSB SCell operation, support at least the following options to deactivate on-demand SSB transmission from a UE perspective. - Option 1: Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication - Deactivation by RRC is up to RAN2 - FFS: Which scenario Option 1 is used - Option 2: Configuration/indication of the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated - FFS: Whether Option 4, 4a is needed in addition to Option 2 - FFS: Whether the value of N can be implicitly determined using a timer

In our understanding, above options can be organized into two categories. Option 1 is explicit deactivation indication, which implies OD-SSB is activated until explicit deactivation indication. Option 2 is implicit deactivation indication, means that the the number of OD-SSB to be transmitted is already configured/indicated by NW. When a UE is indicated either Option 1 or Option 2, it is clear that the UE is not required to measure the deactivated SCell. However, the exact time is not clear especially for Option 1. We would like to suggest clear measurement behaviour as illustrated in Figure 2.

For Option 1, one approach can be that the UE stops the measurement (fast or slow) at the time when the UE receives OD-SSB deactivation indication. However, the indication is containd via MAC-CE, therefore, it may requires some time for processing. Hence, we think it is reasonable that the UE stops the measurement (fast or slow) at the first slot after 3ms transmitting feedback on OD-SSB deactivation indication.

For Option 2, it is rather straight forward that the deactivation time is already configured/indicated. We think the UE stops the measurement(fast or slow) at the end of Nth OD-SSB burst.

Proposal 3: Consider the relation between OD-SSB deactivation indication and measurement as follows:

- In case of Option 1 (Explicit deactivation indication): UE is no required to measure (fast or slow) deactivated SCell at the first slot after 3ms transmitting feedback on OD-SSB deactivation indication

- In case of Option 2 (Configuration of N OD-SSB bursts to be transmitted): UE is not required to measure (fast or slow) deactivated SCell at the end of Nth OD-SSB burst

Figure 2 OD-SSB deactivation indication and measurement

Measurement requirement based on the minimum interval between multiple fast measurement window(T2)

In previous meeting, following agreement regarding minimum interval between multiple fast measurement window has been achieved:

Agreement: RAN4 to define measurement requirement based on T2. - When NW re-activates the OD-SSB within the T2, - Option 1: UE is not required to perform “fast measurement” but follows legacy deactivated SCell measurement requirement. - Option 2: UE is required to perform “fast measurement” without cell search. - The UE is assumed to skip the PSS/SSS detection and/or SSB index acquisition. Option 3: UE is required to perform FMW after T2. - When NW re-activates/re-configures the OD-SSB transmission after T2, UE performs the OD-SSB based fast L3 measurement within FMW - The UE is assumed to meet fast cell identification and measurement. - The start point of T2 is based on - Option 1: the time point Tend - Option 2: the latter point of L3 measurement reporting for FMW and Tend - Option 3: the latter point of latest L3 measurement reporting before NW re-indicating OD-SSB transmission and Tend - Option 4: the time point of OD-SSB deactivation - Other options are not precluded Where, Tend equals the time point of OD-SSB indication plus Tuncertain +Tidentify applies to option 1,2,3. - The duration is - Option 1: 5*max(measCycleSCell, DRX cycles) [– OD-SSB periodicity] - Option 2: 5* measCycle when measCycle =160 ms - Other options are not precluded - The activation and deactivation of OD-SSB is fully up to NW.

The reason T2 is being discussed is to avoid repeatedly perfoming FMW. If NW initiates FMW, it leads to significant power consumption by the UE. Therefore, minimum interval between multiple FMW has been proposed to prevent the UE from starting FMW again immediately after it has ended. Considering UE power consumption in deactivated SCell measurement, it is acceptable to introduce such restriction.

Besides, we would like to point out that introducing minimum interval between multiple FMW (Option 1) does not mean that minimum interval for cell search (Option2) is precluded. We believe that restricting multiple FMWs is one approach, while cell search considered another. That is because the starting point of T2 can be different between minimum interval between multiple FMW, and cell search as shown in Figure 3. As illustrated in Figure 3, T2 Option1 can be defined for minimum interval between FMW, and T2 Option4 can be defined for minimum interval for cell serach. Below, we elaborated the reason of seprating two intervals.

Proposal 4: Discuss minimum interval between FMW and minimum interval for cell search separately

For the starting porint of minimum interval between multiple FMWs, there are four options are on the table. It can be analyzed as below:

- Option 1: the time point Tend

  - Pros: Clear starting point, aligned with motivation

  - Cons: UE behaviour is not clear before Tend, Although SCell is known, FMW can be initiated,

- Option 2: the latter point of L3 measurement reporting for FMW and Tend

  - Pros: SCell can be known based on FMW

  - Cons: UE behaviour is not clear when the report is not provided or triggered

- Option 3: the latter point of latest L3 measurement reporting before NW re-indicating OD-SSB transmission and Tend

  - Pros: As long as SCell is known, FMW can be skipped

  - Cons: Restrictive scheduling, UE behaviour is not clear when the report is not provided or triggered

- Option 4: the time point of OD-SSB deactivation

  - Pros: Clear starting point

  - Cons: UE behaviour is not clear before OD-SSB deactivation

Based on our analysis, we think UE behaviour is not clear due to the uncertainty of the report. Because, after Tend, if the report is triggered baed on certain event, the UE may not know whether the event is met at Tend, which means that the starting point of T2 is not clear for Option 2 and Option 3. On the other hand, the starting point is clear for Option 1 and Option 4. Among two options, we prefer Option 1, since it is more aligned with motivation.

Proposal 5: Regarding the starting point of minimum interval for multiple FMWs, we prefer Option1 (the time point Tend)

Meanwhile, we acknowledged that it is not clear whether re-activating during OD-SSB activation is allowed or not. In our understanding such issue has not been discussed in RAN1. We think RAN4 needs to trigger the discussion whether re-activating during OD-SSB activation is allowed or not before down selecting options for the starting point of T2.

Proposal 6: Send LS to RAN1 to ask whether re-activating is allowed or not

Unlike minimum interval for multiple FMWs, for minimum interval for cell search, we believe it is straight forward that the starting point of T2 for cell search begins at ‘T2 option4’. Since it can be assumed that PSS/SSS and/or SSB can be detected during OD-SSB activation, if the OD-SSB activation is the first activation after SCell addition. However, after OD-SSB deactivation, if the time is long enough to lose the sync, the UE may need to detect PSS/SSS and/or acquire the index of the SSB being measured.

Proposal 7: Regarding the starting point of minimum interval for cell search, we prefer Option4 (the time point Tend)

Figure 3 OD-SSB transmission in Case1

Fast L3 measurement requirements

In the pervious meeting, following wayforward was captured in [2].

Sub-issue 3-1: Lower bound for measurement in FMW Wayforward: - Option 1: RAN4 not to define lower bound to measurement in FMW. - Option 2: Define a lower bound with OD-SSB periodicity as [10ms]. - The lower bound is only applied to measurement in FMW.

For a lower bound of OD-SSB periodicity, the main concern is that the indicated OD-SSB periodicity may be small (e.g. 5ms), so the power consumption of the UE can be increased. In our understanding, even smallest SSB periodicity value has been supported from Rel-15, so there is no feasibility issue. Furthermore, RAN4 already have achieved agreement on the duration of the time window, which limits the ‘fast’ measurement time. Hence, the benefit of the lower bound is not clear.

Proposal 8: Support Option 1: RAN4 not to define lower bound to measurement in FMW

Starting point of the time window

In the previous meeting, RAN4 introduced the time window starting from receiving OD-SSB activation indication. Based on the agreement, in our understanding, it seems that the starting point of the time window is the slot or the symbol of the PDSCH containing the MAC-CE indicating OD-SSB activation. We have a concern that there can be different understanding between NW and UE regarding the starting point of the time window. From NW perspective, staring of the time window is clear. However, from UE perspective, a UE can acknowledge the OD-SSB activation after decoding the PDSCH and proceeding MAC-CE process in the higher layer. Technically, UE acknowledges OD-SSB activation inciation after Tmin, which implies that the time window starts after Tmin. In order to address the issue, we would like to revise the agreement as follows:

Time window means the time duration starting from receiving OD-SSB activation indication→ Time window means the time duration starting from Tmin after receiving OD-SSB activation indication

Proposal 9: Revise the agreement regarding the time window as follows:

- Time window means the time duration starting from receiving OD-SSB activation indication→ Time window means the time duration starting from Tmin after receiving OD-SSB activation indication

On-demand based deactivated SCell L3 requirements (Case 2)

Scenario for On-demand SSB requirements in Case 2

In the previous meeting, RAN4 introduced Scenario 4 for performing combined measurement of AO-SSB and OD-SSB. According to agreement, at least combined measurement can be performed based on the effective measurement periodicity within FMW (fast measurement window) in deactivated SCell L3 measurement.

Issue 2-2: Always-on SSB and OD-SSB time/frequency domain relation Agreement: RAN4 to start from the following scenarios: - Scenario 1: OD-SSB and always-on SSB are with same frequency, same offset but different periodicities. - Scenario 4: OD-SSB and always-on SSB are with same frequency, different offset but same periodicities. - At least the offset between OD-SSB and AO-SSB equals the half of AO-SSB periodicity - The scenarios can be updated based on other WGs’ progress. Issue 2-3: Combine OD-SSB and AO-SSB in measurement Agreement: - In scenario 4, the measurement requirement is defined based on the effective measurement periodicity. - The requirement is only applied in the FMW which includes both OD-SSB and AO-SSB, and - The effective measurement periodicity is half of OD-SSB periodicity - AO-SSB and OD-SSB are on the same frequency with the same spatial relation

In our understanding, applying combination of AO-SSB and OD-SSB can be extend SCell activation delay requirement as well as L1 measurement after SCell activation. It would be helpful to reduce SCell activation dealy especially in unknown condition, and L1 measurement period in Case 2.

Proposal 10: Define SCell activation delay requirement and L1 measurement in Sceanrio 4

Meanwhile, RAN1 agreed to send reply LS[3] as follows:

Agreement Regarding the relation in terms of time location between the always-on SSB and on-demand SSB, - For the case when the center frequency locations of always-on SSB and on-demand SSB are same, - Alt Time-C: RAN1 specification has no restriction with regards to overlapping - From RAN1 perspective, - Alt Time-C1: The case that, during OD-SSB transmission, the union of AO-SSB transmission and OD-SSB transmission has a periodic time domain pattern is supported (the interval between SSB bursts is even and supported in legacy specification). - Alt Time-C2: The case that, during OD-SSB transmission, the union of AO-SSB transmission and OD-SSB transmission has a non-periodic time domain pattern is supported. - It is up to RAN4 to define requirements, if any, corresponding to both or either of Alt Time-C1 or Alt Time-C2 - At least the following is supported: PBCH payload for the same SSB index (other than SFN index, half frame index) is the same for AO-SSB and OD-SSB - FFS: Whether half frame index is the same or different for AO-SSB and OD-SSB - For the case when the center frequency locations of always-on SSB and on-demand SSB are different, - Alt Time-C: RAN1 specification has no restriction with regards to overlapping - UE assumes that frequency resources of always-on SSB are not overlapped with those of on-demand SSB in frequency domain. - AO-SSB and OD-SSB are located in the same BWP - FFS: PBCH payload for the same SSB index (other than SFN index, half frame index) should be the same for AO-SSB and OD-SSB - NOTE: AO-SSB periodicity is not adapted

Per RAN1 agreement, for the case when the center frequency loactions of AO-SSB and OD-SSB are same, there are Time-C1 and Time-C2. In our understanding RAN4 has already defined Scenario 1 and Scenario 4 for Time-C1 as shown in Figure 4.

Meanwhile, RAN1 defined non-periodic time domain pattern which is Time-C2. Regarding Time-C2, similar issue was raised in previous meeting, however, no discussion was made, because non-periodic time domain pattern is complex to be captured in the requirement. In our view, requirement of Time-C2 can be specified at least some limited case in order to increase NW scheduling flexibility. As shown in Figure 5, case can be separated based on the number of OD-SSB within a AO-SSB period (or ratio of OD-SSB). For one OD-SSB in AO-SSB period, measurement period can be derived based on offset between AO-SSB and OD-SSB. For example, if 3 samples are required, and offset=10ms, measurement period can be 40+10=50ms. For multiple OD-SSB in AO-SSB period, different approach, such as measurement period based on the number of OD-SSB within a AO-SSB period can be applied. For example, if 2 samples are required, and 2 OD-SSB are located within a AO-SSB period, the measurement period can be one AO-SSB periodicity, which means that the UE can measure at least 2 samples out of 3 SSBs. Which sample to be measured is up to UE implementation.

In any case, from some example above, we think it is not complex to define measurements for non-periodic Time-C2. So, we suggest RAN4 to specify Time-C2 at least some limited cases.

Figure 4 Example of Time-C1

Figure 5 Example of Time-C2

Proposal 11: RAN4 to specify requirements for non-periodic time domain pattern Time-C2 at least for some limited cases

For the case when the center frequency loactions of AO-SSB and OD-SSB are different, RAN1 defined the case when AO-SSB is CD-SSB, and AO-SSB/OD-SSB are within the same BWP. In case of activated SCell L3 measurement, since AO-SSB and OD-SSB are within the same BWP, a UE can measure both SSB even if they are overlapped. On the other hand, in case of deactivated SCell L3 measurement, BWP is not activated yet, and RF chain bandwidth is only available for SSB bandwidth. If the UE wants to measure AO-SSB and OD-SSB, RF retuning is inevitable. Therefore, certain time offset between AO-SSB and OD-SSB is needed, if the UE is required to measure AO-SSB and OD-SSB (not to combine, but to measure each SSBs independently), otherwise UE needs to apply time scaling for toggle AO-SSB and OD-SSB, or prioritize OD-SSB measurement.

Proposal 12: RAN4 to specify OD-SSB and AO-SSB are with different frequency

Proposal 13: RAN4 to consider minimum gap between AO-SSB and OD-SSB in deactivated SCell L3 measurement with different frequency

  - If the gap is longer than certain thershod, AO-SSB and OD-SSB can be measured independently

  - If the gap is shorter than certain threshold,

    - introduce scaling factor to measurement period, or

    - prioritize OD-SSB measurement

On-demand based SCell activation

Following agreements have been achieved:

RAN4#112bis Issue 1-4-1: General Agreement: - RAN4 focuses on OD-SSB based SCell activation known and unknown requirement for both Case 1 and Case 2 and not specifically differentiate Scenario 2/2A scenarios. Issue 1-4-3: known cell condition design – principle Agreement: - The legacy known condition principle can be reused, which is defined based on if the UE has sent a valid measurement report during a ‘period’ before the reception of the SCell activation command. - When OD-SSB transmission is before/together with the SCell activation command(Scenario #2/Scenario #2A), UE follows the principles of legacy known/unknown SCell activation requirement once the known/unknown condition is met. - FFS how known/unknown condition is defined i.e. based on always-on SSB and/or OD-SSB - Other principles are not precluded. Issue 1-4-5: Which type of SSB is used during SCell activation Agreement: - In Case 1, UE is required to always use OD-SSB for Scell activation. - In Case 2, when both OD-SSB and always-on SSB are transmitted, - RAN4 assumes UE to use OD-SSB to perform SCell activation as a baseline - The agreement can be revisited if different with other WG RAN4#113 Issue 1-4-1: The known cell condition – measurement reporting Agreement: - In case 1, known/unknown condition should be defined based on only OD-SSB. - In case 2, known/unknown condition should be defined based on either always-on SSB or OD-SSB. - Note 1: whether the always-on SSB and OD-SSB can be combined is discussed in another issue. - Note 2: the agreement is valid at least for the scenario with the same frequency for always-on SSB and OD-SSB. - Note 3: RAN4 to further clarify the relation for the SSB measured for known cell condition and the SSB used in SCell activation procedure. Issue 1-4-5: SCell activation requirement – OD-SSB periodicity Agreement: - In Case 1, the SCell activation delay requirement is defined based on OD-SSB periodicity. - Existing SCell activation delay requirement shall apply based on OD-SSB periodicity. - In Case 2, RAN4 to define the SCell activation delay requirement based on OD-SSB periodicity if the time instance receiving OD-SSB activation indication is no later than the time instance when receiving the SCell activation command. Otherwise, RAN4 to define the requirement based on always-on SSB for SCell activation. - This agreement is valid under the assumption of the same spatial relation for OD-SSB and always-on SSB [with the same SSB index], and this can be revisited based on RAN1 LS reply. - Note: T_firstSSB_max for intra-band co-located scenario is discussed in another issue.

Known/unknown cell condition period

In the previous meeting, the principle of known cell and unkown cell using legacy principle has been agreed. In our understanding, as speficied in 38.133 - 8.3.2, Scell in FR1 is considered as ‘known’ when the UE has sent a valid measurement report for the SCell being activated, and the SSB measured remains detectable during the period max(5*measCycleSCell, 5*DRX cycles) before the reception of the SCell activation command. For SCell in FR2 is considered as ‘known’ when the UE has sent a valid L3-RSRP measurement report with SSB index, and SCell activation command is received after L3-RSRP reporting and no later than the time when UE receives MAC-CE command for TCI activation during the period equal to 4s or 3s before UE receives the last activation command, which depends on the UE power class. Otherwise, the SCell is considered to be unknown.

One issue to be resolved is whether to use legacy known/unknown cell condition period. As defined in the legacy requirement, measCycleSCell is applied in the legacy known/unknown condition. We think there is not much motivation to introduce new period for known/unown cell condition.

Proposal 14: RAN4 to specify known/unknown cell condition period as below:

- At least for FR1: max(5*( measCycleSCell), 5*DRX cycles)

Known/unknown cell condition in case of multiple OD-SSB activation indication

The other issue is that the OD-SSB used for known/unknown cell condition when there are multiple OD-SSB activation indication. As illustrated in Figure 6, multiple OD-SSB can be activated at SCell activation command. NW may activate multiple times to renew the time window. In this case, a UE may confuse which activated OD-SSB to be used for determining known/unknown cell condition. From NW perspective, better measurements report is valid for activating SCell. Therefore, the OD-SSB provided the better valid measurement report can be used for SCell activation.

Proposal 15: For the OD-SSB used for known/unknown cell condition when there are multiple OD-SSB activation indication, the OD-SSB provided better valid measurement report can be used for SCell activation

Figure 6 Known/unknown cell condition with multiple OD-SSB activation indication

Restriction during SCell activation

In general, during SCell activation, the UE measures SSB for synchronization, AGC, and time/frequency tracking. If the cell is known, the UE utilizes the information acquaired from measurements, and applies short activation delay. In that sense, if the cell is known, the same SSB should be applied for the OD-SSB used for fulfil known condition and the OD-SSB used for SCell activation. Also, we think OD-SSB should not be changed or deactivated during SCell activation.

Proposal 16: During the SCell activation following should be specified:

- If the cell is known, the same SSB should be applied for both known condition and SCell activation

- OD-SSB should not be changed or deactivated during SCell activation

SCell activation delay requirement

In Rel-19 NES operation, targeted scenario is for both intra-/inter-band CA. Both SCell activation delay requirement for intra-/inter-band needs to be specified. In order to minimize the specification impact, basic principle needs to be followed for TSMTC_MAX, TFirstSSB,TFirstSSB_MAX, and Trs. It is straightforward that periodicity of SMTC of TSMTC_MAX and Trs can be replaced by OD-SSB periodicity in Case 1. For TFirstSSB,TFirstSSB_MAX, it can be the time to the end of the first complete OD-SSB burst, which is no earlier than Time_instance_A+Additional_delay indicated by OD-SSB of the SCell being activated, after slot n +(𝑇𝐻𝐴𝑅𝑄+3𝑚𝑠)/ 𝑁𝑅 𝑠𝑙𝑜𝑡 𝑙𝑒𝑛𝑔𝑡ℎ in Case 1. In Case 2, Scell can be activated based on either AO-SSB or OD-SSB. If AO-SSB is applied, legacy requirement can be used in Case 2. If OD-SSB is applied, same requirement as Case 1 can be used. Furthermore, if SCell activation based on the combination of AO-SSB and OD-SSB is supported, it needs to be specified.

Proposal 17: In order to minimize the specification impact, basic principle needs to be followed for TSMTC_MAX, TFirstSSB,TFirstSSB_MAX, and Trs as follows:

- In Case 1, periodicity of SMTC of TSMTC_MAX and Trs can be replaced by OD-SSB periodicity

- In Case 1, TFirstSSB,TFirstSSB_MAX can be the time to the end of the first complete OD-SSB burst, which is no earlier than Time_instance_A+Additional_delay indicated by OD-SSB of the SCell being activated, after slot n +(𝑇𝐻𝐴𝑅𝑄+3𝑚𝑠)/ 𝑁𝑅 𝑠𝑙𝑜𝑡 𝑙𝑒𝑛𝑔𝑡ℎ

- In Case 2, if AO-SSB is applied, legacy requirement can be used

- In Case 2, if OD-SSB is applied, same requirement as Case 1 can be used

- FSS: In Case 2, specify requirements of combination of AO-SSB and OD-SSB, if it is supported

Miscellaneous

Interruption

Issue 6-6: Interruption Open Issue: - RAN4 to investigate whether there is impact due to OD-SSB on the allowed interruptions during measurements on deactivated Scell.

In previous meeting, some company raised an issue regarding interruptions. We think interruption can be occurred due to RF re-tuning of activating/deactivating a device that receives OD-SSB in Case 1 as well as Case 2. So, interruption can be allowed to other serving cells when OD-SSB is activated and deactivated.

Proposal 18: Allow interruption during OD-SSB activation and deactivation

Additional measurement delay for SCell activation delay requirement

Issue 7-3: OD-SSB based L3 reporting Wayforward: - RAN4 to discuss whether to introduce an OD-SSB based L3 measurement reporting.

We would like to point out the issue for OD-SSB measurement during and after SCell activation. RAN4 agreed to reuse the legacy known cell condition principle. If we apply the legacy known condition principle, sending ‘valid measurement report or L3-RSRP measurement report with SSB index’ may depend on the OD-SSB indication timeline. For example in Figure 7, a UE received OD-SSB activation indication, and SCell activation command. If the UE have enough time to provide valid measurement report, UE may decide SCell as known. However, as shown in Figure 7, the UE cannot provide valid measurement report due to the lack of enough time. In this case, SCell can be remained unkown, and it may lead wasting measured OD-SSB.

In order to avoid wasting resources, additional measurement and reporting after SCell activation command can be provided in Figure 7. For example the UE measures OD-SSB using OD-SSB before SCell activation command and after SCell activation command, and provides valid measurement reports. The UE can apply shorter delay requirements precluding synchronization, AGC, or time/frequency tracking( the UE can also apply known condition requirements). We are not proposing to modify the known condition period, but having impact on SCell activation delay. In other words, additional measurement can be included in SCell activation delay requirements.

Figure 7 Additional measurement for SCell activation

Proposal 19: For SCell activation delay requirement, propose to add additional measurement delay

Conclusion

In this contribution, we provide views on RRM requireiments for on-demand SSB SCell operation, and we propose

Proposal 1: Follow legacy requirement for OD-SSB L1 measurement after SCell activation when DRX is configured

Proposal 2: If Time instance A is located within the first possible OD-SSB burst, start measurement at the next OD-SSB burst

Proposal 3: Consider the relation between OD-SSB deactivation indication and measurement as follows:

- In case of Option 1 (Explicit deactivation indication): UE is no required to measure (fast or slow) deactivated SCell at the first slot after 3ms transmitting feedback on OD-SSB deactivation indication

- In case of Option 2 (Configuration of N OD-SSB bursts to be transmitted): UE is not required to measure (fast or slow) deactivated SCell at the end of Nth OD-SSB burst

Proposal 4: Discuss minimum interval between FMW and minimum interval for cell search separately

Proposal 5: Regarding the starting point of minimum interval for multiple FMWs, we prefer Option1 (the time point Tend)

Proposal 6: Send LS to RAN1 to ask whether re-activating is allowed or not

Proposal 7: Regarding the starting point of minimum interval for cell search, we prefer Option4 (the time point Tend)

Proposal 8: Support Option 1: RAN4 not to define lower bound to measurement in FMW

Proposal 9: Revise the agreement regarding the time window as follows:

- Time window means the time duration starting from receiving OD-SSB activation indication→ Time window means the time duration starting from Tmin after receiving OD-SSB activation indication

Proposal 10: Define SCell activation delay requirement and L1 measurement in Sceanrio 4

Proposal 11: RAN4 to specify requirements for non-periodic time domain pattern Time-C2 at least for some limited cases

Proposal 12: RAN4 to specify OD-SSB and AO-SSB are with different frequency

Proposal 13: RAN4 to consider minimum gap between AO-SSB and OD-SSB in deactivated SCell L3 measurement with different frequency

  - If the gap is longer than certain thershod, AO-SSB and OD-SSB can be measured independently

  - If the gap is shorter than certain threshold,

    - introduce scaling factor to measurement period, or

    - prioritize OD-SSB measurement

Proposal 14: RAN4 to specify known/unknown cell condition period as below:

- At least for FR1: max(5*( measCycleSCell), 5*DRX cycles)

Proposal 15: For the OD-SSB used for known/unknown cell condition when there are multiple OD-SSB activation indication, the OD-SSB provided better valid measurement report can be used for SCell activation

Proposal 16: During the SCell activation following should be specified:

- If the cell is known, the same SSB should be applied for both known condition and SCell activation

- OD-SSB should not be changed or deactivated during SCell activation

Proposal 17: In order to minimize the specification impact, basic principle needs to be followed for TSMTC_MAX, TFirstSSB,TFirstSSB_MAX, and Trs as follows:

- In Case 1, periodicity of SMTC of TSMTC_MAX and Trs can be replaced by OD-SSB periodicity

- In Case 1, TFirstSSB,TFirstSSB_MAX can be the time to the end of the first complete OD-SSB burst, which is no earlier than Time_instance_A+Additional_delay indicated by OD-SSB of the SCell being activated, after slot n +(𝑇𝐻𝐴𝑅𝑄+3𝑚𝑠)/ 𝑁𝑅 𝑠𝑙𝑜𝑡 𝑙𝑒𝑛𝑔𝑡ℎ

- In Case 2, if AO-SSB is applied, legacy requirement can be used

- In Case 2, if OD-SSB is applied, same requirement as Case 1 can be used

- FSS: In Case 2, specify requirements of combination of AO-SSB and OD-SSB, if it is supported

Proposal 18: Allow interruption during OD-SSB activation and deactivation

Proposal 19: For SCell activation delay requirement, propose to add additional measurement delay

Reference

[1] RP-242354, “Revised WID: Enhancements of network energy savings for NR,” Ericsson

[2] R4-2502598, “WF on RRM requirements for Netw_Energy_NR_enh_Part1,” Ericsson

[3] R1-2501633,” LS on time location of on-demand SSB for SCell,” LG Electronics