3GPP TSG-RAN WG4 Meeting # 114bis	                                            R4-2503607

Wuhan, CN, April 7 – 11, 2025

Agenda item:	7.21.2.1

Source:	MediaTek Inc.

Title:	Discussion on RAN4 impacts for RRM measurement prediction

Document for:	Discussion

- Introduction

RAN#105 plenary has agreed the updated version of the SID with the following RAN4 objectives [1]:

According to the agreed work plan last meeting, this meeting RAN4 will focus on RRM measurement prediction and event prediction use-cases. In this contribution, we would like to discuss the related RRM requirements of RRM measurement prediction.

- Discussion

In RAN4#112bis, RAN4 agreed to start the discussion on scenario 2, 3, 4. In the next, we will focus on the three scenarios.

| scenario number | Priority | Evaluation scenario | Target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |

2.1 Metrics used to evaluate the model performance

For AI/ML BM, there were a lot of discussions on the metrics used to define the model performance. Similar as AI/ML BM, the metrics should be discussed for AI/ML mobility too.

RAN4#112bis Issue 2-1-1: Performance Metrics/KPIs for Measurement Prediction use case Agreement: - RAN4 to first study the impacts on requirements for the following case: - L3 cell level RSRP (Point C) prediction - FFS L1 beam-level (Point A1) - FFS L3 beam-level (Point E) - Other cases can be considered based on RAN4 further discussion and RAN2 further progress.

In our view, one of the principles to choose the metrics is that UE will report corresponding results or the metric can be derived through the reported results. No matter which sub-use case is adopted, the final reported results will be L3 cell level RSRP (Point C). So we propose to use cell level RSRP as the performance metric for all the three sub-use cases.

For information Sub-use case 1: L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then cell-level measurement result is generated Sub-use case 2: Cell-level measurement result(s) is predicted based on actual cell-level measurement result(s) Sub-use case 3: Cell-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) Editor Note 1: The RRM measurement refers to L3 cell/beam level measurement and/or L1 beam level measurement

Proposal 1: For RRM measurement prediction, only use cell level RSRP (Point C) as the performance metric for all the three sub-use cases.

2.2 Ground truth

In the past two meetings, RAN4 reached the following progress on the definition of predicted absolute and relative RSRP accuracy. In the next, we would like to discuss how to determine the ground truth.

RAN4#113 Issue 2-1-9a: L3 RSRP prediction accuracy definition Agreement: - For testing, Absolute L3 Predicted RSRP Accuracy = reported predicted L3-RSRP – ground truth of L3-RSRP - FFS Ground truth definition of L3-RSRP for FR1 and FR2 RAN4#114 Issue 2-1-1a: Relative RSRP Prediction Accuracy Agreement: - Predicted relative RSRP accuracy = (reported predicted RSRP of cell 1/beam1 – reported RSRP of cell 2/beam2) – (ground truth of RSRP of cell 1/beam1 – ground truth of RSRP of cell 2/beam2) - FFS: Beam 1 and Beam 2 can be from same cell or different cells. - FFS: whether ground truth can be reported or ideal - FFS: whether the reported RSRP of cell2/beam2 can be predicted or measured or a combination of both predicted and measured samples. Issue 2-1-2: Ground Truth Definition for RSRP accuracy Agreement: Take Option 1-1-FR2 as baseline, and further discuss any additional aspects need to be considered. - Option 1-1-FR2: The ground truth of L3 RPRP is the reported L3 RSRP measurement under sufficient high SNR in FR2. Issue 2-1-2: Ground Truth Definition for RSRP accuracy - Agreement: - Alt 1: Ground truth for FR1 will be based on the transmitted or reception power - Alt 2: Ground truth for FR1 will be based on the reported measurement value under certain conditions - Further discuss the advantages and disadvantages for Alt 1 and Alt 2 considering the aspects such as: - The information at the TE side, on the timing UE performs and/or finishes the measurement and/or prediction - Impact of L3 filtering - Number of samples for L1 filtering - Timing window, if it exists, where UE can’t measure the ground truth in Alt2 - channel condition - Avoid UE cheat in the test - S(I)NR variations in the test at different time instances (for Alt2) - Other aspects are not precluded - Companies are encouraged to provide analysis on all or some of the aspects in the list

The ground truth shall be determined based on the test procedures. Before discussing how to determine the ground truth, we think it is important to have some initial view of what the test cases will be.

To reflect the performance of the model in the real field, the test cases should be designed as similar as the real field. Here is our thought on how RRM measurement prediction work in the real field. We take inter-frequency prediction as an example. NW configures SMTC for both f1 (intra-f ) and f2 (the inter-f to be predicted). MG is not configured. How to perform measurement on f1 is up to UE implementation as long as the requirements can be met. How to predict is also totally up to UE as long as UE can identify the channel condition changes in time.

Similarly, the test case for inter-frequency prediction can be designed as the following:

- NW configures SMTC for both f1 and f2. MG is not configured. A4 event can be used to check the prediction accuracy on f2. TTT can be set to T1 (For simplicity, we will use TTT = 0 for later explanation). Suppose the threshold of A4 is set as X dBm. The absolute prediction accuracy is defined as ±YdB. TE can calculate the filtered L3 result on each SMTC occasion. TE can know when there is a neighbour cell whose RSRP is within [X-Y, X+Y]. TE can also get to know the time range when there will be expected SR based on the configurations. If UE triggers report request during the expected time range, then it can be viewed as the prediction meets the accuracy requirements in this single test.

How to do the filtering can be further discussed and specified in the test case. Here we provide a method for further discussion:

- We propose TE to get L3 RSRP measurement results on f2 assuming SMTC = 20ms for both f1 and f2, and MGRP = 40ms. Then UE is supposed to do the measurement as shown in Fig.1 to meet the delay and accuracy requirements, i.e., performing measurement on either f1 or f2 on each SMTC occasion. Of course, there may be some more powerful UE that can measure less and meet the requirements as well. Anyway, we can use the measurement behaviour in Fig. 1 to determine the ground truth. We choose this configuration as SMTC=20ms is a typical configuration in the real field and in legacy RRM test cases. MGPR=40ms allows the most frequent measurement on f2 and UE measurement behaviour is kind of unique following the minimum measurement requirements defined in 38.133.

- We take occasion#10 as an example to illustrate our thought.

The ground truth of f2 at occasion#10 can be calculated as (1-α)*Fn-1+α*Mn, where

- To determine the latest time to trigger the event

  - Mn: the average transmission power of occasion #2, 4, 6, 8, 10 (L1 filtering with 5 samples).

  - Fn-1: the updated filtered RSRP at occasion#10 – 200ms (assume non-sliding window following the specification of L3 filtering).

  - α is derived according to 38.331 cl. 5.5.3.2.

- To determine the earliest time to trigger the event

  - Mn: the average transmission power of occasion # 6, 8, 10 (L1 filtering with 3 samples).

  - Fn-1: the updated filtered RSRP at occasion#10 – 40ms (assume sliding window).

  - α is derived according to 38.331 cl. 5.5.3.2 (copied in the Appendix).

Later we will explain why we propose two different methods to determine the latest time and the earliest time to trigger the event.

Fig. 1

There were some concerns on deriving the ground truth based on the transmitted power at TE. We would like to address them one by one

- The information at the TE side, on the timing UE performs and/or finishes the measurement and/or prediction

We understand that RRM measurement prediction is the basic for indirect measurement event prediction. But different from L1 measurement report, L3 report is usually triggered by events. RAN2 will discuss how and when to report the predicted measurement results. If the predicted measurement results report is also triggered by events, it may be not necessary to define separate test cases for both RRM measurement prediction and event prediction. The proposed test design can also extend to measurement event prediction. Considering the above, how to perform the measurement and/or prediction can be totally up to UE as long as UE can find the event met in time. So in the test, we propose to check whether reports will be triggered during the expected time range. RAN4 needs to define reasonable prediction accuracy requirements, i.e., the value of Y in the above designed test case.

- Impact of L3 filtering and Number of samples for L1 filtering

The configured L3 filtering reflects the expectation of NW on when UE will report the met event following the defined minimum requirements in spec. UE can adjust L3 filtering and L1 filtering, while after the adjustment UE is not supposed to trigger the report too early or too late.  This is why we propose two different methods to determine the latest time and the earliest time to trigger the event. If UE uses more measurement results in the previous occasions, the event will be met later. Otherwise, the event can be triggered earlier.

Proposal 2: The test case for inter-frequency prediction can be designed as the following:

- NW configures SMTC for both f1 (intra-f) and f2 (inter-f). MG is not configured. A4 event is configured to check the prediction accuracy on f2. The threshold of A4 is set as X dBm. Suppose the absolute prediction accuracy is defined as ±YdB.

- TE calculates filtered L3 result on each SMTC occasion.

- TE can know when there is a neighbour cell whose RSRP is with [X-Y, X+Y]. TE can also get to know the time range when there will be expected SR based on the configurations. If UE triggers report request during the expected time range, then it can be viewed as the prediction meets the accuracy requirements in this single test.

How to define the accuracy requirements (the value of Y) shall be further discussed. Not to mention the test, event when define the accuracy requirements, varied L1 filtering and L3 filtering adopted by UEs may also have impact on the value of Y.  Additional margin can also be added to solve the mismatch issue.

Proposal 3: With the proposed test procedure, the mismatch between TE and UE on L1 filtering and L3 filtering can be reflected on the defined accuracy requirements or some additional margin of the test requirements.

Views on using the reported measurement result as the ground truth

If to use the reported measurement result as the ground truth, for scenario 2 (FR1 to FR1 intra-frequency temporal domain case B) and scenario 3 (FR1 to FR1 inter-frequency (frequency domain)), the test procedures will contain two steps. Here we take scenario 3 as an example too.

- Step 1: MG is not configured. UE measures f1 (intra-f) only and predicts the measurement results on f2 (inter-f).

- Step 2: Configure MG. UE measures both f1 and f2. The measured results on f2 is regarded as ground truth.

The key issue of using the reported measurement result as the ground truth is how to avoid UE reporting a false measurement result in step 2. We didn’t come up any solution yet. We are open to check any proposed solutions.

Proposal 4: How to avoid UE cheating should be resolved at first before concluding to use reported measurement results as ground truth.

2.3 Cell identification

It is observed that there were some proposals to study the impacts on cell identification for L3 cell-level measurement prediction by AI/ML or whether the predicted cell is known or unknown. We agree that this is an important issue for further study. Due to cell identification related discussion is usually conducted in RAN4, RAN2 may not realize the issue yet and it is more appropriate to discuss the issue in RAN4.

For scenario 2 and 4, since they are intra-cell prediction, UE is supposed to detect and measure the cell before prediction. So cell identification is not an issue.

For Scenario 3, FR1 to FR1 inter-frequency (frequency domain), since measurement on target cell is not always conducted, RAN4 should discuss the impact on cell identification. As mentioned above, RAN2 will focus on frequency domain for cluster based approach. Here we also focus on cluster-based approach. As shown in the example in Fig. 2, the inputs are RSRP of 21 cells (assume Cell#0,1, …,20) on frequency#1, the outputs are RSRP of 21 cells (assume Cell#0’,1’, …,20’) on frequency#2. Cell#X and Cell#X’ are co-located.

Fig. 2

Though cell detection, UE can get Cell ID and rough timing. In collocated scenario as shown in Fig. 2, it is possible to inform UE cell ID of Cell#X’ which is co-located with Cell#X. But if extending Scenario#3 to non-collocated cases, we are not sure whether it is workable to inform UE cell ID of the neighbor cell(s) by serving cell.

Observation 1: For scenario 3 (inter-frequency measurement prediction), it is possible to inform UE cell ID of the inter-frequency neighbor cell(s) to be predicted for collocated cases while may be not possible for non-collocated cases.

Regarding timing information, it is not possible to get from serving cell unless collocated cells are well synchronized. So in scenario 3, the cells predicted but not measured recently are unknown.

Proposal 5: For scenario 3 (inter-frequency measurement prediction), the cells predicted but not measured recently are unknown.

In AI/ML BM, there is similar discussion on whether the predicted TCI state is known or unknown. And the final conclusion will have impact on TCI state switching procedures but would have little impact on model monitoring. But for AI mobility, whether the predicted cell is known or not would also have some impact on model monitoring, as UE is supposed to measure the predicted cells during model monitoring. For the cells which have been detected recently, e.g., in 5s, UE can measure the cell directly without cell search. But if the cell has not been detected yet, UE should perform PSS/SSS detection at first and then L3 measurement. So, if model monitoring is performed frequently, the gain will become less. But if UE performs model monitoring every several seconds. The time needed to finish one round of monitoring will be long.

We think it is important to study when UE performs cell detection on the predicted cells as this would have impact on model monitoring.

Proposal 6: Study when UE performs cell detection on the predicted cells, e.g., before model monitoring or during model monitoring.

2.4 Other RRM impacts

UE measurement and/or report behavior will be different from legacy for all the scenarios 2,3,4. RAN4 shall study whether there is any impact on measurement delay and report delay requirements. Regarding the impact on report requirements, there will be similar issue for AI/ML BM. We suggest to wait for more progress in AI/ML BM.

Proposal 7: Wait for more progress in AI/ML BM to study the impact on measurement report delay requirements for AI mobility.

Next, we will focus on the impact on measurement delay requirements.

Scenario 2, FR1 to FR1 intra-frequency intra-cell temporal domain case B prediction, is to predict a subset of measurement instances in temporal domain of the same cell. This scenario is similar as BM case 2. The impact on measurement delay would take BM case 2 as a basis.

Proposal 8: Wait for more progress in AI/ML BM case 2 to study the impact on measurement delay requirements for AI mobility Scenario 2.

Scenario 3, FR1 to FR1 inter-frequency inter-cell (frequency domain), in our understanding, this scenario includes “prediction on an inter-frequency layer based on measurement on an intra-frequency layer” and also “prediction on an inter-frequency layer based on measurement on a different inter-frequency layer”. As discussed in previous part, the scenario is more valuable to predict the measurement on multiple frequency layers. In our understanding, MG still needs to be configured at least for model monitoring. If there are more than one inter-frequency layers involved, RAN4 needs to discuss the gap sharing mechanism among different inter-frequency layers both for measurement and monitoring. There would be some impact on measurement delay requirements.

Proposal 9: For Scenario 3, gap sharing mechanism among different inter-frequency layers should be considered for both for measurement and monitoring when more than one inter-frequency layers are involved.

Scenario 4, FR2 to FR2 intra-frequency intra-cell temporal domain case A, is to predict measurement result(s) in prediction window based on actual measurement result(s) in observation window of the same cell without reducing any measurement in observation window. There is no impact on UE measurement behavior, so there is no impact on the measurement delay.

Proposal 10: Scenario 4 has no impact on measurement delay requirements.

- Conclusion

In this contribution, we provide our views for RRM measurement prediction. Proposals and observations are summarized as follows.

Proposal 1: For RRM measurement prediction, only use cell level RSRP (Point C) as the performance metric for all the three sub-use cases.

Proposal 2: The test case for inter-frequency prediction can be designed as the following:

- NW configures SMTC for both f1 (intra-f) and f2 (inter-f). MG is not configured. A4 event is configured to check the prediction accuracy on f2. The threshold of A4 is set as X dBm. Suppose the absolute prediction accuracy is defined as ±YdB.

- TE calculates filtered L3 result on each SMTC occasion.

- TE can know when there is a neighbour cell whose RSRP is with [X-Y, X+Y]. TE can also get to know the time range when there will be expected SR based on the configurations. If UE triggers report request during the expected time range, then it can be viewed as the prediction meets the accuracy requirements in this single test.

Proposal 3: With the proposed test procedure, the mismatch between TE and UE on L1 filtering and L3 filtering can be reflected on the defined accuracy requirements or some additional margin of the test requirements.

Proposal 4: How to avoid UE cheating should be resolved at first before concluding to use reported measurement results as ground truth.

Observation 1: For scenario 3 (inter-frequency measurement prediction), it is possible to inform UE cell ID of the inter-frequency neighbor cell(s) to be predicted for collocated cases while may be not possible for non-collocated cases.

Proposal 5: For scenario 3 (inter-frequency measurement prediction), the cells predicted but not measured recently are unknown.

Proposal 6: Study when UE performs cell detection on the predicted cells, e.g., before model monitoring or during model monitoring.

Proposal 7: Wait for more progress in AI/ML BM to study the impact on measurement report delay requirements for AI mobility.

Proposal 8: Wait for more progress in AI/ML BM case 2 to study the impact on measurement delay requirements for AI mobility Scenario 2.

Proposal 9: For Scenario 3, gap sharing mechanism among different inter-frequency layers should be considered for both for measurement and monitoring when more than one inter-frequency layers are involved.

Proposal 10: Scenario 4 has no impact on measurement delay requirements.

- Reference

- RP-242393, Revised SID on AIML for mobility in NR, OPPO (rapporteur), RAN#105, Melbourne, Australia, September 9-12, 2024.

Appendix