3GPP TSG-RAN WG4 Meeting # 114bis	                                            R4-2503608

Wuhan, CN, April 7 – 11, 2025

Agenda item:	7.21.2.2

Source:	MediaTek Inc.

Title:	Discussion on RAN4 impacts for measurement event prediction

Document for:	Discussion

- Introduction

RAN#105 plenary has agreed the updated version of the SID with the following RAN4 objectives [1]:

According to the agreed work plan last meeting, this meeting RAN4 will focus on RRM measurement prediction and event prediction use-cases. In this contribution, we would like to discuss the related RRM requirements of measurement event prediction.

- Discussion

2.1 Metrics used to evaluate the model performance

In our view, one of the principles to choose the metrics is that UE will report corresponding metric or the metric can be derived through the reported results.  As far as we know, what to be reported for measurement event prediction is still in discussion in RAN2. It is too early to discuss the metrics to define requirements for in RAN4. It may be better to focus on discussing the metrics for RRM measurement prediction at first. If other quantities except RSRP will be reported too, then RAN4 can discuss whether and how to define requirements for the new quantities.

Observation 1: For measurement event prediction, it is too early to discuss the metrics to define requirements for in RAN4 as the reported quantities are quite open in RAN2.

2.2 Scenarios to define test cases

For measurement event prediction based on the scenario “intra-frequency temporal domain case A”,  legacy measurement event will fall in a time range [T1,T2] as shown in Fig.1 considering measurement uncertainty at UE. If the predicted event occasion can fall in [T1, T2] most of the time which is closer to the ground truth (T0: the occasion that event is met), it may be worth to define the test cases. But if the predicted event occasion will most likely fall out of [T1, T2] which means legacy measurement event reports are more reliable, we don’t think it is meaningful to define the corresponding test cases. Due to lack of simulation results, it is difficult to make a conclusion yet.

Fig. 1

Proposal 1: For measurement event prediction based on the scenario “intra-frequency temporal domain case A”, further study whether there is a need to define the corresponding test cases.

For measurement event prediction based on the scenarios “intra-frequency temporal domain case B” and “inter-frequency”, the test cases should be designed as similar as the real field.  Similar as described in our another contribution R4-2503607. We take inter-frequency prediction as an example to elaborate our thought on how RRM measurement prediction work in the real field. NW configures SMTC for both f1 (intra-f ) and f2 (the inter-f to be predicted). MG is not configured. How to perform measurement on f1 is up to UE implementation as long as the requirements can be met. How to predict is also totally up to UE as long as UE can identify the channel condition changes in time.

Similarly, the test case for inter-frequency measurement event prediction can be designed as the following:

- NW configures SMTC for both f1 and f2. MG is not configured. A4 event (Open to use other events) can be used to check the prediction accuracy on f2. TTT can be set to T1. Suppose the threshold of A4 is set as X dBm. The absolute prediction accuracy is defined as ±YdB. TE can calculate the filtered L3 result on each SMTC occasion. TE can know when there is a neighbour cell whose RSRP is within [X-Y, X+Y]. TE can also get to know the time range when there will be expected SR based on the configurations. If UE triggers report request during the expected time range, then it can be viewed as the prediction meets the accuracy requirements in this single test.

The test procedure will be similar as RRM measurement prediction. We doubt the necessity to define separate test cases for both RRM measurement prediction and measurement event prediction on the same scenario.

Proposal 2: For measurement event prediction and RRM measurement prediction on the scenarios “intra-frequency temporal domain case B” and “inter-frequency”, further study the necessity to define separate test cases for both use cases on the same scenario.

- Conclusion

In this contribution, we provide our views for measurement event prediction. Observation and proposals are summarized as follows.

Observation 1: For measurement event prediction, it is too early to discuss the metrics to define requirements for in RAN4 as the reported quantities are quite open in RAN2.

Proposal 1: For measurement event prediction based on the scenario “intra-frequency temporal domain case A”, further study whether there is a need to define the corresponding test cases.

Proposal 2: For measurement event prediction and RRM measurement prediction on the scenarios “intra-frequency temporal domain case B” and “inter-frequency”, further study the necessity to define separate test cases for both use cases on the same scenario.

- Reference

- RP-242393, Revised SID on AIML for mobility in NR, OPPO (rapporteur), RAN#105, Melbourne, Australia, September 9-12, 2024.