3GPP TSG-RAN WG4 Meeting # 114bis					       			       			R4-2503626

Wuhan, China, April 7-11, 2025

Agenda item:			7.25.3

Source:	Moderator (Huawei)

Title:	Topic summary for [114bis][216] Netw_Energy_NR_enh_Part2

Document for:	Information

Introduction

This topic summary includes RRM core requirements for Rel-19 network energy saving (7.25.2.2 and 7.25.2.3).

Recommended Topics to be treated online (in order of decreasing priority):

Issue 1-3-1: L3 requirements for SSB adaptation

Issue 2-1-1: RRM requirements impacts for OD-SIB1 – paging interruption

Issue 2-1-2: RRM requirements impacts for OD-SIB1 – timing

Issue 1-2-1: L1 requirements at transition

Issue 1-4-1: SCell activation requirement for SSB adaptation – SSB adaptation command related

Issue 1-4-2: SCell activation requirement for SSB adaptation – applicable SCell activation requirements

Topic #1: Adaptation of common signal/channel transmission

Main technical topic overview. The structure can be done based on sub-agenda basis.

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2503125 | Qualcomm Technologies Ireland | Observation 1: RAN1 confirmed that SSB adaptation means adaptation of burst periodicity. There is no change in frequency before and after SSB adaptation. The QCL relation for the same SSB index before and after SSB adaptation is unchanged. It was confirmed that there is no change in transmitted SSB before and after SSB adaptation. Proposal 1: RAN4 should focus on adaptation of SSB periodicity for connected mode, SCell and NCD-SSB under the assumption that frequency location and QCL relation before and after SSB adaptation is unchanged. Proposal 2: RAN4 should not define requirements for SSB adaptation for PCell. Observation 2: The transition period can be defined as Tfirst_SSB + meas_period(P2). Tfirst_SSB is the time till the first SSB with the new period P2 is received. meas_period(P2) is the measurement period based on the new periodicity P2. Observation 3: Tfirst_SSB cannot be defined as the first SSB after the reception of the switch command in DCI 2-9, but additional processing time needs to be considered. Observation 4: Rather than the Rel-18 3ms timeline for DCI 2-9, a timeline like the MAC CE based timeline for OD-SSB may be applicable for DCI 2-9. It remains to be seen by how much the timeline may be shortened since DCI 2-9 is applied. Proposal 3: RAN4 should investigate how to define Tfirst_SSB to leave enough processing/preparation time for the UE to receive the SSB with new periodicity after the reception of the switch command in DCI 2-9. Observation 5: Based on RAN2 agreement, neighbor cell measurements are based on legacy SMTC and are not impacted by SSB adaptation. Proposal 4: RAN4 does not need to define new requirements for L3 neighbor cell measurements. Observation 6: The main task in RAN4 is then to identify for each case a suitable transition time during which another requirement or even no requirement applies. The transition time depends on the details of the RAN2 agreement that are not yet available. Proposal 5: RAN4 should wait till RAN2 has made more progress on the details for introducing additional SMTC for SSB adaptation. |
| R4-2503294 | vivo | Proposal 1: For SSB periodicity of SSB adaptation, support option 1, i.e., consider legacy SSB burst periodicity as starting point to define the requirements. Proposal 2: For SSB adaptation issue 1-2-4, prefer keep (TDRX,TSSB) when DRX cycle≤320 ms in the L1-RSRP for serving cell, L1-SINR requirement. Option 2 is ok. Proposal 3: For L3 requirements for SSB adaptation, support option 2 “same as legacy” for deactivated SCell measurement. |
| R4-2503328 | Xiaomi | Observation 1: In current spec, the total L3 measurement delay requirements depend on the SMTC periodicity which is configured by NW RRC message can be reused for L1/L3 measurement. Observation 2: UE’s measurement requirements can be little impacted because SMTC period in TPSS/SSS_sync and T SSB_measurement_period can be explicitly indicated by NW. Proposal 1: Requirements on L3 measurements based on the adaptation SSB can reuse the current L3 RRM requirements framework in TS38.133 by replacing SMTC configuration with the additional one for the adaptation SSB. Proposal 2: Define the SCell activation requirements based on the SSB periodicity after SSB adaptation. - The start point can be the slot UE receiving SCell activation command if UE receiving SSB adaptation command (MAC) is not later than it |
| R4-2503344 | OPPO | Proposal 1: For SSB adaptation on deactivated SCell measurement, the same L3 requirements as legacy based on measCyclescell can be used. Observation 1: For SSB adaptation on deactivated SCell measurement, additional SMTC configuration is not needed. Observation 2: For SSB adaptation during SCell activation, the requirements are based on the SSB periodicity after SSB adaptation, and additional SMTC configuration is also not needed. Observation 3: After the SCell has been activated, L3 measurement on serving cell or intra-frequency can use legacy SMTC. Proposal 2: RAN4 to discuss the need of additional SMTC since it may be transparent to RAN4. Proposal 3: Any new SMTC impact should be considered for OD-SSB and SSB adaptation together. |
| R4-2503381 | CMCC | Proposal 1: For L3 requirements, reuse the legacy transition framework, that when SMTC configuration changes, the cell identification and measurement period requirements apply based on the longer delay before or after the transition. Proposal 2: Refine the agreement from last meeting: For SCell activation: - Define the SCell activation requirements based on the SSB periodicity after SSB adaptation. - For the case SSB adaptation command comes not later than with SCell activation command |
| R4-2503430 | CATT | Proposal 1: The L3 neighbour cell measurement requirements due to SSB adaptation follows legacy SMTC requirement. Proposal 2: RAN4 need to further discuss how to use additional SMTC configuration to define L3 measurement requirements on SCell with SSB adaptation, more details are needed from RAN2. |
| R4-2503570 | Apple | Observation 1: neither SMTC periodicity nor SSB periodicity appears in requirement of RRM measurement on deactivated SCC in legacy. Observation 2: regardless of actual SSB periodicity, UE always follow measCycleSCell for RRM measurement on deactivated SCC from measurement period point of view. Proposal 1: existing requirements of RRM measurement on deactivated SCC also for SSB periodicity adaptation. Proposal 2: existing requirements of RRM measurement on active serving cell also apply for SSB periodicity adaptation. Only the SMTC may need to be clarify to accommodate the additional SMTC configuration. |
| R4-2503594 | LG Electronics | Proposal 1: RAN4 to focus on L3 measurement requirement issue rather than L1 measurement requirement Proposal 2: Before down select deactivated SCell L3 requirements for SSB adaptation, Option 1 (reusing OD-SSB Case#2) needs to be clarified due to following aspects: - Only OD-SSB based requriements can be applied (no combination of AO-SSB and OD-SSB) - No clear requirments when the SSB periodicity is longer than before Proposal 3: Subsequent to transition period, requirements apply based on the SMTC periodicity after transition for SMTC adaptation of periodicity Proposal 4: Specifcy intra-frequency identification and intra-freqeuncy measurement w/wo gap requirement - For intra-frequency with gap requirement, specify the requirement when intra-frequency measurement becomes partially overlapped with measurement gap Proposal 5: For L3 measurement period, requirements with longer delay is applied corresponding to the SSB before and after adaptation |
| R4-2503867 | Huawei, HiSilicon | Observation 1: DCI-based SSB adaption triggering is supported by RAN1. RAN2 will FFS whether MAC CE based SSB adaptation is supported. Observation 2: DCI-based SSB adaption triggering is supported by RAN1. RAN2 will FFS whether MAC CE based SSB adaptation is supported. Observation 3: RAN2 preference on L3 measurement: - SMTC based SCell measurement with additional SMTC configurations - SMTC based neighbour cell measurement with legacy SMTC Observation 4: The occasionally triggered SSB adaption (maybe also for OD-SSB) may collide pre-defined Cell specific CSI-RS, and gNB may not be able to guarantee that both SSB and CSI-RS are available to UE. Per existing requirements, if UE chooses to measurement CSI-RS, it may cause wrong UE behaviour. Proposal 1: RAN4 to discuss whether existing L1 measurement restriction can apply to the case when adapted SSB colliding with CSI-RS. i.e. Whether UE can choose to measure one of them or only to measure SSB. Proposal 2: RAN4 to confirm the RAN2 agreements on L3 measurement framework on SSB adaptation. And RAN4 to further discuss RAN4 spec impact based on further RAN2 progress. (e.g. whether to define requirements at transition of SMTC change) Proposal 3: For SCell activation requirements for SSB adaptation, RAN4 to wait for more progress from RAN1/2 about triggering command and discuss the conditions that “SSB adaption command and SCell activation command comes together. Proposal 4: Based on agreements in last RAN4 meeting, RAN4 to define SCell activation requirements for SSB adaption for single SCell activation, including: - Normal SCell activation - Direct SCell activation - PUCCH SCell activation - SCell activation with L3 reporting The detailed spec impact can be discussed in CR stage. FFS whether to define requirements for multiple SCells activation. |
| R4-2503918 | China Telecom | Proposal 1: For L1-RSRP measurement based on the SSB periodicity after transition, if the adapted SSB periodicity is less than DRX cycle, UE to measure the SSB resource for adaptation using the measurement period corresponding to non-DRX mode. Proposal 2: For deactivated SCell measurement, it’s proposed to reuse the same conclusion from OD-SSB case#2 (scenario#4) discussion to define RRM requirements for SSB adaptation, at least for the case when the SSB periodicity is adapted to be shorter than before. |
| R4-2504227 | Ericsson | Proposal 1: The L3 neighbour cell measurement follows legacy SMTC requirement. Proposal 2: RAN4 to wait RAN2’s further clarification on how to configure the additional SMTC for SCell. |
| R4-2504247 | ZTE Corporation, Sanechips | Proposal 1: According to the RAN2 LS reply, - For the SCell with SSB adaptation, corresponding SMTC re-configuration/adaptation is expected, and UE performs the SCell measurement based on the re-configured/adaptative SMTC. - For the neighbour cell, UE perform neighbour cell measurement still based on the legacy SMTC, no impact by the SSB adaption. Proposal 2: For the deactivated SCell measurement, when SSB adaptation happens, at least for the case when the SSB periodicity is adapted to be shorter than before, reuse the same conclusion from OD-SSB case#2 (scenario#4) discussion to define RRM requirements |
| R4-2504290 | Samsung | Observation 1: - For SSB burst periodicity adaptation, the indication is signaled by group common DCI, DCI format 2_9 - (Unified) MAC-CE details for SSB burst periodicity adaptation is up to RAN2, depending on the corresponding discussions in OD-SSB feature Observation 2: For L3 RRM measurement on SCell with SSB adaptation. - Existing SMTC based L3 RRM framework can be inherited - Additional SMTC configuration accommodating with SSB adaptation will be introduced and defined Observation 3: For L3 measurement, the adapted SSB on neighbor cell is measured based on legacy SMTC Proposal 1: - Time domain positions of SSBs within a burst is same before and after SSB adaptation - RAN4 to consider legacy SSB burst periodicity as starting point to define the requirements - No RRM impacts for SSB adaptation requirement for SCell CD-SSB. Proposal 2: For SSB adaptation, RAN4 to define requirements considering the following cases - HF index is same before and after SSB adaptation and SFN offset is same before and after SSB adaptation - HF index is same before and after SSB adaptation and SFN offset is same or different before and after adaptation - Wait for RAN1 conclusions on whether to introduce explicit indication or up to UE implementation to perform blind SFN detection Proposal 3: For RRM requirement impact on activated SCell measurement/detection, RAN4 to discuss how to specify time periods for intra-frequency SCell identification and measurement, and inter-frequency SCell identification and measurement after receiving RAN2 agreements on the detailed SMTC configuration mechanism (s) for NES Proposal 4: - If group-common DCI is considered to indicate the adaptation of SSB, for deactivated SCell measurement requirement, same as legacy. - If MAC-CE can also be considered to indicate the adaptation of SSB, for deactivated SCell measurement, impose the same principle from OD-SSB case#2 (same frequency but with different periodicities) discussion to define RRM requirements - Wait for RAN2 conclusions on whether MAC-CE indication for SSB adaptation can be supported Proposal 5: Wait for RAN2 conclusions on whether additional SMTC configuration according to SSB adaptation for measurement on deactivated SCell with SSB adaptation can be supported Proposal 6: - If separate MAC CE to activate SSB adaptation and SCell, define the SCell activation requirements based on the SSB periodicity after SSB adaptation - For the case SSB adaptation command comes together with SCell activation command - If MAC CE to activate SCell but group-common DCI to activate SSB adaptation, RAN4 to discuss the feasibility of defining the SCell activation requirements for the case SSB adaptation command comes together with SCell activation command Proposal 7: For L1-RSRP measurement based on the SSB periodicity after transition, if the adapted SSB periodicity is less than DRX cycle, UE to measure the SSB resource for adaptation using the measurement period corresponding to non-DRX mode Proposal 8: To guarantee the measurement accuracy at transition - UE can to perform measurement based on the SSB resources after transition after transition period, whether to release the measurement results from resources before transition up to UE implementation |
| R4-2504506 | Nokia, Nokia Shanghai Bell | Observation 1: The network must ensure that the adapted SSB periodicity in SCell is not larger than the legacy SMTC periodicity. Proposal 1: RAN4 to consider RRM impacts from SSB adaptation in SCell for NCD-SSB only. - Proposal 2: RAN4 to specify RRM requirements for SSB adaptation based on the existing range of SSB burst periodicities (5ms to 160ms) for SSB periodicity adaptation as the only changeable parameter. Proposal 3: RAN4 to consider specifying RRM requirements for SSB periodicity adaptation for NW triggered DCI and MAC-CE signaling methods, designed by RAN1 and RAN2. Proposal 4: RAN4 to consider the scenario for SSB adaptation command sent together with direct SCell activation command and investigate the impact to SCell addition requirements and align them to OD-SSB based direct SCell activation as much as possible. Proposal 5: RAN4 to consider the impact from SSB periodicity adaptation to Layer 1 measurements such as CBD, L1-RSRP and L1-SINR for active SCells during transition periods only. Proposal 6: RAN4 to agree the measurement period for CBD, L1-RSRP and L1-SINR prior to the SSB received at Tfirst_SSB is based on the SSB periodicity before adaptation. Proposal 7: RAN4 to agree following use case scenarios for specifying L1 transition requirements are distinguished: - Scenario 1: SSB adaptation to lower periodicity with early SSB adaptation command received after completed L1 measurement period. - Scenario 2: SSB adaptation to lower periodicity with late SSB adaptation command received after completed L1 measurement period. - Scenario 3: SSB adaptation to higher periodicity with early SSB adaptation command received after completed L1 measurement period. - Scenario 4: SSB adaptation to higher periodicity with late SSB adaptation command received after completed L1 measurement period. - Proposal 8: RAN4 not to consider any impact to measurement requirements from L1 reporting for SSB adaptation, until further notice from RAN1. Proposal 9: RAN4 to proceed with the assumption that the additional SMTC configuration carries a set of SSB periodicities, from which one will be activated in the SSB adaptation command and only one active at any time in the SCell. Proposal 10: RAN4 to define measurement requirements for deactivated SCell in case of SSB adaptation reusing the legacy requirements for measurements without gaps or with NCSG. Proposal 11: RAN4 to agree measurement period requirements only apply from the first SMTC occasion after reception of the SSB adaptation command, no matter whether the periodicity gets higher or lower. - Proposal 12: RAN4 to define measurement requirements for SCell activation in case of concurrent SCell activation and SSB adaptation command in same RRC message, i.e. based on Tactivation_time = TFirstSSB + X, with X>5ms for measurement period lower or equal to 2400ms and known SCell in FR1. Proposal 13: RAN4 to consider the impact of processing time for decoding the updated SSB burst periodicity, depending on the signaling method specified in RAN1 (DCI), RAN2 (MAC-CE) and RAN4 (RRC), in order to be ready to receive the SSBs with the adjusted SSB burst periodicity, identified as the SSB adaptation delay. Proposal 14: RAN4 to define the MAC-CE based processing delay for SSB adaptation same as for OD-SSB. |

Open issues summary

Before Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Sub-topic 1-1 SSB adaptation – Scenario

Issue 1-1-1: Scenario clarification based on RAN1 progress

Background:

RAN4 agreements in RAN4#112bis R4-2416860 Issue 1-1-1: Scenario clarification based on RAN1 progress Agreement: - Per RAN1 agreements, RAN4 to take following as starting point for SSB adaption discussion: - CONNECTED mode, and - SSB adaptation for SCell, and - SSB adaptation for NCD-SSB, and - SSB periodicity adaptation in time-domain Note: Above could be further updated based on further RAN1 progress (e.g. PCell, other adaptation mechanism) RAN1#118 bis Conclusion There is no consensus on the support of adaptation of SSB for idle mode UEs in Rel-19 Agreement For Cell DTX extension to SSBs not on sync-raster for connected mode UEs, select Option 3, i.e. Cell DTX does not impact UE assumption on SSB transmissions (i.e. legacy behavior). - No spec impact RAN4 agreements in RAN4#113 R4-2420242 Issue 1-1-1: Scenario clarification based on RAN1 progress Agreement: - No RRM impacts for SSB adaptation requirement for IDLE mode. - No RRM impacts for Cell DTX. RAN1#119 Reply to Q1(What is the relation in terms of time location before and after SSB adaptation?): - RAN1 agreed that at least SSB burst periodicity is adapted. - There are no RAN1 agreements to adapt the time location of the SSB burst other than the periodicity but RAN1 is still discussing other options. Reply to Q2(What is the relation in terms of frequency location before and after SSB adaptation?): - The frequency location is same before and after SSB adaptation. Reply to Q3(What is the spatial relation before and after SSB adaptation?): - There is no change to spatial relation (in terms of QCL assumption) for the same SSB index before and after SSB adaptation. At least the case where there is no change to actually transmitted SSBs within a burst before and after SSB adaptation is supported. Conclusion There is no RAN1 consensus to support SSB adaptation in time domain for Rel-19 NES-capable UE’s PCell (connected mode) RAN1#120 Conclusion There is no RAN1 consensus to support the following in Rel-19 - New SSB burst periodicity values other than the legacy values (i.e., 5 ms, 10 ms, 20 ms, 40 ms, 80 ms, or 160 ms). - New UE trigger - Adapting the transmitted number of SSBs within a SSB burst - Adaptation of the time domain positions of SSBs within a burst Conclusion There is no RAN1 consensus to support time domain adaptation for CD-SSB in Rel-19 for SCell RAN4#114 R4-2502689 Issue 1-1-1: Scenario clarification based on RAN1 progress Agreement: - Further clarification on the scope of RRM impact for SSB adaption: - No RRM impacts for SSB adaptation requirement for PCell: - Frequency location is same before and after SSB adaptation. - No change to spatial relation (in terms of QCL assumption) for the same SSB index before and after SSB adaptation - Note: Above could be further updated based on further RAN1 progress if any.

- Proposals

  - Option 1: RAN4 should focus on adaptation of SSB periodicity for connected mode, SCell and NCD-SSB under the assumption that frequency location and QCL relation before and after SSB adaptation is unchanged. (QC)

  - Option 2: RAN4 should not define requirements for SSB adaptation for PCell. (QC)

  - Option 3: For SSB periodicity of SSB adaptation, consider legacy SSB burst periodicity (Vivo, Samsung, Nokia)

  - Option 4: Time domain positions of SSBs within a burst is same before and after SSB adaptation. (Samsung)

  - Option 5: No RRM impacts for SSB adaptation requirement for SCell CD-SSB. (Samsung)

  - Option 6: RAN4 to consider RRM impacts from SSB adaptation in SCell for NCD-SSB only. (Nokia)

- Moderator:

Based on moderator’s understanding, agreements in last RAN4 meeting still hold. Proposals in this meeting above are aligned with latest RAN1 agreements/conclusion. The recommended WF are as follows and no controversial issues needs further discussion. The new added changes are highlighted.

- Recommended WF:

  - Further clarification on the scope of RRM impact for SSB adaption:

    - No RRM impacts for SSB adaptation requirement for PCell:

    - Frequency location is same before and after SSB adaptation.

    - No change to spatial relation (in terms of QCL assumption) for the same SSB index before and after SSB adaptation

    - Existing SSB burst periodicity (i.e., 5 ms, 10 ms, 20 ms, 40 ms, 80 ms, or 160 ms).

    - SSB adaptation in SCell for NCD-SSB.

    - Time domain positions of SSBs within a burst is same before and after SSB adaptation

Issue 1-1-2: Scenario clarification-triggering

- Proposals

  - Option 1(Samsung)

For SSB adaptation, RAN4 to define requirements considering the following cases

    - HF index is same before and after SSB adaptation and SFN offset is same before and after SSB adaptation

    - HF index is same before and after SSB adaptation and SFN offset is same or different before and after adaptation

Wait for RAN1 conclusions on whether to introduce explicit indication or up to UE implementation to perform blind SFN detection

  - Option 2: RAN4 to consider specifying RRM requirements for SSB periodicity adaptation for NW triggered DCI and MAC-CE signaling methods, designed by RAN1 and RAN2. (Nokia)

  - Option 3: RAN4 to consider the impact of processing time for decoding the updated SSB burst periodicity, depending on the signaling method specified in RAN1 (DCI), RAN2 (MAC-CE) and RAN4 (RRC), in order to be ready to receive the SSBs with the adjusted SSB burst periodicity, identified as the SSB adaptation delay. (Nokia)

  - Option 4: RAN4 to define the MAC-CE based processing delay for SSB adaptation same as for OD-SSB. (Nokia)

- Recommended WF: Discuss above proposals and wait for further RAN1/2 progress.

Sub-topic 1-2 SSB adaptation – L1 Requirements impacts

Issue 1-2-1: L1 requirements at transition

RAN4#114 R4-2502689 Issue 1-2-1: Applicable L1 requirements for SSB adaptation Agreement: - No impact of SSB adaptation: - RLM - BFD - Define requirements if SSB adaption happens: - L1-RSRP for serving cell - L1-SINR - CBD - Whether and how to define requirements at transition is discussed in other issues. Issue 1-2-2: Transition requirements applicability & Issue 1-2-3: Transition requirements – details Agreement: - For L1-RSRP/L1-SINR/CBD - The measurement/evaluation period based on SSB periodicity after adaptation apply from Tfirst_SSB. - Tfirst_SSB is the uncertainty time from the timepoint when SSB adaptation occurs till the first SSB after SSB adaptation.

- Proposals

  - Option 1: RAN4 should investigate how to define Tfirst_SSB to leave enough processing/preparation time for the UE to receive the SSB with new periodicity after the reception of the switch command in DCI 2-9. (QC)

  - Option 2: RAN4 to consider the impact from SSB periodicity adaptation to Layer 1 measurements such as CBD, L1-RSRP and L1-SINR for active SCells during transition periods only. (Nokia)

  - Option 3: RAN4 to agree the measurement period for CBD, L1-RSRP and L1-SINR prior to the SSB received at Tfirst_SSB is based on the SSB periodicity before adaptation. (Nokia).

  - Option 4: RAN4 to agree following use case scenarios for specifying L1 transition requirements are distinguished (Nokia)

-	Scenario 1: SSB adaptation to lower periodicity with early SSB adaptation command received after completed L1 measurement period.

-	Scenario 2: SSB adaptation to lower periodicity with late SSB adaptation command received after completed L1 measurement period.

-	Scenario 3: SSB adaptation to higher periodicity with early SSB adaptation command received after completed L1 measurement period.

-	Scenario 4: SSB adaptation to higher periodicity with late SSB adaptation command received after completed L1 measurement period

Option 5: RAN4 not to consider any impact to measurement requirements from L1 reporting for SSB adaptation, until further notice from RAN1. (Nokia)

Option 6: RAN4 to focus on L3 measurement requirement issue rather than L1 measurement requirement. (LGE)

- Moderator:  Option 1, option 3 and Option 4 are related to how to define the Tfirst_SSB, which depends on RAN1/2 design on SSB adaptation command timeline.

  - Recommended WF:

    - RAN4 FFS how to define Tfirst_SSB considering the processing/preparation time based on further RAN1/2 progress on SSB adaptation command timeline.

    - No impacts on L1 reporting requirements for SSB adaptation, unless identified by other WGs.

Issue 1-2-2: L1 requirements after transition  - DRX

- Proposals

  - Option 1: For L1-RSRP measurement based on the SSB periodicity after transition, if the adapted SSB periodicity is less than DRX cycle, UE to measure the SSB resource for adaptation using the measurement period corresponding to non-DRX mode. (CTC, Samsung)

  - Option 2: keep (TDRX,TSSB) when DRX cycle≤320 ms in the L1-RSRP for serving cell. (Vivo)

- Recommended WF:

  - Discuss above options and whether there is RRM requirements impacts in terms of DRX configurations.

Issue 1-2-3: L1 requirements - whether to release the measurement results from resources before transition

- Proposals

  - Option 1: UE can to perform measurement based on the SSB resources after transition after transition period, whether to release the measurement results from resources before transition up to UE implementation. (Samsung)

- Recommended WF:

  - Agree on option 1 and no spec impacts.

Issue 1-2-4: L1 requirements – Measurement restriction impact

- Proposals

  - Option 1: RAN4 to discuss whether existing L1 measurement restriction can apply to the case when adapted SSB colliding with CSI-RS. i.e. Whether UE can choose to measure one of them or only to measure SSB. (Huawei)

- Recommended WF:

  - Discuss above option.

Sub-topic 1-3 SSB adaptation – L3 Requirements impacts

Issue 1-3-1: L3 requirements for SSB adaptation

Sub issue 1: General framework

  - Option 1: Reuse legacy L3 requirements framework. (Xiaomi, Vivo, Oppo, CMCC, Huawei, Apple, Nokia, Samsung for DCI-based deactivated SCell measurement)

    - Option 1a: Requirements on L3 measurements based on the adaptation SSB can reuse the current L3 RRM requirements framework in TS38.133 by replacing SMTC configuration with the additional one for the adaptation SSB. (Xiaomi, Vivo)

    - Option 1b: For SSB adaptation on deactivated SCell measurement, the same L3 requirements as legacy based on measCyclescell can be used. (Oppo)

    - Option 1c: existing requirements of RRM measurement on deactivated SCC also for SSB periodicity adaptation. (Apple)

    - Option 1d: existing requirements of RRM measurement on active serving cell also apply for SSB periodicity adaptation. Only the SMTC may need to be clarify to accommodate the additional. (Apple)

    - Option 1e: If group-common DCI is considered to indicate the adaptation of SSB, for deactivated SCell measurement requirement, same as legacy. (Samsung)

    - Option 1f: RAN4 to define measurement requirements for deactivated SCell in case of SSB adaptation reusing the legacy requirements for measurements without gaps or with NCSG. (Nokia)

  - Option 2: Reuse same principle from OD-SSB case#2 (same frequency but with different periodicities) (ZTE, CTC, Samsung for MAC-CE based)

    - Option 2a: For the deactivated SCell measurement, when SSB adaptation happens, at least for the case when the SSB periodicity is adapted to be shorter than before, reuse the same conclusion from OD-SSB case#2 (scenario#4) discussion to define RRM requirements. (ZTE)

    - Option 2b: If MAC-CE can also be considered to indicate the adaptation of SSB, for deactivated SCell measurement, impose the same principle from OD-SSB case#2 (same frequency but with different periodicities) discussion to define RRM requirements (Samsung)

      - Wait for RAN2 conclusions on whether MAC-CE indication for SSB adaptation can be supported

  - Option 3: (LGE)

    - Before down select deactivated SCell L3 requirements for SSB adaptation, Option 1 (reusing OD-SSB Case#2) needs to be clarified due to following aspects:

      - Only OD-SSB based requriements can be applied (no combination of AO-SSB and OD-SSB)

      - No clear requirments when the SSB periodicity is longer than before

  - Option 4: (LGE)

    - Specifcy intra-frequency identification and intra-freqeuncy measurement w/wo gap requirement

-	For intra-frequency with gap requirement, specify the requirement when intra-frequency measurement becomes partially overlapped with measurement gap

Sub issue 2: Impacts due to additional SMTC/SMTC change

  - Option 1: RAN4 should wait till RAN2 has made more progress on the details for introducing additional SMTC for SSB adaptation. (QC, CATT, Ericsson, Samsung)

  - Option 2: RAN4 to discuss the need of additional SMTC since it may be transparent to RAN4. (Oppo)

  - Option 3: Any new SMTC impact should be considered for OD-SSB and SSB adaptation together. (Oppo)

  - Option 4: when SMTC configuration changes, the cell identification and measurement period requirements apply based on the longer delay before or after the transition. (CMCC, LGE)

  - Option 5: 	For the SCell with SSB adaptation, corresponding SMTC re-configuration/adaptation is expected, and UE performs the SCell measurement based on the re-configured/adaptative SMTC. (ZTE)

  - Option 6: RAN4 to proceed with the assumption that the additional SMTC configuration carries a set of SSB periodicities, from which one will be activated in the SSB adaptation command and only one active at any time in the SCell. (Nokia)

  - Option 7: RAN4 to agree measurement period requirements only apply from the first SMTC occasion after reception of the SSB adaptation command, no matter whether the periodicity gets higher or lower. (Nokia, LGE)

    - Option 7a: Subsequent to transition period, requirements apply based on the SMTC periodicity after transition for SMTC adaptation of periodicity

Sub issue 3: Impacts on neighbour cell measurement

  - Option 1: RAN4 does not need to define new requirements for L3 neighbor cell measurements. (QC, CATT, ZTE)

    - Option 1a: The L3 neighbour cell measurement requirements due to SSB adaptation follows legacy SMTC requirement (CATT)

    - Option 1b: The L3 neighbour cell measurement follows legacy SMTC requirement (Ericsson)

    - Option 1c: For the neighbour cell, UE perform neighbour cell measurement still based on the legacy SMTC, no impact by the SSB adaption. (ZTE)

Moderator: Based on contributions for this meeting, majority view (8/12) is to reuse the legacy L3 measurement framework (i.e. based on SMTC).  For neighbour cell measurement, all companies agree that legacy requirements can apply. For serving cell measurement (deactivated SCell or activated SCell), more progress on additional SMTC is needed from RAN2 before RAN4 can decide whether to define L3 requirements at transition and other RRM requirements impacts.

- Recommended WF:

  - RAN4 to define requirements based legacy L3 measurement framework (i.e. based on SMTC) for SSB adaption.

    - For neighbour cell L3 measurement, legacy requirements apply.

    - For SCell with SSB adaptation (deactivated SCell or activated SCell), wait for more RAN2 progress on additional SMTC configuration, and discuss following aspects:

      - Whether to define L3 measurement requirement at transition.

      - Any other RRM requirements impacts due to additional SMTC configurations.

Sub-topic 1-4 SSB adaptation – Other requirements impacts

Issue 1-4-1: SCell activation requirement for SSB adaptation – SSB adaptation command related

- Proposals

  - Option 1: Define the SCell activation requirements based on the SSB periodicity after SSB adaptation. (Xiaomi, CMCC)

    - The start point can be the slot UE receiving SCell activation command if UE receiving SSB adaptation command (MAC) is not later than it

  - Option 2: For SCell activation requirements for SSB adaptation, RAN4 to wait for more progress from RAN1/2 about triggering command and discuss the conditions that “SSB adaption command and SCell activation command comes together. (Huawei)

  - Option 3: If separate MAC CE to activate SSB adaptation and SCell, define the SCell activation requirements based on the SSB periodicity after SSB adaptation. (Samsung)

    - For the case SSB adaptation command comes together with SCell activation command

  - Option 4: If MAC CE to activate SCell but group-common DCI to activate SSB adaptation, RAN4 to discuss the feasibility of defining the SCell activation requirements for the case SSB adaptation command comes together with SCell activation command. (Samsung)

  - Option 5: RAN4 to consider the scenario for SSB adaptation command sent together with direct SCell activation command and investigate the impact to SCell addition requirements and align them to OD-SSB based direct SCell activation as much as possible. (Nokia)

- Recommended WF:

  - RAN4 to discuss the conditions for SCell activation with SSB adaptation (i.e. based SSB periodicity after SSB adaptation), considering following aspects:

    - SSB adaptation command (e.g. DCI/MAC-CE/RRC), which subject to conclusion from other WG

    - Time relation between SSB adaptation command and SCell activation command.

Issue 1-4-2: SCell activation requirement for SSB adaptation – applicable SCell activation requirements

- Proposals

  - Option 1: Based on agreements in last RAN4 meeting, RAN4 to define SCell activation requirements for SSB adaption for single SCell activation, including (Huawei)

    - Normal SCell activation

    - Direct SCell activation

    - PUCCH SCell activation

    - SCell activation with L3 reporting

The detailed spec impact can be discussed in CR stage.

FFS whether to define requirements for multiple SCells activation.

  - Option 2: RAN4 to consider the scenario for SSB adaptation command sent together with direct SCell activation command and investigate the impact to SCell addition requirements and align them to OD-SSB based direct SCell activation as much as possible. (Nokia)

  - Option 3: RAN4 to define measurement requirements for SCell activation in case of concurrent SCell activation and SSB adaptation command in same RRC message, i.e. based on

    - Tactivation_time = TFirstSSB + X, with X>5ms for measurement period lower or equal to 2400ms and known SCell in FR1. (Nokia)

- Recommended WF:

  - Discuss applicable SCell activation requirements, including

    - Normal SCell activation

    - Direct SCell activation

    - PUCCH SCell activation

    - SCell activation with L3 reporting

    - Multiple SCells activation

Topic #2: OD-SIB1

Main technical topic overview. The structure can be done based on sub-agenda basis.

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2503131 | MediaTek inc. | Proposal 1: Due to UL-WUS transmission for OD-SIB1, RAN4 further discuss the impact on paging interruption requirement in cell re-selection. One possible approach is to extend the interruption time to include the UL WUS transmission. |
| R4-2503126 | Qualcomm Technologies Ireland | Observation 1: Potential impact for RRM requirements is only in idle/inactive mode but not for RRC_CONNECTED. Observation 2: The legacy cell reselection criterion is used as a trigger condition for OD-SIB1 acquisition. Hence, cell reselection related measurement procedures are not impacted by OD-SIB1. Observation 3: The basic flow for OD-SIB1 is that the UE sends a UL WUS to the NES cell and receives OD-SIB1 from the NES cell. After cell reselection the UE camps in the NES cell. Observation 4: It is under discussion in RAN1 whether the UE should also monitor type-0 PDCCH for OD-SIB1 in case that OD-SIB1 is already transmitted by the NES cell. In this case, the UE may not need to transmit UL WUS. Proposal 1: RAN4 should not yet discuss the scenario that the UE does not need to transmit UL WUS to receive OD-SIB1 and wait for further progress in RAN1. Observation 5: Since the core requirement for the maximum interruption time is very generic, the core requirement for paging interruption is not impacted by OD-SIB1. Proposal 2: RAN4 should further investigate whether TSI-NR = 1280 ms can cover UL-WUS transmission and OD-SIB1 reception as well or whether in the performance tests a larger TSI-NR should be defined. Proposal 3: Since the UL WUS is sent to the NES cell, RAN4 should discuss whether the SSB of the NES can be used as the downlink reference signal to define the reference timing. |
| R4-2503295 | vivo | Observation 1 OD-SIB1 related procedure only impacts UE behaviour in RRC idle/inactive states. Observation 2 UE performs cell re-selection to NES cell with OD-SIB1 enabled, when UL WUS triggering condition is met, i.e. UL WUS triggering condition is the same as cell re-selection condition and OD-SIB1 acquisition shall be regarded as a special case of cell re-selection. Proposal 1 RAN4 to further discuss the potential RRM requirement impacts on paging interruption for R19 OD-SIB1 related procedure. Proposal 2 For R19 OD-SIB1 related procedure, clarify DL timing reference of UL-WUS in TS 38.133. RAN4 may further discuss whether additional relaxation or additional applicability restrictions (e.g. SNR lower bound) on Te requirements is needed. Proposal 3 Clarify in TS 38.133 4.2.2.6 that, time for UL WUS msg1 transmission and time for msg2 reception are components of paging interruption for OD-SIB1 cell re-selection requirements. Proposal 4 RAN4 to further discuss the RRM requirements on minimum WUS msg1 attempts during each paging cycle, as part of paging interruption requirements. |
| R4-2503345 | OPPO | Proposal 1: RAN4 to discuss paging interruption due to OD-SIB1 related procedure. Proposal 2: The interruption requirements may need to be extended due to UL WUS and RAR. Proposal 3: Clarify UL timing related requirements/conditions for UL-WUS in RAN4. |
| R4-2503382 | CMCC | Proposal 1: Reuse the current paging interruption requirement and the definition of TSI-NR. Proposal 2: Further discuss whether the current value of TSI-NR in the test need to be extended in perf phase. Proposal 3: For the UL timing requirement of UL-WUS, the DL timing reference of UL-WUS shall be the downlink timing of the reference cell minus NTA+NTA offset×Tc, that: - reference cell is the NES cell - NTA for UL-WUS is defined as 0 - is the n-TimingAdvanceOffset from UL-WUS configuration if configured, if not, follow the default value defined in Table 7.1.2-2 TS 38.133 |
| R4-2503431 | CATT | Observation 1: The UE will receive UL WUS configuration from cell A, and then UE shall send UL WUS to the NES cell after UE meets the WUS triggering conditions. Proposal 1: RAN4 need to clarify downlink reference point of UL-WUS in TS 38.133 for OD-SIB1 related procedure. - The reference cell is the NES cell. - NTA for UL-WUS is defined as 0. - UE identifies NTA offset based on the information n-TimingAdvanceOffset from UL-WUS configuration. Observation 2: In legacy maximum interruption in paging reception requirements, RAN4 didn’t define the exact value for SIB1 receiving in TS 38.133. Proposal 2: For OD-SIB1 reception, RAN4 need to discuss whether there are impact on the interruption in paging reception requirements for cell reselection in TS 38.133. |
| R4-2503571 | Apple | Proposal 1: potential paging interruption due to UL WUS transmission and RAR reception for OD-SIB1 acquisition needs to be reflected in RRM requirements of Maximum interruption in paging reception. |
| R4-2503596 | LG Electronics | Proposal 1: Regarding paging interruption requirement, no RAN4 impact is identified - Detail of TSI-NR is upto RAN2 Observation 1: Current Te requirement can be applied to OD-SIB1 procedure Proposal 2: Fine to update wording of Te requirement to avoid confusion |
| R4-2503868 | Huawei, HiSilicon | Proposal 1: For RRM requirements impacts of OD-SIB1, RAN4 to update the paging interruption requirements as: TSI-NR is the time required for receiving all the relevant system information data according to the reception procedure and the RRC procedure delay of system information blocks defined in TS 38.331 [2] for an NR cell. When the target Cell is an [NES OD-SIB1 Cell], TSI-NR also includes the time for UL WUS transmission and RAR reception for OD-SSB1 reception. |
| R4-2504228 | Ericsson | Observation 1: Paging monitoring in cell A is more important than the UL-WUS transmission to acquire the OD-SIB1. Proposal 1: No RAN4 spec. impact is identified. - UE shall prioritize the paging reception and delay the UL WUS procedure if collision happens. |
| R4-2504507 | Nokia, Nokia Shanghai Bell | Proposal 1: RAN4 to consider impacts to paging interruption during cell reselection to an OD-SIB1 cell due to additional delays for 1) UL-WUS transmission, 2) reception of RAR, 3) time for SIB1 reception. Proposal 2: RAN4 to discuss if the cell reselection delay shall be extended due to UL WUS procedure when UE performs cell reselection to an OD-SIB1 cell. Proposal 3: RAN4 to specify a new test case to verify the cell reselection behaviour to an OD-SIB1 cell. Proposal 4: RAN4 to reuse existing RRM requirements for normal PRACH transmission at system access, for UL-WUS Tx timing, i.e. based on reception of SSB in the NES cell. Proposal 5: RAN4 to consider the additional RRM impact for one or more UL-WUS Msg1 retransmission attempts in the NES cell, if configured. |

Open issues summary

Before Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Sub-topic 2-1 RRM requirements impacts for OD-SIB1

Issue 2-1-1: RRM requirements impacts for OD-SIB1 – paging interruption

- Proposals

  - Option 1: RAN4 to further discuss the potential RRM requirement impacts on paging interruption for R19 OD-SIB1 related procedure. (Vivo, Oppo, CATT, Nokia)

    - Option 1a: 	RAN4 to discuss if the cell reselection delay shall be extended due to UL WUS procedure when UE performs cell reselection to an OD-SIB1 cell. (Nokia)

  - Option 2: clarify that time for UL WUS msg1 transmission and time for msg2 reception are components of paging interruption for OD-SIB1 cell re-selection requirements. (Vivo, MTK, Oppo, Apple, Huawei, Nokia)

    - Option 2a: extend the interruption time to include the UL WUS transmission. (MTK)

    - Option 2b: RAN4 to consider impacts to paging interruption during cell reselection to an OD-SIB1 cell due to additional delays for 1) UL-WUS transmission, 2) reception of RAR, 3) time for SIB1 reception. (Nokia)

  - Option 3: Reuse the current paging interruption requirement and the definition of TSI-NR. Further discuss whether the current value of TSI-NR in the test need to be extended in perf phase. (CMCC, QC)

  - Option 4: No RAN4 impact is identified. Detail of TSI-NR is up to RAN2 (LGE)

  - Option 5: No RAN4 spec. impact is identified. UE shall prioritize the paging reception and delay the UL WUS procedure if collision happens. (Ericsson)

- Recommended WF:

  - Discuss following options:

    - Option A: Clarify that paging interruption includes UL WUS transmission and RAR reception for OD-SIB1 acquisition.

    - Option B: No impacts on core requirements. Discuss whether to extend TSI-NR in test case.

    - Option C: No RRM impacts.

Issue 2-1-2: RRM requirements impacts for OD-SIB1 – timing

- Proposals

  - Option 1: Clarify UL timing related requirements/conditions for UL-WUS in RAN4. (OPPO, Vivo, QC, LGE, Nokia)

    - Option 1a: RAN4 should discuss whether the SSB of the NES can be used as the downlink reference signal to define the reference timing. (QC)

    - Option 1b: clarify DL timing reference of UL-WUS in TS 38.133. RAN4 may further discuss whether additional relaxation or additional applicability restrictions (e.g. SNR lower bound) on Te requirements is needed. (Vivo)

    - Option 1c: the DL timing reference of UL-WUS shall be the downlink timing of the reference cell minus (NTA+NTA offset )×T_c, that: (CATT, CMCC)

      - reference cell is the NES cell

      - NTA for UL-WUS is defined as 0

      - NTA offset is the n-TimingAdvanceOffset from UL-WUS configuration if configured, if not, follow the default value defined in Table 7.1.2-2 TS 38.133

    - Option 1d: RAN4 to reuse existing RRM requirements for normal PRACH transmission at system access, for UL-WUS Tx timing, i.e. based on reception of SSB in the NES cell. (Nokia)

- Recommended WF:

    - Discuss whether to define timing requirements for UL-WUS transmission, and discuss following aspects:

      - DL reference timing

      - NTA offset and NTA

      - Te requirements

Issue 2-1-3: RRM requirements impacts for OD-SIB1 – Others

- Proposals

  - Option 1: 	RAN4 to consider the additional RRM impact for one or more UL-WUS Msg1 retransmission attempts in the NES cell, if configured. (Nokia)

  - Option 2: RAN4 to specify a new test case to verify the cell reselection behaviour to an OD-SIB1 cell. (Nokia)

  - Option 3: RAN4 should not yet discuss the scenario that the UE does not need to transmit UL WUS to receive OD-SIB1 and wait for further progress in RAN1. (QC)

  - Option 4: RAN4 to further discuss the RRM requirements on minimum WUS msg1 attempts during each paging cycle, as part of paging interruption requirements. (Vivo)

- Recommended WF:

    - Discuss above proposals.

Working split

The plan is to trigger CR split for SSB adaption and OD-SIB1 in RAN4#114bis for CR submission in following meetings.

SSB adaptation

| Requirements | Remarks | Responsibility |
| --- | --- | --- |
| 9.5 L1-RSRP measurements for Reporting |  |  |
| 9.8 L1-SINR measurements for Reporting |  |  |
| 8.5.5 Requirements for SSB based candidate beam detection |  |  |
| 8.3 SCell activation requirements | FFS the impacted sub-clause |  |
| 9.2 NR intra-frequency measurement | FFS the impact on L3 measurement |  |
| 9.3 NR inter-frequency measurement | FFS the impact on L3 measurement |  |

OD-SIB1

| Requirements | Remarks | Responsibility |
| --- | --- | --- |
| 4.2.2.6 Maximum interruption in paging reception | Pending on discussion |  |
| 7.1 UE transmit timing | Pending on discussion |  |