3GPP TSG-RAN WG4 Meeting #114bis		R4-2503665

Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda item:		7.22.3.1

Source:		Samsung

Title:	Discussion on RRM requirements of UE-initiated beam management in Rel-19 MIMO phase 5

Document for:		Discussion

- Introduction

In last RAN4#114 meetings, the WF was approved in [1]. In this contribution, we continue to discuss the open issues of RRM requirements for UE-initiated/event-driven beam management.

- Discussion

RRM core requirements of UE-initiated beam management

In last RAN4#114 meeting, the potential issues for further discussion are captured in WF [1].

Issue 1-2: How to define measurement period in Event triggered measurement reporting delay for the basic feature (M instance = 1)? Agreement: The agreements under this issue is for event 2, and further discuss whether these agreements can be applicable to other events agreed in RAN1. TL1-RSRP_event triggered_measurement_Period_SSB or TL1-RSRP_event triggered_measurement_Period_CSI-RS - update based on 9.5.4.1 (SSB based), 9.5.4.2 (CSI-RS based) and 9.13.4 - TReport = 0 or be removed from the measurement period - N: Reuse the legacy definition - M: FFS pending RAN1 further agreement - TSSB (taking SSB as example) - For the same periodicity supported in RAN1, TSSB_current, or TSSB_new - RAN4 to further discuss the case of different periodicities if different periodicities will be [supported or not excluded] in RAN1 - Option A: Max{TSSB_current, TSSB_new} - Option B: When the RS periodicity is not same, UE evaluate the event after each measurement occasion of current and new beam. If the event is triggered after the current beam measurement, TSSB is TSSB_current. If the event is triggered after the new beam measurement, TSSB is TSSB_new. - Option C: others are not precluded Issue 1-3: Same or different periodicities of current beam and new beam(s)? <Way forward > - Option 1: (Qualcomm, Apple, CMCC, Samsung, vivo) - The same periodicities of current beam and new beam(s) to define the requirements, until further RAN1 agreements on different periodicities. - According to RAN1 LS (R1-2410914) - Option 2: (Huawei) - From RAN4 perspective, the periodicity of current beam and new beam(s) can be the same or different which is up to network configurations. Send LS to RAN1 - Option 3: (vivo) - RAN4 may further discuss RRM requirements for Alt-2 and the potential restrictions to the requirement applicability if time allows, based on RAN1 further agreements.

In last RAN4#114 meeting, it is agreed that the basic measurement period are updated based on 9.5.4.1 (SSB based), 9.5.4.2 (CSI-RS based) and 9.13.4.

In RAN1 agreements in previous meetings:

RAN1#119 Agreement Regarding for the evaluation periodicity for determining Event-2 instance [at least when DRX is not configured], at least Alt-1 is supported for the single-CC beam reporting (for case that the CSI report configuration and RS for the current beam and new beam(s) are in the same CC). - Alt-1: The periodicity of the current beam RS should be the same as that of the new beam RS(s). - The evaluation periodicity is the same as the periodicity of the current and new beam RS(s). - Alt-2: The periodicity of the current beam RS can be different from that of the new beam RS(s) - Alt-2_1: The evaluation periodicity is the same as the periodicity of the current beam RS. - Alt-2_2: The evaluation periodicity is the same as periodicity of the new beam RS. - Alt-2_3: The evaluation periodicity is the same as shortest periodicity of the current beam RS and new beam RS(s). - Alt-2_4: The evaluation periodicity is the maximum of {X ms, shortest periodicity of the current beam RS and new beam RS(s)}. - Alt-2_5: The evaluation periodicity is the same as largest periodicity of the current beam RS and new beam RS(s). Note: There is the same periodicity for the new beam RS(s). FFS: Down-selection among above Alt(s) for cross-CC beam reporting. Strong concerns were expressed by Huawei and CATT that if only Alt-1 is supported in the end, the feature will not be practical. Send an LS to RAN4.

RAN1#120: Agreement On cross-CC beam report measurement for UE-initiated/event-driven beam reporting, regarding Event-2, the following is supported - The current beam RS and new beam RS(s) are in the same CC - The above does NOT imply to preclude the cross-CC TCI indication. - The CC on which the indicated TCI state is applied, and the CC in which new beam RS(s) are can be the same or different. - FFS: Whether/how to introduce an RRC parameter to indicate the CC on which the indicated TCI state is applied.

The current beam RS and new beam RS are in the same CC. This configuration plus the periodicity of current beam and new beam are with the same periodicity.

The evaluation periodicity can reuse the legacy L1-RSRP measurement period.

Proposal 1-1: The evaluation periodicity, for event 2, the periodicity of current beam and new beam are the same. the evaluation periodicity can reuse the legacy L1-RSRP measurement period.

Take SSB as example, the Tssb is the periodicity of the SSB-Index in indicated TCI state or QCLed with the indicated TCI state.

For the FFS on M in the L1_measurement_period, if look into the draft CR in RAN1:

The new clause of 5.2.1.5.4 Event-triggered reporting 5.2.1.5.4.1 Event-triggered reporting is added under 5.2.1.5 Triggering/activation of CSI Reports and CSI-RS

For the report configuration, the legacy L1-RSRP are still used as:

##### 5.2.1.4.3 L1-RSRP Reporting For L1-RSRP computation - the UE may be configured with CSI-RS resources, SS/PBCH Block resources or both CSI-RS and SS/PBCH block resources, when resource-wise quasi co-located with 'type C' and 'typeD' when applicable. - the UE may be configured with CSI-RS resource setting up to 16 CSI-RS resource sets having up to 64 resources within each set. The total number of different CSI-RS resources over all resource sets is no more than 128. … When the higher layer parameter groupBasedBeamReporting-r17in CSI-ReportConfig is configured, the UE shall indicate the CSI Resource Set associated with the largest measured value of L1-RSRP, and for each group, CRI or SSBRI of the indicated CSI Resource Set is present first. If the higher layer parameter timeRestrictionForChannelMeasurements in CSI-ReportConfig is set to "notConfigured", the UE shall derive the channel measurements for computing L1-RSRP value reported in uplink slot n based on only the SS/PBCH or NZP CSI-RS, no later than the CSI reference resource, (defined in TS 38.211[4]) associated with the CSI resource setting. If the higher layer parameter timeRestrictionForChannelMeasurements in CSI-ReportConfig is set to "Configured", the UE shall derive the channel measurements for computing L1-RSRP reported in uplink slot n based on only the most recent, no later than the CSI reference resource, occasion of SS/PBCH or NZP CSI-RS (defined in [4, TS 38.211]) associated with the CSI resource setting.

By our understanding, it only added new triggered mechanism. The reporting of legacy L1-RSRP is still applied.

In other words, in RAN4, we can reuse the same requirements of M in L1_measurement periods.

Proposal 1-2: For M in L1_measurement periods, reuse the legacy value:

SSB based: M=1 if higher layer parameter timeRestrictionForChannelMeasurement is configured, and M=3 otherwise

CSI-RS based: For periodic and semi-persistent CSI-RS resources, M=1 if higher layer parameter timeRestrictionForChannelMeasurement is configured, and M=3 otherwise; For aperiodic CSI-RS resources M=1

For P in L1_measurement periods, for event-2, due the periodicity of current beam and new beam are the same. The SMTC and Gap in the configuration of serving cell are also a single value. The sharing factor should be the same as existing requirements.

Proposal 1-3: For event-2, P in L1_measurement periods, reuse the same definition in existing requirements.

Proposal 1: For event-2, for the basic L1_evaluation_period (Mcounter =1), reuse the existing L1 measurement periods. Only Update T_RS definition and remove TReport.

Issue 1-4: Components of Event triggered measurement reporting delay? <Way forward > - Option 1: (Apple, vivo, Samsung, E///, MTK) - The event triggered measurement reporting delay is TL1-RSRP_measure_period + Tfirst UL channel - FFS on whether other components are needed. - FFS the definition of Tfirst UL channel - Option 2: (CMCC, Xiaomi) - The event triggered measurement reporting delay shall be less than Tmeasure, excluding a delay which is caused by no UL resources being available for UE to send the measurement report on and L3 filtering. - Option 3: (Nokia) - The UE shall be able to send the first PUCCH latest at the PUCCH occasion that is available after the maximum L1 event evaluation delay (TL1-RSRP_measure_period).

For the event triggered beam reporting, for both mode A and B, the evaluation of the event is for the UE if there is occasion of the first PUCCH. The PUCCH is periodically. After the L1 measurement, if the condition is met at UE side, the UE is ready to send first PUCCH. If define the Tfirst UL channel as from time point which the RS of L1-RSRP trigger L1 reporting to first PUCCH to the time point of the latest PUCCH occasion, The event triggered measurement reporting delay is TL1-RSRP_measure_period + Tfirst UL channel.

Proposal 2: The event triggered measurement reporting delay is TL1-RSRP_measure_period + Tfirst UL .

- Tfirst UL channel as from time point which the RS of L1-RSRP trigger L1 reporting to first PUCCH to the time point of the latest PUCCH occasion

Issue 1-6: Whether RAN4 to specify requirements for no time window based or cover both no time window based and with time window based with configurable M? <Way forward > - Option 1: (Qualcomm, Apple, Ericsson) - Only define requirements for the number of events is met at once - Option 2: (China Telecom, ZTE, Huawei, Samsung, MediaTek) - Specify requirements to cover both cases of no time window based and with time window based with configurable M - Option 2a: (ZTE) - Event1&Event 2: Case 2: multiple samples M and timer X should be considered - Event1&Event 7: FFS - Option 2b (Samsung) - For within a time window configured case, the L1 evaluation time is defined as multiple times of L1 evaluation time in basic to fulfil M counter is met for the event. - Option 2c (MediaTek) - For multiple events is met by counter: - Tevaluate_L1_event = TL1-RSRP_measure * C + Tuncertainty given that Tevaluate_L1_event ≤ Ttime_window - TL1-RSRP_measure: L1-RSRP measurement delay for either SSB or CSI-RS - C: event instance counter is configured by NW.

For Proposal 1 and 2, they specified the event triggered measurement reporting delay if Mcounter = 1.

RAN1 introduced a time window RAN1#117 Agreement Regarding the triggering event determination for Event 2: - If within a time window (which is configurable), the number of Event-2 instance(s) for at least one same new beam is greater than or equal to a configurable number M, UE initiated beam report occurs. - Note: Event-2 instance for a new beam is determined if the L1-RSRP of the new beam becomes a threshold value better than the current beam Above feature is subject to UE capability. - Basic feature: Once the L1-RSRP of the new beam becomes a threshold value better than the current beam, UE initiated beam report occurs.

The configurable Mcounter, can avoid the instantaneous results to guarantee the better performance with valid counter. In addition, it can avoid like ping-pong behavior if NW use this event-triggered L1-RSRP results.

In last meeting, some companies are discussed this time window are the same as TTT in existing NR therefore no need to define any requirements related to TTT.

We don’t think it is the same as TTT. TTT is a Time during which specific criteria for the event needs to be met in order to trigger a measurement report in each event. In L3 measurement, for each MO, the measurement is based on samples of SMTC. The measurement time are based on the SMTC period for both cells. During some time, UE can decide the event is met or not. In L1 measurement, the time length of the window is the configured time window length. If no definition of how to perform on each or which RS to perform the L1 measurement. The UE behavior is not clear and the performance cannot be guaranteed.

Proposal 3: RAN4 to specify requirements to cover both cases of no time window based and with time window based with configurable M.

Issue 1-7: Whether/how to specify the UE behavior if new L1-RSRP during the UL resource waiting time? <Way forward > - Option 1: (Xiaomi) - For L1 event triggered reporting, it needs to specify that L1 result which triggers L1 reporting will be included in L1 reporting. - Option 2: (LGE, Nokia) - Need to wait RAN1 progress - Option 3: (vivo) - Follow the assumption: the condition which triggers the event is remained fixed during the time after UE sends the first uplink transmission and before UE is scheduled with the second uplink transmission for the L1 reporting. FFS on other cases if time allows

Proposal 4: After UE send first PUCCH, between first PUCCH and PUSCH, there may new L1-RSRP evaluation. No need to update L1-RSRP results for report.

Issue 1-9: CSI-RS based L1-RSRP for inter-cell BM <Way forward > - Option 1: (Qualcomm, Nokia, MediaTek(descoped)) - RAN4 should not introduce in Rel-19 CSI-RS based L1-RSRP measurements for inter-cell beam management as long as RAN1 has not identified a clear use case. - Option 2: (Apple, Huawei, Ericsson) - For UE perspective, for CSI-RS based BM, there is no differentiation for intra-cell and inter-cell. CSI-RS based L1-RSRP measurement requirements applies to both intra-cell and inter-cell without differentiation. - Option 2a (Apple): The measurement delay is the same as that in serving cell in legacy L1-RSRP - Option 2b (Ericsson): reuse the principle of sharing agreed for SSB based measurement. - Option 3: (ZTE) - RAN4 needs to identify whether additional clarifications on the following points are necessary: - Known condition for the NSC, especially for the time offset assumption between serving cell and NSC; - The prerequisite on L3 measurement on NSC. - RAN4 to use principles of SSB based L1-RSRP for inter-cell CSI-RS based L1-RSRP measurements. - Option 4: (Ericsson) - RAN4 to define CSI-RS based L1-RSRP measurements requirements for inter-cell mTRP scenario and inter-cell beam management in FR2 - RAN4 to use principles of sharing agreed for SSB based L1-RSRP for inter-cell CSI-RS based L1-RSRP measurements

From the CSI config, it is the same as Rel-17 ICBM scenario. There is no explicit CSI-RS configuration from inter-cell TRP. For the UE, there is only CSI-RS configuration from the serving cell. If the inter-cell CSI-RS measurement means that the CSI-RS in the CSI config is QCLed with a SSB of inter-cell, the same requirements can be applied.

In Rel-17 requirements, there is only one section of CSI-RS based L1-RSRP measurements in 9.5.

Proposal 5: Reuse the same requirements structure as that in Rel-17 9.5. No need to add sharing factor for serving cell CSI-RS and NSC CSI-RS. From the configuration of CSI-RS perspective, there is only one configuration. If this CSI-RS is QCLed to the SSB of serving cell or SSB of additionPCI cell, they use the same set of requirements.

Issue 1-11: Whether to specify new unified TCI switching delay requirements? <Way forward > - Option 1: (Qualcomm, Apple, China Telecom, Samsung (For known conditions for TCI states), Ericsson, MediaTek) - The existing requirements for TCI state switch delay should be reused for UE-initiated / event-triggered beam management. - Option 2: (Xiaomi, Nokia) - Reduction of beam application delay may have impact on TCI state activation delay, RAN4 shall wait for more RAN1 progress. - Option 3: (LGE) - For unified TCI switching delay requirements, RAN4 needs to discuss whether active TCI state switching delay can be reduced or not - Option 4: (ZTE) - For MAC-CE based DL TCI switching delay: no need to consider TL1-RSRP and Tfirst-SSB+TSSB-proc - Option 5: (Samsung) - Not consider unknown case.

After the UE sent L1 report to gNB, gNB can use the L1 report results to indicate UE to switch to a new beam or not. Until now, there is no any difference whether the L1 reports are UE triggered or NW based. The only difference is when the target TCI state is unknown in the TCI state switching. In legacy requirements, L1-RSRP time is added in the TCI state switching delay. However, in event-triggered L1 measurement, the target TCI is always known if it remains detectable. For unknown case of the switching, it has less usage in event trigger L1 reporting.

Proposal 6: For known conditions for TCI states for TCI states switching based on UEIBM, it can use as legacy.

Proposal 7: No difference by using UE triggered L1 report or NW based L1 report. Existing unified TCI state switching delay requirements are applicable for event triggered measurement reporting based TCI state switching if known case. No unknown case is applied if the TCI states switching is based on UEIBM.

Issue 1-12: Other conditions for L1 measurement and TCI state switching? <Way forward > - Option 1: (Samsung) - The valid L1 evaluation of event-triggered L1-RSRP should be after the MAC-CE command or DCI.

For event-2/event-1, the measurement is based on current beam. The RS of current beam is coming from indicated TCI state. If UE received TCI state switching based on MAC-CE or DCI, it means the indicated TCI state has been changed. The measurement of L1 evaluation time should be changed after the MAC-CE command or DCI indicator. If the event has been triggered, the L1 evaluation time should be restart.

Proposal 8: The valid L1 evaluation of event-triggered L1-RSRP should be after the MAC-CE command or DCI. If the event has been triggered, the L1 evaluation time should be restart.

Issue 1-14: Capabilities for Support of Event Triggering and Reporting Criteria? <Way forward > - Option 1: (Apple, Ericsson) - Defined by RAN1. To be discussed later if any RAN4 specific UE capabilities need to be added. - Option 2: (China Telecom) - Align the agreement on whether to define the capability for support of L1 event triggering and reporting in TS 38.133 for R19 MIMO WI and R19 mobility WI. - Option 3: (CMCC, MediaTek) - Define capabilities similar as legacy L3 (like legacy L3 in section 9.1.4). Issue 1-15: Compatibility of capabilities with LTM <Way forward > - Option 1: (Apple, Ericsson) - Discuss it later. - Option 2: (CMCC, Huawei, Nokia, Samsung) - Do not mix/combine Rel-19 mobility and Rel-19 MIMO. Define the capability separately. - Option 4: (MediaTek) - Define the single capability for total number of L1 events shared by mobility L1 events and MIMO L1 events on intra-frequency.

In legacy L3 measurement in section 9.1.4, the UE shall be able to support in parallel per category up to Ecat reporting criteria according. Such as

is the total number of NR reporting criteria according to table 9.1.4.2-1, and n is the number of configured NR serving frequencies, including PCell, and SCells carrier frequencies,

Unlike L3 measurement, there is no different types of events. In L3, the different types of measurement can be measured and the serving cell and neighbour cells can measure at the same MO, the decided by software for different events whether they are met or not.

In L1 measurement, the measurement type is only based on RSRP.

For different events, they are configured from the CSI config. It will not be support to measure and report the different events at the same reporting. There is no need to define such capability for support of L1 event trigger and reporting as L3.

Proposal 9: For event trigger L1 reporting, do not define such capability in parallel per category like L3.

Spec structure of Rel-19 MIMO phase 5

In this contribution, we also provide the spec impact of core requirements of the Rel-19 MIMO Phase 5, including both part 1 and part 2.

In the objectives of the WID, RAN4 has already agreed:

Obj#1: UEIBM: new RRM requirements.

Obj#2: CSI support for up to 128 CSI-RS ports: No RRM impacts.

Obj#3: UE reporting enhancement for CJT deployments: FFS on RRM core requirements.

Obj#4: 3Tx: No RRM impacts.

Obj#5: Enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios: new RRM requirements.

Based on discussions and agreements, we provided the suggested spec structure of core requirements of Rel-19 MIMO phase 5 in below table.

Proposal 10: Agree on the spec structure of core requirements for Rel-19 MIMO phase 5 as below table:

| Objective | Clauses | Description |
| --- | --- | --- |
| UEIBM | 9.x1 event trigger Reporting based on L1-RSRP measurements 9.x1.1 Introduction 9.x1.2 Requirements applicability 9.x1.3 Measurement Reporting Requirements 9.x1.4 L1-RSRP measurement requirements 9.x1.4.1 SSB based L1-RSRP Reporting 9.x1.4.2 CSI-RS based L1-RSRP Reporting 9.x1.5 Measurement restriction for CSI-RS and SSB for L1-RSRP measurement 9.x1.5.1 Measurement restriction for SSB based L1-RSRP 9.x1.5.2 Measurement restriction for CSI-RS based L1-RSRP 9.x1.6 Scheduling availability of UE during L1-RSRP measurement | New clauses to specify the event trigger reporting based on L1-RSRP measurement |
|  | 9.x2 event trigger Reporting based on L1-RSRP measurements for a cell with different PCI from serving cell 9.x2.1 Introduction 9.x2.2 Requirements applicability 9.x2.3 Measurement Reporting Requirements 9.x2.4 L1-RSRP measurement requirements 9.x2.4.1 SSB based L1-RSRP Reporting 9.x2.5 Measurement restriction for SSB for L1-RSRP measurement 9.x2.5.1 Measurement restriction for SSB based L1-RSRP 9.x2.6 Scheduling availability of UE during L1-RSRP measurement | New clauses to specify the event trigger reporting based on L1-RSRP measurement for a cell with different PCI from serving cell |
|  | 8.15 Active downlink TCI state switching delay for unified TCI | Depends on the open issue: whether new TCI state switching or if existing requirements can be applied, FFS on any clarification |
|  | 8.16 Active uplink TCI state switching delay for unified TCI | Depends on the open issue: whether new TCI state switching or if existing requirements can be applied, FFS on any clarification |
| CJTC reporting | [9.x Inter-TRP time misalignment measurement and reporting 9.x.1 Introduction 9.x.2 Requirements applicability 9.x.3 Measurement Reporting Requirements 9.x.4 Inter-TRP time misalignment measurement requirements 9.x.4.1 CSI-RS based Inter-TRP time misalignment Reporting 9.x.5 Measurement restriction for CSI-RS Inter-TRP time misalignment 9.x.6 Scheduling availability of UE during Inter-TRP time misalignment] | New clauses Whether to add core requirements for CJTC reporting for ‘CJTC-Dd’, depends on the conclusion of open issues. |
|  | [9.x Inter-TRP frequency offset measurement and reporting 9.x.1 Introduction 9.x.2 Requirements applicability 9.x.3 Measurement Reporting Requirements 9.x.4 Inter-TRP frequency offset measurement requirements 9.x.4.1 CSI-RS based Inter-TRP frequency offset Reporting 9.x.5 Measurement restriction for CSI-RS Inter-TRP frequency offset 9.x.6 Scheduling availability of UE during Inter-TRP frequency offset measurement] | New clauses Whether to add core requirements for CJTC reporting for ‘CJTC-F’, depends on the conclusion of open issues. |
|  | [9.x Inter-TRP phase offset measurement and reporting 9.x.1 Introduction 9.x.2 Requirements applicability 9.x.3 Measurement Reporting Requirements 9.x.4 Inter-TRP phase offset measurement requirements 9.x.4.1 CSI-RS based Inter-TRP phase offset Reporting 9.x.5 Measurement restriction for CSI-RS Inter-TRP phase offset 9.x.6 Scheduling availability of UE during Inter-TRP phase offset measurement] | New clauses Whether to add core requirements for CJTC reporting for ‘CJTC-P’, depends on the conclusion of open issues. |
| Asymmetric DL sTRP/UL mTRP deployment | 7 Timing 7.1 UE transmit timing 7.1.1 Introduction 7.1.2 Requirements 7.1.2.1 Gradual timing adjustment | Add the timing requirements for two TA extension. |
|  | 7.5 Maximum Transmission Timing Difference 7.5.1 Introduction 7.5.7 Minimum requirements for multi-TRP 7.6 Maximum Receive Timing Difference 7.6.1 Introduction 7.6.8 Minimum requirements for Multi-TRPs | For two TA extension, update the condition and capability, some parts depend on further RAN1 progress |
|  | 8.16 Active uplink TCI state switching delay for unified TCI 8.16.1 Introduction 8.21 Active downlink TCI state switching delay for unified TCI for single-DCI mTRP 8.21.1 Introduction | Whether to add new TCI state switching delay for unified TCI and single-DCI mTRP, depends on the decision of open issue. Even no new TCI state switching delay requirements, FFS on whether to add the applicability rule in existing requirements. |

- Conclusion

In this contribution, we provide our proposals for RRM requirements for UE-initiated/event-driven beam management in Rel-19 MIMO phase 5 and the spec structure of this WI:

Proposal 1-1: The evaluation periodicity, for event 2, the periodicity of current beam and new beam are the same. the evaluation periodicity can reuse the legacy L1-RSRP measurement period.

Take SSB as example, the Tssb is the periodicity of the SSB-Index in indicated TCI state or QCLed with the indicated TCI state.

Proposal 1-2: For M in L1_measurement periods, reuse the legacy value:

SSB based: M=1 if higher layer parameter timeRestrictionForChannelMeasurement is configured, and M=3 otherwise

CSI-RS based: For periodic and semi-persistent CSI-RS resources, M=1 if higher layer parameter timeRestrictionForChannelMeasurement is configured, and M=3 otherwise; For aperiodic CSI-RS resources M=1

Proposal 1-3: For event-2, P in L1_measurement periods, reuse the same definition in existing requirements.

Proposal 1: For event-2, for the basic L1_evaluation_period (Mcounter =1), reuse the existing L1 measurement periods. Only Update T_RS definition and remove TReport.

Proposal 2: The event triggered measurement reporting delay is TL1-RSRP_measure_period + Tfirst UL .

- Tfirst UL channel as from time point which the RS of L1-RSRP trigger L1 reporting to first PUCCH to the time point of the latest PUCCH occasion

Proposal 3: RAN4 to specify requirements to cover both cases of no time window based and with time window based with configurable M.

Proposal 4: After UE send first PUCCH, between first PUCCH and PUSCH, there may new L1-RSRP evaluation. No need to update L1-RSRP results for report.

Proposal 5: Reuse the same requirements structure as that in Rel-17 9.5. No need to add sharing factor for serving cell CSI-RS and NSC CSI-RS. From the configuration of CSI-RS perspective, there is only one configuration. If this CSI-RS is QCLed to the SSB of serving cell or SSB of additionPCI cell, they use the same set of requirements.

Proposal 6: For known conditions for TCI states for TCI states switching based on UEIBM, it can use as legacy.

Proposal 7: No difference by using UE triggered L1 report or NW based L1 report. Existing unified TCI state switching delay requirements are applicable for event triggered measurement reporting based TCI state switching if known case. No unknown case is applied if the TCI states switching is based on UEIBM.

Proposal 8: The valid L1 evaluation of event-triggered L1-RSRP should be after the MAC-CE command or DCI. If the event has been triggered, the L1 evaluation time should be restart.

Proposal 9: For event trigger L1 reporting, do not define such capability in parallel per category like L3.

Proposal 10: Agree on the spec structure of core requirements for Rel-19 MIMO phase 5 as below table:

| Objective | Clauses | Description |
| --- | --- | --- |
| UEIBM | 9.x1 event trigger Reporting based on L1-RSRP measurements 9.x1.1 Introduction 9.x1.2 Requirements applicability 9.x1.3 Measurement Reporting Requirements 9.x1.4 L1-RSRP measurement requirements 9.x1.4.1 SSB based L1-RSRP Reporting 9.x1.4.2 CSI-RS based L1-RSRP Reporting 9.x1.5 Measurement restriction for CSI-RS and SSB for L1-RSRP measurement 9.x1.5.1 Measurement restriction for SSB based L1-RSRP 9.x1.5.2 Measurement restriction for CSI-RS based L1-RSRP 9.x1.6 Scheduling availability of UE during L1-RSRP measurement | New clauses to specify the event trigger reporting based on L1-RSRP measurement |
|  | 9.x2 event trigger Reporting based on L1-RSRP measurements for a cell with different PCI from serving cell 9.x2.1 Introduction 9.x2.2 Requirements applicability 9.x2.3 Measurement Reporting Requirements 9.x2.4 L1-RSRP measurement requirements 9.x2.4.1 SSB based L1-RSRP Reporting 9.x2.5 Measurement restriction for SSB for L1-RSRP measurement 9.x2.5.1 Measurement restriction for SSB based L1-RSRP 9.x2.6 Scheduling availability of UE during L1-RSRP measurement | New clauses to specify the event trigger reporting based on L1-RSRP measurement for a cell with different PCI from serving cell |
|  | 8.15 Active downlink TCI state switching delay for unified TCI | Depends on the open issue: whether new TCI state switching or if existing requirements can be applied, FFS on any clarification |
|  | 8.16 Active uplink TCI state switching delay for unified TCI | Depends on the open issue: whether new TCI state switching or if existing requirements can be applied, FFS on any clarification |
| CJTC reporting | [9.x Inter-TRP time misalignment measurement and reporting 9.x.1 Introduction 9.x.2 Requirements applicability 9.x.3 Measurement Reporting Requirements 9.x.4 Inter-TRP time misalignment measurement requirements 9.x.4.1 CSI-RS based Inter-TRP time misalignment Reporting 9.x.5 Measurement restriction for CSI-RS Inter-TRP time misalignment 9.x.6 Scheduling availability of UE during Inter-TRP time misalignment] | New clauses Whether to add core requirements for CJTC reporting for ‘CJTC-Dd’, depends on the conclusion of open issues. |
|  | [9.x Inter-TRP frequency offset measurement and reporting 9.x.1 Introduction 9.x.2 Requirements applicability 9.x.3 Measurement Reporting Requirements 9.x.4 Inter-TRP frequency offset measurement requirements 9.x.4.1 CSI-RS based Inter-TRP frequency offset Reporting 9.x.5 Measurement restriction for CSI-RS Inter-TRP frequency offset 9.x.6 Scheduling availability of UE during Inter-TRP frequency offset measurement] | New clauses Whether to add core requirements for CJTC reporting for ‘CJTC-F’, depends on the conclusion of open issues. |
|  | [9.x Inter-TRP phase offset measurement and reporting 9.x.1 Introduction 9.x.2 Requirements applicability 9.x.3 Measurement Reporting Requirements 9.x.4 Inter-TRP phase offset measurement requirements 9.x.4.1 CSI-RS based Inter-TRP phase offset Reporting 9.x.5 Measurement restriction for CSI-RS Inter-TRP phase offset 9.x.6 Scheduling availability of UE during Inter-TRP phase offset measurement] | New clauses Whether to add core requirements for CJTC reporting for ‘CJTC-P’, depends on the conclusion of open issues. |
| Asymmetric DL sTRP/UL mTRP deployment | 7 Timing 7.1 UE transmit timing 7.1.1 Introduction 7.1.2 Requirements 7.1.2.1 Gradual timing adjustment | Add the timing requirements for two TA extension. |
|  | 7.5 Maximum Transmission Timing Difference 7.5.1 Introduction 7.5.7 Minimum requirements for multi-TRP 7.6 Maximum Receive Timing Difference 7.6.1 Introduction 7.6.8 Minimum requirements for Multi-TRPs | For two TA extension, update the condition and capability, some parts depend on further RAN1 progress |
|  | 8.16 Active uplink TCI state switching delay for unified TCI 8.16.1 Introduction 8.21 Active downlink TCI state switching delay for unified TCI for single-DCI mTRP 8.21.1 Introduction | Whether to add new TCI state switching delay for unified TCI and single-DCI mTRP, depends on the decision of open issue. Even no new TCI state switching delay requirements, FFS on whether to add the applicability rule in existing requirements. |

- Reference

[1] R4-2502595, WF on RRM requirements for NR_MIMO_Ph5_Part1, Samsung

[2] RP-242394, WID revision: NR MIMO Phase 5, Samsung