3GPP TSG-RAN WG4 Meeting#114bis		R4-2503671

Wuhan, China, Apr 7th -11th, 2025

Agenda item:	7.24.2.2

Source: 	Spreadtrum, UNISOC

Title: 	Discussion on RF requirements for ambient IoT device 1

Document for:	Discussion

Introduction

In RAN4#114 meeting, a WF [1] was agreed. This contribution continues to discuss remaining issues as follows.

- Topic 3-2: Tx Issue 3-2-1: Transmit output power Agreement: - Measurement configuration including input CW power and potential other conditions need to be defined. Issue 3-2-2: Modulation quality requirements Agreement: - Further discuss whether and how to define the modulation signal quality requirements Issue 3-2-3: Occupied bandwidth Agreement: - Further discuss whether to define the occupied bandwidth for device 1. Issue 3-2-4: Emission requirements Agreement: - Consider one composite emission requirements covering the emission frequency range of SEM and spurious emission as starting point. - Further discuss whether to define TRP or EIRP requirement for unwanted emission. Issue 3-2-5: Other requirements Agreement: - Do not define output power dynamics and intermodulation requirements for device 1. Issue 3-2-6: Other requirements (frequency error) Agreement: - Do not define frequency error requirements. - Small frequency shift error will be discussed in modulation quality requirements. Issue 3-2-7: Other requirements (Transmit ON/OFF time mask) Agreement: - Do not define transmit ON/OFF time mask for device 1 - Topic 3-3: Rx Issue 3-3-1: Reference sensitivity Agreement: - For R2D reference sensitivity, consider following options: - Option 1: define reference sensitivity using legacy approach, e.g. PREFSENSdBm=-174dBm+10×log10BW+NF+IM+SNR - Option 2: define reference sensitivity based on link budget and/or physical limits of the receiver circuitry. - Other options are not precluded. Issue 3-3-2: Maximum input power Agreement: - Further discuss the maximum input power for A-IOT device for R2D signal. Issue 3-3-3: Interference rejection Agreement: - Do not define interference rejection requirements.

Discussion

TX requirements

- Transmitter output power

Backscattered modulated power

In TR, for device1, D2R backscattering is transmitted on a carrier wave provided externally, backscattering power and power backscattering are potentially need to be defined to ensure the coverage range between reader and device. The maximum backscattered modulated power can be specified for device1. The backscatter power is the amount of power reflected back from the device when a successful command sequence is completed at the device at a specific sensitivity level. CW is outside topology, hence, the maximum backscatter modulated power is to ensure the max coverage range between reader and device. In the SI stage, the coverage target is 10m. According to the RAN1’s simulation conclusion about coverage, the results of D2R coverage basically are more than 10m. In this case, we can assume the minimum coverage between device and reader is 10m. From device to reader, including three parts of loss or gain, they are respectively freedom pathloss, shadow fading and antenna gain. The formula of freedom pathloss is L=33.63+21.9log10d3D+20log10fc, shadow fading is 1.346σSF=1.346*4=5.384, the antenna gain of device1 is 0dBi, the antenna gain of A-IoT BS is 6dBi, REFSENS of medium range BS (A-IoT BS) is -101.8dBm, the backscatter loss of OOK is 8~10dB for practically. Through the above calculation, we can get that the minimum backscatter is -37.796dBm. Taking into account a 3~4dB margin, we think the minimum backscatter modulated power is -34dBm could be a starting point. In order to achieve this power, the distance between CW and device needs to be defined.

Proposal 1: The maximum target distance for D2R between device and reader needs to be considered (i.e., 10m).

Proposal 2: The maximum backscatter modulated power needs to be defined. The distance between CW and device needs to be defined. -34dBm for the minimum backscatter modulated power could be a starting point.

Backscattering loss

The backscattering loss is the incident signal minus the backscattered signal. It depends on a mismatch between antenna impedance and load impedance. In our understanding, it belongs to an intermediate process of transmitting output power, as long as the power emitted by the device meets the requirements, no matter what the backscattering loss is. Hence, there is no need to define backscatter loss.

Proposal 3: There is no need to define backscattering loss.

- Modulation quality requirements

Transmitted signal quality includes frequency error, carrier leakage, in-band emission and EVM. In the SI stage, frequency error, carrier leakage and in-band emission don’t need to be defined. However, some companies think that modulation quality requirements are potentially needed. In our understanding, device1 is a low-cost and simple device. Hence, for the device1, it is only necessary to transmit power and meet the requirements. As for the modulation quality, the reader should guarantee it. In addition, small frequency shift error will be discussed in modulation quality requirements. In our understanding, the small frequency shift error comes from sample frequency offset, and sample frequency offset (SFO) is related to channel bandwidth and performance. Therefore, SFO needs to be defined to guarantee the performance.

Proposal 4: There is no need to define the modulation quality requirements and small frequency shift error. SFO needs to be defined (i.e., 105ppm).

- Occupied bandwidth

Occupied bandwidth is defined as the bandwidth containing 99% of the total integrated mean power of the transmitted spectrum on the assigned channel, which directly affects spectrum utilization efficiency and channel interference. Even though it is not continuous spectrum for OOK modulation, we need to define channel bandwidth to avoid interference with other channels because D2R may support FDMA. The specific value is dependent on the channel bandwidth of D2R, which has not yet been finalized at this stage.

Proposal 5: Occupied bandwidth needs to be defined. The specific value is dependent on the channel bandwidth of D2R.

- Emission requirements

One composite emission requirement covers the emission frequency range of SEM and spurious emissions. In FR2, all out of band emissions for frequency range 2 are TRP. In the actual test, TRP and EIRP are needed to scan 3D and they are basically measured at the same time. TRP can also reflect the efficiency of the antenna. Hence, we can define TRP requirements for unwanted emissions.

Proposal 6: Define TRP requirements for unwanted emissions.

RX requirements

- Reference sensitivity

The reference sensitivity power level REFSENS is the minimum mean power applied to each one of the UE antenna ports for all UE categories, at which the throughput shall meet or exceed the requirements for the specified reference measurement channel. PRDCH includes control information and data, BLRE is the performance metric for RAN1’s link budget simulation. Hence, throughput can be a performance metric for REFSENS. We can define throughput ≥ 90% as the performance metric.

Proposal 7: Throughput (i.e., ≥ 90%) can be performance metric for REFSENS.

As to defining reference sensitivity approach, in the last meeting, there were two options. Option1: using legacy approach, e.g. PREFSENSdBm=-174dBm+10×log10BW+NF+IM+SNR. Option2: based on link budget and/or physical limits of the receiver circuitry.According to the co-existence assumptions about device1 of reference sensitivity which is -36dBm. -36dBm=-174+10log10(10MHz)+24+IM+SNR. SNR=40.5dB. Obviously, we cannot get such a high value. Hence, option1 is not appropriate. We can choose option2, based on link budget to define reference sensitivity.

Proposal 8: Support option 2, based on link budget to define reference sensitivity.

As we all known, reference sensitivity is strongly related to the coverage of R2D. According to the co-existence assumptions about device1 of reference sensitivity which is -36dBm. However, reference sensitivity may not be so good for some companies to give feedback, such as -30dBm. Of course, it depends on the actual situation. In addition, if the spectrum for R2D is DL spectrum, the transmitter of A-IoT BS could be 38dBm. In our understanding, whatever the spectrum is used to for R2D, they all need to meet the minimum coverage of R2D (i.e., 10m). Similarly to D2R, we can assume the minimum coverage between device and reader is 10m. From reader to device, including three parts of loss or gain, they are respectively freedom pathloss, shadow fading and antenna gain. The formula of freedom pathloss is L=33.63+21.9log10d3D+20log10fc, shadow fading is 1.346σSF=1.346*4=5.384, the antenna gain of device1 is 0dBi, the antenna gain of A-IoT BS is 6dBi.Through the above calculation, we can get that the reference sensitivity is -31dBm. Similarly, we need to take into account a 3~4dB margin. Hence, -31dBm plus 3dB is -28dBm. Based on all the above analysis, the reference sensitivity could be -28dBm as a starting point.

Proposal 9: The max target distance between Reader and device for R2D should be considered (i.e., 10m). -28dBm for the reference sensitivity could be a starting point.

- Maximum input power

According to the conclusion in TR about maximum input power, the maximum input power needs to be discussed in WI stage. Maximum input power is defined as the maximum mean power received at the UE antenna port. The purpose of the definition is to prevent the whole link from being saturated, thus causing the deterioration of SNR and performance. According to the investigation for envelop detector, the dynamic range of envelop detector is about 45dB, and the reference sensitivity of device 1 is -36dBm. The transmission power of A-IoT BS is 33dBm, assumed for co-existence study. Which means that it may exceed the dynamic range of envelop detector. Hence, it is needed to define the maximum input power for device 1.

Proposal 10: In order to avoid envelop detector being saturated, maximum input power is needed to define. The specific value needs to be further discussed.

Conclusions

In this paper, we provide our views on RF requirements for ambient IoT device, we have made the following proposals.

Proposal 1: The maximum target distance for D2R between device and reader needs to be considered (i.e., 10m).

Proposal 2: The maximum backscatter modulated power needs to be defined. The distance between CW and device needs to be defined. -34dBm for the minimum backscatter modulated power could be a starting point.

Proposal 3: There is no need to define backscattering loss.

Proposal 4: There is no need to define the modulation quality requirements and small frequency shift error. SFO needs to be defined (i.e., 105ppm).

Proposal 5: Occupied bandwidth needs to be defined. The specific value is dependent on the channel bandwidth of D2R.

Proposal 6: Define TRP requirements for unwanted emissions.

Proposal 7: Throughput (i.e., ≥ 90%) can be performance metric for REFSENS.

Proposal 8: Support option 2, based on link budget to define reference sensitivity.

Proposal 9: The max target distance between Reader and device for R2D should be considered (i.e., 10m). -28dBm for the reference sensitivity could be a starting point.

Proposal 10: In order to avoid envelop detector being saturated, maximum input power is needed to define. The specific value needs to be further discussed.

Reference

- RP-2502858, WF on requirements for system parameters and A-IoT device, CMCC, TSG RAN4 meeting #114