# 3GPP TSG-RAN WG4 Meeting # 114bis	R4-2503694

# Wuhan, China, 7th – 11st April, 2025

Source: 	Huawei, HiSilicon

Title: 	Discussion on Rel-19 ATG UE RF with DL inter-band CA_n3-n39

Agenda Item:	7.12.1.1

Document for:	Approval

Introduction

In the RAN4#110bis meeting, RAN4 started to discuss the Rel-19 ATG enhancement. One high level way forward [1] were approved. In our contribution [2], we shared some initial considerations for this topic.

In the RAN4#112 meeting, RAN4 reached a lot of consensus on this topic below, which are captured into the WF [3].

In the RAN4#112bis meeting, RAN4 reached some consensus in the approved WF [4].

In the RAN4#113 meeting, RAN4 updated the MSD test parameters for CA_n3A-n39A with n39 UL based on the approved WF [5].

Issue 3-3: MSD for CA_n3A-n39A with n39 UL Agreement: - Define the MSD value with 40dBm output power - FFS on how to scale MSD for other output power level. - Following the parameters in the table below. UL band; DL band; UL Fc; UL BW; SCS of UL band; UL RB Allocation; DL Fc; DL BW; MSD; Cross-band Interference source. (MHz); (MHz); (kHz); LCRB; (MHz); (MHz); (dB). n39; n3; 1900; 30; 15; 160 (RBstart=0); 1817.5; 25; [TBD]; ACLR2.

In this paper, we’d like to our views on how to scale MSD for different output power level for DL CA_n3-n39 of ATG UE.

Discussion

We’d like to provide two exemplary RF architectures below to allow the transition guard band between DL band n3 and band n39.

figure 1 Common antenna RF architecture for DL CA_n3-n39 with UL in either band n3 or n39

Figure 2 separate antenna RF architecture for DL CA_n3-n39 with UL in either band n3 or n39

For the RF architectures in figures, only partial frequency range can be implemented for the filter(s) in DL band n3 or band n39 or both DL band n3 and n39.

Based on the way forward [3], companies are encouraged to provide the inputs for the frequency gap in the filters.

In China, the spectrum allocation in band n3 and n39 is shown below.

table 1 Band 3 UL spectrum allocation in China

| Band 3 UL |  |  |
| --- | --- | --- |
| CMCC | CU | CTC |
| 1710~1735MHz | 1735~1765MHz | 1765~1785MHz |

table 2 Band 3 DL and band 39 UL/DL spectrum allocation in China

| Band 3 DL |  |  | Band 39 UL/DL |  |
| --- | --- | --- | --- | --- |
| CMCC | CU | CTC | No allocation | CMCC |
| 1805~1830MHz | 1830~1860MHz | 1860~1880MHz | 1880~1885MHz | 1885~1915MHz |

Observation 1: 5MHz frequency gap was reserved for the band 3 and 39 spectrum allocation in China.

Observation 2: From ATG operator’s perspective, the frequency separation between band 3 DL and band 39 UL is about 55MHz.

Thus, we checked the MSD based on 30MHz aggressor located in 1885~1915MHz and 25MHz victim located in 1805~1830MHz, which is the MSD case due to ACLR2 cross band isolation. The evaluation is shown in the table 3. The antenna isolation is assumed as 20dB. The rejection for band n3 Rx filter at 1885~1915MHz is assumed as 35dB and n39 Tx filter rejection at 1805~1830MHz is assumed as 50dB based on the way forward [6] in last meeting. Since there is no volume limitation and power consumption limitation in the airplane, the assumed performance for these parameters are better than the handheld UE.

Issue 2-1: Assumed parameters when supporting n39 UL for CA_n3A-n39A with common antenna. Agreement: - Companies are encouraged to provide MSD values based on the following parameters. Parameters; Values. Antenna Isolation; 20dB. The rejection for band n3 Rx filter at 1885~1915MHz; [35dB]. n39 Tx filter rejection at 1805~1830MHz; [50dB]. - Above parameters are only applicable for ATG UE with dedicated filter design

The MSD values for different output power are summarized below.

table 4 MSD values of DL band n3 with band n39 UL interference for different output powers

| Output power | MSD (Based on Antenna Architecture with common antenna) |
| --- | --- |
| 23dBm | 3.4 |
| 24dBm | 3.57 |
| 25dBm | 3.74 |
| 26dBm | 3.94 |
| 27dBm | 4.16 |
| 28dBm | 4.42 |
| 29dBm | 4.72 |
| 30dBm | 5.07 |
| 31dBm | 5.49 |
| 32dBm | 5.96 |
| 33dBm | 6.51 |
| 34dBm | 7.13 |
| 35dBm | 7.84 |
| 36dBm | 8.63 |
| 37dBm | 9.50 |
| 38dBm | 10.46 |
| 39dBm | 11.50 |
| 40 dBm | 12.63 |

Proposal 1: For ATG UE, RAN4 can specify the following MSD requirements for DL band n3 with band n39 UL interreference under different output powers considering operator’s spectrum allocation and 5MHz frequency gap in 1880~1885MHz.

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n39 | n3 | 1900 | 30 | 15 | 160 (RBstart=0) | 1817.5 | 25 | MSD Table 4 | ACLR2 and >ACLR2 |

In addition, RAN4 had a way forward on studying the MSD value of band n39 DL with band n3 UL aggressor.

Issue 2-3: Assumed parameters for CA_n3A-n39A with n3 UL WF: - Use the following parameters in the approved Tdoc R4-2410641 with 20dB antenna isolation as baseline to analysis the MSD value for n39 DL. - Other parameters are not precluded. Parameters; Value. Antenna isolation; [20dB]. Tx attenuation at 1880~1920MHz for band n3 duplexer; [30dB]. Rx attenuation at 1710~1785MHz for band n3+n39 Rx filter; [40dB].

The MSD values for different output power are summarized below.

table 5 MSD values of DL band n39 with band n3 UL interference for different output powers

| Output power | MSD (Based on Antenna Architecture with common antenna) |
| --- | --- |
| 23dBm | 0.16 |
| 24dBm | 0.19 |
| 25dBm | 0.24 |
| 26dBm | 0.29 |
| 27dBm | 0.36 |
| 28dBm | 0.44 |
| 29dBm | 0.55 |
| 30dBm | 0.68 |
| 31dBm | 0.84 |
| 32dBm | 1.04 |
| 33dBm | 1.28 |
| 34dBm | 1.58 |
| 35dBm | 1.94 |
| 36dBm | 2.36 |
| 37dBm | 2.87 |
| 38dBm | 3.46 |
| 39dBm | 4.16 |
| 40 dBm | 4.95 |

Proposal 2: For ATG UE, RAN4 can specify the following MSD requirements for DL band n39 with band n3 UL interreference

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n3 | n39 | 1722.5 | 25 | 15 | 133 (RBstart=0) | 1900 | 30 | MSD Table 5 | >ACLR2 |

Summary

Observation 1: 5MHz frequency gap was reserved for the band 3 and 39 spectrum allocation in China.

Observation 2: From ATG operator’s perspective, the frequency separation between band 3 DL and band 39 UL is about 55MHz.

Proposal 1: For ATG UE, RAN4 can specify the following MSD requirements for DL band n3 with band n39 UL interreference under different output powers considering operator’s spectrum allocation and 5MHz frequency gap in 1880~1885MHz.

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n39 | n3 | 1900 | 30 | 15 | 160 (RBstart=0) | 1817.5 | 25 | MSD Table 4 | ACLR2 and >ACLR2 |

Proposal 2: For ATG UE, RAN4 can specify the following MSD requirements for DL band n39 with band n3 UL interreference

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n3 | n39 | 1722.5 | 25 | 15 | 133 (RBstart=0) | 1900 | 30 | MSD Table 5 | >ACLR2 |

- References

[1] R4-2406594	WF on Rel-19 ATG UE requirements, CMCC

[2] R4-2405344	General discussion on Rel-19 ATG UE RF, Huawei, HiSilicon

[3] R4-2414322 WF on Rel-19 ATG UE RF requirements, CMCC

[4] R4-2417121	WF on UE RF for ATG enhancement, CMCC

[5] R4-2420338 WF on NR ATG UE RF, CMCC

[6] R4-2502952, WF on UE RF requirements for ATG, CMCC