3GPP TSG-RAN WG4 Meeting #114bis                                     	            R4-2503715

Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda Item:   	7.30.2

Source:   	LG Electronics

Title: 	Discussion on feasibility of TDM DMRS for 3.75kHz SCS OCC

Document for:   	Discussion

- Introduction

In this contribution, we will provide our view on the feasibility of TDM DMRS pattern for 3.75kHz SCS OCC for NPUSCH format 1, which is a question in RAN1 LS [1].

- Discussion

The following question regarding the support of TDM DMRS pattern was asked for RAN4 to provide an answer in LS [1]:

RAN1 has made the following working assumption on the DMRS pattern for OCC for 3.75kHz SCS OCC for NPUSCH format 1: For 3.75kHz SCS OCC for NPUSCH format 1, support TDM DMRS over 4 slots where DMRS are transmitted in the first 2 slots and DMRS REs are blanked in the next 2 slots, or vice-versa, where the DMRS REs are as in legacy NB-IoT and the guard period within the slot is as in legacy NB-IoT.. - Question: From a RAN4 perspective, is it feasible to introduce support of TDM DMRS, as per the description above, in Rel-19?

In the case when an IoT device perform NPUSCH transmission using TDM DMRS pattern, we believe that it is problematic to simply assume that blanked DMRS symbol(s) do not affect its phase continuity maintenance. To be specific, for example, according to the current specification, even for an NR UE not indicating the capability of dmrs-BundlingNonBackToBackTX, a non-zero symbol gap shall not be scheduled between any two consecutive PUSCH transmissions in order to maintain the phase continuity across PUSCH transmissions within the nominal TDW. Additionally, for IoT devices, one of the important operations is to extend the lifespan by minimizing the power consumption, and this can be performed based on UE implementation in the duration of blanked DMRS symbol(s), which could be another cause of phase continuity error. Considering these factors mentioned above and the limited hardware capabilities of IoT devices, it would be reasonable to conclude that blanked DMRS symbol(s) may cause issues in maintaining the phase continuity. Figure 1 shows the simulation result showing the performance degradation when the phase continuity error due to blanked DMRS symbol(s) occurs randomly within the range of -π/8 to π/8.

Figure 1

Proposal: RAN4 provides an answer to the question on RAN1 LS [1] as follows:

- When applying TDM DMRS pattern for 3.75kHz SCS OCC for NPUSCH format 1, blanked DMRS symbol(s) may cause issues in maintaining the phase continuity.

- Conclusion

In this contribution, we discussed how RAN4 provides feedback on RAN1’s question in LS [1]. The following proposal is given.

Proposal: RAN4 provides an answer to the question on RAN1 LS [1] as follows:

- When applying TDM DMRS pattern for 3.75kHz SCS OCC for NPUSCH format 1, blanked DMRS symbol(s) may cause issues in maintaining the phase continuity.

- Reference

[1] R4-2503109, “LS on TDM DMRS for 3.75kHz SCS OCC,” RAN1.