3GPP TSG-RAN WG4 Meeting#114bis		R4-2503732

Wuhan, China, Apr 7th -11th, 2025

Agenda item:	7.29.2.1

Source: 	Spreadtrum, UNISOC

Title: 	Discussion on NTN (e)Redcap UE RF requirements

Document for:	Discussion

Introduction

In RAN4#114 meeting, a WF [1] was approved, in which there were remaining open issues as listed. This contribution continues to be discussed as follows.

Discussion

PC3 output power for (e)Redcap

Agreement: - Rel-18 power boosting is applicable for NR NTN including (e)RedCap - No reference sensitivity specification impact (e.g. RSD) from power boosting. - No changes to the PC3 tolerance for HD-FDD and FD-FDD compared non-RedCap UE NR NTN - FFS on solutions on additional new output power enhancement - supported modulation orders - potential downselection between HD-FDD and FD-FDD - simultaneous support with rel-18 power boosting - Background: output power enhancement (e.g., FE IL improvement) - Other solutions not precluded Note: No change for (e)RedCap MOP tolerance to enhance the output power does not preclude any solution in other Rel-19 WI.

According to the last meeting agreement, R18 power boosting is applicable for NR NTN including (e) Redcap, which is applicable to HD-FDD and FD-FDD. Compared to FD-FDD, HD-FDD with insertion loss reduction of TX filter can increase the output power. Regarding this additional new output power enhancement, which can be a new capability (i.e., R19 power boosting. In our understanding, this value has nothing to do with the modulation order theoretically. This specific power boosting could be 0.5dB or other values.

In addition, if the UE supports R18 power boosting(1dB for PC2 and 0.5dB for PC2) and supports this new output power enhancement (i.e., 0.5dB) at the same time, the output power of UE will be increased by 1.5~2dB, which may need additional ACLR/new emission requirements. Additional spec work is needed.

Proposal 1: Additional new output power enhancement such as R19 power boosting can only be applied to HD-FDD (e)Redcap NTN. Which can be supported for all modulation orders.

Proposal 2: This new output power enhancement is no simultaneous support with R18 power boosting.

Conclusions

In this paper, we provide our views on on NTN (e)Redcap UE RF requirements, we have made the following

proposals.

Proposal 1: Additional new output power enhancement such as R19 power boosting can only be applied to HD-FDD (e)Redcap NTN. Which can be supported for all modulation orders.

Proposal 2: This new output power enhancement is no simultaneous support with R18 power boosting.

- Reference

- R4-2502281, WF on NR_NTN_Ph3_UERF, Qualcomm INC.TSG-RAN4#114 meeting